3GPP TSG RAN WG1 # 110bis-e 	R1- 2209248

e-Meeting, October 10th – 19th, 2022

Agenda item:		9.14.3

Source:		Mavenir

Title:	Discussion on solutions for NR dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:		Discussion and Decision

# 1	Introduction

3GPP Work Item Description [1] provides a potential target in R18. The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1).

One source (R1-2008626) provides a method for dynamic switching between DFT-S-OFDM and CP-OFDM according to MCS table. We want to use other methods to indicate the DFT-S-OFDM and CP-OFDM waveform and give two options.

# 2	Dynamic switching between DFT-S-OFDM and CP-OFDM

5G NR PUSCH introduces two kinds of UL waveform: DFT-S-OFDM and CP-OFDM.

CP-OFDM offers a lower bler when SINR is high. It can be used for improving throughput for cell-center UEs and when network operators need to maximize network capacity in dense urban environments.

Compared with CP-OFDM, DFT-S-OFDM adds transform precoding after layer mapping. It has a lower PAPR and increased transmission power. It is used to address greater range requirements and used for power-limited scenarios.

We can see that DFT-S-OFDM and CP-OFDM waveforms both have advantages and disadvantages.

As per the present 5G NR specification, CP-OFDM or DFT-S-OFDM is indicated by transformPrecoder which is involved in (or related to) RRC parameters configured in an RRC message (e.g., pusch-Config, configuredGrantConfig, msg3-transformPrecoder and msgA-TransformPrecoder-r16).

When transformPrecoder is included in pusch-Config or configuredGrantConfig, transformPrecoder enabled means using DFT-S-OFDM for PUSCH and transformPrecoder disabled means using CP-OFDM for PUSCH.  If transformPrecoder is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder or msgA-TransformPrecoder-r16, as follows.

transformPrecoder                     ENUMERATED {enabled, disabled}      OPTIONAL,   -- Need S

msgA-TransformPrecoder-r16            ENUMERATED {enabled, disabled}      OPTIONAL, -- Need R

msg3-transformPrecoder                ENUMERATED {enabled}                OPTIONAL,   -- Need R

Now the switching is by RRC reconfigure message which is similar to a static method. when gNB wants to change the PUSCH waveform, needs to reconfigure RRC messages to UE which results in a long latency and easily fails to UE because of the uncertainty of RRC reconfigure message. So, it is beneficial to achieve dynamic switching between DFT-S-OFDM and CP-OFDM.

Observation 1. Dynamic switching between DFT-S-OFDM and CP-OFDM could reduce switch latency

we focus our attention on how to achieve dynamic switching between DFT-S-OFDM and CP-OFDM. This contribution provides two options to address the problem, as follows:

Option1: By MAC CE

UL Waveform Activation/Deactivation MAC CE are new defined MAC CEs, whose details should be discussed with RAN2.

Option2: By DCI

UL waveform indicator is added to DCI format 0_0 and 0_1, as follows:

- UL waveform indicator: 1 bit according to Table 1.

Table 1: UL waveform indicator

| Value of UL waveform indicator | UL waveform |
| --- | --- |
| 0 | CP-OFDM |
| 1 | DFT-S-OFDM |

In conclusion, there are two options to indicate the UL waveform:

- Option1: by MAC CE

- Option2: by DCI

Propose1. In conclusion, there are two options to achieve dynamic switching between DFT-S-OFDM and CP-OFDM:

- Option1: by MAC CE

- Option2: by DCI

# 3	Conclusions

In this contribution, the following observation has been made:

Observation 1. Dynamic switching between DFT-S-OFDM and CP-OFDM could reduce switch latency.

In addition, the following proposal has been made:

Propose1. In conclusion, there are two options to achieve dynamic switching between DFT-S-OFDM and CP-OFDM:

- Option1: by MAC CE

- Option2: by DCI

# 4. References

[1] RP-220937  Revised WID on Further NR coverage enhancements

[2] R1-2008626 Potential coverage enhancement techniques for PUSCH.