3GPP TSG-RAN WG1 Meeting #110b	R1-2209250

# e-Meeting, Oct 10 – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1 and DCI 1-1 for FR 2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No LBT was introduced in R17 for FR 2-2. Based on the description of TS 38.331, when parameter channelAccessMode2-r17 is absent, UE shall not apply any channel access procedure.UE will transmit UL channels in no-LBT mode on shared spectrum, and none of the Type 1/2/3 LBT defined for FR 2-2 will be used. Based on the description of TS 38.212, for a cell operate on shared spectrum, the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1/1-1 is log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1/1-1, and I>=1. The current TS 38.212 assumes as long as the cell operate on shared spectrum, ul-AccessConfigListDCI-0-1/1-1 should be configured at least one entry. However, for FR2-2,even on shared spectrum, when channelAccessMode2-r17 is absent, UE shall not apply any channel access procedure. And ul-AccessConfigListDCI-0-1/1-1 should not be configured and the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1/1-1 should be 0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in TS 38.212 the restriction that when channelAccessMode2-r17 is absent, ul-AccessConfigListDCI-0-1/1-1 should not be configured and the bit length of ChannelAccess-CPext-CAPC field in DCI 0-1/1-1 is 0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Contradictory descriptions between TS 38.331 and TS 38.212 . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.2/7.3.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

<omitted text>

- ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access for FR1 and FR 2-1 and when channelAccessMode2-r17 is provided for FR 2-2  ; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

<omitted text>

7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

<omitted text>

- ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access for FR1 and FR 2-1 and when channelAccessMode2-r17 is provided for FR 2-2; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

<omitted text>