3GPP TSG RAN WG1 #110bis-e			R1-2209254

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | draft | rev | - | Current version: | 15.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the RSRP of the downlink pathloss reference for PRACH power control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 7.4 of TS 38.213, it describes that the pathloss of PRACH is calculated by the UE in dB as referenceSignalPower - higher layer filtered RSRP in dBm, where the higher layer filtered RSRP configuration is defined in TS 38.331. And, the higher layer filtering, i.e., L3 filtering, is defined as follows according to clause 5.5.3.2 of TS 38.331: Fn = (1 – a)*Fn-1 + a*Mn Where, a = 1/2(ki/4), and ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR However, since MeasObjectNR is only configured in the RRC connected state, the filterCoefficient can’t be obtained in RRC idle/inactive state and the L3 filtering can’t be applied for the pathloss determination of PRACH. Even though the default value fc4 is configured for filterCoefficient, our RAN2 colleagues believe that the idle/inactive state can’t use a default value of the connected state. Besides, for RSRP measurements for random access procedure, it has been discussed in RAN2 NR AdHoc 1807 meeting and the following agreement was achieved: For the purpose of Random Access the UE uses unfiltered L1 measurements for RSRP. Thus, we propose to change the “higher layer filtered RSRP” in clause 7.4 of TS 38.213 to “L1 RSRP” for the calculation of pathloss for PRACH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change ‘higher layer filtered RSRP’ in Clause 7.4 to ‘L1 RSRP’ for the pathloss determination of PRACH channel |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between TS 38.213 and TS 38.331 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

- 7.4	Physical random access channel

A UE determines a transmission power for a physical random access channel (PRACH), , on active UL BWP  of carrier  of serving cell  based on DL RS for serving cell  in transmission occasion  as

[dBm],

where  is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [38.101-3] for carrier  of serving cell  within transmission occasion ,  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers [11, TS 38.321] for the active UL BWP  of carrier  of serving cell , and  is a pathloss for the active UL BWP  of carrier  based on the DL RS associated with the PRACH transmission on the active DL BWP of serving cell  and calculated by the UE in dB as referenceSignalPower – higher layer filtered L1 RSRP in dBm, where RSRP is defined in [7, TS 38.215] and the higher layer filter configuration is defined in [12, TS 38.331]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in Clause 13, the UE determines  based on the SS/PBCH block associated with the PRACH transmission.

<Unchanged parts omitted>