3GPP TSG RAN WG1 #110bis-e			R1-2209265

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.11.2

Source:		Xiaomi

Title:	Discussion on the network verified location in NR NTN

Document for:	Discussion

Introduction

In RAN1#110 meeting [1], the following agreements were made on the network verified location.

Agreement

The following 3GPP defined RAT dependent positioning methods shall be considered as starting point for the study on Network verified UE location in case of NGSO based NTN deployment:

- Multi-RTT

- DL/UL-TDOA

Note-1: Other methods (e.g. AoA based) are not precluded

Note-2: RAT independent positioning methods are not under the scope of the study

Agreement

For evaluating positioning performance in NTN, the following metrics apply.

- Horizontal accuracy:

- Horizontal accuracy is the difference between a calculated horizontal position by the network and the actual horizontal position of a UE (for evaluation purposes)

- At least CDFs of horizontal positioning errors are used as a performance metrics in NR positioning evaluations

- At least the following percentiles of positioning error is analyzed 50%, 67%, 80%, 90%, 95%

Agreement

The following parameters are assumed for the evaluation of RAT dependent positioning methods study in NTN:

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km, optional: 1200km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1: 2GHz, S-band (n256). Optional: FR2 |
| BW | To be reported by companies |
| Subcarrier spacing, kHz | 15 for FR1, optional: 120 kHz for FR2 |
| Number of satellite in view | 1 for single satellite case, [3] for multi-satellite case |
| Orbit inclination | To be reported by companies |
| UE type | Handheld terminal, Optional: VSAT |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration as agreed under AI 9.12.1 |
| Positioning signals (Note 1) | To be reported |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | To be reported |
| RS type of sequence/number of ports | To be reported |
| Number of symbols used per occasion | To be reported |
| number of occasions used per positioning estimate | To be reported |
| Time window for measurement collection | To be reported |
| Interference modelling (ideal muting, or other) | To be reported |
| Reference Signal Transmission Bandwidth | To be reported |
| Reference point for timing measurement | Satellite |
| Description of positioning technique / applied positioning algorithm | To be reported |
| UE speed | 3km/h |
| Maximum timing measurement error | To be reported |
| Performance metrics | Horizontal accuracy (UE 2D position accuracy) |
| Additional notes, if any | Note 1: Time-related measurements can be performed via other downlink and uplink signals than PRS and SRS Note 2: The corresponding link budget should also be reported and the verification procedure should be done within the restriction of minimum elevation angle for service, e.g., 30 degree for LEO |

In this contribution, we present the discussion on the network verified location.

Discussion

The accurate UE location information is necessary to be known at the network. When a UE attaches to the mobile network, the RAN selects the appropriate core network for the UE taking into account, among other things [2]:

-	UE identifiers;

-	UE's selected PLMN;

-	UE location information (including the serving cell as known to the serving RAN node).

In NR NTN, the UE is assumed to have GNSS capability. The UE may send GNSS measurements to the RAN over RRC, but this has at least the following drawbacks [2]:

-	In principle, just as a malicious UE could fake its selected PLMN, it could also fake its GNSS measurements;

-	Sending GNSS measurements over RRC before AS security is set up raises security and privacy issues.

Thus it is necessary to support the network verified location in Rel-18. The UE location information is considered verified if the reported UE location is consistent with the network based assessment to within 5-10 km (similar to terrestrial network macro cell size), enabling country discrimination and selection of an appropriate core network in order to support all the regulatory services (i.e. emergency call, lawful intercept, public warning, charging/billing).

One issue is which node should be responsible for the location verification. In the legacy TN system, the AMF or the UE is responsible for the location calculation. However, in NTN, the UE’s location should be known to the gNB either to perform the time compensation. In order to save the latency, the gNB could also be considered to perform the location verification

Proposal 1: Both the gNB and the AMF could be responsible for the location verification.

Considering that in the initial phase of deployment, it is hard for the UE to have multiple satellites in view, and then the scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority. It was agreed that the multi-RTT and DL/UL-TDOA can be used for the location verification.

For the DL-TDOA based solution, the target UE can measure the TDOA at different time instants with single satellite in view as illustrated in the figure below:

Figure 1: DL-TDOA based positioning with single satellite

For the multi-RTT based solution [3], it makes use of the UE Rx-Tx time difference measurements of downlink signals received from the same satellite at multiple different instants (i.e. DL-PRS), measured by the UE reported to the gNB and the measured gNB Rx-Tx time difference measurements, of uplink signals transmitted from UE (i.e. UL-SRS) at multiple time instants. However, the movement of the satellite have impact on the RTT estimation as transmission delay between UE and satellite at the gNB’s PRS transmission timing with that at the gNB’s SRS reception timing. The RTT estimation error is related to the timing difference between tUE-Rx and tUE-Tx. For the RTT estimation error on the feeder-link, it can be handled by the gNB, while for the RTT estimation error on the service link, the UE can report the timing difference accompanying the SRS transmission.

Figure 2: Illustration of the RTT estimation error

Proposal 2: The RTT estimation error due to the movement of the satellite should be taken into account.

Proposal 3: The RTT estimation error on the feeder-link can be handled the gNB.

Proposal 4: The RTT estimation error on the service-link can be reported by the UE.

In the following, the simulation result using DL-TDOA is provided on the positioning accuracy in single satellite scenario. Detailed simulation assumption can be found in the appendix. The measurement period is the time interval to perform a TDOA measurement while the observation times is the number of TDOA parameter used to perform the positioning.

Table 1: Evaluation results for LEO 600KM

| Measurement intervals/s | Observation times | Positioning error/m | Measurement error/ns |
| --- | --- | --- | --- |
| 5 | 3 | 3.90*103 | 32.09 |
| 10 | 3 | 8.73*102 | 31.91 |
| 15 | 3 | 3.62*102 | 31.52 |
| 20 | 3 | 1.99*102 | 32.08 |

Table 2: Evaluation results for LEO 1200KM

| Measurement intervals/s | Observation times | Positioning error/m | Measurement error/ns |
| --- | --- | --- | --- |
| 10 | 3 | 8.29*103 | 30.50 |
| 15 | 3 | 3.51*103 | 32.41 |
| 20 | 3 | 2.34*103 | 31.49 |
| 30 | 3 | 1.11*103 | 31.91 |
| 40 | 3 | 6.29*102 | 31.06 |

The positioning performance with different measurement interval is illustrated in the figure below:

Figure 3: Positioning performance

It can be observed that the measurement interval has significant impact on the positioning accuracy. The positioning accuracy is improved with the increase of the measurement interval. At least 10s and 20s is required to meet the accuracy requirement for LEO600 and LEO1200 cases respectively.

Observation:

- The measurement interval, and the satellite orbit have significant impact on the positioning accuracy

- The delay for verifying the location is at least 10s and 20s for LEO600 and LEO1200 cases respectively.

Proposal 5: The DL-TDOA solution is feasible to support the network verified location.

Proposal 6: The delay required for verifying the location needs to be further considered to avoid the impact to the service.

Conclusions

In this contribution, we present the discussion on the network verified location and some preliminary simulation results are provided. Based on our analysis, we have the following observations and proposals:

Observation:

- The measurement interval, and the satellite orbit have significant impact on the positioning accuracy

- The delay for verifying the location is at least 10s and 20s for LEO600 and LEO1200 cases respectively.

Proposal 1: Both the gNB and the AMF could be responsible for the location verification.

Proposal 2: The RTT estimation error due to the movement of the satellite should be taken into account.

Proposal 3: The RTT estimation error on the feeder-link can be handled the gNB.

Proposal 4: The RTT estimation error on the service-link can be reported by the UE.

Proposal 5: The DL-TDOA solution is feasible to support the network verified location.

Proposal 6: The delay required for verifying the location needs to be further considered to avoid the impact to the service.

Appendix

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km, 1200km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1: 2GHz, S-band (n256). |
| BW | 20MHz (106 RBs) |
| Subcarrier spacing, kHz | 15 |
| Number of satellite in view | 1 |
| Orbit inclination | 90° |
| UE type | Handheld terminal |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration as agreed under AI 9.12.1 |
| Positioning signals (Note 1) | PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | comb-2, 2 symbols, 104 RBs |
| RS type of sequence/number of ports | Gold, 1 port |
| Number of symbols used per occasion | 2 |
| number of occasions used per positioning estimate | 1 |
| Time window for measurement collection | 1 symbol |
| Interference modelling (ideal muting, or other) | ideal muting |
| Reference Signal Transmission Bandwidth | 104 RBs |
| Reference point for timing measurement | Satellite |
| Description of positioning technique / applied positioning algorithm | Taylor series |
| UE speed | 3km/h |
| Maximum timing measurement error | 32.55ns |
| Performance metrics | Horizontal accuracy (UE 2D position accuracy) |

References

- 3GPP RAN1 #110 meeting chairman-note, 3GPP TSG RAN WG1 #110, Toulouse, France, August 22nd – 26th, 2022.

- 3GPP TR 38.882, Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR, V18.0.0, 2022-06

- 3GPP R1-2207631, FL Summary #4: Network verified UE location for NR NTN, Moderator (Thales), 3GPP TSG RAN WG1 Meeting #110, Toulouse, France, August 22nd – 26th, 2022