3GPP TSG RAN WG1 #110bis-e		R1-2209344

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.5.2.3

Source: 	CMCC

Title: 	Discussion on low power high accuracy positioning

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new SID on study on expanded and improved NR positioning was approved [1], in which the intention to evaluate whether the Rel-17 RRC_INACTIVE positioning can satisfy the SA1 requirements for high accuracy and extreme low power consumption with battery life sustainable up to one or more years was justified.

The objectives on LPHAP are listed as follows:

- Improved accuracy, integrity, and power efficiency: - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In the last two RAN1 meetings, evaluation methodology on LPHAP was thoroughly discussed. In this contribution, we continually discuss the remaining open issue on the power model for ultra-deep sleep, and provide our updated evaluation results and observations based on the agreed evaluation methodology, and also our views on potential enhancements of LPHAP.

Remaining issues on evaluation methodology

In RAN1#110 meeting, it was agreed to consider a new ultra-deep sleep state in addition to the three UE sleep types defined in TR 38.840 for the LPHAP evaluation [2]:

Agreement For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed: - Option 1: - The relative power unit: 0.015 - Additional transition energy: 2000 - Total transition time: 400ms - Option 2: - The relative power unit: 0.01 - Additional transition energy: 450; - Total transition time: 25ms - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state - Notes: the values above can be further discussed Agreement For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value.

During the discussion, two options of the power model of the ultra-deep sleep state were proposed, where option 1 considers the evaluation assumption of “Power Saving State” in the study of NB-IoT as a starting point, and option 2 represents an optimized wake-up procedure for positioning. In addition, as companies shared quite diverse views on the additional transition energy for option 1, two different values of additional transition energy were proposed and left for further study.

Regarding the two values of additional transition energy for option 1, we prefer 2000 over 20000 (as in NB-IoT). The reasons are twofold:

- The UE architecture, capabilities and bandwidth of NB-IoT and LPHAP devices are quite different, it seems not reasonable to completely reuse the values defined for NB-IoT in the evaluation for LPHAP;

- From our evaluations and analysis (details please refer to Section 3), if additional transition energy as 20000 is adopted, there is no way to achieve the target requirement of battery life of 6~12 months for the baseline LPHAP Type A.

Hence, we prefer to set the additional transition energy value as 2000 for option 1.

Proposal 1: For option 1 of ultra-deep sleep state, consider the following power consumption model:

  - The relative power unit: 0.015

  - Additional transition energy: 2000

  - Total transition time: 400ms

For option 2 of the ultra-deep sleep state, during the last RAN1 meeting, supporting companies argued that the current sleep states in TR38.840 are defined for normal eMBB UEs; while option 2 assumes a UWB-like LPHAP UE with limited communication capability, which can quickly wake up from an ultra-deep sleep mode. This kind of low power device type indeed exists in the industry, but we still prefer to further discuss the values. Based on the evaluation, we propose to adopt additional transition energy of larger than [800] and total transition time of larger than [50]ms.

Proposal 2: For option 2 of ultra-deep sleep state, consider the following power consumption model:

  - The relative power unit: 0.01

  - Additional transition energy: 800

  - Total transition time: 50ms

In addition, another issue is whether the power consumption model of ultra-deep sleep mode should be down-select to one. During the discussion in the last RAN1 meeting, it was argued that the two options may represent different UE capabilities and wake-up procedures, and hence no down-selection is pursued. In our views, our first preference is still to down-select between the two, otherwise, it may over-complicate the evaluations. If companies are convinced by adopting two models, it is acceptable to us for making progress.

Proposal 3: For the power consumption model of the ultra-deep sleep state, RAN1 strives to down-select between the following two options:

  - Option 1:

    - The relative power unit: 0.015

    - Additional transition energy: 2000

    - Total transition time: 400ms

  - Option 2:

    - The relative power unit: 0.01

    - Additional transition energy: [800]

    - Total transition time: [50]ms

Evaluation results

In the following, evaluation results of UE power consumption for the baseline Rel-17 RRC_INACTIVE positioning and potential enhancements are analysed. We assume that the TDD pattern is 7D1S2U with the periodicity of 5ms, as shown below:

Figure 1: TDD pattern in the evaluation

## 3.1 Baseline evaluation cases

In this sub-section, we investigate several evaluation cases of the baseline Rel-17 RRC_INACTIVE positioning techniques, and the evaluation assumptions are captured in the following table.

Table 1: Evaluation cases and assumptions for Rel-17 RRC_INACTIVE positioning

| Evaluation assumption | [B101], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B102], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B103], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B104], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B105], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B106], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| --- | --- | --- | --- | --- | --- | --- |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 1.28s (1 RS occasion per 1 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 1.28s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B111], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B112], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B113], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B114], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B115], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B116], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 10.24s (1 RS occasion per 8 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 10.24s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B121], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B122], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B123], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B124], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B125], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B126], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 1.28s (1 RS occasion per 1 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | RA-SDT; Reporting interval = 1.28s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B131], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B132], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B133], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [B134], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B135], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [B136], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 10.24s (1 RS occasion per 8 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | RA-SDT; Reporting interval = 10.24s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B201], [FR1], [UE-based DL positioning], [LPHAP Type A] | [B202], [FR1], [UE-based DL positioning], [LPHAP Type A] | [B203], [FR1], [UE-based DL positioning], [LPHAP Type A] | [B204], [FR1], [UE-based DL positioning], [LPHAP Type B] | [B205], [FR1], [UE-based DL positioning], [LPHAP Type B] | [B206], [FR1], [UE-based DL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 1.28s (1 RS occasion per 1 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B211], [FR1], [UE-based DL positioning], [LPHAP Type A] | [B212], [FR1], [UE-based DL positioning], [LPHAP Type A] | [B213], [FR1], [UE-based DL positioning], [LPHAP Type A] | [B214], [FR1], [UE-based DL positioning], [LPHAP Type B] | [B215], [FR1], [UE-based DL positioning], [LPHAP Type B] | [B216], [FR1], [UE-based DL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 10.24s (1 RS occasion per 8 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B301], [FR1], [UL positioning], [LPHAP Type A] | [B302], [FR1], [UL positioning], [LPHAP Type A] | [B303], [FR1], [UL positioning], [LPHAP Type A] | [B304], [FR1], [UL positioning], [LPHAP Type B] | [B305], [FR1], [UL positioning], [LPHAP Type B] | [B306], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 1.28s (1 RS occasion per 1 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B311], [FR1], [UL positioning], [LPHAP Type A] | [B312], [FR1], [UL positioning], [LPHAP Type A] | [B313], [FR1], [UL positioning], [LPHAP Type A] | [B314], [FR1], [UL positioning], [LPHAP Type B] | [B315], [FR1], [UL positioning], [LPHAP Type B] | [B316], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 10.24s (1 RS occasion per 8 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B321], [FR1], [UL positioning], [LPHAP Type A] | [B322], [FR1], [UL positioning], [LPHAP Type A] | [B323], [FR1], [UL positioning], [LPHAP Type A] | [B324], [FR1], [UL positioning], [LPHAP Type B] | [B325], [FR1], [UL positioning], [LPHAP Type B] | [B326], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 1.28s (1 RS occasion per 1 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | RA-SDT for SRS configuration (per power cycle) |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [B331], [FR1], [UL positioning], [LPHAP Type A] | [B332], [FR1], [UL positioning], [LPHAP Type A] | [B333], [FR1], [UL positioning], [LPHAP Type A] | [B334], [FR1], [UL positioning], [LPHAP Type B] | [B335], [FR1], [UL positioning], [LPHAP Type B] | [B336], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | 38.840 |  |  |  |  |  |
| DRX cycle | 1.28s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 10.24s (1 RS occasion per 8 I-DRX cycle) |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | RA-SDT for SRS configuration (per power cycle) |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |

Summary

The results of baseline Rel-17 RRC_INACTIVE positioning are summarized in the following table. The detailed evaluation result and analysis for each baseline case is shown in Appendix A1.

Table 2: Summary for UE power consumption results of baseline Rel-17 RRC_INACTIVE positioning

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| [B101], [baseline assumption] | 2.59 | 0.29 | No; | No; |
| [B102] |  | 0.57 | No; | No; |
| [B103] |  | 1.14 | No; | No; |
| [B104] |  | 1.61 | No; | No; |
| [B105] |  | 3.22 | No; | No; |
| [B106] |  | 6.44 | Yes | No; |
| [B111] | 1.60 | 0.46 | No; | No; |
| [B112] |  | 0.93 | No; | No; |
| [B113] |  | 1.85 | No; | No; |
| [B114] |  | 2.60 | No; | No; |
| [B115] |  | 5.21 | No; | No; |
| [B116] |  | 10.42 | Yes | No; |
| [B121] | 3.96 | 0.19 | No; | No; |
| [B122] |  | 0.37 | No; | No; |
| [B123] |  | 0.75 | No; | No; |
| [B124] |  | 1.05 | No; | No; |
| [B125] |  | 2.10 | No; | No; |
| [B126] |  | 4.21 | No; | No; |
| [B131] | 1.77 | 0.42 | No; | No; |
| [B132] |  | 0.84 | No; | No; |
| [B133] |  | 1.67 | No; | No; |
| [B134] |  | 2.35 | No; | No; |
| [B135] |  | 4.71 | No; | No; |
| [B136] |  | 9.42 | Yes | No; |
| [B201], [baseline assumption] | 1.67 | 0.44 | No; | No; |
| [B202] |  | 0.89 | No; | No; |
| [B203] |  | 1.77 | No; | No; |
| [B304] |  | 2.50 | No; | No; |
| [B305] |  | 4.99 | No; | No; |
| [B306] |  | 9.98 | Yes | No; |
| [B211] | 1.49 | 0.50 | No; | No; |
| [B212] |  | 0.99 | No; | No; |
| [B213] |  | 1.99 | No; | No; |
| [B214] |  | 2.80 | No; | No; |
| [B215] |  | 5.59 | No; | No; |
| [B216] |  | 11.19 | Yes | No; |
| [B301], [baseline assumption] | 1.85 | 0.40 | No; | No; |
| [B302] |  | 0.80 | No; | No; |
| [B303] |  | 1.60 | No; | No; |
| [B304] |  | 2.25 | No; | No; |
| [B305] |  | 4.50 | No; | No; |
| [B306] |  | 9.01 | Yes | No; |
| [B311] | 1.51 | 0.49 | No; | No; |
| [B312] |  | 0.98 | No; | No; |
| [B313] |  | 1.96 | No; | No; |
| [B314] |  | 2.76 | No; | No; |
| [B315] |  | 5.52 | No; | No; |
| [B316] |  | 11.04 | Yes | No; |
| [B321] | 4.11 | 0.18 | No; | No; |
| [B322] |  | 0.36 | No; | No; |
| [B323] |  | 0.72 | No; | No; |
| [B324] |  | 1.01 | No; | No; |
| [B325] |  | 2.03 | No; | No; |
| [B326] |  | 4.06 | No; | No; |
| [B331] | 1.79 | 0.41 | No; | No; |
| [B332] |  | 0.83 | No; | No; |
| [B333] |  | 1.66 | No; | No; |
| [B334] |  | 2.33 | No; | No; |
| [B335] |  | 4.66 | No; | No; |
| [B336] |  | 9.31 | Yes | No; |

Based on the above evaluation results, the following observations can be made:

Observation 1: With baseline evaluation assumptions, none of the existing Rel-17 RRC_INACTIVE state positioning functionalities meets the target requirement of 6~12 months battery life.

Observation 2: For UL positioning, frequent (re)configuration for UL SRS significantly increases the power consumption:

- The slot-averaged relative power unit increases from 1.85 to 4.11 for a power cycle of 1.28s;

- The slot-averaged relative power unit increases from 1.51 to 1.79 for a power cycle of 10.24s.

From the evaluation results and observations, it can safely conclude that the existing Rel-17 RRC_INACTIVE positioning functionalities cannot meet the stringent battery life requirement developed by SA1 and it is necessary to pursue further enhancements on power saving solutions.

Proposal 4: Enhancements on power saving solutions should be studied for low power and high accuracy positionings.

## 3.2 Enhanced evaluation cases

In the last RAN1 meeting, assumptions for eDRX and ultra-deep sleep were additionally agreed for the evaluation:

Agreement For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed: - Option 1: - The relative power unit: 0.015 - Additional transition energy: 2000 - Total transition time: 400ms - Option 2: - The relative power unit: 0.01 - Additional transition energy: 450; - Total transition time: 25ms - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state - Notes: the values above can be further discussed Agreement For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value. Agreement - For the purpose of LPHAP evaluation, the following assumptions on eDRX configuration and/or paging reception can be optionally considered: - The eDRX cycle to evaluate: 20.48s; 30.72s; - For paging reception: - 1 paging occasion is included in one eDRX cycle - 10% paging rate - No paging reception can be optionally evaluated; - 1 DL PRS and/or UL SRS for positioning occasion per 1 eDRX cycle - Minimizing the gap between PRS measurement, SRS transmission and/or measurement reporting with paging monitoring in time domain can be evaluated.

In the following sub-section, we further consider some enhanced evaluation cases, and the evaluation assumptions are captured in Table 3.

Table 3: Evaluation cases and assumptions for potential enhancements

| Evaluation assumption | [E101], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E102], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E103], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E104], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E105], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E106], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| --- | --- | --- | --- | --- | --- | --- |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 20000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E111], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E112], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E113], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E114], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E115], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E116], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 2000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E121], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E122], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E123], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E124], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E125], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E126], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 2) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E131], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E132], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E133], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E134], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E135], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E136], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 20000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E141], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E142], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E143], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E134], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E135], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E136], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 2000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E151], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E152], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E153], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E154], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E155], [FR1], [UE-assisted DL positioning], [LPHAP Type B] | [E156], [FR1], [UE-assisted DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 2) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E161], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E171], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E181], [FR1], [UE-assisted DL positioning], [LPHAP Type A] |  |  |  |
| Sleep state | Ultra-deep sleep (Option 1, 20000) | Ultra-deep sleep (Option 1, 2000) | Ultra-deep sleep (Option 2) |  |  |  |
| DRX cycle | 20.48s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 1 | 1 |  |  |  |
| Evaluation assumption | [E201], [FR1], [UE-based DL positioning], [LPHAP Type A] | [E202], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E203], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E204], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E205], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E206], [FR1], [UE- based DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 20000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E211], [FR1], [UE-based DL positioning], [LPHAP Type A] | [E212], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E213], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E214], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E215], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E216], [FR1], [UE- based DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 2000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E221], [FR1], [UE-based DL positioning], [LPHAP Type A] | [E222], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E223], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E224], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E225], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E226], [FR1], [UE- based DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 2) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E231], [FR1], [UE-based DL positioning], [LPHAP Type A] | [E232], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E233], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E234], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E235], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E236], [FR1], [UE- based DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 20000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E241], [FR1], [UE-based DL positioning], [LPHAP Type A] | [E242], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E243], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E244], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E245], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E246], [FR1], [UE- based DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 2000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E251], [FR1], [UE-based DL positioning], [LPHAP Type A] | [E252], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E253], [FR1], [UE- based DL positioning], [LPHAP Type A] | [E254], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E255], [FR1], [UE- based DL positioning], [LPHAP Type B] | [E256], [FR1], [UE- based DL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 2) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E261], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E271], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E281], [FR1], [UE-assisted DL positioning], [LPHAP Type A] |  |  |  |
| Sleep state | Ultra-deep sleep (Option 1, 20000) | Ultra-deep sleep (Option 1, 2000) | Ultra-deep sleep (Option 2) |  |  |  |
| DRX cycle | 20.48s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT; Reporting interval = 20.48s |  |  |  |  |  |
| implementation factor K | 1 | 1 | 1 |  |  |  |
| Evaluation assumption | [E301], [FR1], [UL positioning], [LPHAP Type A] | [E302], [FR1], [UL positioning], [LPHAP Type A] | [E303], [FR1], [UL positioning], [LPHAP Type A] | [E304], [FR1], [UL positioning], [LPHAP Type B] | [E305], [FR1], [UL positioning], [LPHAP Type B] | [E306], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 20000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E311], [FR1], [UL positioning], [LPHAP Type A] | [E312], [FR1], [UL positioning], [LPHAP Type A] | [E313], [FR1], [UL positioning], [LPHAP Type A] | [E314], [FR1], [UL positioning], [LPHAP Type B] | [E315], [FR1], [UL positioning], [LPHAP Type B] | [E316], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 2000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E321], [FR1], [UL positioning], [LPHAP Type A] | [E322], [FR1], [UL positioning], [LPHAP Type A] | [E323], [FR1], [UL positioning], [LPHAP Type A] | [E324], [FR1], [UL positioning], [LPHAP Type B] | [E325], [FR1], [UL positioning], [LPHAP Type B] | [E326], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 2) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E331], [FR1], [UL positioning], [LPHAP Type A] | [E332], [FR1], [UL positioning], [LPHAP Type A] | [E333], [FR1], [UL positioning], [LPHAP Type A] | [E334], [FR1], [UL positioning], [LPHAP Type B] | [E335], [FR1], [UL positioning], [LPHAP Type B] | [E336], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 20000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E341], [FR1], [UL positioning], [LPHAP Type A] | [E342], [FR1], [UL positioning], [LPHAP Type A] | [E343], [FR1], [UL positioning], [LPHAP Type A] | [E344], [FR1], [UL positioning], [LPHAP Type B] | [E345], [FR1], [UL positioning], [LPHAP Type B] | [E346], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 1, additional transition energy = 2000) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E351], [FR1], [UL positioning], [LPHAP Type A] | [E352], [FR1], [UL positioning], [LPHAP Type A] | [E353], [FR1], [UL positioning], [LPHAP Type A] | [E354], [FR1], [UL positioning], [LPHAP Type B] | [E355], [FR1], [UL positioning], [LPHAP Type B] | [E356], [FR1], [UL positioning], [LPHAP Type B] |
| Sleep state | Ultra-deep sleep (Option 2) |  |  |  |  |  |
| DRX cycle | 20.48s (Gap between paging, RS or reporting is minimized) |  |  |  |  |  |
| paging reception | N/A |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 2 | 4 | 1 | 2 | 4 |
| Evaluation assumption | [E361], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E371], [FR1], [UE-assisted DL positioning], [LPHAP Type A] | [E381], [FR1], [UE-assisted DL positioning], [LPHAP Type A] |  |  |  |
| Sleep state | Ultra-deep sleep (Option 1, 20000) | Ultra-deep sleep (Option 1, 2000) | Ultra-deep sleep (Option 2) |  |  |  |
| DRX cycle | 20.48s |  |  |  |  |  |
| paging reception | Yes |  |  |  |  |  |
| RS periodicity | 20.48s |  |  |  |  |  |
| M-sample | 1 |  |  |  |  |  |
| RRM measurement | N/A |  |  |  |  |  |
| BWP switching | N/A |  |  |  |  |  |
| SRS configuration | N/A |  |  |  |  |  |
| implementation factor K | 1 | 1 | 1 |  |  |  |

Summary

The results of evaluation cases for potential enhancements are summarized in the following table. The detailed evaluation result and analysis for each enhanced case is shown in Appendix A2.

Table 4: Summary of enhanced evaluation results

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| [E101] | 0.59 | 1.26 | No; | No; |
| [E102] |  | 2.52 | No; | No; |
| [E103] |  | 5.04 | No; | No; |
| [E104] |  | 7.09 | Yes | No; |
| [E105] |  | 14.19 | Yes | Yes |
| [E106] |  | 28.37 | Yes | Yes |
| [E111] | 0.15 | 5.01 | No; | No; |
| [E112] |  | 10.01 | Yes | No; |
| [E113] |  | 20.03 | Yes | Yes |
| [E114] |  | 28.16 | Yes | Yes |
| [E115] |  | 56.33 | Yes | Yes |
| [E116] |  | 112.67 | Yes | Yes |
| [E121] | 0.11 | 7.03 | Yes | No; |
| [E122] |  | 14.06 | Yes | Yes |
| [E123] |  | 28.12 | Yes | Yes |
| [E124] |  | 39.75 | Yes | Yes |
| [E125] |  | 79.08 | Yes | Yes |
| [E126] |  | 158.16 | Yes | Yes |
| [E131] | 0.58 | 1.29 | No; | No; |
| [E132] |  | 2.58 | No; | No; |
| [E133] |  | 5.15 | No; | No; |
| [E134] |  | 7.24 | Yes | No; |
| [E135] |  | 14.49 | Yes | Yes |
| [E136] |  | 28.97 | Yes | Yes |
| [E141] | 0.14 | 5.46 | No; | No; |
| [E142] |  | 10.91 | Yes | No; |
| [E143] |  | 21.83 | Yes | Yes |
| [E144] |  | 30.69 | Yes | Yes |
| [E145] |  | 61.38 | Yes | Yes |
| [E146] |  | 122.75 | Yes | Yes |
| [E151] | 0.09 | 7.95 | Yes | No; |
| [E152] |  | 15.89 | Yes | Yes |
| [E153] |  | 31.78 | Yes | Yes |
| [E154] |  | 44.70 | Yes | Yes |
| [E155] |  | 89.39 | Yes | Yes |
| [E156] |  | 178.78 | Yes | Yes |
| [E161] | 0.66 | 1.12 | No; | No; |
| [E171] | 0.22 | 3.30 | No; | No; |
| [E181] | 0.18 | 4.06 | No; | No; |
| [E201] | 0.53 | 1.40 | No; | No; |
| [E202] |  | 2.80 | No; | No; |
| [E203] |  | 5.60 | No; | No; |
| [E204] |  | 7.87 | Yes | No; |
| [E205] |  | 15.74 | Yes | Yes |
| [E206] |  | 31.47 | Yes | Yes |
| [E211] | 0.09 | 8.22 | Yes | No; |
| [E212] |  | 16.45 | Yes | Yes |
| [E213] |  | 32.89 | Yes | Yes |
| [E214] |  | 46.25 | Yes | Yes |
| [E215] |  | 92.50 | Yes | Yes |
| [E216] |  | 185.00 | Yes | Yes |
| [E221] | 0.05 | 15.59 | Yes | Yes |
| [E222] |  | 31.17 | Yes | Yes |
| [E223] |  | 62.35 | Yes | Yes |
| [E224] |  | 87.66 | Yes | Yes |
| [E225] |  | 175.32 | Yes | Yes |
| [E226] |  | 350.65 | Yes | Yes |
| [E231] | 0.52 | 1.44 | No; | No; |
| [E232] |  | 2.88 | No; | No; |
| [E233] |  | 5.75 | No; | No; |
| [E234] |  | 8.09 | Yes | No; |
| [E235] |  | 16.18 | Yes | Yes |
| [E236] |  | 32.35 | Yes | Yes |
| [E241] | 0.08 | 9.78 | Yes | No; |
| [E242] |  | 19.56 | Yes | Yes |
| [E243] |  | 39.13 | Yes | Yes |
| [E244] |  | 55.01 | Yes | Yes |
| [E245] |  | 110.03 | Yes | Yes |
| [E246] |  | 220.05 | Yes | Yes |
| [E251] | 0.03 | 22.33 | Yes | Yes |
| [E252] |  | 44.66 | Yes | Yes |
| [E253] |  | 89.31 | Yes | Yes |
| [E254] |  | 125.58 | Yes | Yes |
| [E255] |  | 251.16 | Yes | Yes |
| [E256] |  | 502.32 | Yes | Yes |
| [E261] | 0.61 | 1.22 | No; | No; |
| [E271] | 0.17 | 4.44 | No; | No; |
| [E281] | 0.12 | 5.94 | No; | No; |
| [E301] | 0.53 | 1.40 | No; | No; |
| [E302] |  | 2.80 | No; | No; |
| [E303] |  | 5.61 | No; | No; |
| [E304] |  | 7.88 | Yes | No; |
| [E305] |  | 15.77 | Yes | Yes |
| [E306] |  | 31.54 | Yes | Yes |
| [E311] | 0.09 | 8.32 | Yes | No; |
| [E312] |  | 16.65 | Yes | Yes |
| [E313] |  | 33.30 | Yes | Yes |
| [E314] |  | 46.82 | Yes | Yes |
| [E315] |  | 93.64 | Yes | Yes |
| [E316] |  | 187.28 | Yes | Yes |
| [E321] | 0.05 | 15.95 | Yes | Yes |
| [E322] |  | 31.91 | Yes | Yes |
| [E323] |  | 63.82 | Yes | Yes |
| [E324] |  | 89.73 | Yes | Yes |
| [E325] |  | 179.47 | Yes | Yes |
| [E326] |  | 358.93 | Yes | Yes |
| [E331] | 0.52 | 1.43 | No; | No; |
| [E332] |  | 2.86 | No; | No; |
| [E333] |  | 5.73 | No; | No; |
| [E334] |  | 8.05 | Yes | No; |
| [E335] |  | 16.11 | Yes | Yes |
| [E336] |  | 32.21 | Yes | Yes |
| [E341] | 0.08 | 9.51 | Yes | No; |
| [E342] |  | 19.01 | Yes | Yes |
| [E343] |  | 38.02 | Yes | Yes |
| [E344] |  | 53.46 | Yes | Yes |
| [E345] |  | 106.93 | Yes | Yes |
| [E346] |  | 213.85 | Yes | Yes |
| [E351] | 0.04 | 20.94 | Yes | Yes |
| [E352] |  | 41.88 | Yes | Yes |
| [E353] |  | 83.77 | Yes | Yes |
| [E354] |  | 117.78 | Yes | Yes |
| [E355] |  | 235.56 | Yes | Yes |
| [E356] |  | 471.13 | Yes | Yes |
| [E361] | 0.62 | 1.20 | No; | No; |
| [E371] | 0.18 | 4.16 | No; | No; |
| [E381] | 0.14 | 5.45 | No; | No; |

Based on the evaluation results, we have the following observations:

Observation 3: By extending the DRX cycle and RS periodicity, the power consumption significantly reduced, as the UE is able to go into a deeper sleep state and stays asleep as much as possible.

Observation 4: Minimizing the gap between paging reception, DL PRS/UL SRS occasion, and measurement reporting helps reduce the power consumption, as the UE reduces the number of transitions from active to ultra-deep sleep and stays asleep as much as possible.

Observation 5: For option 1 of ultra-deep sleep state with additional transition power 20000:

- None of the UE-assisted DL positioning, UE-based DL positioning and UL positioning techniques meets the battery life of 6 nor 12 months for LPHAP Type A device.

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type B device.

Observation 5: For option 1 of ultra-deep sleep state with additional transition power 2000:

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type A device.

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type B device.

Observation 6: For option 2 of ultra-deep sleep state:

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type A device.

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type B device.

Potential enhancements

During the discussion in RAN1#110 meeting, as clarified on the study scope of the objective, the study will include both RRC_INACTIVE and RRC_IDLE state; while in the normative stage, the enhancement in RRC_INACTIVE state or RRC_IDLE state will be specified. In our views, at last the support of DL PRS measurement in RRC_IDLE state should be considered from RAN1 perspective. For the UL positioning in RRC_IDLE state, we think that it should be up to RAN2 to determine.

Proposal 5: From RAN1 perspective, support of DL measurement for UEs in RRC_IDLE state.

From the evaluation results in Section 3, it can be observed that a UE “wakes up” during the ON duration of each DRX cycle to monitor the PDCCH and the corresponding power mode transit energy consumes a big part of the power for both DL and UL positioning. Note that for typical use cases of LPHAP, such as the plant asset management, the positioning UE can barely have 5G communication traffic, the service types are mainly focused on positioning and the related request/response signalling. In such cases, the UE may not be required to wake up every time when the ON duration comes.

In addition, for the RS monitoring and transmissions, if the RS occasions are configured without taking the DRX cycle into account, the UE has to additionally ramp up to measure the DL PRS or sends the UL SRS for positioning, and the transitions between the sleep mode and active mode consumes extra power. From this perspective, a smarter design is to align the RS configurations with the DRX pattern.

Proposal 6: The following DRX related enhancements should be considered:

- Introduction of the eDRX mode in LPHAP

- Reduce the number of PDCCH monitoring occasions in RRC_INACTIVE/IDLE state for LPHAP

- Align the DRX pattern and the DL PRS / UL SRS occasions

In Rel-17, the UL SRS for transmission in RRC_inactive state is configured by RRC_Release message, and if it is determined to be invalid, the UE has to enter the RRC_connected mode for the update of the SRS configurations. According to the initial evaluation results, to reduce the UE power consumption of UL positioning, the SRS configurations in RRC_inactive state should be kept valid for a period of time as long as possible. One straightforward solution is that, the SRS resources are (pre-)configured within an area (e.g., an indoor factory hall). Optionally, the SRS resources within this area are orthogonal to avoid the potential conflict of SRS resources. NW configures the SRS resource for a positioning UE and the SRS resource keeps valid within this area.

Proposal 7: The following enhancement of SRS transmission in RRC_INACTIVE state should be considered:

- SRS resources are (pre-)configured within an area in RRC_INACTIVE state.

- FFS: How to define this area.

Conclusions

In this contribution, we provide our views on the low power and high accuracy positioning, and the following observations and proposals are made:

Observation 1: With baseline evaluation assumptions, none of the existing Rel-17 RRC_INACTIVE state positioning functionalities meets the target requirement of 6~12 months battery life.

Observation 2: For UL positioning, frequent (re)configuration for UL SRS significantly increases the power consumption:

- The slot-averaged relative power unit increases from 1.85 to 4.11 for a power cycle of 1.28s;

- The slot-averaged relative power unit increases from 1.51 to 1.79 for a power cycle of 10.24s.

Observation 3: By extending the DRX cycle and RS periodicity, the power consumption significantly reduced, as the UE is able to go into a deeper sleep state and stays asleep as much as possible.

Observation 4: Minimizing the gap between paging reception, DL PRS/UL SRS occasion, and measurement reporting helps reduce the power consumption, as the UE reduces the number of transitions from active to ultra-deep sleep and stays asleep as much as possible

Observation 5: For option 1 of ultra-deep sleep state with additional transition power 20000:

- None of the UE-assisted DL positioning, UE-based DL positioning and UL positioning techniques meets the battery life of 6 nor 12 months for LPHAP Type A device.

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type B device.

Observation 5: For option 1 of ultra-deep sleep state with additional transition power 2000:

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type A device.

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type B device.

Observation 6: For option 2 of ultra-deep sleep state:

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type A device.

- The battery life of 6 and 12 months can be met with UE-assisted DL positioning, UE-based DL positioning and UL positioning for LPHAP Type B device.

Proposal 1: For option 1 of ultra-deep sleep state, consider the following power consumption model:

- The relative power unit: 0.015

- Additional transition energy: 2000

- Total transition time: 400ms

Proposal 2: For option 2 of ultra-deep sleep state, consider the following power consumption model:

- The relative power unit: 0.01

- Additional transition energy: 800

- Total transition time: 50ms

Proposal 3: For the power consumption model of the ultra-deep sleep state, RAN1 strives to down-select between the following two options:

  - Option 1:

    - The relative power unit: 0.015

    - Additional transition energy: 2000

    - Total transition time: 400ms

  - Option 2:

    - The relative power unit: 0.01

    - Additional transition energy: [800]

    - Total transition time: [50]ms

Proposal 4: Enhancements on power saving solutions should be studied for low power and high accuracy positionings.

Proposal 5: From RAN1 perspective, support of DL measurement for UEs in RRC_IDLE state.

Proposal 6: The following DRX related enhancements should be considered:

- Introduction of the eDRX mode in LPHAP

- Reduce the number of PDCCH monitoring occasions in RRC_INACTIVE/IDLE state for LPHAP

- Align the DRX pattern and the DL PRS / UL SRS occasions

Proposal 7: The following enhancement of SRS transmission in RRC_INACTIVE state should be considered:

- SRS resources are (pre-)configured within an area in RRC_INACTIVE state.

- FFS: How to define this area.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e.

- Draft Report of 3GPP TSG RAN WG1 #110 v0.2.0

- Appendix A: Detailed evaluation for each case

## A1. Baseline evaluation cases

UE-assisted DL positioning

In the following, the power consumption results for UE-assisted DL positioning are captured. Typical UE power state transition during a power cycle for UE-assisted DL positioning is illustrated in Figure A1, where A1(a) shows CG-SDT for measurement reporting and A1(b) shows RA-SDT for measurement reporting.

Figure A1: Evaluation cases of UE power state transition for UE-assisted DL positioning with (a) CG-SDT for measurement reporting and (b) RA-SDT for measurement reporting.

The corresponding UE power consumptions are evaluated in the following table.

Table A1: UE power consumptions for UE-assisted DL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B101 | SSB synchronization | 50 | 4 | 2 | 8 | 400 | 6.04% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 3.44% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 1.81% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 21.13% |
|  | Deep sleep | 1 |  |  | 2409 | 2409 | 36.35% |
|  | Micro sleep | 45 |  |  | 16 | 720 | 10.86% |
|  | Transition energy - Deep sleep | 450 | 40 | 3 | 120 | 1350 | 20.37% |
|  | Total (every 1.28s) |  |  |  | 2560 | 6627 | 100% |
|  | Slot-averaged power unit |  |  |  | 2.59 |  |  |
|  | Battery life (in month) |  |  |  | 0.29 |  |  |
| B102 | Battery life (in month) |  |  |  | 0.57 |  |  |
| B103 | Battery life (in month) |  |  |  | 1.14 |  |  |
| B104 | Battery life (in month) |  |  |  | 1.61 |  |  |
| B105 | Battery life (in month) |  |  |  | 3.22 |  |  |
| B106 | Battery life (in month) |  |  |  | 6.44 |  |  |
| B111 | SSB synchronization | 50 | 4 | 9 | 36 | 1800 | 5.48% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 8 | 32 | 1824 | 5.56% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.37% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 4.27% |
|  | Deep sleep | 1 |  |  | 19937 | 19937 | 60.74% |
|  | Micro sleep | 45 |  |  | 72 | 3240 | 9.87% |
|  | Transition energy - Deep sleep | 450 | 40 | 10 | 400 | 4500 | 13.71% |
|  | Total (every 10.24s) |  |  |  | 20480 | 32821 | 100% |
|  | Slot-averaged power unit |  |  |  | 1.60 |  |  |
|  | Battery life (in month) |  |  |  | 0.46 |  |  |
| B112 | Battery life (in month) |  |  |  | 0.93 |  |  |
| B113 | Battery life (in month) |  |  |  | 1.85 |  |  |
| B114 | Battery life (in month) |  |  |  | 2.60 |  |  |
| B115 | Battery life (in month) |  |  |  | 5.21 |  |  |
| B116 | Battery life (in month) |  |  |  | 10.42 |  |  |
| B121 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 1.97% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 2.25% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 1.18% |
|  | RA-SDT |  |  |  | 72 | 5520 | 54.50% |
|  | 1) SSB | 50 | 4 | 2 | 8 |  |  |
|  | 2) CORESET0+SIB1 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 3) PRACH | 210 | 2 | 1 | 2 |  |  |
|  | 4) RAR (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 5) Msg3 (UL) | 700 | 2 | 1 | 2 |  |  |
|  | 6) Msg4 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 7) RRCRelease (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 8) Micro sleep | 45 |  |  | 52 |  |  |
|  | Deep sleep | 1 |  |  | 2351 | 2351 | 23.21% |
|  | Micro sleep | 45 |  |  | 8 | 360 | 3.55% |
|  | Transition energy - Deep sleep | 450 | 40 | 3 | 120 | 1350 | 13.33% |
|  | Total (every 1.28s) |  |  |  | 20480 | 32821 | 100% |
|  | Slot-averaged power unit |  |  |  | 3.96 |  |  |
|  | Battery life (in month) |  |  |  | 0.19 |  |  |
| B122 | Battery life (in month) |  |  |  | 0.37 |  |  |
| B123 | Battery life (in month) |  |  |  | 0.75 |  |  |
| B124 | Battery life (in month) |  |  |  | 1.05 |  |  |
| B125 | Battery life (in month) |  |  |  | 2.10 |  |  |
| B126 | Battery life (in month) |  |  |  | 4.21 |  |  |
| B131 | SSB synchronization | 50 | 4 | 8 | 32 | 1600 | 4.40% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 8 | 32 | 1824 | 5.02% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.33% |
|  | RA-SDT |  |  |  | 72 | 5520 | 15.20% |
|  | 1) SSB | 50 | 4 | 2 | 8 |  |  |
|  | 2) CORESET0+SIB1 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 3) PRACH | 210 | 2 | 1 | 2 |  |  |
|  | 4) RAR (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 5) Msg3 (UL) | 700 | 2 | 1 | 2 |  |  |
|  | 6) Msg4 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 7) RRCRelease (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 8) Micro sleep | 45 |  |  | 52 |  |  |
|  | Deep sleep | 1 |  |  | 19879 | 19879 | 54.73% |
|  | Micro sleep | 45 |  |  | 64 | 2880 | 7.93% |
|  | Transition energy - Deep sleep | 450 | 40 | 10 | 400 | 4500 | 12.39% |
|  | Total (every 10.24s) |  |  |  | 20480 | 36323 | 100% |
|  | Slot-averaged power unit |  |  |  | 1.77 |  |  |
|  | Battery life (in month) |  |  |  | 0.42 |  |  |
| B132 | Battery life (in month) |  |  |  | 0.84 |  |  |
| B133 | Battery life (in month) |  |  |  | 1.67 |  |  |
| B134 | Battery life (in month) |  |  |  | 2.35 |  |  |
| B135 | Battery life (in month) |  |  |  | 4.71 |  |  |
| B136 | Battery life (in month) |  |  |  | 9.42 |  |  |

UE-based DL positioning

The power consumption results for UE-based DL positioning are captured below. Typical UE power state transition during a power cycle for UE-based DL positioning is illustrated in Figure A2.

Figure A2: UE power state transition for UE-based DL positioning.

The corresponding UE power consumptions are evaluated in the following table. Note that only the power consumption of DL measurement is evaluated for UE-based DL positioning, and that of location estimation is not taken into account.

Table A2: UE power consumptions for UE-based DL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B201 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 4.68% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 5.34% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.81% |
|  | Deep sleep | 1 |  |  | 2463 | 2463 | 57.67% |
|  | Micro sleep | 45 |  |  | 8 | 360 | 8.43% |
|  | Transition energy - Deep sleep | 450 | 40 | 2 | 80 | 900 | 21.07% |
|  | Total (every 1.28s) |  |  |  | 2560 | 4271 | 100% |
|  | Slot-averaged power unit |  |  |  | 1.67 |  |  |
|  | Battery life (in month) |  |  |  | 0.44 |  |  |
| B202 | Battery life (in month) |  |  |  | 0.89 |  |  |
| B203 | Battery life (in month) |  |  |  | 1.77 |  |  |
| B204 | Battery life (in month) |  |  |  | 2.50 |  |  |
| B205 | Battery life (in month) |  |  |  | 4.99 |  |  |
| B206 | Battery life (in month) |  |  |  | 9.98 |  |  |
| B211 | SSB synchronization | 50 | 4 | 8 | 32 | 1600 | 5.25% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 8 | 32 | 1824 | 5.99% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.39% |
|  | Deep sleep | 1 |  |  | 19991 | 19991 | 65.62% |
|  | Micro sleep | 45 |  |  | 64 | 2880 | 9.45% |
|  | Transition energy - Deep sleep | 450 | 40 | 9 | 360 | 4050 | 13.29% |
|  | Total (every 10.24s) |  |  |  | 20480 | 30465 | 100% |
|  | Slot-averaged power unit |  |  |  | 1.49 |  |  |
|  | Battery life (in month) |  |  |  | 0.50 |  |  |
| B212 | Battery life (in month) |  |  |  | 0.99 |  |  |
| B213 | Battery life (in month) |  |  |  | 1.99 |  |  |
| B214 | Battery life (in month) |  |  |  | 2.80 |  |  |
| B215 | Battery life (in month) |  |  |  | 5.59 |  |  |
| B216 | Battery life (in month) |  |  |  | 11.19 |  |  |

UL positioning

The power consumption results for UL positioning are captured in the following. Typical UE power state transition during a power cycle for UL positioning are shown in Figure A3, where A3(a) considers no SRS (re)configuration in a power cycle and A3(b) shows RA-SDT for SRS (re)configuration every power cycle.

Figure A3: UE power state transition for UL positioning with (a) no SRS (re)configuration in a power cycle and (b) RA-SDT for SRS (re)configuration in a power cycle.

The corresponding UE power consumptions are evaluated in the table below.

Table A3: UE power consumptions for UL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B301 | SSB synchronization | 50 | 4 | 2 | 8 | 400 | 8.45% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 4.82% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 4.44% |
|  | Deep sleep | 1 |  |  | 2463 | 2463 | 51.87% |
|  | Micro sleep | 45 |  |  | 12 | 540 | 11.41% |
|  | Transition energy - Deep sleep | 450 | 40 | 2 | 80 | 900 | 19.02% |
|  | Total (every 1.28s) |  |  |  | 2560 | 4733 | 100% |
|  | Slot-averaged power unit |  |  |  | 1.85 |  |  |
|  | Battery life (in month) |  |  |  | 0.40 |  |  |
| B302 | Battery life (in month) |  |  |  | 0.80 |  |  |
| B303 | Battery life (in month) |  |  |  | 1.60 |  |  |
| B304 | Battery life (in month) |  |  |  | 2.25 |  |  |
| B305 | Battery life (in month) |  |  |  | 4.50 |  |  |
| B306 | Battery life (in month) |  |  |  | 9.01 |  |  |
| B311 | SSB synchronization | 50 | 4 | 9 | 8 | 1800 | 5.82% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 8 | 4 | 1824 | 5.90% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.68% |
|  | Deep sleep | 1 |  |  |  |  | 64.61% |
|  | Micro sleep | 45 |  |  | 68 | 3060 | 9.89% |
|  | Transition energy - Deep sleep | 450 | 40 | 9 | 360 | 4050 | 13.10% |
|  | Total (every 10.24s) |  |  |  | 20480 | 30927 | 100% |
|  | Slot-averaged power unit |  |  |  | 1.51 |  |  |
|  | Battery life (in month) |  |  |  | 0.49 |  |  |
| B312 | Battery life (in month) |  |  |  | 0.98 |  |  |
| B313 | Battery life (in month) |  |  |  | 1.96 |  |  |
| B314 | Battery life (in month) |  |  |  | 2.76 |  |  |
| B315 | Battery life (in month) |  |  |  | 5.52 |  |  |
| B316 | Battery life (in month) |  |  |  | 11.04 |  |  |
| B321 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 3.80% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 2.16% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 2.00% |
|  | RA-SDT |  |  |  | 72 | 5520 | 52.41% |
|  | 1) SSB | 50 | 4 | 2 | 8 |  |  |
|  | 2) CORESET0+SIB1 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 3) PRACH | 210 | 2 | 1 | 2 |  |  |
|  | 4) RAR (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 5) Msg3 (UL) | 700 | 2 | 1 | 2 |  |  |
|  | 6) Msg4 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 7) RRCRelease (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 8) Micro sleep | 45 |  |  | 52 |  |  |
|  | Deep sleep | 1 |  |  | 2357 | 2357 | 22.55% |
|  | Micro sleep | 45 |  |  | 20 | 900 | 8.54% |
|  | Transition energy - Deep sleep | 450 | 40 | 2 | 80 | 900 | 8.54% |
|  | Total (every 1.28s) |  |  |  | 2560 | 10533 | 100% |
|  | Slot-averaged power unit |  |  |  | 4.11 |  |  |
|  | Battery life (in month) |  |  |  | 0.18 |  |  |
| B322 | Battery life (in month) |  |  |  | 0.36 |  |  |
| B323 | Battery life (in month) |  |  |  | 0.72 |  |  |
| B324 | Battery life (in month) |  |  |  | 1.01 |  |  |
| B325 | Battery life (in month) |  |  |  | 2.03 |  |  |
| B326 | Battery life (in month) |  |  |  | 4.06 |  |  |
| B331 | SSB synchronization | 50 | 4 | 9 | 36 | 1800 | 4.90% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 8 | 32 | 1824 | 4.97% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.57% |
|  | RA-SDT |  |  |  | 72 | 5520 | 15.03% |
|  | 1) SSB | 50 | 4 | 2 | 8 |  |  |
|  | 2) CORESET0+SIB1 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 3) PRACH | 210 | 2 | 1 | 2 |  |  |
|  | 4) RAR (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 5) Msg3 (UL) | 700 | 2 | 1 | 2 |  |  |
|  | 6) Msg4 (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 7) RRCRelease (PDCCH + PDSCH) | 120 | 2 | 1 | 2 |  |  |
|  | 8) Micro sleep | 45 |  |  | 52 |  |  |
|  | Deep sleep | 1 |  |  | 19903 | 19903 | 54.19% |
|  | Micro sleep | 45 |  |  | 76 | 3420 | 9.31% |
|  | Transition energy - Deep sleep | 450 | 40 | 9 | 360 | 4050 | 11.03% |
|  | Total (every 10.24s) |  |  |  | 20480 | 36727 | 100% |
|  | Slot-averaged power unit |  |  |  | 1.79 |  |  |
|  | Battery life (in month) |  |  |  | 0.41 |  |  |
| B332 | Battery life (in month) |  |  |  | 0.83 |  |  |
| B333 | Battery life (in month) |  |  |  | 1.66 |  |  |
| B334 | Battery life (in month) |  |  |  | 2.33 |  |  |
| B335 | Battery life (in month) |  |  |  | 4.66 |  |  |
| B336 | Battery life (in month) |  |  |  | 9.31 |  |  |

## A2. Enhanced evaluation cases

DL positioning

Figures A4(a) and A4(b) depict the UE power consumption transition models of the UE-assisted DL positioning with gaps between paging reception, RS and measurement reporting is minimized. For the UE-based DL positioning, no measurement reporting is considered.

Figure A4: UE power state transition for UE-assisted DL positioning (a) with eDRX cycle of 20.48s; (b) without paging monitoring

UL positioning

Figures A5(a) and A5(b) depict the UE power consumption transition models of the UL positioning with gaps between paging reception, RS and measurement reporting is minimized.

Figure A5: UE power state transition for UL positioning (a) with eDRX cycle of 20.48s; (b) without paging monitoring

Based on the UE power state transition models shown above, the corresponding UE power consumptions with enhanced assumptions are evaluated in Table A4 to Table A6.

Table A4: UE power consumptions for UE-assisted DL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E101 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 0.83% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 0.95% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.50% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 5.82% |
|  | Ultra-deep sleep | 0.015 |  |  | 40079 | 601.19 | 2.50% |
|  | Light sleep | 20 |  |  | 48 | 960 | 3.99% |
|  | Micro sleep | 45 |  |  | 10 | 450 | 1.87% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 83.13% |
|  | - Light sleep | 100 | 12 | 1 | 12 | 100 | 0.42% |
|  | Total (every 20.48s) |  |  |  | 40960 | 24059.19 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.59 |  |  |
|  | Battery life (in month) |  |  |  | 1.26 |  |  |
| E102 | Battery life (in month) |  |  |  | 2.52 |  |  |
| E103 | Battery life (in month) |  |  |  | 5.04 |  |  |
| E104 | Battery life (in month) |  |  |  | 7.09 |  |  |
| E105 | Battery life (in month) |  |  |  | 14.19 |  |  |
| E106 | Battery life (in month) |  |  |  | 28.37 |  |  |
| E111 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 3.31% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 3.76% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 1.98% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 23.11% |
|  | Ultra-deep sleep | 0.015 |  |  | 40079 | 601.19 | 9.92% |
|  | Light sleep | 20 |  |  | 48 | 960 | 15.84% |
|  | Micro sleep | 45 |  |  | 10 | 450 | 7.43% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 33.01% |
|  | - Light sleep | 100 | 12 | 1 | 12 | 100 | 1.65% |
|  | Total (every 20.48s) |  |  |  | 40960 | 6059.19 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.15 |  |  |
|  | Battery life (in month) |  |  |  | 5.01 |  |  |
| E112 | Battery life (in month) |  |  |  | 10.02 |  |  |
| E113 | Battery life (in month) |  |  |  | 20.03 |  |  |
| E114 | Battery life (in month) |  |  |  | 28.16 |  |  |
| E115 | Battery life (in month) |  |  |  | 56.33 |  |  |
| E116 | Battery life (in month) |  |  |  | 112.67 |  |  |
| E121 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 4.63% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 5.28% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.78% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 32.44% |
|  | Ultra-deep sleep | 0.01 |  |  | 40829 | 408.29 | 9.46% |
|  | Light sleep | 20 |  |  | 48 | 960 | 22.24% |
|  | Micro sleep | 45 |  |  | 10 | 450 | 10.43% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 10.43% |
|  | - Light sleep | 100 | 12 | 1 | 12 | 100 | 2.32% |
|  | Total (every 20.48s) |  |  |  | 40960 | 4316.29 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.11 |  |  |
|  | Battery life (in month) |  |  |  | 7.03 |  |  |
| E122 | Battery life (in month) |  |  |  | 14.06 |  |  |
| E123 | Battery life (in month) |  |  |  | 28.12 |  |  |
| E124 | Battery life (in month) |  |  |  | 39.54 |  |  |
| E125 | Battery life (in month) |  |  |  | 79.08 |  |  |
| E126 | Battery life (in month) |  |  |  | 158.16 |  |  |
| E131 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 0.85% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.51% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 5.94% |
|  | Ultra-deep sleep | 0.015 |  |  | 40089 | 601.34 | 2.55% |
|  | Light sleep | 20 |  |  | 48 | 960 | 4.07% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 0.76% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 84.88% |
|  | - Light sleep | 100 | 12 | 1 | 12 | 100 | 0.42% |
|  | Total (every 20.48s) |  |  |  | 40960 | 23561.34 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.58 |  |  |
|  | Battery life (in month) |  |  |  | 1.29 |  |  |
| E132 | Battery life (in month) |  |  |  | 2.58 |  |  |
| E133 | Battery life (in month) |  |  |  | 5.15 |  |  |
| E134 | Battery life (in month) |  |  |  | 7.24 |  |  |
| E135 | Battery life (in month) |  |  |  | 14.49 |  |  |
| E136 | Battery life (in month) |  |  |  | 28.97 |  |  |
| E141 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 3.60% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.16% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 25.17% |
|  | Ultra-deep sleep | 0.015 |  |  | 40089 | 601.34 | 10.81% |
|  | Light sleep | 20 |  |  | 48 | 960 | 17.26% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 3.24% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 35.96% |
|  | - Light sleep | 100 | 12 | 1 | 12 | 100 | 1.80% |
|  | Total (every 20.48s) |  |  |  | 40960 | 5561.34 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.14 |  |  |
|  | Battery life (in month) |  |  |  | 5.46 |  |  |
| E142 | Battery life (in month) |  |  |  | 10.91 |  |  |
| E143 | Battery life (in month) |  |  |  | 21.83 |  |  |
| E144 | Battery life (in month) |  |  |  | 30.69 |  |  |
| E145 | Battery life (in month) |  |  |  | 61.38 |  |  |
| E146 | Battery life (in month) |  |  |  | 122.75 |  |  |
| E151 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 4.63% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.78% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 32.44% |
|  | Ultra-deep sleep | 0.01 |  |  | 40839 | 408.39 | 9.46% |
|  | Light sleep | 20 |  |  | 48 | 960 | 22.24% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 10.43% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 10.43% |
|  | - Light sleep | 100 | 12 | 1 | 12 | 100 | 2.32% |
|  | Total (every 20.48s) |  |  |  | 40960 | 3818.39 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.09 |  |  |
|  | Battery life (in month) |  |  |  | 7.95 |  |  |
| E152 | Battery life (in month) |  |  |  | 15.89 |  |  |
| E153 | Battery life (in month) |  |  |  | 31.79 |  |  |
| E154 | Battery life (in month) |  |  |  | 44.70 |  |  |
| E155 | Battery life (in month) |  |  |  | 89.39 |  |  |
| E156 | Battery life (in month) |  |  |  | 178.78 |  |  |
| E161 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 1.47% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 0.84% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.44% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 5.15% |
|  | Ultra-deep sleep | 0.015 |  |  | 37600 | 564 | 2.07% |
|  | Deep sleep | 1 |  |  | 2409 | 2409 | 8.86% |
|  | Micro sleep | 45 |  |  | 16 | 720 | 2.65% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 73.55% |
|  | - Deep sleep | 450 | 40 | 3 | 120 | 1350 | 4.96% |
|  | Total (every 20.48s) |  |  |  | 40960 | 27191 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.66 |  |  |
|  | Battery life (in month) |  |  |  | 1.12 |  |  |
| E171 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 4.35% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 2.48% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 1.31% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 15.23% |
|  | Ultra-deep sleep | 0.015 |  |  | 37600 | 564 | 6.14% |
|  | Deep sleep | 1 |  |  | 2409 | 2409 | 8.86% |
|  | Micro sleep | 45 |  |  | 16 | 720 | 7.83% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 21.76% |
|  | - Deep sleep | 450 | 40 | 3 | 120 | 1350 | 14.69% |
|  | Total (every 20.48s) |  |  |  | 40960 | 9191 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.22 |  |  |
|  | Battery life (in month) |  |  |  | 3.30 |  |  |
| E181 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 5.36% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 3.06% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 1.61% |
|  | UL | 700 | 2 | 1 | 2 | 1400 | 18.77% |
|  | Ultra-deep sleep | 0.01 |  |  | 38350 | 383.50 | 5.14% |
|  | Deep sleep | 1 |  |  | 2409 | 2409 | 32.29% |
|  | Micro sleep | 45 |  |  | 46 | 720 | 9.65% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 0.60% |
|  | - Deep sleep | 450 | 40 | 3 | 120 | 1350 | 18.10% |
|  | Total (every 20.48s) |  |  |  | 40960 | 7460.50 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.18 |  |  |
|  | Battery life (in month) |  |  |  | 4.07 |  |  |

Table A5: UE power consumptions for UE-based DL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E201 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 0.92% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 1.05% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.55% |
|  | Ultra-deep sleep | 0.015 |  |  | 40139 | 602.09 | 2.78% |
|  | Micro sleep | 45 |  |  | 12 | 540 | 2.49% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 92.21% |
|  | Total (every 20.48s) |  |  |  | 40960 | 21609.09 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.53 |  |  |
|  | Battery life (in month) |  |  |  | 1.40 |  |  |
| E202 | Battery life (in month) |  |  |  | 2.80 |  |  |
| E203 | Battery life (in month) |  |  |  | 5.60 |  |  |
| E204 | Battery life (in month) |  |  |  | 7.87 |  |  |
| E205 | Battery life (in month) |  |  |  | 15.74 |  |  |
| E206 | Battery life (in month) |  |  |  | 31.47 |  |  |
| E211 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 5.42% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 6.18% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 3.25% |
|  | Ultra-deep sleep | 0.015 |  |  | 40139 | 602.09 | 16.32% |
|  | Micro sleep | 45 |  |  | 12 | 540 | 14.63% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 54.20% |
|  | Total (every 20.48s) |  |  |  | 40960 | 3690.09 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.09 |  |  |
|  | Battery life (in month) |  |  |  | 8.22 |  |  |
| E212 | Battery life (in month) |  |  |  | 16.45 |  |  |
| E213 | Battery life (in month) |  |  |  | 32.89 |  |  |
| E214 | Battery life (in month) |  |  |  | 46.25 |  |  |
| E215 | Battery life (in month) |  |  |  | 92.50 |  |  |
| E216 | Battery life (in month) |  |  |  | 185.00 |  |  |
| E221 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 10.52% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 11.99% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 11.04% |
|  | Ultra-deep sleep | 0.01 |  |  | 40889 | 408.89 | 21.50% |
|  | Micro sleep | 45 |  |  | 12 | 540 | 21.29% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 23.66% |
|  | Total (every 20.48s) |  |  |  | 40960 | 1946.89 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.05 |  |  |
|  | Battery life (in month) |  |  |  | 15.59 |  |  |
| E222 | Battery life (in month) |  |  |  | 31.17 |  |  |
| E223 | Battery life (in month) |  |  |  | 62.35 |  |  |
| E224 | Battery life (in month) |  |  |  | 87.66 |  |  |
| E225 | Battery life (in month) |  |  |  | 175.32 |  |  |
| E226 | Battery life (in month) |  |  |  | 350.65 |  |  |
| E231 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 0.95% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.57% |
|  | Ultra-deep sleep | 0.015 |  |  | 40151 | 602.27 | 2.85% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 0.85% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 94.78% |
|  | Total (every 20.48s) |  |  |  | 40960 | 21102.27 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.52 |  |  |
|  | Battery life (in month) |  |  |  | 1.44 |  |  |
| E232 | Battery life (in month) |  |  |  | 2.88 |  |  |
| E233 | Battery life (in month) |  |  |  | 5.75 |  |  |
| E234 | Battery life (in month) |  |  |  | 8.09 |  |  |
| E235 | Battery life (in month) |  |  |  | 16.18 |  |  |
| E236 | Battery life (in month) |  |  |  | 32.35 |  |  |
| E241 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 6.45% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 3.87% |
|  | Ultra-deep sleep | 0.015 |  |  | 40151 | 602.27 | 19.41% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 5.80% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 64.47% |
|  | Total (every 20.48s) |  |  |  | 40960 | 3102.27 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.08 |  |  |
|  | Battery life (in month) |  |  |  | 9.78 |  |  |
| E242 | Battery life (in month) |  |  |  | 19.56 |  |  |
| E243 | Battery life (in month) |  |  |  | 39.13 |  |  |
| E244 | Battery life (in month) |  |  |  | 55.01 |  |  |
| E245 | Battery life (in month) |  |  |  | 110.03 |  |  |
| E246 | Battery life (in month) |  |  |  | 220.05 |  |  |
| E251 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 14.72% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 8.83% |
|  | Ultra-deep sleep | 0.01 |  |  | 40901 | 409.01 | 30.10% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 13.24% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 33.11% |
|  | Total (every 20.48s) |  |  |  | 40960 | 1359.01 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.03 |  |  |
|  | Battery life (in month) |  |  |  | 22.33 |  |  |
| E252 | Battery life (in month) |  |  |  | 44.66 |  |  |
| E253 | Battery life (in month) |  |  |  | 89.31 |  |  |
| E254 | Battery life (in month) |  |  |  | 125.58 |  |  |
| E255 | Battery life (in month) |  |  |  | 251.16 |  |  |
| E256 | Battery life (in month) |  |  |  | 502.32 |  |  |
| E261 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 0.81% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 0.92% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.48% |
|  | Ultra-deep sleep | 0.015 |  |  | 37600 | 564 | 2.27% |
|  | Deep sleep | 1 |  |  | 2463 | 2463 | 9.92% |
|  | Micro sleep | 45 |  |  | 8 | 360 | 1.45% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 80.53% |
|  | - Deep sleep | 450 | 40 | 2 | 80 | 900 | 3.62% |
|  | Total (every 20.48s) |  |  |  | 40960 | 24835 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.61 |  |  |
|  | Battery life (in month) |  |  |  | 1.22 |  |  |
| E271 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 2.92% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 3.34% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 1.76% |
|  | Ultra-deep sleep | 0.015 |  |  | 37600 | 564 | 8.25% |
|  | Deep sleep | 1 |  |  | 2463 | 2463 | 36.03% |
|  | Micro sleep | 45 |  |  | 8 | 360 | 5.27% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 29.26% |
|  | - Deep sleep | 450 | 40 | 2 | 80 | 900 | 13.17% |
|  | Total (every 20.48s) |  |  |  | 40960 | 6835 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.17 |  |  |
|  | Battery life (in month) |  |  |  | 4.44 |  |  |
| E281 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 3.92% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 4.47% |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.35% |
|  | Ultra-deep sleep | 0.01 |  |  | 38350 | 383.50 | 7.51% |
|  | Deep sleep | 1 |  |  | 2463 | 2463 | 48.25% |
|  | Micro sleep | 45 |  |  | 8 | 360 | 7.05% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 8.82% |
|  | - Deep sleep | 450 | 40 | 2 | 80 | 900 | 17.63% |
|  | Total (every 20.48s) |  |  |  | 40960 | 5104.50 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.12 |  |  |
|  | Battery life (in month) |  |  |  | 5.94 |  |  |

Table A6: UE power consumptions for UL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E301 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 9.24% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 1.05% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.97% |
|  | Ultra-deep sleep | 0.015 |  |  | 40142 | 602.13 | 2.78% |
|  | Micro sleep | 45 |  |  | 9 | 405 | 1.87% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 92.40% |
|  | Total (every 20.48s) |  |  |  | 40960 | 21645.13 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.53 |  |  |
|  | Battery life (in month) |  |  |  | 1.40 |  |  |
| E302 | Battery life (in month) |  |  |  | 2.80 |  |  |
| E303 | Battery life (in month) |  |  |  | 5.61 |  |  |
| E304 | Battery life (in month) |  |  |  | 7.88 |  |  |
| E305 | Battery life (in month) |  |  |  | 15.77 |  |  |
| E306 | Battery life (in month) |  |  |  | 31.54 |  |  |
| E311 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 5.49% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 6.25% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 5.76% |
|  | Ultra-deep sleep | 0.015 |  |  | 40142 | 602.13 | 16.52% |
|  | Micro sleep | 45 |  |  | 9 | 405 | 11.11% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 54.87% |
|  | Total (every 20.48s) |  |  |  | 40960 | 3645.13 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.09 |  |  |
|  | Battery life (in month) |  |  |  | 8.32 |  |  |
| E312 | Battery life (in month) |  |  |  | 16.65 |  |  |
| E313 | Battery life (in month) |  |  |  | 33.30 |  |  |
| E314 | Battery life (in month) |  |  |  | 46.82 |  |  |
| E315 | Battery life (in month) |  |  |  | 93.64 |  |  |
| E316 | Battery life (in month) |  |  |  | 187.28 |  |  |
| E321 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 10.52% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 11.99% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 11.04% |
|  | Ultra-deep sleep | 0.01 |  |  | 40892 | 408.92 | 21.50% |
|  | Micro sleep | 45 |  |  | 9 | 405 | 21.29% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 23.66% |
|  | Total (every 20.48s) |  |  |  | 40960 | 1901.92 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.05 |  |  |
|  | Battery life (in month) |  |  |  | 15.95 |  |  |
| E322 | Battery life (in month) |  |  |  | 31.91 |  |  |
| E323 | Battery life (in month) |  |  |  | 63.82 |  |  |
| E324 | Battery life (in month) |  |  |  | 89.73 |  |  |
| E325 | Battery life (in month) |  |  |  | 179.47 |  |  |
| E326 | Battery life (in month) |  |  |  | 358.93 |  |  |
| E331 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 0.94% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.99% |
|  | Ultra-deep sleep | 0.015 |  |  | 40151 | 602.27 | 2.84% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 0.85% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 94.37% |
|  | Total (every 20.48s) |  |  |  | 40960 | 21192.27 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.52 |  |  |
|  | Battery life (in month) |  |  |  | 1.43 |  |  |
| E332 | Battery life (in month) |  |  |  | 2.86 |  |  |
| E333 | Battery life (in month) |  |  |  | 5.73 |  |  |
| E334 | Battery life (in month) |  |  |  | 8.05 |  |  |
| E335 | Battery life (in month) |  |  |  | 16.11 |  |  |
| E336 | Battery life (in month) |  |  |  | 32.21 |  |  |
| E341 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 6.27% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 6.58% |
|  | Ultra-deep sleep | 0.015 |  |  | 40151 | 602.27 | 18.87% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 5.64% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 62.65% |
|  | Total (every 20.48s) |  |  |  | 40960 | 3192.27 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.08 |  |  |
|  | Battery life (in month) |  |  |  | 9.51 |  |  |
| E342 | Battery life (in month) |  |  |  | 19.01 |  |  |
| E343 | Battery life (in month) |  |  |  | 38.02 |  |  |
| E344 | Battery life (in month) |  |  |  | 53.46 |  |  |
| E345 | Battery life (in month) |  |  |  | 106.93 |  |  |
| E346 | Battery life (in month) |  |  |  | 213.85 |  |  |
| E351 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 13.80% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 14.49% |
|  | Ultra-deep sleep | 0.01 |  |  | 40901 | 409.01 | 28.23% |
|  | Micro sleep | 45 |  |  | 4 | 180 | 12.42% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 31.06% |
|  | Total (every 20.48s) |  |  |  | 40960 | 1449.01 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.04 |  |  |
|  | Battery life (in month) |  |  |  | 20.94 |  |  |
| E352 | Battery life (in month) |  |  |  | 41.88 |  |  |
| E353 | Battery life (in month) |  |  |  | 83.77 |  |  |
| E354 | Battery life (in month) |  |  |  | 117.78 |  |  |
| E355 | Battery life (in month) |  |  |  | 235.56 |  |  |
| E356 | Battery life (in month) |  |  |  | 471.13 |  |  |
| E361 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 1.58% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 0.90% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.83% |
|  | Ultra-deep sleep | 0.015 |  |  | 37600 | 564 | 2.23% |
|  | Deep sleep | 1 |  |  | 2455 | 2455 | 9.70% |
|  | Micro sleep | 45 |  |  | 12 | 540 | 2.13% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 20000 | 800 | 1 | 800 | 20000 | 79.06% |
|  | - Deep sleep | 450 | 40 | 2 | 80 | 900 | 3.56% |
|  | Total (every 20.48s) |  |  |  | 40960 | 25297 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.62 |  |  |
|  | Battery life (in month) |  |  |  | 1.20 |  |  |
| E371 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 5.48% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 3.12% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 2.88% |
|  | Ultra-deep sleep | 0.015 |  |  | 37600 | 564 | 7.73% |
|  | Deep sleep | 1 |  |  | 2455 | 2455 | 33.64% |
|  | Micro sleep | 45 |  |  | 12 | 540 | 7.40% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 2000 | 800 | 1 | 800 | 2000 | 27.41% |
|  | - Deep sleep | 450 | 40 | 2 | 80 | 900 | 12.33% |
|  | Total (every 20.48s) |  |  |  | 40960 | 7297 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.18 |  |  |
|  | Battery life (in month) |  |  |  | 4.16 |  |  |
| E381 | SSB synchronization | 50 | 4 | 1 | 4 | 200 | 7.19% |
|  | Paging | 0.1*120 + 0.9*50 = 57 | 4 | 1 | 4 | 228 | 4.10% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 3.77% |
|  | Ultra-deep sleep | 0.01 |  |  | 38350 | 383.50 | 6.89% |
|  | Deep sleep | 1 |  |  | 2455 | 2455 | 44.10% |
|  | Micro sleep | 45 |  |  | 12 | 540 | 9.70% |
|  | Transition energy |  |  |  |  |  |  |
|  | - Ultra-deep sleep | 450 | 50 | 1 | 50 | 450 | 8.08% |
|  | - Deep sleep | 450 | 40 | 2 | 80 | 900 | 16.17% |
|  | Total (every 20.48s) |  |  |  | 40960 | 5566.50 | 100% |
|  | Slot-averaged power unit |  |  |  | 0.14 |  |  |
|  | Battery life (in month) |  |  |  | 5.45 |  |  |