3GPP TSG RAN WG1 #110bis-e	R1-2209373

e-Meeting, October 10th – 19th, 2022

Agenda item:		8.7

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on Idle/Inactive mode power saving

Document for:		Discussion and Decision

# Introduction

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

  - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1]

NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

The release 17 work item [1] on UE power saving includes the following objective:

In this contribution, we discuss about the remaining aspects related to the TRS occasion for IDLE/Inactive UEs.

# Behaviour and signaling related to TRS occassions

In RAN1#109e it was agreed to confirm the understanding the that the TRS occasions configured by TRS-ResourceSet do not impact in any way to the UE in CONNECTED mode operation. I.e. should not restrict or limit the CONNECTED mode configuration of CSI-RS or otherwise. In RAN1#110, a further agreement was made to capture this in TS38.211 as follows:

Agreement Adopt the following correction to clause 7.3.1.5 of TS 38.211 for alignment with RAN1#109-e agreement, “TRS-ResourceSetConfig IE does not affect the UE behaviour in CONNECTED mode and can be ignored by UE in CONNECTED mode” #### < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with to resource elements in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index over the assigned virtual resource blocks, where is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index . #### < Unchanged parts are omitted >.

The afore agreed change was implemented in CR [2].

Focusing in the bullet to which the change was agreed in last meeting, it unfortunately appears that there is two ways to read this. First way, as intended by the CR and earlier agreements; For the DL physical channel mapping assume that the resource blocks are available when (in the bullet in question, omitting others):

-  not used by NZP-CSI-RS as per clause 7.4.1.5 unless configured by TRS-ResourceSet IE

- not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource;

Second way; For the DL physical channel mapping assume that the resource blocks are available when (in the bullet in question, omitting others):

- not used by NZP-CSI-RS as per clause 7.4.1.5

- not configured by TRS-ResourceSet IE

- not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource;

Thus it would appear that there is some risk of miss-interpreting the spesification text. Hence, to ensure unabiquous interpretation, as intended by earlier agreement, following change could be considered to the afore text:

Proposal 1: Adopt following TP to Section 7.3.1.5 of TS38.211:

#### < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - unless the non-zero power CSI-RS are configured by TRS-ResorceSet IE, in which case the resource elements are considered available for PDSCH - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. #### < Unchanged parts are omitted >

Corresponding draft CR is in [3].

# Conclusion

In section 2 we discussed the remaining open items relared to the TRS occasions for the IDLE/INACTIVE mode UEs and propose a following TP to ensure consistent interpretation of the spesification:-

Proposal 1: Adopt following TP to Section 7.3.1.5 of TS38.211:

#### < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - unless the non-zero power CSI-RS are configured by TRS-ResorceSet IE, in which case the resource elements are considered available for PDSCH - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. #### < Unchanged parts are omitted >

Corresponding draft CR is in [3].

# References

- Mediatek, ” New WI: UE Power Saving Enhancements”, RP-193239, December 2019

- R1-2208276, Corrections on UE Power Saving Enhancements for NR in TS 38.211, Moderator (MediaTek) et al

- R1-2209375, Clarification of virtual resource block mapping in respect of TRS-ResourceSet IE, Nokia, Nokia Shanghai Bell