3GPP TSG RAN WG1 #110bis-e	R1-2209381

e-Meeting, October 10th – October 19th, 2022

Source:	Sharp

Title:	CSI enhancement

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Background

In RAN#94e, a new work item “MIMO Evolution for Downlink and Uplink” was approved, and the following objectives were agreed for unified TCI framework extension [1].

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking … - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we share our views on CSI enhancements.

Discussions

2.1 CSI enhancement for coherent joint transmission

At RAN1#110 meeting, the following agreements were made for CSI enhancement for CJT.

Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, support NTRP={1, 2, 3, 4} with equal priority. Agreement For the Rel-18 Type-II codebook for CJT mTRP, support RI={1,2,3,4}. Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported: - The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP) - Each of the CSI-RS resources has a same number of CSI-RS ports - Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied). Agreement For the Rel-18 Type-II codebook for CJT mTRP, support the following two modes: - Mode 1: Per-TRP/TRP-group SD/FD basis selection which allows independent FD basis selection across N TRPs / TRP groups. Example formulation (N = number of TRPs or TRP groups): - Mode 2: Per-TRP/TRP group (port-group or resource) SD basis selection and joint/common (across N TRPs) FD basis selection. Example formulation (N = number of TRPs or TRP groups): - Striving for the two modes to share commonality in detailed designs such as parameter combinations, basis selection, TRP (group) selection, reference amplitude, W2 quantization schemes. - FFS: Depending on the decision on SCI design, whether additional per-TRP/TRP-group amplitude scaling and/or co-phase is needed or not, and whether they are a part of W2s Agreement On the Type-II codebook refinement for CJT mTRP, down-select from the following TRP selection/determination schemes (where N is the number of cooperating TRPs assumed in PMI reporting) by RAN1#110bis-e: - Alt1. N is gNB-configured via higher-layer (RRC) signalling - The N configured TRPs are gNB-configured via higher-layer (RRC) signalling - Note: only one transmission hypothesis is reported - Alt2. N is UE-selected and reported as a part of CSI report where N{1,..., NTRP} - N is the number of cooperating TRPs, while NTRP is the maximum number of cooperating TRPs configured by gNB - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme) - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit - Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses. - Alt3. The UE reports CSI corresponding to K transmission hypotheses - The N configured TRPs per hypothesis are gNB-configured via higher-layer (RRC) signalling - FFS: supported value(s) of K, and whether the K transmission hypotheses are gNB-configured or UE-reported - FFS: Whether the same N value or possibly different N values - Alt4. The UE reports CSI corresponding to K transmission hypotheses where N is UE-selected and reported as a part of CSI report where N{1,..., NTRP} - N is the number of cooperating TRPs per hypothesis, while NTRP is the maximum number of cooperating TRPs configured by gNB - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme) - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit - FFS: Whether the same N value or possibly different N values FFS: Whether S-TRP transmission hypothesis is also reported Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, for each layer, down-select one from the following alternatives by RAN1#110bis-e: - Alt1. One group comprises one polarization across all TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2), one (common) SCI across all TRPs/TRP groups - Alt2. One group comprises one polarization for one TRP/TRP-group (Cgroup,phase=N, Cgroup,amp=2N), per-TRP/TRP-group SCI - FFS: Quantization of N strongest coefficients - Alt3. One group comprises one polarization for one TRP/TRP-group with a common phase reference across TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2N) - FFS: SCI, per-TRP/TRP-group vs. one (common) SCI across all TRPs/TRP groups - FFS: Quantization of N strongest coefficients - Alt4. For a selected TRP/TRP-group, one group comprises one polarization, and for remaining N-1 TRPs/TRP-groups, one group comprises one polarization across remaining N-1 TRPs/TRP-groups (Cgroup,amp=2+2=4), with a common phase reference across all of N TRPs/TRP-groups (Cgroup,phase=1) - FFS: The selected TRP/TRP-group FFS: The need for “strongest” TRP/TRP-group indicator in addition to SCI(s) For the Rel-18 Type-II codebook for CJT mTRP based on the Rel-16 Type-II codebook, SD basis and FD basis are separate, each fully reusing the legacy Rel-16 DFT-based design Agreement The Rel-18 Type-II codebook for CJT mTRP comprises refinement of the following codebooks: - Refinement of the Rel-16 eType-II regular codebook - Refinement of the Rel-17 FeType-II port selection (PS) codebook, based on the same design details as the refinement of the Rel-16 eType-II regular codebook, except for the supported set of parameter combinations Strive to maintain as much commonality between the Rel-16 and Rel-17 codebook enhancements to minimize workload. Vivo and Lenovo raised concerns on the workload due to this agreement

One of the remaining topics is SD/FD basis selection. In our view, for SD basis selection, layer-common and polarization-common selection is preferred. For FD basis selection, the same solution (i.e. layer-specific selection for refinement of Rel-16 eType II and layer-common selection for refinement of Rel-17 FeType II PS) is preferred.

Proposal 1: For CSI enhancements for CJT,

- Layer-common and polarization-common selection are supported for SD basis selection

- Layer-specific selection is supported for FD basis selection for refinement of Rel-16 eType II CB

- Layer-common selection is supported for FD basis selection for refinement of Rel-17 FeType II PS

2.2	CSI enhancement for high/medium UE velocities

2.2.1 Type-II CSI refinement

At RAN1#10 meeting, the following agreements were made for Type-II CSI refinement for high/medium UE velocities [2].

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, down-select one from the following codebooks structures: - Alt2A: Doppler-domain basis commonly selected for all SD/FD bases, e.g. - Note that Wd may be the identity as a special case - Alt2B: Doppler-domain basis independently selected for different SD/FD bases - Note that Wd may be the identity as a special case - Alt3. Reuse Rel-16/17 (F)eType-II codebook with multiple W2 and a single W1 and Wf report. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, on the DD/TD basis waveforms: - Down-select or combine from the following Doppler-/time-domain basis waveforms: - Alt1. Orthogonal DFT - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - Alt2. Identity (i.e. no Doppler-/time-domain compression) - FFS: Whether Doppler-/time-domain (DD/TD) basis vector length (N4) is RRC-configured or reported by the UE - FFS: Whether the number of selected DD/TD basis vectors (for Alt1) is RRC-configured or reported by the UE Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, down-select one from the following alternatives by RAN1#110bis-e: - Alt1.B: l ≥ nref - nref (a CSI reference resource slot) as boundary - Alt2.B: l ≥ n - n (report slot) as boundary

In our view, the time domain interpolation is based on a single Doppler. Therefore, layer-common Doppler domain basis is preferred.

Proposal 2: Doppler-domain basis commonly selected for all SD/FD bases, e.g.

Regarding the assumption of UE-side prediction, our view is to support Alt.1 B because CQI is calculated based on the CSI reference resource.

Proposal 3: For UE-side prediction, l ≥ nref is preferred

2.2.2 Reporting of TRS-based time-correlation/Doppler information via TRS

At RAN1#110 meeting, the following agreements were made for UE reporting of TRS-based time-correlation/Doppler information via TRS.

Agreement The Rel-18 TRS-based TDCP reporting comprises stand-alone auxiliary feedback information to enable refinement of CSI reporting configuration, and/or codebook configuration parameters, and/or (to be confirmed in RAN1#110) gNB-side CSI prediction - Not conditioned on other UCI parameters - Not reported together with CQI/PMI/RI/(CRI) associated with a codebook - Note: This does not prevent TDCP reporting from being multiplexed with other UCI parameters on PUCCH and/or PUSCH - Note: Aperiodic reporting is supported (per agreed Alt1 in RAN1#109-e) Agreement For the Rel-18 TRS-based TDCP reporting, down select one of the following alternatives by RAN1#110bis-e: - AltA. Based on Doppler profile - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc - AltB. Based on time-domain correlation profile - E.g. Correlation within one TRS resource, correlation across multiple TRS resources - Note: The correlation over one or more lags of TRS resource may be considered. The lags may be within one TRS burst or different TRS bursts - AltC: CSI-RS resource and/or CSI reporting setting configuration parameter(s) to assist network - E.g. gNB configures UE with multiple choices on what to assist (e.g. two or more CSI-RS/report periodicities, or precoding schemes depending mainly on UE velocity), then UE report according to configuration; parameters correspond to CSI reporting periodicity, codebook type, etc. Note: Different alternatives may or may not apply to different use cases Agreement For the Rel-18 TRS-based TDCP reporting, the use case of “aiding gNB-side CSI prediction” is refined to “aiding gNB implementation in CSI prediction for TDD”

Regarding TDCP parameter, we prefer Alt. B (time domain correlation).

Proposal 4: For the Rel-18 TRS-based TDCP reporting, TDCP parameter should be based on time-domain correlation profile

Conclusion

In this contribution, we have the following proposal:

Proposal 1: For CSI enhancements for CJT,

- Layer-common and polarization-common selection are supported for SD basis selection

- Layer-specific selection is supported for FD basis selection for refinement of Rel-16 eType II CB

- Layer-common selection is supported for FD basis selection for refinement of Rel-17 FeType II PS

Proposal 2: Doppler-domain basis commonly selected for all SD/FD bases, e.g.

Proposal 3: For UE-side prediction, l ≥ nref is preferred

Proposal 4: For the Rel-18 TRS-based TDCP reporting, TDCP parameter should be based on time-domain correlation profile

References

RAN Meeting#94, RP-213598, New WID: MIMO Evolution for Downlink and Uplink

RAN1 Meeting#110, Chair's notes RAN1#110.