3GPP TSG RAN WG1#110-bis-e	           R1-2209393

E-meeting, 10th – 19th October 2022

Agenda item:	9.5.1.2

Source: 	Lenovo

Title: 	Potential SL Positioning Solutions

Document for:	Discussion

# Introduction

The Rel-18 NR positioning evolution SID was agreed upon during the RAN#94-e [1] meeting, where one of the objectives included the potential solutions to enable SL Positioning. Agreements were made during RAN1#109-e [2] to further study potential SL Positioning Solutions.

This contribution provides a detailed discussion into the potential solutions across different areas to support SL Positioning.

# Terminologies

A few terminologies were agreed upon during the RAN1#109-e [2] to help further the study:

RAN1#109-e Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface). - Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information. - Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE. - Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes. - SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity. - Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion. RAN1#109-e Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. - FFS: clarification of the knowledge of the location of the anchor UE

In addition, the LS from SA2 has informed RAN1 about the following additional terminology [3]:

- Reference UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. using sidelink. - NOTE 1: SL Reference UE is understood as "Anchor UE" in RAN WG1 TR 38.859 . - NOTE 2: "Reference UE" mentioned in KIs and Solutions of this TR refers to "SL Reference UE". - Assistant UE: A UE supporting Ranging/Sidelink Positioning between a SL Reference UE and a Target UE over PC5, when the direct Ranging/Sidelink positioning between the SL Reference UE and Target UE cannot be supported. The measurement/result of Ranging/Sidelink Positioning between the Assistant UE and the SL Reference UE and that between the Assistant UE and the Target UE are determined and used to derive the Ranging/Sidelink Positioning result between Target UE and SL Reference UE. - Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning. - SL Positioning Server UE: A UE offering location calculation, for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if location calculation is supported. - SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it. - NOTE 3: The SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result.

It can be noted that the SA2 definition of Reference UE and Located UE aligns with the RAN1 and RAN2 definition of an Anchor UE. Reference UE /Reference device has been used in the past discussions to refer to a positioning reference unit, which may be confusing. Therefore, the fragmented definition of the same concept may be cause ambiguity and it is suggested to inform SA2 to adopt the “Anchor UE” terminology instead of the “Reference UE”, where the “Located UE” falls under the definition of an “Anchor UE”.

Proposal 1: Request SA2 to adopt the terminology of the “Anchor UE” in their discussions to avoid fragmented definitions of the same concept of “Reference UE” and “Located UE”.

On the remaining issues for discussion, there are few additional terminologies, which require additional consideration and alignment. In the context of SL, there can be some ambiguity on which nodes are initiate and request for time-frequency resources within a given SL positioning session since both the so-called Tx and Rx UE(s) can potentially transmit, measure and report SL-PRS. Therefore, terminology is needed as to which UEs are triggering and responding to the SL positioning procedures.

Further to that, the configuration entity/node may also vary depending on the coverage scenario (e.g., in-coverage vs out-of-coverage) and positioning mode (UE-assisted vs. UE-based positioning). Therefore, terminology is needed as to which node is capable of configuring SL-PRS time-frequency resources and related SL positioning configurations

Observation 1: Terminology needs to be defined as to which UE starts a SL positioning/ranging session and which UE(s) is/are the intended recipient or responder of the SL positioning/ranging session.

Therefore, the following definitions are proposed in addition to the those agreed in RAN1#109-e.

- Initiator UE initiates a SL positioning/ranging session, e.g. anchor UE, target-UE, assistance UE

- Responder UE responds to a SL positioning/ranging session from an initiator UE, , e.g. anchor UE, target-UE, assistance UE.

In this case, both the Initiator and Responder UE can be both a TX and RX UEs at a given time within the same SL positioning session.

Proposal 2: RAN1 to consider the following additional terminology during the study:

- Initiator UE initiates a SL positioning/ranging session, e.g. anchor UE, target-UE, assistance UE

- Responder UE responds to a SL positioning/ranging session from an initiator UE, , e.g. anchor UE, target-UE, assistance UE.

# SL Positioning Models

The traditional 3GPP Uu positioning framework consists of 2 models, whereby the position estimate of a UE is computed, which includes:

- Uu Model 1- UE-assisted positioning: The configuration and positioning calculation entity is the LMF

- Uu Model 2 - UE-based positioning: The configuration entity is the LMF, while the positioning calculation entity is the UE.

These aforementioned models only catered to in-coverage UEs. Considering the additional out of coverage scenarios for SL positioning, the following 4 SL positioning models are to be supported during the study:

- SL Positioning Model 1a - UE-assisted positioning: The configuration entity and positioning calculation entity is a network node comprising either of the LMF or gNB .

- SL Positioning Model 1b – UE-assisted positioning: The configuration entity is a UE, while the positioning calculation entity is a network node comprising either of a LMF or gNB.

- SL Positioning Model 2a – UE-based positioning: The configuration entity and positioning calculation entity is a UE.

- SL Positioning Model 2b - UE-based positioning: The configuration entity is a network node comprising either of a LMF or gNB, while the positioning calculation entity is a UE.

The configuration entity may configure the assistance data (including time-frequency resources in which to measure the SL positioning measurements). On the other hand, the positioning calculation entity is responsible for SL measurement requests as well as the reception of the SL measurements. The above models mentioned ensure that SL positioning is supported in in-coverage, partial coverage as well as out-of-coverage scenarios.

Proposal 3: Support the following 4 different models for SL positioning, which distinguish the configuration entity and positioning calculation entity for in-coverage, and partial and out-of-coverage scenarios:

- SL Positioning Model 1a - UE-assisted positioning (network-configured)

- SL Positioning Model 1b – UE-assisted positioning (UE-configured )

- SL Positioning Model 2a – UE-based positioning (UE-configured)

- SL Positioning Model 2b - UE-based positioning (network-configured)

# SL Positioning Physical Layer Structure

The following SL physical layer structure aspects was agreed upon during the previous RAN1#109-e [2] and RAN1#110-e [4] meetings:

RAN1#109-e Agreements With regards to the numerologies of the SL-PRS, limit the study to those supported for NR Sidelink. - Note 1: NR Sidelink supports {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2 - Note 2: This doesn’t imply that SL-PRS FR2-specific optimization(s) are expected to be studied Study new reference signal for SL positioning/ranging using the existing PRS/SRS design and SL design framework as a starting point. - The study could at least include: Sequence design, frequency domain pattern, time domain pattern (e.g. number of symbols, repetitions, etc), time domain behavior, configuration/triggering/activation/de-activation of the SL-PRS, AGC time, Tx-Rx Turanround time, supportable bandwidth(s), multiplexing options with other SL channels, randomization/orthogonalization options. - Note: The study of existing SL reference signal for SL positioning/ranging is not precluded. Companies are encouraged to perform performance evaluation/comparison to investigate whether such reference signals can meet the positioning accuracy requirements. With regards to the frequency domain pattern, study further a Comb-N SL-PRS design. Study at least the following aspects: - N>=1 (where N=1 corresponds to full RE mapping pattern) - Fully staggered SL-PRS pattern (e.g., M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used), Partially staggered SL-PRS pattern (e.g., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used), Unstaggered SL-PRS patterns (e.g., M symbol(s) of SL-PRS with comb- N, at each symbol a same RE offset is used, N > 1) - The number of symbols of SL-PRS within a slot - Any relation to the comb-N option - RE offset pattern repetitions within a slot - FFS: Other frequency domain pattern(s) For a potential new SL PRS, study further the following - Number of symbol(s) for AGC and/or Rx-Tx turnaround time - Conditions under which AGC training and/or Rx-Tx turnaround time are needed RAN1#110 Agreements A new reference signal should be introduced for supporting SL positioning/ranging. For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2: - Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point. - Alt. 2: ZC-based (SRS sequence as a starting point) With regards to the frequency domain pattern, a Comb-N SL-PRS occupying M symbol(s) design should be introduced for the support of NR SL positioning - Note: there could be multiple values for M, N With regards to the frequency domain pattern for multi-symbol SL-PRS, prioritize partially and fully staggered SL-PRS. - Note: this does not preclude comb N=1 - FFS: single symbol SL-PRS, if supported

## Sequence Type

A new reference signal for SL positioning has been agreed to be introduced of SL Positioning. A key issue is to down-select between the existing pseudorandom DL-PRS sequence design and a ZC-based sequence design that uses the current SRS sequence as a starting point.

The Zadoff-chu sequence offers additional cyclic shift-based orthogonality, which offers the flexibility of generating additional orthogonal signals from the same base sequence, and can be advantageous for SL positioning where positioning may have to be performed amongst a large number of distributed UEs while minimising interference. ZC sequences exhibit constant amplitude in both frequency and time and therefore offer desirably low PAPR, when compared to Gold sequences. Due to the short range of SL as well as the limited propagation delays, it would be possible, to distinguish different CIRs to support the required SL Positioning techniques at both the Initiator and Responder UEs.

Proposal 4: RAN1 to adopt ZC-based sequences with SRS as a baseline design for SL Positioning.

## Comb Design

Another key issue is to study the design of the ZC sequence in terms of whether a comb structure containing staggered, partial or non-staggered mapping for enabling SL PRS transmissions across the frequency/time domain. RAN1 should at least consider the re-use of the staggered design that a key design element in the positioning framework.

A key issue to consider for both fully staggered and partially staggered SL PRS pattens is the number of available SL symbols within a SL slot. According to the Uu positioning design a maximum comb-size of 12 could be supported, however due to the nature of SL communication, this may be challenging to support. Considering that an AGC may occupy 1 symbol, PSCCH may occupy 2 or 3 symbols, Tx-Rx turnaround time (Guard symbol) may occupy a further 1 symbol, the number of available slots for SL-PRS may be 9 or 10 symbols. As such it may only be feasible to support a comb size of maximum 10 symbols.

Proposal 5: Partial and Fully staggered comb patterns should consider the number of available symbols within a SL slot considering number of available symbols of 9 or 10. FFS whether a comb size of 12 symbols may be still supported.

In addition, the use of Partial staggered design may result in an inefficient use of SL resources as certain subcarriers will be potentially left blank due to the structure of the partial staggered pattern. This can lead to a degradation of the positioning performance depending on the symbol length (M) and comb-N permutation.

Proposal 6: RAN1 to consider at least the staggered SL PRS design option. FFS the positioning performance of partially staggered designs.

# SL Positioning Techniques

Progress was also made with regard to the study of positioning techniques in RAN1#109-e [2] and RAN1#110 [4]:

RAN1#109-e Agreement With regards to the Positioning methods supported using SL measurements study further the following methods: - RTT-type solutions using SL - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT - SL-AoA - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study - SL-TDOA - SL-AoD - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu. - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study - Consider in the study at least the following aspects: - Definition(s) of the corresponding SL measurements for each method - Which method is applicable to absolute or relative positioning or ranging, including whether such categorization is needed to be discussed. - For angle-based methods, antenna configuration consideration(s) using practical UE capabilities - Per-panel location, if UE uses multiple panels. - UE’s mobility, especially for V2X scenarios - Impact of synchronization error(s) between UEs - Existing SL measurements (e.g. RSSI, RSRP), and UE ID information etc, may be used. - Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method. - Note: When the study of carrier phase positioning and the evaluations of sidelink positioning have progressed, it can be reviewed whether carrier phase for sidelink can be considered in further work. Checkpoint at RAN1#110-e-Bis to see if sufficient information is available for this review. Note: Companies are encouraged to describe the role of SL nodes and their interaction/coordination participating in each method. RAN1#110 Agreement With regards to the Positioning methods supported using at least SL measurements, potential candidate positioning methods include at least the following: - RTT-type solution(s) using SL - SL-AoA - SL-TDOA - Note: other methods can still be studied - Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified.

The potential SL positioning techniques should be able to support different scenarios, especially given that anchor nodes (whose location is usually known) and non-anchor nodes (whose location is usually unknown or only coarsely known) can be considered given the distributed nature of SL communications. The SL RAT-dependent positioning techniques to be supported may be summarized as follows:

- SL timing-based positioning methods

  - SL-TDoA

  - SL-RTT (including one-way and two-way RTT methods)

- SL angular-based positioning methods

  - SL-AoD

  - SL-AoA

- SL Location fingerprinting methods

  - Exploiting RRM measurements such as SL RS RSRP/RSRQ measurements

## SL Timing-based methods

### SL-TDoA

NR DL-TDoA has already been employed since Rel-16 on the Uu-interface and requires at least 3 or more anchor nodes (including a reference node) and can be easily extended to support SL absolute positioning of the target-UE. The anchor nodes may comprise of RSUs with known fixed locations and are usually deployed alongside a highway or at traffic intersections in the context of V2X scenarios.  In the case of the other use cases, static UEs may also act as fixed anchor nodes. Some challenges include achieving the tight synchronisation requirements among RSUs and UEs as well as localizing highly mobile UEs. An additional challenge would involve the mobility of anchor nodes along with the target UE.

According to the to the previous RAN1#110 meeting agreement [4], SL-TDoA has been considered as a viable SL Positioning Technique. It is also straight-forward for the UE to measurement the SL reference signal time difference measurements between a reference anchor UE/node and another anchor UE/node. RAN1 may also consider the definition of SL RSTD measurements. Therefore, in order to perform RSTD measurements, a reference UE/node is also required similar to the “Reference TRP” concept for Uu positioning. In other words, a reference anchor node should be selected with respect to of available SL anchor UE/s/nodes.

In addition, support is required for signalling the relative time differences (RTD) between a reference anchor UE/node and the other respective anchor UEs/nodes. Such information will assist in the correction of any synchronization errors arising from the transmission of SL-PRS from the different anchor nodes.

Proposal 7: RAN1 to define SL RSTD measurements for supporting SL-TDOA.

Proposal 8: RAN1 to at least introduce the support for a reference anchor node/UE for SL RSTD measurements and SL relative time difference (RTD) information.

### SL-RTT

Multi-cell Round trip time (Multi-RTT) has also been supported on the Uu interface since Rel-16 to perform position estimation using the round trip signal time at the gNB (using gNB Rx-Tx time difference measurements) and at the UE (using UE Rx-Tx time difference measurements). RTT requires at least one anchor or non-anchor to determine its absolute or relative location estimate, and the positioning performance is enhanced as more nodes are involved. In the case of SL positioning, two variants of RTT are envisioned to be supported and described below.

Observation 2: RTT requires at least one anchor or non-anchor node for absolute/relative location estimation.

#### One-way SL-RTT

The one-way (single-sided) RTT method is similar in principle to the Multi-RTT technique whereby an initiator UE transmits a SL PRS signal and the responder UE transmits a reply signal, which may include a corresponding SL PRS transmission, and thereafter the initiator UE performs the one-way RTT measurement considering transmit and receive timing of the SL PRS. The overall procedure is illustrated in Figure 1. The signal propagation time between a pair of UEs is used to derive the distance/range between the devices as a function of the UE-1 Ranging Round Duration (A) and UE-2 Reply Time Duration (B), where ε is sum of the absolute clock frequency offset errors, each independently measured at the Initiator and Responder UE. This can be calculated as follows:

Tpropogation-delay=A-B-ε2,

Figure 1: One-way SL-RTT procedure

It can be noted that the propagation delay computation between two SL UEs is also dependent on timing errors such as clock offset at each UE, which require compensation for an accurate range estimate.

Observation 3: One-way RTT is susceptible to UE clock offset errors at the Initiator and Responder UE.

#### Two-way SL-RTT

The two-way (double-sided) RTT method draws from the same principles of one-way RTT but can eliminate the effects of the clock offset errors via the introduction of 2nd stage RTT round as noted in Figure 2. This method is essentially a two-stage RTT procedure where the TX-UE of the 1st phase is the Initiator UE and the TX-UE of the 2nd phase is the Responder UE. This brings about an improved distance/ranging estimate between the pair of UEs and provides robust timing measurements in the presence of uncompensated clock offsets, which is an issue for the one-way SL-RTT method. This ranging method can also operate based asymmetric reply times, depending on the SL traffic.

The signal propagation time (Tpropagation-delay) can be used to derive the range between the Initiator-UE and Responder-UE as a function of the TX-Device RTT Round Durations (A and D) and RX-Device Reply Time Durations (B and C) and is represented as follows:

Tpropogation-delay=A×D-B×C(A+B+C+D),

The independent clocks at the Initiator and Responder UEs can be assumed to run at a factor of δ1 and δ2 , respectively with respect to an ideal/master clock, which is assumed to be constant over the course of the two-way SL-RTT exchange and has been taken into account. The two-way RTT procedure may incur additional latency when compared to one-way SL-RTT method.

Figure 2: Two-way SL-RTT procedure

Observation 4: Two-way RTT can compensate for UE clock offsets at both Initiator and Responder UEs.

Proposal 9: Support the following two types of SL-RTT type solutions for SL positioning:

- One-way SL-RTT

- Two-way SL-RTT

In addition, it also feasible to support Multi-node RTT, where multiple anchor UE or non-anchor UEs may perform RTT with the target-UE.

Proposal 10: Support the Multi-node SL-RTT type solution where multiple anchor nodes/UEs may perform SL-RTT with a target-UE.

## SL Angular-based methods

In addition to timing-based methods, angular-based methods can also provide a good absolute and relative location performance. In general, the use of multi-antenna transceivers are required to support the angular-based methods which are readily supported in SL systems. Depending on the antenna array systems deployed at the UE/RSU, the following angles may be measured:

- 1D Antenna Arrays – Better suited to measure the azimuth (θ) angle, assuming the target-UE moves along the same plane. e.g., ULAs

- 2D Antenna Arrays – Better suited to measure azimuth (θ) and elevation (φ) angles.

Multiple anchor node nodes can be involved to perform triangulation of the target-UE, whereas relative ranges and directions can also be determined with a single anchor node.

### SL-AoA

SL-Angle-of-Arrival rather requires that the SL transmitting node(s) at least be equipped with a single antenna, whereas the responder/target-UE should have at least a multi-antenna implementation. The relative AoA and range with respect to each of the SL transmitting node(s) can be determined by the target-UE by measuring the AoA phase difference at each antenna element as seen in Figure 4. In this the case, SL transmitting node(s) can comprise of anchor or non-anchor nodes.

Observation 5: In the case of SL-AoA, anchor or non-anchor node(s) may be employed to determine the relative range and relative AoA with respect to the target-UE.

In the case of SL Positioning Mode 1b (network configured UE-assisted methods) and SL Positioning Mode 2b (network configured UE-based methods), the SL AoA may also be computed at the anchor nodes where thereafter the positioning calculation entity collects the relative AoAs from each of the anchor nodes to perform triangulation to determine the absolute location estimate of the target-UE.

Figure 4:  SL-AoA positioning

Based on the RAN1#110 agreement, SL-AoA was agreed to be introduced for SL Positioning. Further to this agreement, the measurements for SL-AoA may be defined and agreed upon. Similar to the Uu UL-AoA measurement definition, the SL-AoA may be defined in terms of the estimated azimuth and vertical angle an anchor or target UE for both LCS and GCS coordinate systems.

Proposal 11: RAN1 to define SL-AoA measurements in terms of azimuth and vertical (elevation) angles for both Local coordinate systems (LCS) and Global coordinate systems (GCS).

Additional assistance information by SL-AoA measurement UE, e.g., target-UE or anchor UE can be used to improve the accuracy of SL-AoA measurements. This information may include expected AoA ranges based on a priori location information as well as per path indications to support SL-AoA measurements in multipath and NLOS scenarios.

Proposal 12:  Support angular-based SL positioning methods, SL-AoA and SL-AoD, considering antenna calibrated entities/nodes and fixed anchor nodes, e.g., gNBs or RSUs.

## SL Fingerprinting methods

Fingerprinting is a well-known low complexity positioning method, which exploits the received signal strength measurements to determine a UE’s location. Although, not as robust and accurate as the timing-based and angular-based methods, e.g., in the presence of the multipath, this method has the potential to be a low latency and low complexity positioning for SL positioning in cases where course accuracy of the target-UE is desired. In addition, SL offers additional granularity in terms of geographically defined regions such as zones (over and above cell IDs), which may be used to assist in mapping the SL RSSI measurements, e.g., using SL measurements to location of the target-UE. This method may also be used in conjunction with other RAT-dependent/RAT-independent methods in a hybrid fashion to improve the overall UE’s absolute/relative location estimate. This is in some sense, similar to the current E-CID technique over the Uu interface but exploits additional features in SL to determine the location of the UE.

Observation 6: SL fingerprinting, E-CID type methods can satisfy low latency and coarse SL positioning requirements.

The target-UE may use existing SL-RS RSRP measurements, e.g. PSBCH-RSRP, PSSCH-RSRP as a basis to determine the absolute location as well as the Tx-Rx distance between an initiator UE (Tx) and responder UE (Rx). The target-UE shall be localized using further granularity shall be added using the Zone ID, which can supplement the cell of origin technique employed in the Uu interface. The SL-RS measurements shall be used to estimate the TX-RX distance between the reference nodes, i.e. UE-1 and UE-2 and thus derive the absolute (for anchor nodes) and relative (non-anchor) location with respect to each of these UEs. In addition, the responder UEs may be configured to report such measurements, whenever it is available.

Proposal 13: RAN1 to support the SL fingerprinting method to satisfy coarse positioning requirements and to be used as input for other SL positioning methods, e.g., SL-TDoA, to enhance UE position estimation.

## Hybrid Positioning

It also well established that no single positioning technique can satisfy all the target positioning requirements as documented in our companion contribution [3]. The SL Positioning framework should be able to support and leverage hybrid positioning in order extract the best gains from different interfaces, positioning techniques as well as technologies. Accordingly, we define hybrid positioning based on two categories:

- Hybrid Positioning Model A (using hybrid interfaces) – This hybrid positioning model leverages measurements from both Uu and SL interface to aid and enhance either the Uu positioning estimate (based on the LPP framework) or SL positioning estimate.

- Hybrid Positioning Model B (using hybrid positioning technologies) - This hybrid positioning model exploits the position computation using a combination of RAT-dependent and RAT-independent methods (e.g., GNSS, WLAN, Bluetooth, IMU, etc.). Since RAT-independent methods have already been supported within Uu positioning framework, it can be also straight-forward to enable SL positioning to leverage RAT-independent methods without significant additional work.

Proposal 14: RAN1 to support hybrid positioning under the following models:

- Hybrid Positioning Model A - Hybrid positioning using hybrid interfaces including the support of Uu and SL measurements for SL position calculation.

- Hybrid Positioning Model B - Hybrid positioning using hybrid technologies including the support of RAT-dependent and RAT-independent methods for SL position calculation.

# SL Positioning Resource Configuration and Allocation

## SL PRS configuration/activation/deactivation/triggering

RAN1#110 Agreement[4] With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed.

The SL PRS transmission(s) may be supported to be dynamically and semi-statically configured. The associated triggering and configuration procedures need therefore to be considered, depending on the type of coverage scenarios. In the case of SL communication, the trigger for layer-1 procedures originated from the MAC-layer (higher layers). Similarly, the triggering for SL positioning/ranging may originate from higher layer signalling. Therefore Option 2, most closely aligns with existing SL functionality by considering both higher-layer and lower-layer signalling.

Proposal 15: RAN1 to support Option 2 of configuration/activation/deactivation/triggering of SL-PRS, i.e., higher and lower layer involvement.

Proposal 16: RAN1 to consider the resource allocation triggering mechanism of SL PRS transmission(s) in terms of which entity, e.g., anchor-UE, target-UE, gNB, LMF and the type of trigger, e.g., higher-layer SL positioning/ranging service request. RAN2 input may be required.

The type of trigger may indicate whether the SL-PRS transmission request is a periodic/semi-persistent/aperiodic transmission, which will have to align with the SL resources to be reserved in both Mode 1 and Mode 2 SL transmissions.  In addition, SL communications have been reliant on source and destination-IDs to identify transmissions in a unicast, groupcast and broadcast sessions.

Proposal 17: RAN1 to consider at least the following configurability options for SL PRS:

- Support of periodic/semi-persistent/aperiodic transmission of SL PRS

- Unambiguous identification of initiator UEs and responder UEs, e.g., defining new IDs or re-using existing IDs

## Power control and Interference

The following agreement [2] was made with respect to SL positioning power control and interference aspects:

RAN1#109-e Agreement Study power control mechanisms for SL-PRS transmission, including whether it is necessary.

In order to improve the Uu PRS detectability/hearability of further out base stations with respect to the serving cell, muting of the Uu PRS was specified, which also served to mitigate inter-cell interference. Furthermore, additional power control measures were also enabled to further improve detectability/hearabality of the PRS, e.g., power boosting. Due to the short range of SL, interference amongst nodes transmitting SL PRS may also degrade positioning performance. The current SL power control methods rely on open loop methods, to manage the transmit power and in the context of SL positioning further studies are also required in terms of controlling the interference between anchor nodes, which are in close proximity.

Therefore, RAN1 is also encouraged to discuss power control and interference mitigation methods to improve overall SL PRS detectability.

Proposal 18: RAN1 to further study SL power control (e.g., based on open-loop methods) and interference mitigation mechanisms (e.g., muting) to improve SL PRS detectability/hearability between anchor nodes and the target-UE.

## Resource Pool Structure

RAN1#109-e Agreement With regards to the SL Positioning resource allocation, study further the following 2 options for SL Positioning resource (pre-)configuration: - Option 1: Dedicated resource pool for SL-PRS - Include in the study at least the following aspects: - which slots can be used, SL frame structure, SL positioning slot structure, multiplexing of SL-PRS with control information (if included in the same slot) - positioning measurement report - whether a dedicated frequency allocation (e.g., layer/BWP) is needed for SL PRS - resource allocation procedure(s) of SL-PRS - This option may or may not include control information (i.e., configuration/activation/deactivation/triggering of SL-PRS) for the purpose of SL positioning operation - Option 2: Shared resource pool with sidelink communication. - Include in the study at least the following aspects: - co-existence between SL communication and SL positioning, backward compatibility - Multiplexing considerations of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH) and any modifications in the SL-slot structure RAN1#110 Agreement With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging: - Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS - Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

The above options were agreed in [2] and [4] for further study with respect to the resource pool structure. In Rel-16 NR sidelink, multiple resource pools are configured with a SL BWP and only one SL BWP is configured in a sidelink carrier which is common for all sidelink devices. The DL PRS configuration consists of positioning frequency layer since the DL PRS is not limited to within a DL BWP configuration so that the DL PRS transmission can be measured over a wide enough bandwidth when compared to the BWP configuration. The resource pool design may be restrictive since the SL PRS is limited according to the resource pool size and is not attractive for forward compatibility in the case aggregating different SL PRS resources.

Firstly, RAN1 need to study the possibility of multiplexing between PRS and existing physical channels in a frequency resource considering that such multiplexing may degrade positioning performance due to interference. Also, PRS can span across a longer bandwidth compared to data and for the same UE transmitting data and SL PRS, data REs can be punctured to allow PRS transmission. However, when PRS occupies other UE’s data resource then puncturing those data REs to transmit PRS becomes difficult. In such shared resource pool, the PRS bandwidth is limited according to the data resource pool size which is not optimum to achieve higher positioning accuracy. Considering multiple comb patterns and frequency shifts needed for PRS such data REs puncturing negates the advantage of resource efficient SL PRS and data multiplexing in a shared resource.  However, based on the RAN1#110 agreement, we would be supportive Alt.2 as a reasonable comprise and enable flexibility.

Proposal 19: Support Alt.2 of SL positioning resource allocation where dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

Further study is also required to analyse the pros and cons of SL PRS resource configuration granularity of a SL BWP or resource pool or SL positioning frequency layers (PFL). A SL PRS resource configuration that limits the overall positioning bandwidth is undesirable for meeting the stringent target positioning requirements.

Proposal 20: RAN1 to further study the SL PRS resource configuration considering the relationship between SL Positioning frequency layers or SL BWPs with the agreed SL Positioning dedicated resource pools.

In shared resource pool with sidelink communication, the TDM multiplexing of SL-PRS and existing physical channel can be considered allowing SCI and SL-PRS TDM multiplexing to reuse the existing slot structure by mapping SL-PRS onto legacy PSSCH region.  For the sidelink slot with PSFCH region, whether the PSFCH region can be indicated/configured to transmit 2 symbols SL-PRS can be further studied.

Proposal 21: In a shared resource pool, TDM multiplexing of PSCCH and SL PRS may be supported.

Proposal 22: Further study whether SL-PRS is transmitted on legacy PSSCH region and/or PSFCH region.

In a dedicated resource pool, if data transmission is not supported, this may generate dummy transmissions with blank symbols along with a SL-PRS transmission within a slot. For example, except one symbol AGC, one symbol Tx-Rx turnaround time (Guard symbol) and 2 symbols potential PSCCH transmission (if any), it needs 8, 6 and 4 symbols of dummy transmissions (blank symbols) corresponding to the transmission of2,4 and 6 SL-PRS symbols respectively within a 14 symbols slot. It obviously wastes large amounts of resources and may be resource inefficient. A solution to this is to adopt a mini-slot structure considering various symbol number of SL-PRS e.g., for 2, 4 and, 6 symbols, which can be studied for a dedicated SL Positioning resource pool. In this way, the potential SL-PRS transmission occasions within one slot is increased.

Proposal 23: Mini-slot of various symbol length of SL-PRS e.g., 2, 4, and 6 can be further studied for a dedicated resource pool.

In order to support Hybrid positioning model A (as outlined above) using both Uu and SL interfaces, it also necessary to discuss the relationship of the Uu PFL with SL PRS in order to efficiently process and measure both Uu and SL PRS. Currently, the Uu PRS can be measure with or without (specified in Rel-17) a measurement gap configuration.

Proposal 24: RAN1 to discuss the relationship between Uu PRS and SL PRS configuration and associated measurements for hybrid positioning model A (using both Uu and SL interfaces). FFS whether such measurements are performed in a joint manner or separately measured.

## Resource Allocation

RAN1#109-e Agreement [2] With regards to the SL-PRS resource allocation, study the following two schemes: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS - Applicable regardless of the network coverage - FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting UEs (e.g. IUC-like solutions). - Note: Other Schemes are not precluded to be studied - FFS how to handle resource allocation of SL-Positioning measurement report RAN1#110 Agreement [4] Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g., gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

It should be possible to extend the current Mode 1 and Mode 2 resource allocation schemes in SL to configure SL PRS in all coverage scenarios, i.e., in-coverage, partial coverage and out-of-coverage. Mode 1 offers a centralized approach to configure SL PRS for in-coverage and partial coverage scenarios, while Mode 2 enables SL PRS configuration especially in out-of-coverage scenarios.

### Mode 1

In this Mode, the gNB may configure the SL PRS resources in a centralised manner depending on the configured positioning technique. For techniques requiring relative position estimation amongst multiple pair of UEs (e.g., between an anchor node UE and target-UE), gNB coordination should be sufficient. The gNB may also coordinate SL PRS for multiple anchor node UEs when performing triangulation using timing-based methods such as SL-TDoA or angular-based position estimation, for the purposes of absolute location estimation.

In addition, depending on the type of location request the gNB may configure one of the following:

- Dynamic grants for one shot SL positioning estimates (based on one shot SL PRS transmissions) (e.g., in the case of immediately triggered requests)

- Configured grant schemes in the case that periodic location requests are required, e.g., Type 1 and 2 CG grants (based on periodic SL PRS transmissions).

Proposal 25: RAN1 to use the existing Mode 1 resource allocation mechanisms bas baseline including dynamic grants and configured grants for the transmission of SL-PRS and SL positioning messages (e.g., assistance data, measurement reports).

### Mode 2

Mode 2 is beneficial for scenarios not requiring network infrastructure, where the Initiator UE may autonomously select and reserve resource for the purposes of SL positioning. This relies on a pre-configuration of resource pools within a UE. Given the distributed nature of this scheme, Mode 2 can sure stability of SL PRS for a given period by selecting and reserving resources for SL PRS transmission. However, this may depend on the type of SL PRS configuration (e.g., bandwidth, number of symbols, etc.).

The selection triggers may be different in the case of SL PRS when compared to the existing schemes for SL data (TB) transmission, where the selection is dependent on the generation of a new TB or a pre-emption mechanism depending on the SL traffic priority. Similarly, in the case SL Positioning, the QoS may also have an impact on the resource selection mechanism in Mode 2. The sensing window should also be able to accommodate a SL PRS transmission based on the accuracy requirements of the SL positioning service request.

Proposal 26:   RAN1 to use the existing Mode 2 sensing, (re-) selection and reservation as baseline for the transmission of SL-PRS and SL positioning messages (e.g., assistance data, measurement reports).

### Resource Allocation for a SL Positioning Group

A SL positioning group may comprise of a number of UEs including Anchor UEs, Target-UE, Assistance UE, etc. and therefore tight coordination may be required amongst UEs depending on the positioning techniques, e.g., SL-TDoA, etc. Therefore, coordination resource allocation schemes amongst member UEs of a SL positioning group should be further investigated over and above the existing SL positioning resource schemes. This may include recommending a set of resources (inter-UE coordination), scheduling/coordination by one member UE on behalf of other member group UEs, forwarding of resource grants one member UE to other member UEs.

Proposal 27:  RAN1 to support resource allocation/coordination schemes for a SL Positioning Group, e.g., resource configuration sharing by a member UE, inter-UE coordination by a member UEs.

# SL Measurement, Processing and Reporting

RAN1#109-e Agreement [2] With regards to the Sidelink Positioning measurement report, - Study the contents of the measurement report (e.g., time stamp(s), quality metric(s), ID(s), angular/timing/power measurements, etc) - Study the time domain behavior of the measurement report (e.g. one-shot, triggered, aperiodic, semi-persistent, periodic) - FFS whether the Sidelink Positioning measurement can be a high-layer report and/or a lower layer report. RAN1#110 Agreement [4] For the content of the sidelink positioning measurement report, potential elements may include at least the following: - One or more sidelink positioning measurement(s) - Timestamp(s) associated with a sidelink positioning measurement - Quality metric(s) associated with a sidelink positioning measurement - Identification Information for a sidelink positioning measurement - FFS any detail for the above

SL positioning measurements can be performed by the target-UE in a similar fashion to the current Uu positioning framework. The target-UE is usually configured separately to report certain measurements based on the positioning method and can also be extended to the SL positioning framework. In the context of relative positioning, it can be expected that the SL positioning measurements may be performed either by the Initiating or Responding UEs, depending on the type of configured positioning methods, e.g., the two-way SL-RTT requires that both the Initiator and Responder UE perform the timestamp measurements for enhanced accuracy. Figure 5 is a general conceptual view of the measurement, processing and reporting flow for SL positioning.

Observation 7: In SL positioning, both the initiator and responding UEs can be supported to perform SL positioning measurements within the same session.

Figure 5: Measurement, processing and reporting flow for SL positioning

A portion of the time has to also be dedicated for processing the SL PRS measurements depending on different UE capability types. The measurement UE may indicate to the configuration entity, the duration of SL PRS symbols N in units of ms that it may process every T ms assuming a maximum SL PRS bandwidth. The type of capabilities can affect the amount of SL PRS resources a UE can process in a given time as well as the latency of processing the SL PRS. The configuration entity may then configure a set of SL PRS resources based on the SL PRS processing capabilities of the UE.

Proposal 28: RAN1 to consider different SL PRS processing capabilities depending on the UE-type.

Another key issue is the prioritization of SL positioning measurements depdninding on the transmitting entity/node, e.g., anchor-UEs, initiator-UE, responder -UE or others UEs , which are deemed mobile or static. It is assumed that the transmitted SL-PRS originates from nodes known to be generally mobile (depdening on the class of mobility, e.g., low speeds/high speeds), even though there can be fixed SL transmitters such as roadside units (RSU) or UEs that are temporarily stationary. Given the distributed nature of SL, the may be a host of candidate nodes available for selection by a SL measurement UE. There is a therefore a need to prioritize SL-PRS measurements that originate from a SL-PRS transmitter whose position is well known with good accuracy/confidence interval.

Proposal 29: RAN1 to further study the prioritization of SL positioning measurements depending on the known position and accuracy of the SL transmitter.

The SL Positioning framework should also be capable of supporting different types of reporting of both SL positioning measurements and absolute/relative location estimate. Table 2 shows the various reporting types depending on the reporting configuration.

Table 2: Types of SL Positioning Reporting

| SL Positioning Reporting Types | Overview |
| --- | --- |
| One-shot | - The UE/device performing measurements may report to the positioning calculation entity in a one-shot manner to support immediate reporting. - The UE (positioning calculation) entity may report the absolute/relative location estimate to the requesting node/network entity, e.g., UE, LMF, etc. |
| Triggered | - The UE/device performing measurements may report to the positioning calculation entity based on defined event-based criteria, e.g., based on a location change/cell change. - The UE/device performing measurements may report to the positioning calculation entity upon trigger of a configured event. |
| Periodic | - The UE/device may provide periodic positioning measurement or location estimate reports with the same time interval between two consecutive reports. |

Proposal 30: Support different SL Positioning reporting types including one-shot, triggered (event-based) and periodic reports.

It is also important to assess the quality of the SL PRS measurements, e.g., in the form of confidence intervals relating to a specific measurement, predefined margins of error, or other range reliability metrics. This can be applied to timing-based, e.g., SL-TDoA -RTD quality, etc, angle-based and other SL positioning measurements. This serves as additional beneficial assistance information for the positioning calculation entity. The quality metrics can be finalized upon the definition of the different measurements applicable to each of the supported positioning techniques.

Proposal 31: Measurement quality metrics should be considered based on the required measurements for each supported positioning techniques, e.g., SL-TDoA, SL-AoA, SL-RTT.

# SL Positioning Reference Units and Assistance UEs

The positioning reference unit (PRU) with a known fixed location has been extensively discussed Rel-17 Uu positioning to assist in improving the accuracy of the location estimate, especially for timing-based positioning methods. However, due to lack of time and consensus among WGs, this feature was not further specified.

In the context of SL and considering the challenges in achieving stable location accuracy performance for multiple mobile (mobile) nodes, the SL PRU can be considered to help mitigate timing clock/offsets errors using reference measurements from a fixed known location, e.g., an RSU may act as a PRU. This achieved via compensating the pseudo-range errors arising from the Rx/Tx timing delays from the transmitting and receiving UEs. UEs may be more prone to clock drifts at the transmitter and receiver end depending on the hardware implementations.

Observation 8: Transmitter and receiving UEs may be susceptible to Tx/Rx errors impacting positioning performance due to uncompensated clocks based on different hardware implementations.

The positioning calculation entity (e.g., UE or network node) can exploit the PRU’s known precise location to compensate for timing-based errors such as delays embedded within the received measurements by applying (double) differential computations. This is similar in concept to the reference stations employed by GNSS systems to compensate for pseudo-range errors by the GNSS receivers by exploiting the fact that receivers in the same general area, e.g., within a few hundred km between the receivers, experience the same atmospheric impairments.  The actual known precise location of the reference stations is therefore an important pre-requisite.

Due to the distributed nature of SL and the type of environment, many of the pair of UEs may not have a good quality link, e.g., LOS, in order to perform accurate ranging. In such cases, such scenarios may require cooperation with respect to an assistance UE, in order to facilitate the ranging process. In another example, network assistance UEs, may also facilitate a SL positioning/ranging session between an in-coverage and out-of-coverage UE.

Proposal 32: RAN1 to further discuss the benefits of PRUs and assistance UEs in SL positioning/ranging procedures.

# Conclusion

The following observations regarding potential SL positioning solutions have been noted as part of the discussion:

Observation 1: Terminology needs to be defined as to which UE starts a SL positioning/ranging session and which UE(s) is/are the intended recipient or responder of the SL positioning/ranging session.

Observation 2: RTT requires at least one anchor or non-anchor node for absolute/relative location estimation.

Observation 3: One-way RTT is susceptible to UE clock offset errors at the Initiator and Responder UE.

Observation 4: Two-way RTT can compensate for UE clock offsets at both Initiator and Responder UEs.

Observation 5: In the case of SL-AoA, anchor or non-anchor node(s) may be employed to determine the relative range and relative AoA with respect to the target-UE.

Observation 6: SL fingerprinting, E-CID type methods can satisfy low latency and coarse SL positioning requirements.

Observation 7: In SL positioning, both the initiator and responding UEs can be supported to perform SL positioning measurements within the same session.

Observation 8: Transmitter and receiving UEs may be susceptible to Tx/Rx errors impacting positioning performance due to uncompensated clocks based on different hardware implementations.

The proposals relating to the SL Positioning framework are summarized as follows:

Proposal 1: Request SA2 to adopt the terminology of the “Anchor UE” in their discussions to avoid fragmented definitions of the same concept of “Reference UE” and “Located UE”.

Proposal 2: RAN1 to consider the following additional terminology during the study:

- Initiator UE initiates a SL positioning/ranging session, e.g. anchor UE, target-UE, assistance UE

- Responder UE responds to a SL positioning/ranging session from an initiator UE, , e.g. anchor UE, target-UE, assistance UE

Proposal 3: Support the following 4 different models for SL positioning, which distinguish the configuration entity and positioning calculation entity for in-coverage, and partial and out-of-coverage scenarios:

- SL Positioning Model 1a - UE-assisted positioning (network-configured)

- SL Positioning Model 1b – UE-assisted positioning (UE-configured )

- SL Positioning Model 2a – UE-based positioning (UE-configured)

- SL Positioning Model 2b - UE-based positioning (network-configured)

Proposal 4: RAN1 to adopt ZC-based sequences with SRS as a baseline design for SL Positioning.

Proposal 5: Partial and Fully staggered comb patterns should consider the number of available symbols within a SL slot considering number of available symbols of 9 or 10. FFS whether a comb size of 12 symbols may be still supported.

Proposal 6: RAN1 to consider at least the staggered SL PRS design option. FFS the positioning performance of partially staggered designs.

Proposal 7: RAN1 to define SL RSTD measurements for supporting SL-TDOA.

Proposal 8: RAN1 to at least introduce the support for a reference anchor node/UE for SL RSTD measurements and SL relative time difference (RTD) information.

Proposal 9: Support the following two types of SL-RTT type solutions for SL positioning:

- One-way SL-RTT

- Two-way SL-RTT

Proposal 10: Support the Multi-node SL-RTT type solution where multiple anchor nodes/UEs may perform SL-RTT with a target-UE.

Proposal 11: RAN1 to define SL-AoA measurements in terms of azimuth and vertical (elevation) angles for both Local coordinate systems (LCS) and Global coordinate systems (GCS).

Proposal 12:  Support angular-based SL positioning methods, SL-AoA and SL-AoD, considering antenna calibrated entities/nodes and fixed anchor nodes, e.g., gNBs or RSUs.

Proposal 13: RAN1 to support the SL fingerprinting method to satisfy coarse positioning requirements and to be used as input for other SL positioning methods, e.g., SL-TDoA, to enhance UE position estimation.

Proposal 14: RAN1 to support hybrid positioning under the following models:

- Hybrid Positioning Model A - Hybrid positioning using hybrid interfaces including the support of Uu and SL measurements for SL position calculation.

- Hybrid Positioning Model B - Hybrid positioning using hybrid technologies including the support of RAT-dependent and RAT-independent methods for SL position calculation.

Proposal 15: RAN1 to support Option 2 of configuration/activation/deactivation/triggering of SL-PRS, i.e., higher and lower layer involvement.

Proposal 16: RAN1 to consider the resource allocation triggering mechanism of SL PRS transmission(s) in terms of which entity, e.g., anchor-UE, target-UE, gNB, LMF and the type of trigger, e.g., higher-layer SL positioning/ranging service request. RAN2 input may be required.

Proposal 17: RAN1 to consider at least the following configurability options for SL PRS:

- Support of periodic/semi-persistent/aperiodic transmission of SL PRS

- Unambiguous identification of initiator UEs and responder UEs, e.g., defining new IDs or re-using existing IDs

Proposal 18: RAN1 to further study SL power control (e.g., based on open-loop methods) and interference mitigation mechanisms (e.g., muting) to improve SL PRS detectability/hearability between anchor nodes and the target-UE.

Proposal 19: Support Alt.2 of SL positioning resource allocation where dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

Proposal 20: RAN1 to further study the SL PRS resource configuration considering the relationship between SL Positioning frequency layers or SL BWPs with the agreed SL Positioning dedicated resource pools.

Proposal 21: In a shared resource pool, TDM multiplexing of PSCCH and SL PRS may be supported.

Proposal 22: Further study whether SL-PRS is transmitted on legacy PSSCH region and/or PSFCH region.

Proposal 23: Mini-slot of various symbol length of SL-PRS e.g., 2, 4, and 6 can be further studied for a dedicated resource pool.

Proposal 24: RAN1 to discuss the relationship between Uu PRS and SL PRS configuration and associated measurements for hybrid positioning model A (using both Uu and SL interfaces). FFS whether such measurements are performed in a joint manner or separately measured.

Proposal 25: RAN1 to use the existing Mode 1 resource allocation mechanisms bas baseline including dynamic grants and configured grants for the transmission of SL-PRS and SL positioning messages (e.g., assistance data, measurement reports).

Proposal 26:   RAN1 to use the existing Mode 2 sensing, (re-) selection and reservation as baseline for the transmission of SL-PRS and SL positioning messages (e.g., assistance data, measurement reports).

Proposal 27:  RAN1 to support resource allocation/coordination schemes for a SL Positioning Group, e.g., resource configuration sharing by a member UE, inter-UE coordination by a member UEs.

Proposal 28: RAN1 to consider different SL PRS processing capabilities depending on the UE-type.

Proposal 29: RAN1 to further study the prioritization of SL positioning measurements depending on the known position and accuracy of the SL transmitter.

Proposal 30: Support different SL Positioning reporting types including one-shot, triggered (event-based) and periodic reports.

Proposal 31: Measurement quality metrics should be considered based on the required measurements for each supported positioning techniques, e.g., SL-TDoA, SL-AoA, SL-RTT.

Proposal 32: RAN1 to further discuss the benefits of PRUs and assistance UEs in SL positioning/ranging procedures.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN#94-e, Dec. 2021.

- R1-2205574, “Session notes for 9.5 (Study on expanded and improved NR positioning)”, Ad-Hoc Chair (Huawei), RAN1#109-e, May 2022.

- R2-2209351, “LS on Terminology Alignment for Ranging/Sidelink Positioning”, S2-152-E, Xiaomi, Aug. 2022.

- R1-2205574, “Session notes for 9.5 (Study on expanded and improved NR positioning)”, Ad-Hoc Chair (Huawei), RAN1#109-e, May 2022.

- 3GPP TR 38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases (Release 17), V2.0.0, September 2021.