3GPP TSG-RAN1 #110-bis-e	              R1-2209423

October 10th – 19th, 2022

Source:	NEC

Title:	Evaluation of UE-UE CLI for NR SBFD operation

Agenda Item:	9.3.1

Document for: 	Discussion

# Introduction

At RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The objectives are provided below.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss the evaluation results for SBFD operation.

# Discussion

Subband Non-overlapping Full Duplex (SBFD) operation has been agreed to be studied in 3GPP owing to its various advantages over legacy TDD frame structures which are provided below:

- SBFD has the potential to reduce UL delays observed in legacy TDD frame structures. This is because most of the current networks setup their operations for DL dominant traffic and it becomes to difficult to adapt the frame structure configuration when higher UL performance is required.

- SBFD improves cell capacity by allowing to use more UL resources in TDD band. Note that this is mainly expected from scenarios where UL traffic is significantly higher than DL traffic.

- With increased UL time resource available for repetitions, even UL coverage performance is expected to be better at cell edge for SBFD.

Note that in traditional NR TDD structure, the time domain resources are split between downlink and uplink for both gNB and UE either by semi-static TDD configuration (e.g., tdd-UL-DL-ConfigurationCommon/tdd-UL-DL-ConfigurationDedicated) and/or by dynamic TDD configuration (e.g., SFI in DCI format 2_0). The TDD configurations can be dedicatedly provided to UE, hence, gNB can potentially schedule DL and UL for different UEs at the same time achieving functionality of full duplex. However, various UE side and gNB side physical layer procedures may need to be enhanced for SBFD on account of higher interference expected.

If RAN1#109e, it has been agreed to study SBFD operation within a single TDD carrier. There are currently two approaches possible for SBFD operation within a TDD carrier (as mentioned below) which were also discussed in RAN1#109e.

- SBFD operation performed across different BWPs where each BWP contains either UL or DL sub-band

- SBFD operation performed within a single BWP where BWP contains both UL and DL subband(s)

Each of the above-mentioned options may impact other physical layer procedures, e.g., BWP switching operation. Below figure illustrates the mechanism for the two options.

Figure 1 Possible approaches for sub-band non-overlapping full duplex operation

Currently following options can be considered for SBFD within a TDD carrier:

  - Single BWP containing either UL or DL sub-band (but not both). UL and DL sub-bands are present in different BWPs for enabling SBFD

  - Single BWP containing both UL and DL sub-band(s)

To understand the benefits and drawbacks of SBFD operation, SLS evaluations are required to study the performance of SBFD operation for different network scenarios and SBFD approaches. Also, evaluation should consider different types of interference scenarios which are captured in RAN1#109e as follows:

- gNB self-interference

- inter-site and co-site gNB-gNB co-channel inter-subband CLI

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI

- inter-site and co-site gNB-gNB co-channel intra-subband CLI

- (inter-cell) UE-UE co-channel intra-subband CLI (complex)

- gNB-gNB adjacent-channel CLI

- UE-UE adjacent-channel CLI.

Out of the above, gNB self-interference, inter-subband and adjacent channel CLI modeling are still pending on RAN4. However, for initial discussions it is still worth the effort to try to evaluate which of the CLI scenarios are expected to cause more performance deterioration.

In the RAN1#109e meeting, majority of the discussion around evaluation was centered on gNB self-interference mitigation. In this document, we provide a basic model for UE-UE inter-subband CLI and try to identify its impact on DL performance of UEs due to SBFD operation. Note that these results can be revisited again after RAN4 concludes on CLI modeling.

# Evaluation Assumptions

For the modeling of inter-subband UE CLI whether analog filter can be used or not is unclear. However, to get an idea of the worst-case impact we assume that filter is not used at UE to isolate DL and UL subbands for SLS evaluations. In such case, we assume that PSD of inter-subband emissions can be approximated as following [2]:

Pf=1TskGkf21+2CPN+CPcos⁡(2πf∆f)

Here, N is number of subcarriers, CP is cyclic prefix duration Gk(f) is the power spectral density of the modulation pulse on sub-carrier k. Assuming a brickwall filter, Gk(f) becomes the sinc2 function centered on the active sub-carrier. We use the above model to determine inter-subband CLI on each DL subcarrier due to each PRB of UL transmission from a proximity UE. To consider non-orthogonality between UL and DL subband, a random frequency error is introduced (uniformly distributed between -0.1ppm and 0.1ppm). The resultant PSD from this model is shown in Figure 2. Based on this model, there is expected to be an isolation of at least 50dB for leakage power emissions to adjacent subband by using a single PRB frequency gap. So, we expect that inter-subband inter-UE CLI is not apparently a serious concern for SBFD operation. Note that in the figure, for each frequency point a random frequency error was used (for better illustration purpose), but in SLS the frequency error is randomly selected once per UE UL transmission so that all the UL PRBs of an UL transmission are associated with same frequency error. We may still need to consider impact due to other non-orthogonalities e.g. due to UL symbol end point during an ongoing DL symbol.

Figure 2 Inter-subband emission model for 1 PRB UL transmission

For the evaluation framework, we model the scenario where SBFD is performed over different BWPs in the same cell (i.e. UL subband and DL subband are present in separate BWPs of the cell). Also, we assume 2 BWPs per cell of equal PRB size (100 PRBs) where during SBFD operation one bandwidth part is allotted to UL subband and other BWP is allotted to DL subband.

# Initial Evaluation Results for UE-UE CLI

We carry out 2 set of simulations. In the first set, we try to evaluate the DL peformance of UEs when all cells of the network follow the same SBFD configuration. While, in the second set we evaluate the impact of inter-cell UE intra-subband interference by using 2 different SBFD configurations applied to the cells in the network.

Common SBFD Configuration across all cells

In this set, we use SBFD configuration as DXXXU, where all cells in the network share the same SBFD configuration. We model a frequency guard band of N number of PRBs between the UL and DL subbands (same SCS of 30kHz is modeled for DL and UL).

For SLS simulation, we evaluate PDSCH performance while changing the frequency guard band value between the UL and DL subbands. Metrics that we are considering are DL SINR and DL throughtput experienced by UEs for full buffer traffic. Simulation assumptions are listed in Table I in Appendex.

We specify SINR results from 2 scenarios- one where UEs are randomly dropped in the cell coverage and second scenario where UE are dropped near cell coverage edge (with minimum distance from TRP=200m). Four different values of frequency guard band are used: 0 PRB, 10PRB, 20 PRB, 40 PRB. The results for this test case are depicted in Figure 3. We can observe that the SINR values do not vary much even with increasing guard band values, this is because the amount of interference expected because of the modeled inter-subband PSD is insignificant as compared to the interference observed due to gNB’s DL received power values. Although, for the case where users are dropped near cell edge boundary, there is a small difference observed between the different guard band values, but again the difference is not expected to cause major impact on DL throughput.

Inter-UE inter-subband CLI is not expected to be a serious concern for DL SINR and throughput performance

Figure 3 DL SINR performance with respect to guard band size between UL and DL subband

Common SBFD Configuration across all cells

In this test set, we use different SBFD configurations across the cells of the SLS. Although all the gNBs use the SBFD configuration of DXXXU, but the central gNB’s DL and UL sub-band positions are reveresed as compared to other gNBs. This is to understand the impact of inter-cell co-subband UE interference from one UE’s UL to other UE’s DL reception. The description of the SBFD configuration is depicted in below figure.

The SINR results in this scenario are shown in Figure 4. Here, we compare this scenario against the case where all cells share the same SBFD configuration and UL/DL subband position. We can immediately observe that there is larger impact on SNR due to co-subband interference from UE UL transmissions connected to neighbor cells to UE DL reception. And as expected, the interference is more severe for the case when users are dropped near cell edge. The impact of this interference is observed in the average DL throughput which decreases from 97Mbps to 93Mbps (4% reduction) for uniformly distributed users and 65Mbps to 48Mbps (26% reduction) for the case of users dropped near cell edge. Hence, from the observations, our understanding is that it is more critical to resolve co-subband inter-cell UE-UE interference for SBFD for SBFD operation to be successfully deployed.

Higher DL performance impact is expected due to co-subband inter-cell UE-UE CLI when different cells operate with different SBFD configurations

Figure 4 DL SINR performance between same SBFD configuration used by all cells and different SBFD configuration used by the cells

# Conclusion

In this contribution, we discussed on the evaluation of NR duplex operation. Based on the discussion we made the following observations.

Observation 1	Currently following options can be considered for SBFD within a TDD carrier:

a.	Single BWP containing either UL or DL sub-band (but not both). UL and DL sub-bands are present in different BWPs for enabling SBFD

b.	Single BWP containing both UL and DL sub-band(s)

Observation 2	Inter-UE inter-subband CLI is not expected to be a serious concern for DL SINR and throughput performance

Observation 3	Higher DL performance impact is expected due to co-subband inter-cell UE-UE CLI when different cells operate with different SBFD configurations

# References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- Toon van Waterschoot et al., “Analytical expressions for the power spectral density of CP-OFDM and ZP-OFDM signals”.

- Draft_Minutes_report_RAN1#109-e_v030.

- Draft_Minutes_report_RAN1#110_v020.

# Appendix

## Simulation Assumptions

Simulation assumptions for SLS are listed in Table.1.

Table I. Simulation assumptions for SLS.

| Parameter | Value |
| --- | --- |
| Traffic | Full Buffer |
| Channel Model | Urban Macro TR 38.901 - ISD=500m - TRPs per site=3 - Hexagonal grid layout, 7 macro sites gNB height: 25m, UE height: 1.5m For inter-UE CLI modeling, path loss and shadow fading are modeled |
| UE mobility model | Stationary with random drop (80% indoor) Minimum distance from TRP=35m Number of users: 4 UEs per sector |
| Center frequency | 4 GHz |
| Bandwidth | 80MHz with 2 BWPs (each BWP=100PRBs of SCS=30kHz) |
| SCS | 30 kHz |
| LoS/NLoS | Based on LOS probability of TR 38.901 |
| Antenna configuration | gNB: 8*8 - M, N, P, Mg, Ng = 8, 8, 2, 1, 1 - (dH, dV) = (0.5, 0.5)λ - ±45°polarization - TxRU: (Mp, Np, P, Mg, Ng = 8, 8, 2, 1, 1) UE: 1 antenna - M, N, P, Mg, Ng = 1, 1, 2, 1, 1 - 0°, 90° polarization |
| Antenna gain element | gNB: 8dB UE: 0dB |
| Antenna element gain pattern | gNB: According to TR 38.901 UE: Omnidirectional |
| Transmission power | gNB: 50dBm UE (max): 23dBm |
| Noise figure | gNB: 5dB UE: 7dB |
| Thermal noise level | -174 dBm/Hz |
| Number of ports | PDSCH:16. PUSCH:16 |
| Target BLER | 10% |
| Max HARQ retransmissions | 4 |
| MIMO Scheme | MU-MIMO, Regularized ZF |
| CSI scheme | Reciprocity based; 4T SRS; 10ms SRS interval |
| Scheduler scheme | Proportional fair (PF) |

## 3GPP RAN1 Agreements

RAN1#110

Agreement Two types of RU (Resource utilization) are defined for SBFD evaluation. - Type-1 RU: DL/UL Type-1 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of all the RBs per cell including DL, UL and guard bands over observation time. - Type-2 RU (Follow TR 36.814): DL/UL Type-2 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of RBs per cell available for traffic for the given link direction over observation time - Note: In case of MU-MIMO, one RB allocated to N users within a cell is only counted as used once. - Companies are to submit results for both RU definitions - FFS: RU definition for dynamic TDD evaluations Agreement For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline: (UE clustering) - 10 users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped throughout the macro geographical area - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - FFS the values of X, Dmacro-to-cluster, Dinter-cluster, R, Y% - Optional: - 10 users per macro TRP, and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - FFS: FR2 details Agreement For Dense Urban with 2-layer for FR1, consider micro cell TRPs are deployed as following - Step 1: Randomly drop [3] micro TRP centers within one macro cell geographical area considering the minimum distance between micro TRP centers (Dinter-micro-center) and the minimum distance between macro TRP and micro TRP center (Dmacro-to-micro-center). - Step 2: Randomly deploy one micro TRP on the area circle around each micro TRP center with the radius of half of Dinter-micro-center - Step 3: Determine the horizontal angle of the micro TRPs with the planer facing to the micro TRP center. - Dinter-micro-center =[57.9 m], Dmacro-to-micro-center = [105 m] Agreement For latency related performance metric for FTP model 3 in SLS, option 1 is baseline, it is up to companies to report the latency with option 2. - Packet latency: defined as the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver. - (baseline) Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs. - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs. - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs. - (optional) Option 2: Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE - UE-Average-Latency: defined as the average packet latency for a UE - UE-Average-Latency CDF: The CDF of the UE-Average-Latency for all users. - Mean/5%/50%/95% UE-Average-Latency: The mean/5%/50%/95% value of UE-Average-Latency for all users. - Note: HARQ re-transmission should be considered for latency evaluation. - Unfinished/dropped FTP packets are not incorporated in the packet latency calculation. - Unfinished/dropped Packet Rate is defined as the number of the unfinished packets for all users divided by the total number of generated packets for all users - To be reported as part of the system level simulation results Agreement For UPT (user perceived throughput) related performance metrics for FTP model 3 in SLS, adopt the following option. - Option 1: UPT is defined as the size of an FTP packet divided by the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver [Refer to TR36.814]. - Unfinished FTP packets should be incorporated in the UPT calculation. The number of served bits (possibly zero) of an unfinished FTP packet by the end of the simulation is divided by the served time (simulation end time – file arrival time) [Refer to TR36.889]. - Consider zero bit for dropped FTP packets. - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814]. - Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814]. - Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user. - Average-UPT CDF: The CDF of the Average-UPTs for all users. - Tail-UPT CDF: The CDF of the Tail-UPTs for all users. - Median-UPT CDF: The CDF of the Median-UPTs for all users. - Mean/5%/50%/95% Average-UPT: The mean/5%/50%/95% value of Average-UPTs for all users. - Mean/5%/50%/95% Tail-UPT: The mean/5%/50%/95% value of Tail-UPTs for all users. - Mean/5%/50%/95% Median-UPT: The mean/5%/50%/95% value of Median-UPTs for all users. Agreement - Adopt the following table for traffic model of FTP model 3 for scenarios in deployment case 1 for SBFD. Indoor office (FR1&FR2); Urban Macro (FR1); Dense Urban Macro layer (FR1&FR2); Dense Urban Micro layer (FR2); Dense Urban with 2-layer (FR1). General; UL and DL are simulated simultaneously. Companies to report which option is used. - Option 1: Each UE is either assigned UL traffic or DL traffic. - assume the same number of UEs for UL and DL, FFS the total number of UEs - FFS how to handle the UE clustering case - Option 2: Each UE is assigned both UL traffic and DL traffic.. FTP packet size; Both symmetric and asymmetric packet size for UL and DL can be considered. Companies to report which option is used. - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL - Option 2: Asymmetric packet size: - 4Kbytes for DL and 1Kbyte for UL, 0.5Mbyte for DL and 0.125 Mbytes for UL. UL arrival rate for legacy TDD; - The UL arrival rate is selected to reach a target UL traffic load (RU). - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used; - The UL arrival rate#1 of Macro cell and UL arrival rate#2 of Micro cell are selected to reach target UL traffic load (RU)#1 of Macro cell and target UL traffic load (RU)#2 of Micro cell, respectively - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used. DL arrival rate for legacy TDD; - The DL arrival rate is selected to reach a target DL traffic load (RU). - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used; - The DL arrival rate#1 of Macro cell and DL arrival rate#2 of Micro cell are selected to reach target DL traffic load (RU)#1 of Macro cell and target DL traffic load (RU)#2 of Micro cell, respectively - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used. Arrival rate for SBFD; The UL and DL FTP packet arrival rate for SBFD are the same as legacy TDD.. Working assumption: - Adopt the following table for gNB-gNB channel model and gNB-UE channel model. Dense urban, Urban macro; Indoor office. Large-scale channel parameters; FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m), - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m); FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m). Fast fading parameters; FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0; FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m), ASA and ZSA statistics updated to be the same as ASD and ZSD. Agreement - For evaluation of SBFD and dynamic/flexible TDD, adopt the following evaluation assumptions. FR1; FR2-1. System bandwidth; 100MHz; 100MHz. Numerology; 14 OFDM symbol slot SCS = 30kHz; 14 OFDM symbol slot SCS = 120kHz. UE Tx power; 23dBm; 23 dBm. EIRP should not exceed 43 dBm [refer to TR 38.802 Table A.2.1-1]. Open loop power control parameters; Companies to report power control parameters. For calibration: - P0= -60 dBm, alpha = 0.6 for InH [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] - P0= -86 dBm, alpha = 0.9 for Dense Urban [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] - P0= -80 dBm, alpha = 0.8 for Urban Macro. BS receiver noise figure; 5dB [refer to TR 38.802 Table A.2.1-1]; 7dB [refer to TR 38.802 Table A.2.1-1]. UE receiver noise figure; 9 dB [refer to TR 38.802 Table A.2.1-1]; 13 dB (baseline), 10 dB (optional) [refer to TR 38.802 Table A.2.1-1]. UE receiver; MMSE-IRC as the baseline receiver. Note: Advanced receiver is not precluded. [refer to TR 38.802 Table A.2.1-1]. Feedback assumption; Realistic [refer to TR 38.802 Table A.2.1-1]. Channel estimation; Companies to report the option used. Option 1: Ideal Option 2: Realistic [refer to TR 38.802 Table A.2.1-1]. UE processing capability; UE processing capability 1 as baseline; UE processing capability 1 as baseline. Handover margin; 3 dB [refer to TR 38.828 Table 5.2.1.4-1]. UE attachment; Based on RSRP from port 0 [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip]; Based on RSRP from port 0. The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip]. Polarized antenna model; Model-1 in clause 7.3.2 in TR 38.901. DL/UL Modulation; Up to 256QAM. Transmission scheme; Companies to report transmission schemes (e.g., SU-MIMO, MU-MIMO, maximum layers for SU-MIMO/MU-MIMO, etc) For calibration, consider SU-MIMO with single layer for both DL and UL. Scheduling; PF. Overhead; Companies to report the overhead assumption. Agreement Update the previous agreement as below: For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline: (UE clustering at least for FR1) - M users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - FFS: Indoor UEs height - Y%=80% - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R - Optional: - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) [refer to TR 36.873 Table 6-1] - FFS: FR2 details Agreement For LOS probability of gNB-gNB channel, - For Macro-gNB-to-Macro-gNB case - Option 3: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. - X = 0.75 - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901. Agreement For Dense Urban Micro layer for FR2-1, - Regarding the layout, only consider the Micro TRPs of Dense Urban 2-layer network. All users communicate with micro TRPs, i.e. macro cell is only used for determining position of micro TRP. - Regarding UE distribution, all users are randomly and uniformly dropped around Micro TRP center with the radius of R (R = [28.9m]). Agreement For UE distribution of Dense Urban with 2-layer, reuse the modeling in TR38.802 as much as possible. - For FTP traffic model 3: 2/3 users randomly and uniformly dropped around micro TRP centers with radius of R (R = [28.9m]), 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area. - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) Agreement - For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case. FR1; FR2-1. Urban macro; - Option 1: [53] dBm for 100MHz - Option 2: [49] dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1]; N.A.. Dense Urban Macro layer; - Option 1: [53] dBm for 100MHz - Option 3: [44] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 1: [43] dBm for 200MHz [refer to TR 38.828 Table 5.2.2.4-1]. Dense Urban Micro layer; - Option 3: [40] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 2: [33] dBm for 200MHz. EIRP should not exceed 68 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Indoor hotspot; - Option 2: [24] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1]; - Option 1: [23] dBm for 200MHz. EIRP should not exceed 58 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Agreement For evaluation of SBFD and dynamic/flexible TDD, use BS antenna radiation pattern as following: - InH: reuse Table 10 in Report ITU-R M.2412 for both FR1&FR2-1 (Table A.2.1-7 in TR 38.802) - Urban Macro/ Dense Urban Macro layer / Dense Urban Micro layer: reuse Table 9 in Report ITU-R M.2412 for both FR1&FR2-1 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) - Companies can also consider evaluation with other realistic BS antenna radiation pattern Agreement For evaluation of SBFD and dynamic/flexible TDD, use UE antenna radiation pattern as following: - FR1: Omni-directional with 0 dBi element gain - FR2: reuse Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802) Working Assumption Parameters; Indoor office; Urban macro / Dense Urban Macro layer; Dense Urban with 2-layer. Layout; Single layer Indoor floor: (12BSs per 120m x 50m); Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around.; Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: According to previous agreement - Baseline: 3 Micro BSs per Macro BS - Optional: 6, or 9 Micro BSs per Macro BS. Inter-BS (2D) distance; 20m [TR 38.802 Table A.2.1-11]; 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1]; Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 105m Minimum Micro-center-to-micro-center distance: 57.9m. Minimum BS-UE (2D) distance; 0m [TR 38.802 Table A.2.1-11]; 35m [TR 38.802 Table A.2.1-11]; Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11]. Minimum UE-UE (2D) distance; FFS; FFS :3m [TR 38.802 Table A.2.1-11]; FFS: 3m [TR 38.802 Table A.2.1-11]. BS antenna height; 3 m [TR 38.802 Table A.2.1-1]; 25 m [TR 38.802 Table A.2.1-1]; 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1]. Conclusion - For SLS of NR duplex evolution, Rural scenario is not considered in Rel-18. - For NR duplex evolution evaluation, FR2-2 is not considered in Rel-18. Agreement For SBFD evaluation from RAN1 perspective, the evaluation assumptions that are specific for Deployment Case 2 and Case 3-1 can be discussed with low priority. Agreement RAN1 strives to agree on system level simulation parameters for SBFD deployment case 4 by RAN1#110bis-e with specific focus on different power levels and load levels between two operators in adjacent carriers. Agreement For evaluation of SBFD operation, separate-Tx/Rx antenna array can be modelled by two panel groups. - Legacy parameters M,N,P,Mg,Ng, dH,dV and dg,H,dg,V are used for description of each panel group: - M: Number of vertical antenna elements within a panel, on one polarization - N: Number of horizontal antenna elements within a panel, on one polarization - P: Number of polarizations - Mg: Number of panels in a column within a panel group. - Ng: Number of panels in a row within a panel group. - dg,H: Antenna panel spacing in horizontal direction within a panel group. - dg,V: Antenna panel spacing in vertical direction within a panel group. - Companies are to report the separation of the two panel groups. Introduce new parameters da,H,da,V as illustrated in the following figure. - da,H: Panel group spacing in the horizontal direction. Typically, da,H = 0. - da,V: Panel group spacing in the vertical direction. Agreement For evaluation and comparison between SBFD and legacy TDD, the two options for the SBFD antenna configuration agreed in RAN1#109 are further clarified as below: - SBFD antenna configuration option-1 (same as Opt 1 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-2 (same as Opt 2 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-3 (new): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD. These options are further clarified with examples in the following: - For legacy TDD with shared-Tx/Rx antenna array, assume the antenna configuration is M,N,P,Mg,Ng;Mp,Np. The total number of TxRUs is K=PMpNpMgNg, and the total number of antenna elements is L=PMNMgNg. - For SBFD antenna configuration option-1, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). One method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols is illustrated as below. Other methods are not precluded and can be reported by companies. - Method 1: - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Tx chains in TxRU group#2. - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2. - For SBFD antenna configuration option-2, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is 2L=2PMNMgNg(two times of that for legacy TDD). Two methods on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies. - Method 2-1: - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains. - In UL slots, L antenna elements on panel group#2 are connected to K Rx chains. - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains. - Method 2-2: - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains. - In UL slots, L antenna elements on panel group#1 are connected to K Rx chains. - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains. - For SBFD antenna configuration option-3, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K/2=PMpNpMgNg/2 (half of that for legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). The method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies. - Method 3-1: - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains. - In UL slots, L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains. - Method 3-2: - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1. - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#1. Working Assumption For UE-UE channel model, reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 as baseline, and other models are not precluded. UE-UE channel model Dense urban, Urban macro; Indoor hotspot. Large-scale channel parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-13 in TR38.802 FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802; FR1: - Option1 : UE-to-UE: A.2.1.2 in TR36.843 (*) - Option 2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m). Fast fading parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Optioin 2: UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA.; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA. (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802.. Agreement For evaluation of adjacent-channel coexistence between two networks for Urban Macro and Dense Urban Macro layer scenarios in RAN1, consider grid shifts between two networks of 0% and 100%. - the topologies shown below can be used for the 0% and 100% grid shift for RAN1 evaluation. Agreement For evaluation of SBFD operation, it is up to companies to report the BS antenna configurations used in their simulations. The BS antenna configurations in the following table can be considered for calibration purpose. Scenarios; FR; Legacy TDD; SBFD. BS antenna configuration for Indoor office; FR1; M,N,P,Mg,Ng;Mp,Np = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (2,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ. FR2-1; M,N,P,Mg,Ng;Mp,Np=(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ. BS antenna configuration for Urban Macro/ Dense Urban Macro layer/ Dense Urban Micro layer; FR1; M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ. FR2-1; M,N,P,Mg,Ng;Mp,Np= (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ.

RAN1#109e

Agreement For discussion purpose for evaluation, define the following deployment cases for SBFD: - Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration. - Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations. - Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration. - Deployment Case 3-1: Only 1-layer is considered - Deployment Case 3-2: 2-layer is considered - Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration. Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2 Note: SBFD subband configuration is from gNB perspective. Agreement For SBFD Deployment Case 1, at least consider the following scenarios for evaluation: - For FR1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Urban macro (use Urban macro defined in TR38.802/TR38.901 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban with 1-layer or 2-layer (use Dense Urban defined in TR38.802/TR38.901 as starting point) - FFS: Rural - For FR2-1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban micro (use Dense Urban micro defined in TR38.802/TR38.901 as starting point) - FFS: Whether FR2-2 is considered or not in Rel-18. Note: For optional scenarios, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. Agreement: Regarding gNB self-interference modelling for system level simulation purpose, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n, can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - FFS: Model for link level simulations and relevant questions to ask RAN4 - FFS: details of gNB self-interference modelling using RSI in SLS. As one example based on per-RB-RSI, the gNB self-interference on a single receiver chain at UL RB n can be modelled as - ISIn=m≠nm∈DL subbandISIm,n, wherein, - dBmISIm,n=Ptxm-dBαSIm,n - ISIm,nis the gNB self-interference on a single receiver chain at UL RB n caused by DL transmission on DL RB m. - m is the DL RB index in DL subbands. - Ptxm is gNB’s DL transmission power across all transmit chains at RB m (in dBm). - αSIm,n is the per-RB-RSI. - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA. - The following should be asked to RAN4: - What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided? - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity. - Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate estimates for spatial isolation, subband frequency isolation, beamform nulling/isolation, and digital cancellation, etc., as below? - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-digtial)+… - αSI-spatial denotes the spatial isolation. - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n. - αSI-beam denotes the beamform nulling or beam isolation. - αSI-digtial denotes the digital cancellation capability. - Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored? - The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc. - Does RSI have any dependency with the following factors or any other factors? What are the dependencies? - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc. - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc. - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc. - Note: RAN1’s consideration on the frequency locations and sizes of SBFD DL subband and SBFD UL subband assumed in SBFD operation can be provided to RAN4. Agreement For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc) - The following questions should be asked to RAN4: - Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following two cases: - inter-site gNB-gNB co-channel inter-subband CLI - co-site inter-sector co-channel inter-subband CLI - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver? FFS: Usage of the above model provided by RAN4 in the evaluation Agreement At least the following metrics are considered for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) using SLS - Latency (CDF or {mean, 5%, 50%, 95%}) using SLS - Resource utilization using SLS - DL/UL received SINR using SLS - Coverage metric - FFS: MPL to achieve a certain bit rate in UL and DL - FFS: definitions of the above metrics - FFS: other metrics Agreement Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic. - FFS: other traffic models, e.g., XR, VoIP - FFS: Packet size, traffic load, ratio of DL/UL traffic - FFS: additionally consider different amount of input traffic at least for adjacent-channel/co-channel coexistence studies Agreement For discussion for duplex evolution study (all agenda items), consider the following as RAN1’s common understanding: - Co-channel interference: The interference is from the aggressor to the victim in the same carrier. - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on a set of contiguous RBs in a carrier to reception of the victim on the same set of contiguous RBs in the same carrier. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in a first set of contiguous RBs in a carrier to reception of the victim in a second set of contiguous RBs in the same carrier, where the two contiguous RB sets are non-overlapping in frequency. - Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers. Note 1: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’. Agreement For discussion for duplex evolution study (all agenda items), consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation: - gNB self-interference (SI): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency. - gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. - UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. - (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier. - (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier. - (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier. - (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency. - gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier. - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI. - UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier. Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs. Note: This does not imply we need to consider all the above interference types in evaluation for SBFD. Agreement Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc) The following questions should be asked to RAN4: - Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following cases: - the two gNBs are from the same sector of the same site in adjacent carriers, i.e., co-site co-sector gNB-gNB adjacent-channel CLI - the two gNBs are from different sectors of the same site in adjacent carriers, i.e., co-site inter-sector gNB-gNB adjacent-channel CLI - the two gNBs are from different sites in adjacent carriers, i.e., inter-site gNB-gNB adjacent-channel CLI - Whether it is feasible to define a similar interference ratio as BS-BS ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above? - For example, whether it is feasible to define gNB-gNB-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor gNB on DL frequency unit m to the interference received by the victim gNB on UL frequency unit n? If it is feasible, then what is the value range of the gNB-gNB-adjacent-channel-per-RB/subband interference ratio for each frequency range? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver? - Whether it is feasible to define a similar interference ratio as UE-UE ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above? - For example, whether it is feasible to define UE-UE-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor UE on UL frequency unit n to the interference received by the victim UE on DL frequency unit m? If it is feasible, then what is the value range of the UE-UE-adjacent-channel-per-RB/subband interference ratio for each frequency range? FFS: How to make use of the interference model in RAN1 Agreement For SBFD evaluation, consider the following for SBFD subband configurations: - SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. - SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth. - Use the following parameters for description of SBFD subband configuration in evaluation assumptions: - ND: the number of RBs in one DL subband - NU: the number of RBs in one UL subband - NG: the number of RBs in one guard band between one UL subband and one DL subband Agreement For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives: - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison. Agreement For gNB-gNB co-channel/adjacent-channel channel model and UE-UE co-channel/adjacent-channel channel model in RAN1 SLS, - Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modelled in their simulation. - Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modelled. - FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model. - FFS: How to set aligned channel model amongst companies for SLS calibration (if needed). Agreement For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification - Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread Agreement For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1. Agreement For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption. - Companies can report the separation of the Tx panel and Rx panel assumed in their simulation. - Companies can report how the antenna elements are used for transmission or reception in a slot if BS does not perform simultaneous downlink transmission and uplink reception. Agreement For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption. Agreement For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used. - Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. - Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. - Companies report which option is assumed in their simulation. Agreement For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective: - FR1: Urban Macro - FR2-1: Dense Urban Macro layer - FFS: UE outdoor/indoor proportion, clustering, etc - FFS: the grid shift between two networks, e.g., 0%, 100% - FFS: Indoor hotspot, Dense Urban Micro layer