3GPP TSG RAN WG1 #110bis-e			R1-2209437

e-Meeting, October 10th – 19th, 2022

Agenda Item:	8.2

Source: 	LG Electronics

Title: 	Discussion on how to indicate GSCN ranges where CD-SSB does not exist using NCD-SSB in FR2-2

Document for:	Discussion and decision

- Discussion

In RAN1#110 meeting, the following text proposal was agreed [1] considering the fact that the number of GSCNs in FR2-2 is about three times greater than that of GSCNs in FR2-1. By introducing a step size 3 for all SCS values in FR2-2, the number of GSCN entries that can be indicated by NCD-SSB is increased to sufficiently cover the whole FR2-2.

13 UE procedure for monitoring Type0-PDCCH CSS sets <Unchanged parts are omitted> If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNReference+NGSCNSize⋅NGSCNOffset. NGSCNReference is the GSCN of the first SS/PBCH block, NGSCNSize=1 in FR1 and FR2-1, NGSCNSize= 3 in FR2-2, and NGSCNOffset is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If the UE detects the second SS/PBCH block and the second SS/PBCH block does not provide a CORESET for Type0-PDCCH CSS set, as described in clause 4.1, the UE may ignore the information related to GSCN of SS/PBCH block locations for performing cell search. <Unchanged parts are omitted>

As captured from TS 38.213 Section 13 below, it is noted that if a UE detects NCD-SSB in FR2 and determines that a CORESET for Type0-PDCCH CSS set is not present by kSSB=15, the UE determines that there is no CD-SSB within a GSCN range NGSCNReference-NGSCNStart, NGSCNReference+NGSCNEnd where NGSCNStart and NGSCNEnd are up to 16, which seems to be too narrow for FR2-2 case.

13 UE procedure for monitoring Type0-PDCCH CSS sets <Unchanged parts are omitted> If a UE detects a SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for kSSB=31 for FR1 or for kSSB=15 for FR2, the UE determines that there is no SS/PBCH block having an associated Type0-PDCCH CSS set within a GSCN range NGSCNReference-NGSCNStart, NGSCNReference+NGSCNEnd. NGSCNStartand NGSCNEnd are respectively determined by controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1. If the GSCN range is NGSCNReference, NGSCNReference, the UE determines that there is no information for a second SS/PBCH block with a CORESET for an associated Type0-PDCCH CSS set on the detected SS/PBCH block.

Therefore, with the same motivation for CD-SSB frequency indication using NCD-SSB, it can be considered to introduce a step size to make it possible to indicate wider GSCN ranges where CD-SSB does not exist using NCD-SSB, as follows and also proposed in [2].

13 UE procedure for monitoring Type0-PDCCH CSS sets <Unchanged parts are omitted> If a UE detects a SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for kSSB=31 for FR1 or for kSSB=15 for FR2, the UE determines that there is no SS/PBCH block having an associated Type0-PDCCH CSS set within a GSCN range NGSCNReference-NGSCNSize⋅NGSCNStart, NGSCNReference+NGSCNSize⋅NGSCNEnd where NGSCNSize=1 in FR1 and FR2-1, NGSCNSize= 3 in FR2-2. NGSCNStartand NGSCNEnd are respectively determined by controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1. If the GSCN range is NGSCNReference, NGSCNReference, the UE determines that there is no information for a second SS/PBCH block with a CORESET for an associated Type0-PDCCH CSS set on the detected SS/PBCH block.

- Conclusion

Proposal #1: Adopt the following text proposal in TS 38.213.

13 UE procedure for monitoring Type0-PDCCH CSS sets <Unchanged parts are omitted> If a UE detects a SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for kSSB=31 for FR1 or for kSSB=15 for FR2, the UE determines that there is no SS/PBCH block having an associated Type0-PDCCH CSS set within a GSCN range NGSCNReference-NGSCNSize⋅NGSCNStart, NGSCNReference+NGSCNSize⋅NGSCNEnd where NGSCNSize=1 in FR1 and FR2-1, NGSCNSize= 3 in FR2-2. NGSCNStartand NGSCNEnd are respectively determined by controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1. If the GSCN range is NGSCNReference, NGSCNReference, the UE determines that there is no information for a second SS/PBCH block with a CORESET for an associated Type0-PDCCH CSS set on the detected SS/PBCH block.

- Reference

R1-2208241, Correction on CD-SSB frequency indication using NCD-SSB in TS38.213, Moderator (Intel Corporation), Samsung, ZTE, Sanechips

R1-2209436, Draft CR for indicating GSCN ranges where CD-SSB does not exist using NCD-SSB in FR2-2, LG Electronics