3GPP TSG RAN WG1 #110bis-e	        		   R1-2209441

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for type-1 HARQ-ACK codebook when time domain bundling is configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For type-1 HARQ-ACK CB pseudo code when time domain bundling is configured, - To follow the interpretation that “a PDSCH associated with occasion m” implies PDSCH(s) of which the corresponding HARQ-ACK information maps to occasion m - To clarify binary AND operation when some of scheduled PDSCHs are collided with semi-static UL symbol(s) |  |  |  |  |  |  |  |  |
| Summary of change: |  | For type-1 HARQ-ACK CB pseudo code when time domain bundling is configured, - “a PDSCH associated with occasion m” is changed to “more than one PDSCH associated with occasion m” and if statement to check whether or not the PDSCH is associated with the last SLIV is removed. - Scheduled PDSCHs that are collided with semi-static UL symbol(s) are assumed as correctly received for binary AND operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behaviour for type-1 HARQ-ACK codebook generation when time domain bundling is configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

*** Unchanged text is omitted ***

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cells including, when applicable, cells in the set S0 and the set S1

Set j=0- HARQ-ACK information bit index

Set NcellsDL to the number of serving cells configured by higher layers for the UE

while c<NcellsDL

Set m=0 – index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update

while m<Mc

if enableTimeDomainHARQ-Bundling is provided for serving cell c and more than one PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions scheduled by the DCI format on serving cell c, by assuming ACK for first transport blocks in PDSCH receptions that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated;

j=j+1;

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions scheduled by the DCI format on serving cell c, by assuming ACK for second transport blocks in PDSCH receptions that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated;

ojACK=

j=j+1

ojACK=

j=j+1;

elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs scheduled by the DCI format of serving cell c, by assuming ACK for all transport blocks in PDSCHs that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated

if the UE receives one transport block, the UE assumes ACK for the second transport block;

ojACK

j=j+1;

else

ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs scheduled by the DCI format of serving cell c, by assuming ACK for all transport blocks in PDSCHs that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated

ojACK

j=j+1;

end if