3GPP TSG RAN WG1 #110b-e		                                    		   R1-2209491

e-Meeting, Oct. 10th – 19th, 2022

Agenda Item:	9.5.3

Source:	InterDigital, Inc.

Title:	Discussions on positioning for RedCap UEs

Document for:	Discussion and Decision

# Introduction

In RAN1-110e, the following agreements were made [1]. The agreement detailing simulation assumptions is omitted for the sake of brevity.

Agreement For the purpose of the Rel-18 study - The target accuracy requirements for RedCap UEs for commercial use cases are defined as follows: - Indoor and outdoor - Horizontal position accuracy (< 3 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - The target accuracy requirements for RedCap UEs for IIoT use cases are defined as follows: - Horizontal position accuracy (<1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - Note: the requirements may not be met in all scenarios and use cases Agreement CDF values for evaluations of Redcap UE Positioning scenarios are derived based on: - The reported CDF points used as performance metrics in the evaluation include at least the 50%, 67%, 80%, 90% percentiles. - For indoor scenarios - (Required): The UEs inside the convex hull of the horizontal BS deployment area. - (Optional): All the UEs Agreement The following table is endorsed to capture the evaluation scenarios and parameters in the evaluation results section of the TR: Table 3.2-2 evaluation scenarios and parameters template Parameter; Case XYZ (channel model, FRx). Scenario (baseline, otherwise state any modifications). Carrier frequency. Subcarrier spacing. Reference Signal Transmission Bandwidth. Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution). Reference signal (type of sequence, number of ports, …). Number of sites. Number of symbols used per occasion. number of occasions used per positioning estimate. Power-boosting level. Uplink power control (applied/not applied). interference modelling (ideal muting, or other). Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….). Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc). Network synchronization assumptions. UE/gNB RX and TX timing error. Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides). Precoding assumptions (codebook, nrof antenna elements used, etc). UE antenna configuration. Number of UE branches. Description of enhancement solutions, if any. gNB antenna configuration. UE noise figure. UE antenna height. gNB antenna height. Additional notes, if any. Agreement Endorse the templates in section 7 in R1-2207749 to collect RedCap UE positioning simulation results, with the following notes: - The first table as endorsed in previous agreement - Add a column to the second table for capturing whether the requirement is met or not met - The TR editor can adjust the sections/sub-sections arrangement - Adjust the titles of the tables to refer to RedCap UE positioning Agreement For the evaluation of redcap UEs in the RMa scenarios, companies should report their evaluations parameters with their results. Agreement The potential benefits and performance gains of frequency hopping of the DL PRS and UL SRS can be investigated in release 18, which may take into account at least the following: - The impact of Doppler, phase offset, timing offset, power imbalance among hops - RedCap UE capability and complexity considerations - Impact of RF retuning during frequency hopping Details of frequency hopping (including Tx hopping and/or Rx hopping, BWP switching) for the study are FFS

In this contribution, evaluation assumptions are discussed and initial evaluation results for enhancements for positioning of RedCap UEs are shown. Details of potential enhancements are described in the contribution.

# Evaluation assumptions for DL positioning techniques

## Potential enhancements

To improve the performance of positioning methods for reduced BW, one potential solution may be to investigate frequency hopping for DL-PRS so that the RedCap UE can make measurements on PRS at reduced BW. Frequency hopping patterns, configurations or signaling details can be discussed during the study item phase. The following proposal is made.

Proposal 1: Study enhancements related to frequency hopping for DL-PRS to improve positioning accuracy for RedCap UEs

In this contribution, frequency hopping with the following hop pattern is considered. In the evaluation, 2-hop pattern is considered where each hopping bandwidth is not overlapping.

Figure 1 An illustration of frequency hopping (FH) pattern

## Evaluation results

In this section, horizontal accuracy results for RedCap UEs are compared with and without frequency hopping technique implemented. All common scenario parameters for evaluations agreed in RAN1#109e[1] are described in Table A1 and antenna configuration of RedCap UE is presented in Table A2. In this contribution, we evaluate positioning accuracy of RedCap UEs with the following parameters:

- Frequency range: FR1

- Bandwidth(s): 20 MHz & 5 MHz (optional)

- Positioning technique: DL-TDOA

- Enhancement: frequency hopping

To analyse the performance of RedCap UE positioning, we have evaluated 5 cases shown in Table 1. Evaluation results for each case are presented in the table.

Table 1 Horizontal accuracy performance comparison for RedCap UE positioning

| Simulation Case | Horizontal Positioning accuracy (meter) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% ile | 67% ile | 80% ile | 90% ile |
| (1) DL-TDOA, InF-SH, 100 MHz (Normal UE) bandwidth, convex hull | 0.6275 | 1.1785 | 1.8569 | 2.5681 |
| (2) DL-TDOA, InF-SH, 20 MHz bandwidth (RedCap), without frequency hopping, convex hull | 1.2968 | 1.6847 | 2.5333 | 4.1500 |
| (3) DL-TDOA, InF-SH, 5 MHz bandwidth (RedCap), without frequency hopping, convex hull | 3.2729 | 4.0432 | 4.9324 | 6.7101 |
| (4) DL-TDOA, InF-SH, 20 MHz bandwidth (RedCap), frequency hopping over 2 hops, convex hull | 1.0280 | 1.6992 | 2.5045 | 2.9372 |
| (5) DL-TDOA, InF-SH, 5 MHz bandwidth (RedCap), frequency hopping over 2 hops, convex hull | 2.5783 | 3.5926 | 4.3058 | 5.1143 |

The following observations are made.

Observation 1: In FR1, for 20 MHz bandwidth, RedCap UE positioning with frequency hopping compared to without frequency hopping leads to ~ 1.22 m accuracy gain for 90%ile UEs.

Observation 2: In FR1, for 5 MHz bandwidth, RedCap UE positioning with frequency hopping compared to without frequency hopping leads to ~ 1.6 m accuracy gain for 90%ile UEs.

In addition, evaluation details are described in Table 1, in the format agreed in RAN1#110.

Table 2 Evaluation scenarios and parameters template

| Parameter | FR1 |
| --- | --- |
| Scenario (baseline, otherwise state any modifications) | Baseline IIoT |
| Carrier frequency | 3.5 GHz |
| Subcarrier spacing | 30kHz |
| Reference Signal Transmission Bandwidth | 5MHz, 20MHz, 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS, comb-2, 2-symbol |
| Reference signal (type of sequence, number of ports, …) | PN base sequence, 1 port |
| Number of sites | 18 |
| Number of symbols used per occasion | 2 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 0 dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Correlation based |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Chan’s algorithm |
| Network synchronization assumptions | No synchronization error |
| UE/gNB RX and TX timing error | No timing error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Not Applied |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No precoding |
| UE antenna configuration | Panel model 1 dH = 0.5λ, for 1Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) |
| Number of UE branches | 1 |
| Description of enhancement solutions, if any | Frequency hopping, 2 hops, non-overlapping bandwidth |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |
| UE noise figure | 9 dB |
| UE antenna height | 1.5 m |
| gNB antenna height | 8 m |
| Additional notes, if any |  |

The following proposal is made.

Proposal 2: Capture the evaluation results  and assumptions shown in Table 1 and Table 2, respectively, in TR 38.859

# Conclusion.

In this contribution, the following proposals and observations are made.

Proposal 1: Study enhancements related to frequency hopping for DL-PRS to improve positioning accuracy for RedCap UEs

Proposal 2: Capture the evaluation results  and assumptions shown in Table 1 and Table 2, respectively, in TR 38.859

Observation 1: In FR1, for 20 MHz bandwidth, RedCap UE positioning with frequency hopping compared to without frequency hopping leads to ~ 1.22 m accuracy gain for 90%ile UEs.

Observation 2: In FR1, for 5 MHz bandwidth, RedCap UE positioning with frequency hopping compared to without frequency hopping leads to ~ 1.6 m accuracy gain for 90%ile UEs.

# Reference

[1] RAN1 Chairman’s note, RAN1#109e, May, 2022

# Appendix

Table A1: Common scenario parameters applicable for all scenarios for Redcap UEs evaluations

|  | FR1 |
| --- | --- |
| Carrier frequency, GHz | 3.5GHz |
| Bandwidth, MHz | 100 MHz ,20 MHz, 5 MHz |
| Subcarrier spacing, kHz | 30KHz |
| gNB model parameters |  |
| gNB noise figure, dB | 5dB |
| UE model parameters |  |
| UE noise figure, dB | 9dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 |
| UE antenna radiation pattern | Omni, 0dBi |
| UE horizontal drop procedure | Uniformly distributed within the convex hull of the horizontal BS deployment. |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized) |
| Note 1: According to TR 38.802 |  |

Table A2: RedCap UE antenna model

|  | FR1 |
| --- | --- |
| UE model parameters |  |
| UE antenna configuration | Panel model 1 – Note 1 dH = 0.5λ, for 1Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |