3GPP TSG RAN WG1 Meeting #110bis-e 	R1-2209510

Online, October 10th – 19th, 2022

Agenda Item: 9.2.4.1

Source: MediaTek Inc.

Title: Evaluation on AI/ML for positioning accuracy enhancement

Document for: Discussion and Decision

# Introduction

In the RAN1#110 meeting, it was agreed that two approaches are studied and evaluated for AI/ML-based positioning [1]. In this contribution, we discuss AI/ML assisted positioning, and AI/ML plays the role of LOS/NLOS identification.

Agreement For AI/ML-based positioning, both approaches below are studied and evaluated by RAN1: - Direct AI/ML positioning - AI/ML assisted positioning

# Evaluation methodology

## AI/ML LOS/NLOS identification assisted positioning

LOS/NLOS detection is an important method to improve the positioning accuracy. As shown in Figure1, we use non-normalized PDP (path delay profile) as AI/ML model input. Non-normalized PDP means we don’t normalize PDP power to 1. To provide the soft value as the LOS/NLOS probability from the model output to the LMF, the measurement results under potential NLOS condition maybe removed from the UE position calculation, resulting in the improvement in accuracy. In this contribution, DL-TDOA positioning method is used as the baseline for comparison purpose.

Figure 1. AI/ML assisted positioning based on LOS/NLOS identification

Proposal 1: For AI/ML assisted positioning, LOS/NLOS identification based on AI/ML should be evaluated for positioning accuracy enhancements.

Proposal 2: For evaluation of the AI/ML-based LOS/NLOS identification, the positioning accuracy should be compared with the performance of an existing conventional algorithm.

# Evaluation results

## Dataset description

For the AI/ML-based LOS/NLOS classification, clutter parameters of {40%, 2m, 2m} were used to generate dataset. We generate 4 drops and 1,800 UEs are dropped in every drop. When one DL-PRS antenna port at TX of each gNB, and one RX branch at UE is configured, the sample number is 1800UE*18BS*4drop= 129600 samples within the dataset. Where 1 sample means 1 PDP between 1 UE and 1 gNB.

90% data in every drop are randomly selected for training and 10% data for inference. The total training data size is 129600*90%=116640 and inference data size is 129600*10%=12900.

Different systems may have different sampling rates. For comparing the prediction accuracy of different sampling rates, we also generate dataset of sampling period 1ns and 9.3ns. In dataset, ToA(time-of-arrival) is represented as integer-values in time units of sampling period. Every path interval of PDP is the units of sampling period.

## Simulation results

### Sampling period 1ns

When sampling period is 1ns, we compared accuracy between non-normalized PDP and normalized PDP as model input.

For non-normalized PDP, LOS detection probability is 99.13% and NLOS detection probability is 99.39%.

For normalized PDP, LOS detection probability is 87.16% and NLOS detection probability is 90.58%.

As shown in Table1, horizontal positioning accuracy at CDF=90% of non-normalized PDP is 0.51meter, and normalized PDP is 5.9 meters. For comparison, horizontal positioning accuracy at CDF=90% of conventional TDoA is 13.8 meters.

Table 1. UE side model/CNN

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity | Computational complexity | AI/ML |
| non-normalized PDP(512*1) | Los/Nlos probability | Ideal Los/Nlos | {40%, 2m, 2m} | 1800UE*18BS*4drop*0.9 =116640 | 1800UE*18BS*4drop*0.1 =12960 | 4166 | 73.74k | 0.51 |
| normalized PDP(512*1) | Los/Nlos probability | Ideal Los/Nlos | {40%, 2m, 2m} | 1800UE*18BS*4drop*0.9 =116640 | 1800UE*18BS*4drop*0.1 =12960 | 4166 | 73.74k | 5.9 |

Observation 1: From the evaluation results, it is observed that non-normalized power delay profile (PDP) as the model inputs provide better LOS/NLOS classification accuracy than normalized PDP.

Proposal 3: For further study the model inputs, comparison of evaluation on AI/ML-based LOS/NLOS identification, support the non-normalized power delay profile (PDP) as the model inputs.

### Sampling period 9.3ns

When sampling period is 9.3ns, 256-points PDP was used. LOS detection probability is 93.74% and NLOS detection probability is 82.85%, which is smaller than 1ns. And horizontal positioning accuracy is bigger than 1ns sampling period as shown in Table2.

Table 2. UE side model/CNN

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity | Computational complexity | AI/ML |
| non-normalized PDP(256*1) | Los/Nlos probability | Ideal Los/Nlos | {40%, 2m, 2m} | 1800UE*18BS*4drop*0.9 =116640 | 1800UE*18BS*4drop*0.1 =12960 | 2118 | 36.876k | 17.2 |

If sampling period is 9.3ns, the ToA error is (9.3e-9)*(3e8) = 2.79meters; but if sampling period is 1ns, the ToA error is (1e-9)*(3e8) = 0.3meter. When we use conventional TDoA algorithm to estimate UE location, the positioning accuracy should be better with 1ns sampling period.

Observation 2: From the evaluation results, it is observed that smaller sampling period provide better positioning accuracy.

Proposal 4: For comparison of evaluation on AI/ML-based LOS/NLOS identification, it is suggested to align the sampling period in dataset.

# Conclusion

In this contribution, we summarize the following observations and proposals.

Proposal 1: For AI/ML assisted positioning, LOS/NLOS identification based on AI/ML should be evaluated for positioning accuracy enhancements.

Proposal 2: For evaluation of the AI/ML-based LOS/NLOS identification, the positioning accuracy should be compared with the performance of an existing conventional algorithm.

Observation 1: From the evaluation results, it is observed that non-normalized power delay profile (PDP) as the model inputs provide better LOS/NLOS classification accuracy than normalized PDP.

Observation 2: From the evaluation results, it is observed that smaller sampling period provide better positioning accuracy.

Proposal 3: For further study the model inputs, comparison of evaluation on AI/ML-based LOS/NLOS identification, support the non-normalized power delay profile (PDP) as the model inputs.

Proposal 4: For comparison of evaluation on AI/ML-based LOS/NLOS identification, it is suggested to align the sampling period in dataset.

# References

- RAN1 Chair’s Notes, RAN1#110, August 2022