3GPP TSG RAN WG1 #110bis-e 	                                                 R1-2209519 e-Meeting, October 10th – 19th, 2022

Agenda item:	9.6.1

Source: 	MediaTek Inc.

Title: 	On further UE complexity reduction for RedCap

Document for:	Discussion and decision

# Introduction

With the conclusion of Rel-18 RedCap study item in [1], a work item on further NR UE complexity reduction was approved with the following objectives in [2]:

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2 and CT1 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#98-e regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone - Whether or not/how a separate early indication can be supported - Other restrictions of the WI (e.g., connectivity restrictions, band, etc.)

# UE baseband bandwidth reduction

At RAN #97e, the following proposal was agreed [3]. In this section, we share our understanding of the definition of the UE BB bandwidth reduction for Rel-18 eRedCap UEs.

Proposal: Approve the WID in RP-222612 (Inbox, Docs), with the understanding that the exact definition of the UE BB bandwidth reduction will be revisited in RAN1.

PDSCH transmission bandwidth

Based on the coverage evaluation results in Table 8.2.1-7 of TR 38.865 [1], when SIB1 is transmitted with a bandwidth larger than 5MHz (e.g. 48 PRBs) but a Rel-18 eRedCap UE only receives 5MHz of it, a link performance loss of 11.24dB is observed in Urban scenario at 2.6GHz with 30kHz SCS compared with Rel-17 RedCap UE. The reason is that a huge portion of SIB1 has been truncated at the UE receiver side. By confining SIB1 transmission bandwidth to 5MHz, a Rel-18 eRedCap UE can receive and decode the entire SIB1 message without truncation. The link performance is hence improved by 2.24dB. However, this still results in a performance degradation of 9dB due to the increase of MCS compared with Rel-17 RedCap UE. In addition, the 2.24dB improvement is at the cost of a dedicated SIB1 transmission which increases system’s overhead.

Observation 1: In Urban scenario at 2.6GHz with 30kHz SCS, link performance degradation of SIB1 compared to Rel-17 RedCap UE is about (1) 11.24 dB when SIB1 transmission BW is larger than 5MHz, and is about (2) 9 dB when SIB1 is confined within 5MHz.

Taking the coexistence with non-RedCap UEs and Rel-17 RedCap UEs into consideration, as agreed in the WID, we propose that the transmission bandwidth of PDSCH shall not be confined to 5MHz. Furthermore, from gNB’s perspective, the transmission bandwidth of PDSCH for Rel-18 eRedCap UEs is allowed up to 20MHz.

Proposal 1: Clarify that “5MHz BB bandwidth” at UE does not suggest the bandwidth span of PDSCH transmitted at gNB is confined within 5MHz.

Proposal 2: From gNB’s perspective, transmission bandwidth of PDSCH can be up to 20MHz for Rel-18 eRedCap UEs.

UE BB bandwidth(s)

Though the UE baseband bandwidth is agreed to be reduced to 5MHz for data channels PDSCH and PUSCH; however, for other signals/channels, the maximum UE bandwidth for both baseband and RF is still assumed to be 20MHz for Rel-18 eRedCap UEs. Since UE is expected to receive and process PDCCH, it is reasonable to assume the post-FFT data buffering size remains 20MHz at least for the first several symbols of a slot. How much complexity reduction in terms of the post-FFT data buffering reduction in the remaining symbols in a slot requires some further discussion in RAN1. For example, it should be clarified whether UE can know which 5MHz in a BWP is to be scheduled before a scheduling PDCCH is decoded. In addition, since CSI-RS can be transmitted in symbols latter than the first three symbols, it is reasonable for UE to have a post-FFT data buffering size of 20MHz for all the symbols in a slot. Furthermore, if we take dual-mode support for NR and LTE into consideration, it implies the Rel-18 eRedCap UE will likely be capable of receiving and buffering 20MHz of signal for all symbols in a slot. In other words, we can say that the UE reception bandwidth for Rel-18 RedCap UE is assumed to be 20MHz for all symbols in a slot. As to the “UE BB bandwidth reduction to 5MHz” in the WID, it can be considered as the UE processing bandwidth reduction. What we mean by UE processing bandwidth is the number of resource elements (REs) that UE can process within a slot which can be different from UE reception bandwidth.

Proposal 3: Rel-18 eRedCap UEs can receive and buffer 20MHz of signals for all symbols in a slot. In other words, the UE reception bandwidth for Rel-18 RedCap UE is 20MHz for all symbols in a slot.

Proposal 4: Clarify that “UE BB bandwidth” in the WID refers to UE processing bandwidth which represents the number of REs that UE can process in a slot.

Proposed UE reception schemes

Since both the UE’s RF bandwidth and post-FFT data buffering can support up to 20MHz, it implies the maximum “UE reception bandwidth” is 20MHz so that UE can receive and buffer PDSCH of 20MHz. Therefore, as illustrated in Figure 1, two UE reception schemes can be considered when the UE processing bandwidth is reduced to 5MHz per slot. In UE reception scheme 1 as illustrated in the left-hand side of Figure 1, the transmission bandwidth of PDSCH is confined within 5MHz to accommodate UE’s baseband processing capability. In the UE reception scheme 2 as illustrated in the right-hand side of Figure 1, the transmission bandwidth of PDSCH can be up to 20MHz. Note that in both schemes, UE is expected to be able to receive and process PDCCH with a bandwidth up to 20MHz. Also note that in the second UE reception scheme, UE can still receive and buffer a PDSCH of 20MHz (i.e. with a reception bandwidth of 20MHz). However, because the UE processing bandwidth is reduced to 5MHz, UE should be allowed with more slots to process a 20MHz PDSCH received in a slot. In the second scheduling scheme, once UE has received a PDSCH scheduled in slot n, UE is not required to receive another PDSCH in the following three slots, i.e. slots n+1 to n+3 and hence it can turn off it RF for saving power.

Proposal 5: Support both UE reception schemes 1 and 2:

- In UE reception scheme1, PDSCH transmission bandwidth is confined to UE’s processing bandwidth of 5MHz.

- In UE reception scheme 2, PDSCH transmission bandwidth can be wider than UE’s processing bandwidth. Once UE receives a PDSCH, UE can turn of its RF for power saving in the following N slots. FFS: N.

Figure 1: Illustration of the two proposed UE reception schemes

Analysis of UE power consumption for the proposed reception schemes

In this sub-section, system-level (SLS) evaluations on power consumption with the two proposed UE reception schemes are conducted. See Figure 1 for illustrations of the two proposed UE reception schemes. To analyze the power consumption under different UE reception schemes, we also provide the derivation of power states with data BB bandwidth reduction which can be found in Appendix A.

Table 1 compares the evaluation results of the two proposed UE reception schemes. It can be observed that UE reception scheme 2 can provide significant power saving gain w.r.t scheme 1, especially in DRX setting 2. Compared to the DRX setting suggested in TR38.840 [4], a setting with shorter inactivity timer highlight the benefit provided by UE reception scheme 2. A significant power saving gain (22.7%~50.5%) is observed if the UE is configured with UE reception scheme 2. Thus, we have the following observations.

Observation 2: UE reception scheme 2 has higher power saving gain than UE reception scheme 1 in the evaluated scenarios.

Observation 3: UE power saving gain can be further enhanced significantly if UE reception scheme 2 is configured with other power saving methods. (e.g., a short inactivity timer)

Table 1: UE power consumption evaluation of the two proposed UE reception schemes in different scenarios

| Traffic | Instant Messaging (IM) |  |  |  |
| --- | --- | --- | --- | --- |
| Power (unit) | DRX setting 1: Value from TR38.840 (period, on-duration, inactivity timer) = (320, 10, 80) msec |  | DRX setting 2: Reduce inactivity timer (period, on-duration, inactivity timer) = (320, 10, 10) msec |  |
| Scenario/Scheme | Scheme 1 | Scheme 2 | Scheme 1 | Scheme 2 |
| TDD 2RX | 20.68 | 18.32 (11.4%↓) | 8.08 | 5.72 (29.2%↓) |
| FDD 2RX | 21.05 | 19.37 (8.0%↓) | 7.39 | 5.71 (22.7%↓) |
| TDD 1RX | 23.65 | 18.29 (22.7%↓) | 12.20 | 6.04 (50.5%↓) |
| FDD 1RX | 22.61 | 19.57 (13.4%↓) | 9.08 | 5.92 (34.8%↓) |

# UE peak data rate reduction

The target peak data rate for Rel-18 RedCap is 10Mbps. In contrast, the peak data rate requirement for Rel-17 RedCap is at least 150Mbps. If the peak rate constraint (vLayers(j)⋅Qmj⋅fj) can be relaxed to 1, the maximum modulation order can be reduced to 2 instead of 4. With further reduction on modulation order, both SNR and RF requirements can be relaxed which can lead to potential further cost reduction.

Proposal 6: Relax constraint  vLayers(j)⋅Qmj⋅fj from 4 to 1 for Rel-18 eRedCap.

Proposal 7: 16QAM is mandatorily supported while 64QAM can be optionally supported by Rel-18 eRedCap.

Table 2: Peak data rate calculation for different bandwidths and different values of constraint (vLayers(j)⋅Qmj⋅fj)

| SCS | BW (NPRBBW) | Peak rate constraint vLayers(j)⋅Qmj⋅fj | DL peak rate (Mbps) | UL peak rate (Mbps) |
| --- | --- | --- | --- | --- |
| 15KHz | 20MHz (106) | 4 | 56.7 | 60.7 |
|  |  | 1 | 14.2 | 15.2 |
|  | 5MHz (25) | 4 | 12.8 | 13.7 |
|  |  | 1 | 3.2 | 3.4 |
| 30KHz | 20MHz (51) | 4 | 54.6 | 58.4 |
|  |  | 1 | 13.6 | 14.6 |
|  | 5MHz (11) | 4 | 11.8 | 12.6 |
|  |  | 1 | 2.95 | 3.15 |

# Conclusions

In this contribution, we have made the following observations and proposals.

Observation 1: In Urban scenario at 2.6GHz with 30kHz SCS, link performance degradation of SIB1 compared to Rel-17 RedCap UE is about (1) 11.24 dB when SIB1 transmission BW is larger than 5MHz, and is about (2) 9 dB when SIB1 is confined within 5MHz.

Observation 2: UE reception scheme 2 has higher power saving gain than UE reception scheme 1 in the evaluated scenarios.

Observation 3: UE power saving gain can be further enhanced significantly if UE reception scheme 2 is configured with other power saving methods. (e.g., a short inactivity timer)

Proposal 1: Clarify that “5MHz BB bandwidth” at UE does not suggest the bandwidth span of PDSCH transmitted at gNB is confined within 5MHz.

Proposal 2: From gNB’s perspective, transmission bandwidth of PDSCH can be up to 20MHz for Rel-18 eRedCap UEs.

Proposal 3: Rel-18 eRedCap UEs can receive and buffer 20MHz of signals for all symbols in a slot. In other words, the UE reception bandwidth for Rel-18 RedCap UE is 20MHz for all symbols in a slot.

Proposal 4: Clarify that “UE BB bandwidth” in the WID refers to UE processing bandwidth which represents the number of REs that UE can process in a slot.

Proposal 4: Clarify that “UE BB bandwidth” in the WID refers to UE processing bandwidth which represents the number of REs that UE can process in a slot.

Proposal 5: Support both UE reception schemes 1 and 2:

- In UE reception scheme1, PDSCH transmission bandwidth is confined to UE’s processing bandwidth of 5MHz.

- In UE reception scheme 2, PDSCH transmission bandwidth can be wider than UE’s processing bandwidth. Once UE receives a PDSCH, UE can turn of its RF for power saving in the following N slots. FFS: N.

Proposal 6: Relax constraint  vLayers(j)⋅Qmj⋅fj from 4 to 1 for Rel-18 eRedCap.

Proposal 7: 16QAM is mandatorily supported while 64QAM can be optionally supported by Rel-18 eRedCap.

# References

- 3GPP TR 38.865, “Study on further NR RedCap UE complexity reduction (Release 18),” V18.0.0, 2022-09

- RP-222675, “New WID on enhanced support of reduced capability NR devices,” Ericsson, 3GPP TSG RAN meeting #97e, September 2022

- RP-222633, “Moderator’s summary for discussion [97e-15-R18-RedCap],” Ericsson, 3GPP TSG RAN meeting #97e, September 2022

- 3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR (Release 16),” V16.0.0, 2019-06

# Appendix A: UE power consumption evaluation

In Appendix A, we provide the SLS simulation parameters (Table 2) and the power derivation for baseband bandwidth reduction.

Table 3: Parameter of System Level Simulation

| Simulation parameters |  | value |  |
| --- | --- | --- | --- |
| Deployment |  | 3GPP Dense Urban, 7 sites, 3 sectors per site |  |
| Channel model |  | 3D-UMa |  |
| SCS |  | 30 kHz |  |
| Frame structure for TDD |  | DDDDDDDSUU (S: 6D:4G:4U) |  |
| BS Tx antennas number |  | 64TX; (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) |  |
| UE Rx antennas number, (MIMO layer) |  | 2 RX; (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), (2 layer) 1 RX; (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1), (1 layer) |  |
| Power class |  | PC 3 |  |
| Modulation |  | DL 64 QAM |  |
| bandwidth |  | 20MHz - Scheduling scheme 1: 20 MHz PDCCH, 5 MHz PDSCH - Scheduling scheme 2: 20 MHz PDCCH, 20 MHz PDSCH |  |
| Traffic |  | Instant messaging (IM) | VoIP |
|  | Model | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
|  | Packet size | 0.1 Mbytes |  |
|  | Mean inter-arrival time | 2 seconds |  |
|  | DRX setting | (Period, On duration, IAT) ms Setting 1: (320, 10, 80) Setting 2: (320, 10, 10) | (Period, On duration, IAT) ms Setting: (40, 4, 10) |

In both scheduling schemes, the data baseband is reduced from 20MHz to 5MHz. To analyse the power consumption, the power of baseband reduction is derived as the following.

Firstly, it can be observed from the value of Table 3, the power of “PDCCH + PDSCH” (PPDCCH+PDSCH) and “PDSCH-only” (PPDSCH) are scaling down 0.4 times when the bandwidth is scaling down 0.2 times (100MHz to 20MHz). Then, we can simply get the scaling factor from 20MHz to 5MHz (S(20→5)MHz) is 0.5.

| S20→5MHz=0.5 | (1) |
| --- | --- |

Table 4: UE power consumption model for FR1.

| Power State | Relative power for 100MHz UE (from TR 38.840 [Error! Bookmark not defined.]) | Relative power for 20MHz UE (from TR 38.875 Error! Reference source not found.) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 0.8 |
| Light Sleep (PLS) | 20 | 18 |
| Micro sleep (PMS) | 45 | 31 |
| PDCCH-only () | 100 for same-slot scheduling 70 for cross-slot scheduling | 50 for same-slot scheduling, 40 for cross-slot scheduling |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 | 50 |

Secondly, to get the power only scaling down on data baseband, we decouple the power of power states into PF, control baseband, data baseband. Figure 2 illustrates the components of power states. Then, we can have the following formulas:

| PPDCCH= αRF+control baseband | (2) |
| --- | --- |
| PPDCCH+PDSCH= RF+control baseband | (3) |
| PPDSCH=βRF+data baseband | (4) |

where α and β equal the time portion of RF in “PDCCH only” and “PDSCH only”, respectively

Figure 2: Components of power states

Thirdly, we derive the power of each component by Table 3 and equation (1), (2) and (3).

From Table 3, we can get the power of “PDCCH + PDSCH” equals

| PPDCCH+PDSCH-PPDSCH=120-112=8 | (5) |
| --- | --- |

From equation (2) - (3), the power of  PPDCCH+PDSCH-PPDSCH=1-βRF+control baseband equals (5).

Assume that β=13/14 (turn on RF part 13 symbols within a slot), we can derive the power of control baseband equals to 3.33, and the power of data baseband equals to 51.33.

Assume that α=10/13 (turn on RF part 10 symbols within a slot), the power of RF = 65.33

To simplify the power state, we round off the value of each component as Table 4.

Table 5: Power value of RF and BB

| Power value (unit) | 20MHz RF and Control 20MHz BB BW | 20MHz RF and Control 5MHz BB BW (PDSCH) |
| --- | --- | --- |
| RF | 65 | 65 |
| control baseband | 3 | 3 |
| data baseband | 51 | 31 from (7) |

Finally, we can get the power of PPDCCH+PDSCH with data baseband scaling down from 20MHz to 5MHz equals to

| PPDCCH+PDSCH= RF+control baseband+ data baseband × S20→5MHz =65+3+52×0.5=94 | (6) |
| --- | --- |

The power of PPDSCH in scheduling scheme 2 can be calculated by the following equation.

| PPDSCH=max PMS, PPDSCH×S20→5MHz= max 31, 52×0.5=31 | (7) |
| --- | --- |