3GPP TSG RAN WG1 #110bis-e		    R1-2209527

e-Meeting, October 10th – 19th, 2022

Source: 	MediaTek Inc.

Title:	Remaining issues on NR MBS

Agenda item:	8.12

Document for:	Discussion & Decision

## Introduction

In previous e-meeting [1], the remaining issues for Rel-17 NR MBS were discussed and achieved some agreements. However, there are still some critical remaining issues needed to be discussed and to finalize the Rel-17 MBS. In this contribution, we will further discuss the detailed design for the remaining issue of Rel-17 NR MBS.

## Discussion

  - PDSCH/PUCCH processing timeline for multicast NACK-only based feedback

Regarding the PDSCH/PUCCH processing procedure timeline for NACK-only based feedback, it was discussed in previous meeting with controversial views and achieved the following agreements for further study in RAN1#109-e meeting.

Agreement Regarding the PDSCH and/or PUCCH processing procedure timeline for NACK-only based feedback, further discuss on whether/how to extending the PDSCH and/or PUCCH processing procedure timeline. - Note1: The current PDSCH processing procedure timeline defined in clause 5.3 TS38.214 is for ACK/NACK based feedback. - Note2: For NACK-only based feedback for more than 1 TB, one possible issue is that the PUCCH resource for NACK-only can only be determined after obtaining the decoding result of all the related PDSCHs for the UE.

The issue was extensively further discussed in the RAN1#110 meeting, and the following agreement was achieved for resolving the issue:

Agreement When NACK-only PUCCH overlaps with other PUCCH/PUSCH transmission, NACK-only is transformed into ACK/NACK and multiplexed with other PUCCH/PUSCH transmission: - Opt4: All PUCCHs have the same starting symbol and the same time duration

However, even though the UE can know the PUCCH resource time domain information in advance since all the PUCCHs for NACK only have the same starting symbol and same time duration, the UE still cannot know the PUCCH frequency RB position and cannot determine the PUCCH TPC calculation. So, the following proposal was raised for further resolving the issue [2]:

Proposal: Extending PDSCH processing procedure time for NACK-only based feedback by adding d3 into TProc,1 as follows: TProc,1=N1+d1,1+d2+d32048+144∙ƙ2-μ∙TC+Text, where d3=0 when UE is configured with the same PRB for the PUCCH resources configured for NACK-only mode2. Otherwise d3=[x]. FFS on value of x. - Note all PUCCH resources have the same starting symbol and the same time duration

However, the proposal was not agreed due to controversial views. From the UE processing perspective, this is a critical and valid issue. If there is not any agreement for prolonging the processing timeline to consider the UE behavior, the UE will have to report not support the NACK-only feedback mode based on the UE capability, and this also will delay the MBS commercial deployment and is not expected.

Observation 1: For the multicast NACK-only mode 2, the UE cannot know which PUCCH frequency resource will be used before decoding all the related PDSCH.

Observation 2: The UE has to report not support NACK-only mode 2 feedback based on UE capability if the corresponding processing timeline is not extended, which is not desirable since the motivation to introduce the NACK-only mode 2 case is to simplify the HARQ-ACK operation and accelerate the MBS commercial deployment.

Figure 1 PDSCH/PUCCH processing timeline

If the gNB can ensure all the PUCCH resources configured for NACK-only mode 2 have the same PRB, it may be not needed to extend the processing timeline and the current timeline can be reused for NACK-only mode 2 feedback. However, if the gNB cannot guarantee all the PUCCHs for NACK-only mode 2 has the same PRB, the current processing timeline is not sufficient as we mentioned above. For example, as illustrated in Figure 1, the legacy unicast or multicast ACK/NACK UE can know which PUCCH time/frequency domain resource will be used after decoding the corresponding PDCCH and parsing the corresponding PRI information, then it can determine the corresponding PUCCH operation, e.g., TPC determination and calculation. However, for the more than one NACK only mode 2 case, the UE still needs more time to process TPC procedure since it cannot know the exact PUCCH frequency resource position even if the PUCCHs has the same starting symbol and duration. It also can be seen that the delayed time duration for the NACK only mode 2 feedback is d3, i.e. d3=N1+t2, where N1 is the PDSCH processing time as defined in current spec, and d2=PDSCHdurationtime. Thus, it is better to prolong the same time duration after UE decoding all the related PDSCHs. Meanwhile, considering that the same starting symbol and time duration for NACK-only mode 2 PUCCH have been agreed, it means that UE can know some part of PUCCH time domain resource in advance, so, d3=N1 is sufficient for UE to process the frequency domain resource. Based on the above discussion, we can directly confirm the previous proposal with d3 modification as follows:

Proposal 1: Extending PDSCH processing procedure time for NACK-only based feedback by adding d3 into TProc,1 as follows:

TProc,1=N1+d1,1+d2+d32048+144∙ƙ2-μ∙TC+Text, where d3=0  when UE is configured with the same PRB for the PUCCH resources configured for NACK-only mode2. Otherwise d3=N1.

- Note all PUCCH resources have the same starting symbol and the same time duration

  - PUCCH resource mapping for the NACK-only case

For the NACK only based HARQ-ACK feedback, the following agreement about the mapping table between HARQ-ACK bits and PUCCH resource was achieve in last meeting.

Agreement For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows: - The mapping is applied to both a single G-RNTI - FFS: multiple configured G-RNTIs cases. - FFS: how to order the TBs, whether/how to account for missed or non-scheduled TBs - FFS whether/how PRI is used for indication. HARQ-ACK Value; PUCCH resource. {0}; {0,0}; {0,0,0}; {0,0,0,0}; 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList. {1,0}; {1,0,0}; {1,0,0,0}; 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList. {0,1}; {0,1,0}; {0,1,0,0}; 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList. {1,1,0}; {1,1,0,0}; 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList. {0,0,1}; {0,0,1,0}; 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList. {1,0,1}; {1,0,1,0}; 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList. {0,1,1}; {0,1,1,0}; 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList. {1,1,1,0}; 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList. {0,0,0,1}; 9th PUCCH resource provided by pucch-ResourceId obtained from the 9th value of resourceList. {1,0,0,1}; 10th PUCCH resource provided by pucch-ResourceId obtained from the 10th value of resourceList. {0,1,0,1}; 11th PUCCH resource provided by pucch-ResourceId obtained from the 11th value of resourceList. {1,1,0,1}; 12th PUCCH resource provided by pucch-ResourceId obtained from the 12th value of resourceList. {0,0,1,1}; 13th PUCCH resource provided by pucch-ResourceId obtained from the 13th value of resourceList. {1,0,1,1}; 14th PUCCH resource provided by pucch-ResourceId obtained from the 14th value of resourceList. {0,1,1,1}; 15th PUCCH resource provided by pucch-ResourceId obtained from the 15th value of resourceList.

One remaining issue is whether the mapping table will be applied to multiple configured G-RNTIs. From our understanding, the main purpose of introducing the NACK only based feedback is to decrease the PUCCH resource overhead, and multiple UE can share the same PUCCH resource to feedback the only NACK feedback. Besides, assuming the mapping table is applied to G-RNT, if one UE is configured multiple G-RNTIs and will feedback the NACK in the same PUCCH slot, the UE also needs to choose one PUCCH and multiplex all the feedback in the same PUCCH resource. For example, as illustrated in Figure 2, if UE is configured three G-RNTIs, e.g., G-RNTI 1, G-RNTI 2 and G-RNTI 3, and the PDSCHs corresponding to the configured G-RNTIs are feedback in the same PUCCH slot, the UE will choose one PUCCH resource to transmit the corresponding PDSCH decoding status information. If multiple mapping table is used for the case, it will need more discussion how to choose which mapping table is used and how to map between the TB and PUCCH resource and so on, which will make the issue more complexity.

Figure 2 multiple G-RNTIs for one PUCCH slot

Considering the above reason, we prefer that the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only can be applied to multiple G-RNTIs.

Proposal 2: For the NACK-only case, the mapping table between HARQ-ACK values and the PUCCH resources can be applied to multiple configured G-RNTIs.

  - Codebook for NACK only

For the codebook design for the NACK only case, the following agreements were achieved for addressing how to order the TBs, whether/how to account for missed or non-scheduled TBs.

Agreement For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4: - Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI (from network and UE perspective) - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. - FFS: whether Opt1-1 can also work when different G-RNTIs are used for different UEs - Note: other cases are not precluded Agreement For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, further down-selection on the following cases/options in the next meeting: - Case 2: for the case of all UEs configured with the same set of G-RNTIs - Opt2-1: support Alt4 for this case - Opt2-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. - Opt2-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs - Opt2-2: does not support Alt4 for this case. - Case 3: for the case of different UEs configured with different G-RNTIs - Opt3-1: support Alt4 for this case - Opt3-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. - Opt3-2: does not support Alt4 for this case.

It can be seen that, for a single G-RNTI, the counting and ordering the HARQ-ACK bits for NACK-only is based on the C-DAI included in DL DCI. One remaining issue is that whether it can still work when different G-RNTIs are used for different UEs.

From our view, the issue is more related to whether the DAI is counted per a single G-RNTI or per all configured G-RNTIs. If DAI is counted per G-RNTI, it may cause the codebook misalignment between gNB and UE side when some G-RNTIs only schedule one PDSCHs in the HARQ-ACK feedback window. For example, as illustrated in Figure 3, assuming UE1 is configured with three G-RNTIs, e.g., G-RNTI1 corresponding to TB1 and TB2, G-RNTI2 corresponding to TB3 and G-RNTI3 corresponding to TB4, and UE2 is only configured with 2 G-RNTIs, e.g., G-RNTI2 and G-RNTI3, if TB3 is missed for the UE1 and DAI is counted per DAI as Opt 2 in the figure, the UE cannot aware that it missed the TB3, and will choose the PUCCH resource based on 3 PDSCH receiving status, which is not aligned with gNB side. Instead, if Opt1 is used, e.g., DAI is counted for all G-RNTIs, the UE can know that it missed the TB3 and will choose the PUCCH resource based on 4 PDSCH receiving status with TB3 is NACK. For the UE2, if Opt 1 is used, it has the similar issue with UE1, and Opt 2 can avoid the issue, e.g., missed TB3. To be honest, Opt2 has some disadvantages, e.g., even though UE2 does not receive the TB1 and TB2, it needs to feedback these two TB’s HARQ-ACK bits. Comparing the two option’s pros and cons, we prefer the DAI is counted per all G-NRTIs to resolve the DCI missing issue.

Proposal 3: For the NACK-only HARQ codebook generation, the DAI is counted per all G-RNTIs.

Figure 3 The relationship between DAI and G-RNTI

  - NACK-only feedback collides with SR

Regarding the NACK only collide with SR in the same slot, the following proposal was discussed in previous meeting:

Proposal 3.1.1: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two TBs/HARQ-ACK bits of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. - PUCCH resources configured for unicast will be used subject to legacy procedure as Rel-15/16. - For the case of more than two TBs of NACK-only, SR is dropped.

From our perspective, the proposal is basically reasonable except for the last sub-bullet. As discussed in previous meeting, the NACK-only feedback can be transformed into ACK/NACK based feedback for more than 2 TBs if configured by RRC signaling. And it will use the PF 2/3/4 resource for more than two TB case. Therefore, it is no need to drop SR since the PF 2/3/4 can transit multiple bits. Thus, we suggest the proposal as following:

Proposal 4: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

  - Corrections on TS 38.202

Regarding the FDMed case MCCH/MTCH/multicast MTCH and SIB PDSCH/PBCH, the following agreement was achieved in the last meeting.

Related PBCH FDMed case: Agreement For RRC_IDLE/INACTIVE UEs, a UE is required to support reception of FDMed MCCH PDSCH and PBCH in PCell. Agreement For RRC_IDLE/INACTIVE UEs, a UE is not required to support reception of FDMed MTCH PDSCH and PBCH in PCell.. Related SIB FDMed case: Agreement: For RRC_CONNECTED UEs, • a UE is not required to support reception of FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in Pcell. • a UE is required to support reception of FDMed MCCH PDSCH and PBCH in Pcell. • a UE is not required to support reception of FDMed MTCH PDSCH and PBCH in Pcell. • a UE is not required to support reception of FDMed multicast PDSCH and PBCH in Pcell. Agreement For RRC_IDLE/INACTIVE UEs, a UE is not required to support reception of FDMed MCCH/MTCH PDSCH and SIB PDSCH in PCell..

However, the latest spec of 38.202 h20 does not capture the above agreement, e.g., based on the following Table 2, for the RRC CONNECTED state, it also has the combination of "B(SI-RNTI) + D1(C-RNTI) +( D3(multicast G-RNTI) or ( D5(MCCH-RNTI) or D6(broadcast G-RNTI) ) )", it means the UE can receive the SIB PDSCHC and MCCH/MTCH PDSCH simultaneously, which is against the agreements above. For the PBCH related FDMed case, it also has the similar situation, the detailed comparison between existing combinations and corresponding against agreements are summarized in Table 1 Considering the combination is so complicated, maybe the simple way to add a note to capture the above agreement to resolve the issue and the corresponding can refer to R1-2209524[3].

Table 1: The comparison b/w the existing combinations and corresponding against agreements

| Existing combinations | Notes | Against agreements |
| --- | --- | --- |
| B + D3 | B: SI-RNTI D3: multicast G-RNTI | Agreement: For RRC_CONNECTED UEs, • a UE is not required to support reception of FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in Pcell. |
| B + D5 | B: SI-RNTI D5: MCCH-RNTI | Agreement: For RRC_CONNECTED UEs, • a UE is not required to support reception of FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in Pcell. |
| B + D6 | B: SI-RNTI D6: broadcast G-RNTI | Agreement: For RRC_CONNECTED UEs, • a UE is not required to support reception of FDMed MCCH/MTCH/multicast PDSCH and SIB PDSCH in Pcell. |
| A+D6 | A: PBCH D6: broadcast G-RNTI | Agreement For RRC_IDLE/INACTIVE UEs, a UE is not required to support reception of FDMed MTCH PDSCH and PBCH in PCell. Agreement: For RRC_CONNECTED UEs, a UE is not required to support reception of FDMed MTCH PDSCH and PBCH in Pcell. |
| A+D3 | A: PBCH D3: multicast G-RNTI | Agreement: For RRC_CONNECTED UEs, • a UE is not required to support reception of FDMed multicast PDSCH and PBCH in Pcell. |

Table 2: The current Downlink "Reception Type" combinations in TS 38.202 h20

| Supported Combinations |  |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCell | PSCell |  | SCell |  |  |  |
| 1. RRC_IDLE |  |  |  |  |  |  |
| 1.1 All UEs |  |  |  |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  |  |  | Note 1 |  |
| 1.2 UEs supporting MBS broadcast reception |  |  |  |  |  |  |
| A+D5 |  |  |  |  |  |  |
| 2. RRC_INACTIVE |  |  |  |  |  |  |
| 2.1 All UEs |  |  |  |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  |  |  | Note 1 |  |
| 2.2 UEs supporting MBS broadcast reception |  |  |  |  |  |  |
| A+D5 |  |  |  |  |  |  |
| 3. RRC_CONNECTED |  |  |  |  |  |  |
| (A + ((C0 + (B and/or (D0 or (m1*D1+m2*D2+((m3*D3+m4*D4) or m5*(D5 or D6))))) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M + N + P) or D5)) | (A + (D0 or (m1*D1+m2*D2)) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + N + P) |  | m1*D1 + m2*D2 + ((m3*D3+m4*D4) or m5*(D5 or D6)) + E + n*F1 + G + H + J0 + J1 + J2 + K + O + L0 + L1 + M + P |  | Note 2, Note 3, Note 4, Note 5, Note 6, Note 7, Note 8, Note 9, Note 10, Note 11, Note 12 |  |
| Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: For PCell, UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. Note 3: Supported combinations are subject to UE capabilities for dual connectivity, carrier aggregation, receiving of group TPC commands, pre-emption indication and dynamic SFI monitoring. Note 4: The values of m2 ≥ 0 and n≥ 0 in the supported combinations are subject to the UE capability. Note 5: Support of monitoring PDCCH with SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI are subject to UE capability. Note 6: The values of m1 ≥ 1 in the supported combinations are subject to the UE capability. Note 7: In Active time, a UE is not expected to monitor the DCI format for the PDCCH scrambled by PS-RNTI. Note 8: The PDCCH scrambled by PS-RNTI can only be configured on the PCell and PSCell. Note 9: For a UE supporting MBS multicast reception, the values of 1 ≥ m3 ≥ 0 and m4 ≥ 0 are subject to UE capability and applicable to RRC connected UEs. If m3 = 1, then m1 ≤ 1. Note 10: For a UE supporting MBS multicast or broadcast reception, the UE is not expected to be configured simultaneously with more than one component carrier for multicast or broadcast reception. Note 11: For a UE supporting MBS broadcast reception, the values of 1≥m5 ≥ 0 are subject to UE capability and applicable to RRC connected UEs. If m5=1, then m1≤1. Note 12: For a UE supporting MBS broadcast reception in RRC_CONNECTED state, it is required to support reception of FDMed MCCH PDSCH and PBCH in Pcell. |  |  |  |  |  |  |

Proposal 5: Adopt the draft CR in R1-2209524.

## Conclusion

In this contribution, it further discusses the MBS remaining issues with following observations and proposals:

Observations:

Observation 1: For the multicast NACK-only mode 2, the UE cannot know which PUCCH frequency resource will be used before decoding all the related PDSCH.

Observation 2: The UE has to report not support NACK-only mode 2 feedback based on UE capability if the corresponding processing timeline is not extended, which is not desirable since the motivation to introduce the NACK-only mode 2 case is to simplify the HARQ-ACK operation and accelerate the MBS commercial deployment.

Proposals:

Proposal 1: Extending PDSCH processing procedure time for NACK-only based feedback by adding d3 into TProc,1 as follows:

TProc,1=N1+d1,1+d2+d32048+144∙ƙ2-μ∙TC+Text, where d3=0 when UE is configured with the same PRB for the PUCCH resources configured for NACK-only mode2. Otherwise d3=N1.

- Note all PUCCH resources have the same starting symbol and the same time duration

Proposal 2: For the NACK-only case, the mapping table between HARQ-ACK values and the PUCCH resources can be applied to multiple configured G-RNTIs.

Proposal 3: For the NACK-only HARQ codebook generation, the DAI is counted per all G-RNTIs.

Proposal 2: For the NACK-only case, the mapping table between HARQ-ACK values and the PUCCH resources can be applied to multiple configured G-RNTIs.

Proposal 3: For the NACK-only HARQ codebook generation, the DAI is counted per all G-RNTIs.

Proposal 4: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

Proposal 5: Adopt the draft CR in R1-2209524.

## Reference

- RAN1 Chairman’s Notes, RAN1 #110, August 2022.

- R1-2207714, FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs, Moderator (Huawei), RAN1#110, Toulouse, France, August 22 – 26, 2022.

- R1-2209524, Corrections on the MBS reception type combinations in TS 38.202, MediaTek, RAN1#110bis-e, October 10th – 19th, 2022