3GPP TSG RAN WG1 #110bis-e	R1-2209536

e-meeting, 10 – 19 October, 2022

Agenda item:		9.10.2

Source:	Nokia, Nokia Shanghai Bell

Title:	XR-specific capacity enhancements

Document for:		Discussion and Decision

# Introduction

RAN#95 approved a revised SID on XR Evaluations for NR [1] with the following objectives:

### 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution we present our views on the enhancements related to capacity. Particularly, CG, DG, CBG-based CQI, and restrictions due to measurements are discussed in more details below.

During 109-e, the following agreements were made in order to assess capacity related enhancements:

Agreement - For each candidate capacity enhancement technique for XR traffic, companies are encouraged to consider the following common principle for assessment of the candidate capacity enhancement technique: - Identify the XR-specific issue(s) that the enhancement technique is addressing - Identify the necessity of the enhancement technique to address the issues - Identify whether/how the enhancements provide benefit/performance capacity gain. - Consider at least feasibility, complexity, and system level performance evaluations in comparing the enhancement techniques. Power saving gains for a given enhancement technique can optionally be evaluated and considered in addition to these other aspects. - The baseline scheduling scheme when comparing the proposed capacity enhancements techniques is: - Dynamic scheduling and/or - Semi-persistent scheduling / Configured grant scheduling - Note: Companies are encouraged to additionally use DG scheduling as the baseline scheduling scheme when showing the capacity performance gain Agreement - To support a candidate capacity enhancement technique for XR traffic, capacity performance gain by the technique as compared to baseline should be shown. - Capacity performance gain by the candidate technique as compared to baseline is a necessary condition to consider supporting the candidate technique. For future meetings: Companies are requested to follow the following agreement and conclusion from RAN1#109-e. Check final FL summary for details. - Agreement - Rel-17 evaluation methodology for XR capacity enhancement captured in TR 38.838 is used as the baseline evaluation methodology for XR capacity enhancement of Rel-18 SI on XR enhancements. - Conclusion - Companies are encouraged to use the capacity Excel sheet attached with TR 38.838 in RP-213652 for recording the simulation results that are provided in their contributions.

# CG enhancements

During RAN1 #109-e and #110 the following agreements and conclusion were made:

Agreement To study whether/how to support a candidate capacity enhancement technique for XR traffic based SPS/CG transmissions, companies are encouraged to consider the following studies: - Study enhancements related to multiple PDSCHs SPS transmission occasions in a period - Study enhancements related to multiple PUSCHs CG transmission occasions in a period - Study enhancements related to dynamic adaptation of SPS/CG parameters/configurations - Study enhancements related to non-integer periodicity for SPS/CG transmissions. - Note: Other studies are not precluded, as well as the combination of the above studies. Follow the common principle for assessment of the candidate capacity enhancement technique

Conclusion There is no consensus in RAN1 on the benefits of enhancing SPS for the purpose of XR capacity enhancement Agreement When DG is used as the baseline scheme, for the performance evaluation scheduling, after SR is triggered, both BSR and UL data can be transmitted using the UL grant after SR. - Companies are encouraged to provide the size of resources by the first UL grant after SR

The benefits of using CG for UL transmissions is two-fold: (i) it involves shorter latency as compared to having to first send a scheduling grant (DCI) in the downlink followed by a scheduling request (SR) received the UL, and (ii) it saves DCI transmission overhead in the downlink. The CG is particularly useful for XR applications as per the currently agreed traffic models for pose/control and uplink video, no time-domain jitter is present. This makes it easy for the gNB to configure the UE with CG patterns in line with the traffic characteristics. It should, however, be kept in mind that some uplink XR traffic may be subject to some degrees of time-domain jitter, although not incorporated in the currently defined 3GPP XR traffic models. For CG to be fully applicable for XR use cases, the periodicity at which the gNB can configure a UE with CG resources shall be in line with the XR traffic periodicity. That means including cases with 4 ms periodicity as for Pose/Control and 1/fps for Video where fps=60, 90, or 120. Furthermore, 3GPP TR 38.838 also lists more advanced cases where a UE may have three uplink streams as follows: Stream 1 with pose/control, Stream 2 with the scene and video aggregation, and Stream 3 with aggregating of audio and data. So the options for configuration of CG resource periodicity shall support such cases. ​

Proposal 1: The options for configuration of the periodicity of CG resources shall match those of the uplink XR traffic models from 3GPP TR 38.838. Among others, this shall include a periodicity of every 16.6 ms (as required for 60 fps uplink video).

Especially for the uplink video cases with 10Mbps, the XR frames are often so large that they have to be transmitted in multiple TTIs. That is especially the case for the uplink direction, where UEs are often scheduled on a reduced bandwidth as compared to the total carrier of bandwidth, e.g., using 10 MHz or 20 MHz due to uplink transmit power limitations and how the network has set the open loop Tx power control parameters such as Po and Alpha (pathloss compensation factor). We have e.g., found that for UEs transmitting over only 10 MHz, typically 8 TTIs are needed to transmit an XR frame for 10 Mbps video. Similarly, 4 TTIs are typically needed for 20 MHz transmissions. This leads to the following suggestion:

Proposal 2: It should be possible to have CG configuration that includes an integer number of TTI transmissions per periodicity, e.g., in the range from 1 to 8.

In the scenarios where multiple TTIs are configured per periodicity for UL video traffic transmission, one of the open issues to be studied further is the impact of variable packet size for one video frame. Suppose that the CG resource is configured according to the average size of the XR frame, in case the coming XR frame is larger than the configured size, one possible way is to inform gNB earlier enough to get more resources via, e.g., MAC CE carrying BSR sending together with UL data in one of the earlier CG resources. However, it is more challenging considering the scenario where the size of the coming XR frame is smaller than the configured CG resources. One simple and straightforward way is applying simple zero padding or UL PUSCH skipping, which obviously leads to resource waste. Depending on the deployment scenario, it is also possible that UE informs gNB that part of the configured CG resources will not be used for XR frame delivery and can be used for other purposes or even for another UE. This could be more feasible, for example, in case of TDD.

Proposal 3: RAN1 to study potential CG enhancements, for example, temporarily deactivating selected CG resources within one CG period.

In the following, we present simple calculations for the DCI savings of using enhanced CG transmissions compared to the current dynamic grant (DG). Each DG transmission is assumed to require a DCI transmission. For both DG and CG cases, HARQ retransmissions are assumed to happen with DG transmissions and hence also require a DCI transmission. The first transmission error probability is assumed to be equal 10%. We consider XR uplink video model with 10 Mbps and a UE with 20 MHz transmission bandwidth. We found that approximately 264 DCI transmissions are required per second to serve each user. This number accounts for both first transmissions and the 10% probability of having HARQ transmissions. This is assuming the agreed truncated Gaussian UL XR traffic model with 10 Mbps on average. For CG, we have configured 4 TTIs per 16.6 ms (with 20 MHz transmission BW). If an incoming XR UL frame is too large to be fully transmitted within the configured CG resources, the excessive data are sent using the DG that requires a DCI transmission. With our assumed setting of the CG resources allocation, we find approximately a 25% probability of this happening. On the contrary, if the incoming XR frame is smaller than the average data size of the CG, it is sent in line with CG allocation by applying simple zero padding to fit with CG allocation or simply skipping some of the configured TTIs. On average, 92% of the allocated resources are utilized. Moreover, we find that an average of 21 DCI transmissions per second are needed for the CG case. Those account for HARQ retransmissions and the dynamic transmissions of additional data that do not fit within the allocated CG allocation. Overall, this example illustrates that we gain a factor of 264/21=12.6 of a reduced number of DCI transmissions. The reduced number of DCI transmissions will translate to savings in the DL signaling overhead and potentially also some benefits in terms of UE power consumption as the UE would need to monitor for fewer PDCCH transmissions and decode fewer PDCCH transmissions.

Observation 1: Using CG presents attractive savings in DL signaling overhead as much fewer DCI transmissions for UL traffic are needed. This may also materialize in reduced UE power consumption from having to monitor and decode fewer PDCCH transmissions.

# Dynamic grant enhancements

## Multi-PUSCH/PDSCH scheduling

During RAN1 #109-e the following agreement was made:

Agreement To study whether/how to support a candidate capacity enhancement technique for XR traffic based dynamic scheduling/grant transmissions, companies are encouraged to consider the following studies: - Study enhancements related to extending capability of single DCI scheduling multi-PDSCHs/PUSCHs for FR2-2 to FR1/FR2. - Note: whether and how to discuss enhancements may depend on the outcome of Rel-17 B52.6G UE feature discussion - Study enhancements related to HARQ-ACK and/or CBG transmissions for single DCI scheduling one or multi PDSCH(s). - Study enhancements related to allowing different configurations per PDSCH/PUSCH - Study enhancement related to scheduling request and/or BSR with the focus on L1 enhancements. - Note: Other studies are not precluded as well as the combination of the above studies. - Follow the common principle for assessment of the candidate capacity enhancement technique.

Given a large size of one video frame (e.g., 62.5 kB for 60 fps and 30 Mbit/s) multiple TBs may be required to transmit one such video frame. In order to avoid a row of DCI commands scheduling the consecutive packets of a frame, a single DCI can schedule the resources for multiple PDSCH/PUSCH.

One of the benefits from employing the above solution is the reduced PDCCH monitoring occasions for scheduling DCIs which would be very beneficial to maintain the power saving at UE device. Moreover, in case of sparse PDCCH monitoring, there will be a scheduling delay which will unnecessary decrease the time left before the PDB is expired. The latter is one of the factors reducing the capacity – number of satisfied UEs.

Multi-PDSCH scheduling with single DCI assumes that the HARQ feedback is sent after all scheduled PDSCHs. However, such configuration reduces the time left before the PDB expires if the packet needs a ReTx. Given maximum of 8 slots to be scheduled and lower SCSs the delay before the ReTx will be possible can be significant. In order to reduce the delay and thus increase number of supported users, it is beneficial to study solutions that reduces the HARQ ACK/NACK delay.

Proposal 4: Study solutions that reduces HARQ ACK/NACK delay for multi-PDSCH/PUSCH scheduling with a single DCI for XR uses cases with large frame size, e.g., early HARQ ACK/NACK feedback.

### Extending capability of single DCI scheduling multi-PDSCHs/PUSCHs for FR2-2 to FR1/FR2

During RAN1 #110 the following agreement was made:

Agreement RAN1 to make decision on the following in RAN1#110bis-e - Support single DCI scheduling multi-PDSCHs/PUSCHs which is currently supported for FR2-2 to other SCS in FR1/FR2

Current multi-PxSCH scheduling with single DCI is supported for the following SCSs:

-	For multi-PUSCH/PDSCH scheduling: SCSs 120 kHz, 480 kHz, 960 kHz

-	For multi-PUSCH scheduling: SCSs 15 kHz, 30 kHz and 60 kHz are also supported in FR1.

Release 17 SI on XR considered SCS of 30 kHz (FR1) as well as 120 kHz (FR2) for the simulation purposes. The conclusion of TR 38.838 confirmed that NR can support XR. In case that multi-PDSCH scheduling is found beneficial for XR, we propose to extend the capability of multi-PDSCH scheduling to support lower SCS and frequency range. Moreover, the multi-PUSCH scheduling already supports the lower SCS and frequency ranges. In case there is consensus in RAN1 to extend the capability of supporting single DCI scheduling multi-PxSCHs for XR, it is worth to investigate further any essential gaps considering fulfilling the QoS requirements at least PDB and reliability of XR TBs. For example, multiple TBs over multi-PxSCHs corresponding to the same XR frame are actually transmitted in different TTIs which leads to different available time window for transmission of different TBs.

Proposal 5: Extend the capability of multi-PDSCH scheduling to lower SCS and frequency range in order to better serve XR use cases in various deployments.

### CBG indication in Multi-PUSCH/PDSCH scheduling

Considering the key characteristics of XR data traffic with very large payload, the re-transmission of the whole TB in case of an error is not the best solution. Therefore, a CBG-based Re-Tx can be considered as an attractive option for XR uses cases. Moreover, one video frame in many cases might be conveyed via multiple slots due to its large frame size. Thereby, multi-PxSCH scheduling with single DCI is also considered as a valuable approach as it can reduce the DCI overhead, decrease the power consumption due to reduced PDCCH monitoring occasions.

Current multi-PxSCH scheduling with single DCI does not support CBG-based transmission which limits the applicability of multi-PxSCH scheduling for XR use cases with very large payloads. We thus propose to study and consider the CBG-based transmission to be supported by multi-PxSCH scheduling with single DCI.

Proposal 6: Consider CBG-based transmission to be supported by multi-PxSCH scheduling with single DCI.

One of the changes compared to the legacy CBG-based single-PDSCH scheduling is the increase in the HARQ feedback overhead. For instance, if a total number of N PDSCH transmissions are co-scheduled and each contain M CBGs, the total number of HARQ ACK/NACK feedback is of size of M*N. When M = 8 and N = 8, the total HARQ ACK/NACK feedback can be signifant.

Observation 2: Size of HARQ feedback increases as in CBG-based multi-PDSCH transmissions.

One way to limit this additional overhead is to use some compression techniques to reduce the number of feedback bits. For instance, one possible approach is to consider to send the per CBG HARQ ACK/NACK feedback for the TBs that are in error.

Proposal 7: Consider the per CBG HARQ ACK/NACK feedback for the TBs that are in error to reduce the size of the HARQ feedback in case of CBG-based multi-PDSCH scheduling schemes are specified.

# Enhanced CQI feedback for CBG-based transmissions

During RAN1 #109-e the following agreement was made:

Agreement The following lists the candidate enhancements techniques for link adaptation to improve XR capacity that are proposed by companies RAN1#109-e. - At least the proponents are encouraged to justify the corresponding capacity benefits for XR traffic for considering potential study of these candidate enhancements techniques. - Delta MCS - Soft HARQ-ACK feedback - Cooperative MIMO scheme via precoding technique - bi-directional training - Enhanced link adaptation for CBG-based transmission - CSI report enhancements to address the different BLER requirements of different XR flows - Follow the common principle for assessment of the candidate capacity enhancement technique.

As discussed at RAN1#109-e and RAN1#110, there are opportunities to improve the XR capacity by relying on CBG-based transmissions if doing so in combination with enhanced link adaptation in the form of enhanced CQI feedback. This is considered attractive for XR cases where the transmitted payload (i.e. transport block – TB), is typically rather large, and hence retransmitting only the failed CBGs as compared to the full TB is a clear advantage.

The currently defined CQI feedback schemes for NR all rely on providing information to guide the gNB on the maximum modulation and coding scheme (MCS) that it can use without exceeding a certain first transmission BLER target. Such CQI solutions are attractive for the legacy HARQ transmission modes, where the full transport block (TB) is retransmitted in case of errors, and hence it makes sense to have CQI feedback that expresses the recommended MCS for a certain BLER of the first transmission of the TB. However, for CBG-based HARQ operation, the full TB is seldom retransmitted as only the potentially erroneously received CBGs are subject to retransmission. Given the large payloads for XR cases, CBG-based transmissions are therefore attractive as a mean to have resource efficient transmissions, and hence helps improve the overall capacity. However, in order to fully gain from CBG-based transmissions, we need to have efficient link adaptation. Our proposal is therefore to enhance CQI (eCQI) so guides the gNB on the maximum MCS scheme it can use while ensuring that only a certain maximum subset of CBGs will need retransmission with a controllable probability. E.g. have an eCQI scheme that guides the gNB to use a MCS index such that at most 4 CBGs (out of 8 CBGs) will require retransmission with P=0.1 probability (10%). This is clearly different from current CQI designs that only offers the possibility to select the MCS corresponding to a certain TB block error probability (BLEP), without controlling the CBG error probability and hence how many CBGs of a TB are in error. At RAN1#110 it was commented that the CBG error probability could be derived directly from the TB BLER via a simple equation as e.g. presented in R1-2205917. However, this is only true if we assume that the errors on CBG’s are independently identically distributed (i.i.d.) which is not the case in reality. Figure 1 is a snapshot of the CB and CBG SINR experienced by one UE in an indoor hotspot scenario. As can be seen, the assumption of similar CB and consecutively similar CBG SINR values does not hold. Therefore, assuming that the error probabilities are identical is not accurate.

| (a) CB SINR values in one TB | (b) CBG SINR values in the same TB |
| --- | --- |

Figure 1. The experienced CB and CBG SINR values for one example TB of a UE at 30Mbps in an Indoor Hotspot deployment in FR1 scenario.

Secondly, it was discussed that sub-band based CQI could also be used as mean to gain more insight on the CBG error probability. However, as CBGs are not localized in frequency such that a CBG is mapped to only one or few subbands, it is hard to estimate and control the CBG error probability of PDSCH transmission from the sub-band legacy CQI schemes. This leads us to the following observations

Observation 3: The legacy CQI scheme with a recommended MCS to obtain a certain TB BLER, offers no direct mean for the gNB to control the CBG error probability for PDSCH transmissions. Only under certain conditions of i.i.d. CBG errors is this feasible, which is usually not the case.

Observation 4: As the name indicates, sub-band based CQI offers the channel quality per sub-based. However, as the CBG mapping to PHY is not strictly localized in frequency, there is no one-to-one mapping of the sub-band per sub-band to the CBG error probability.

In order to overcome those shortcomings, we propose an eCQI as follows: The gNB configures the UE to use eCQI (CBG optimized) reporting where the UE shall estimate highest supported MCS (expressed via a CQI index), assuming that downlink transmissions occupy a group of downlink physical resource blocks termed the CSI reference resource with M code block groups, while the error probability of at most N failed code block groups does not exceed P. Parameters M, N, and P are may configured by the network, or fixed to values that are attractive for XR services. The configuration of the UE to use eCQI may be conducted with RRC signaling (note that configuration of current CQI schemes for a UE to use is also via RRC). For the NR, the maximum number of CBGs for a transmission is 8. For transmission of large XR payloads, the PDSCH transmissions are therefore often sent with 8 CBGs, and hence a reasonable choice is M=8. We have furthermore found that setting N=4 and P=50% (i.e. corresponding to half of the 8 CBGs are in error with 50% probability) results in a good performance. But, as mentioned, we suggest having those eCQI parameters configurable by the network, so they don’t need to be fixed in the specifications.

A general overview of the eCQI scheme can be summarized as the following: The UE performs measurements on the CSI reference resources to determine the received post-detection SINR (this is the same as done for the current legacy CQI schemes). Based on these measurements, the UE estimates the effective SINR for the M-different CBGs. The UE thereafter determines the highest MCS that it can support, while at most N out of the M CBGs are in error with probability P. This may be implemented in the UE for example having a table with CBG error rate vs effective SINR for the different MCS’s (just as current UE implementations has a table with effective TB error rate vs effective SINR). The reporting of the eCQI can be in the form of an eCQI index that points to a new eCQI table that enumerates the supported modulation scheme, effective code rate, and overall efficiency that it recommends the gNB to use for its PDSCH transmissions.

Notice that the eCQI does not involve any changes to the definitions of CSI measurement resources, and it does not results in high uplink reporting overhead, as the eCQI also is just a pointer to a CQI index table. Hence, it requires only modest specification changes as building on the existing CQI measurement and reporting framework. We summarize the suggested eCQI as follows:

Proposal 8: An enhanced CQI (eCQI) shall be further considered, where the UE measures on the CSI reference resources to determine the highest supported MCS, while at most N out of M CBGs are in error with probability P. The reporting of the eCQI is in form of an index to an eCQI table. Other options for eCQI schemes to gain the most for CBG-based XR transmissions are not excluded.

Proposal 9: Note that the proposed eCQI does not include any changes to the CSI-RS transmission, the specification changes are mainly limited to 3GPP TS 388.214 clause 5.2.2.1 as follows:

- The UE shall derive for each CQI value reported in uplink slot n the highest CQI index which satisfies the following condition: A single PDSCH transport block with a combination of modulation scheme, target code rate and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource with M code block groups, could be received with an error probability of at most N failed code block groups not exceeding:

Proposal 10: Signaling of the eCQI configuration parameters M, N, P may be included in CSI-ReportConfig and sent via RRC signaling (needs to be further discussed with RAN2).

To quantify the benefits of the improved link adaptation for CBG-based transmissions from the eCQI, we have run a series of XR system-level simulations in line with the agreed assumptions (3GPP TR 38.838 [2]).

| (a) AR/VR in FR1 at 30Mbps with X=99% | (b) CG in FR1 at 30Mbps with X=99% |
| --- | --- |

Figure 2. DL Capacity evaluation of the different link adaptation schemes (with/without eCQI) for CG traffic (Video) and AR/VR (Video) at 30Mbps in Indoor Hotspot deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

| (a) AR/VR in FR1 at 45Mbps with X=99% | (b) CG in FR1 at 45Mbps with X=99% |
| --- | --- |

Figure 3. DL Capacity evaluation of the different link adaptation schemes (with/without eCQI) for CG traffic (Video) and AR/VR (Video) at 45Mbps in Indoor Hotspot deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Figure 2 shows the percentage of satisfied users for Indoor Hotspot scenario in FR1 for different link adaptation algorithms with and without eCQI reporting. The detailed set of parameters is given in Appendix. For this simulation, a maximum rank of 1 is set. Moreover, the scenarios with data rate of 30Mbps with PDB = 10 ms for AR/VR and 15ms for CG are considered. Similarly, Figure 3 illustrates the results for the data rate of 45 Mbps. Two different algorithms are compared: legacy link adaptation with TB-based transmission and legacy CQI reporting and enhanced link adaptation with CBG-based transmission and enhanced CQI reporting with two parameter set versions. The CQI mechanism in the legacy algorithm is set to fix the BLER of the first transmission to 10% for the TBs. On the other hand, enhanced CQI is working with a fixed CBG error probability to have at most N/M =25% N/M =50% (blue curve in Figures 2 and 3) failed CBGs. As can be seen from the figure, the number of satisfied users has been increased in both CG and AR/VR use cases for both data rates. Moreover, the gain is still achieved for the stricter PDB of 10 ms for the AR/VR applications. We can also observe that for these simulation scenarios, the best N/M ratio is around 50% where the MCS selection is done in a way that in the worst-case scenarios at most half of the CBGs in one TB transmission will be successfully decoded. It is worth noting that the results presented in [4], the case with a rank adaptation mechanism with a maximum rank of 2 were presented that showed an improved XR capacity for the eCQI method. The main reason for eCQI gain is due to the ability of the eCQI method to tolerate a certain probability of CBG failure, P, that can be compensated (via retransmission) with very small resource consumption. On the other hand, this tolerance allows the possibility of choosing much higher MCS indices compared to legacy CQI method, leading to much better resource efficiency outcome.

# Discussion of soft HARQ feedback

Another method to improve the HARQ operation is to replace the Boolean ACK/NACK feedback with multi-bit feedback that expresses the decoder state information (DSI). The DSI conveys information on “how close” the receiver was at being able to correctly decode a failed HARQ transmission. DSI-rich HARQ feedback allows more accurate redundancy version matching of the retransmission. As an example, for the log maximum a-posteriori (LogMAP) receivers (decoder), the output LLRs (Log Likelihood ratios) can be used as an estimate of the DSI. Such options of HARQ enhancements were earlier studied for NR Rel-15 and Rel-16, but without being standardized, while instead the CBG-based HARQ enhancement got standardized (that also introduces multi-bit HARQ feedback in the form of per CBG ACK/NACK). For further studying soft HARQ feedback schemes with quantized feedback of DSI/LLRs, we therefore recommend that the CBG-based HARQ retransmissions is taken as the baseline reference. This is particularly relevant for XR use cases where the TB size is typically rather large, and hence CBG-based HARQ is attractive.

Proposal 11: The baseline for comparing the potential performance benefits of soft HARQ feedback schemes (e.g. with quantized DSI/LLR feedback) shall be CBG-based HARQ as this one is attractive for XR cases with large TB sizes.

# Scheduling restrictions due to RRM measurements

In this section we discuss the impacts of scheduling restrictions imposed by RRM measurements in intra- and inter-frequency case and discuss possible methods to avoid the capacity impact due to these. In Section 6.1 we discuss the UE based methods to avoid the impact due to scheduling restrictions, and in Section 6.2 the network based schemes.

## UE based scheduling restrictions avoidance

As discussed at RAN1#109-e, UEs performing RRM measurements does not come for free, as there are cases where this imposes scheduling restrictions. Either due to potential measurement gaps for inter-frequency RRM measurements, of alike restrictions for FR2 intra-frequency RRM measurements as is discussed in greater details in this section.

In line with the Rel-17 XR simulation assumptions (3GPP TR 38.838 [2]) for FR2, UEs are equipped with multiple antenna panels, where UEs may only be able to measure (and Tx/Rx) on a single panel at a time. The UE selects its best antenna panel for Tx/Rx with its serving cell based on local RRM (RSRP) measurements. The UE also perform intra-frequency RRM measurements for mobility and beam management purposes. However, for FR2 operation, it is important to notice that there are additional scheduling restrictions due to intra-frequency RRM measurements that challenge the XR performance. Notice that earlier FR2 XR performance results included in the Rel-17 XR TR 38.838 did not consider the effects of such measurement restrictions.

As per the current NR specifications, the network configures the UE when to measure RSRP from e.g., SSBs by means of RRC signalling of the so-called SMTC (see section 5.5.2.10 in 38.331). The time-resolution of SMTC is on subframe level, corresponding to 1 ms intervals. It should be noted that the SMTC only instruct the UE when (in time domain) it could/should measure RSRP, while it is left completely open for UE implementation exactly when to measure, and which antenna panel to be used for conducting such measurement during those “SMTC measurement windows”.

Scheduling restrictions apply for the UEs during time-intervals where it may be performing RSRP measurements as per the SMTC configuration appears in 38.133, Section 9.5.6.3. In particular for FR2 and L1-RSRP on SSB, “The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS….”. Typical network configurations use a setting with SMTC windows of 5 ms every 20 ms (aligned to SSB periodicity), meaning that this poses serious scheduling restrictions that likely challenge the networks capability to efficiently schedule and serve its XR users according to their QoS constraint, severely limiting the XR capacity if such scheduling restrictions are valid. Accounting that scheduling restrictions would apply on SSBs to be measured, starting from one symbol before and ending one symbol after, it in practice means that every slot where SSBs are to be measured is restricted from PDSCH perspective, resulting that nearly 20% of the time (i.e., SMTC windows of 5ms every 20ms time-period) can be blocked if 64 SSBs are to be measured.

Secondly, current NR specs does not mention anything related to how/when the UE shall/may measure on different antenna panels, and when to only measure on its currently selected best antenna panel. The timing of the SMTC windows of 5 ms with scheduling restrictions, as well as the arrival of XR frames is illustrated in Fig. 4. As can be seen, the SMTC induced scheduling restrictions come every 20 ms, while the average XR frame inter-arrival time is 16.6 ms (assuming 60 fps). On top of that, each XR frame arrival is subject to +/-4ms jitter as illustrated with the dashed line. From that figure it is visible that the SMTC windows with scheduling restrictions often collides with time periods where the gNB would have preferred to schedule the XR transmission. This will impact the experienced XR QoE, as well as negatively impacting the obtained network XR capacity.

Figure 4. Sketch of timing of SMTC windows with scheduling restrictions as well as arrival of XR frames.

In order to further assess the system-level performance degradation from the mentioned scheduling restrictions, we have been running a series of FR2 system-level simulations in line with the assumptions in 3GPP TR 38.838 [2], considering only DL XR traffic. We have conducted simulations without any scheduling restrictions (as companies also report in TR 38.838) and simulations with scheduling restrictions every 20 ms time period for an SMTC window of 5 ms. These simulations are conducted for traffic with a PDB of 10 ms and 15 ms and a source data rate of 30 Mbps, assuming the default setting of 60 fps. The simulated scenario is Dense Urban (DU) with details given in Appendix. Fig. 5 summarizes the performance, and it is clearly observed that including the effects of such scheduling restrictions will severely reduce the XR capacity. For these particular results, we observe an XR capacity reduction of 4 users per cell both for CG and AR/VR applications (i.e., from 10 CG users per cell down to 6 CG users per cell and 7 AR/VR users per cell down to 3 AR/VR users per cell) when accounting for the measurement restrictions. This leads us to draw the following observation:

Observation 5: Scheduling XR users with 60 fps according to the agreed QoS constraints in 3GPP TR 38.838 is seriously challenged for FR2 if subject to scheduling restrictions with SMTC windows of 5 ms every 20 ms time-period. System-level performance results confirm that this severely impacts network XR capacity.

| (a) CG in FR2 at 30Mbps with X=99% | (b) AR/VR in FR2 at 30Mbps with X=99% |
| --- | --- |

Figure 5. Percentage of satisfied XR users obtained from system-level simulations for DU at FR2 with 30 Mbps and PDBs of 10ms and 15 ms, with/without scheduling restrictions during SMTC windows of 5 ms for every 20 ms time period.

The current NR specs also allow the network to configure the UE with a search threshold (s-MeasureConfig) for a UE in connected mode to enable reduction of the intra-frequency measurement effort. 3GPP TS 38.331 defines:

- s-MeasureConfig: Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS

If the network has configured the s-MeasureConfig threshold allowing the UE not to perform measurements on non-serving cells, including the intra-frequency neighbor cells, there may be unused scheduling opportunities in those cases where the UE is not performing intra-frequency measurements, but the network is not aware of this, and hence obeys the defined scheduling restrictions. It would therefore be an advantage to “build awareness” at the network on whether the UE performs intra-freq measurements (as per the s-MeasureConfig criteria), so it knows if scheduling restrictions apply. It is therefore proposed that the UE informs the gNB (network) when it is, or is not, having scheduling restrictions due to performing intra-frequency measurements if the UE is configured with s-MeasureConfig. Such UE to gNB signaling may be specified by means of PHY or MAC layer signaling. There needs to be one indication to inform when the UE applies the scheduling restrictions due to such RRM measurements, and one to indicate when such scheduling restrictions no longer applies. This way, the gNB MAC scheduler can account for the potentially applied scheduling restrictions on UE (due to performing intra-frequency measurements), and benefit from the increased scheduling opportunities when no scheduling restrictions are present. This leads to the following proposal:

Proposal 12: For UEs that are configured with s-MeasureConfig, additional UE-to-gNB signaling shall be introduced to make the gNB scheduler aware of when scheduling restrictions apply. The solution may include signaling when the UE starts and stops making intra-freq measurements as per the s-MeasureConfig.

As UE evaluated the s-MeasureConfig against its L3 filtered serving cell measurement, the proposed UE-2-gNB signaling does not necessary need to be very fast, and hence higher-layer signaling could be considered depending on RAN2 views as follows:

Proposal 13: The UE-to-gNB signaling to make the gNB scheduler aware of when s-MeasureConfig induced scheduling restrictions apply could be realized with higher-layer signaling such MAC CE or RRC signaling. RAN2 shall be asked for further guidance.

The exact gain of the proposal depends on the network topology and how the operator chose to set s-MeasureConfig. If s-MeasureConfig is set so that only few percentage of the UEs per cell actually perform intra-freq measurements, the XR capacity gains shown in Fig. 5 would be largely be valid. For cases where the network operator e.g. set s-MeasureConfig so that on average 50% of the UEs are making intra-freq RRM measurements, the effective gains would be at least half of those results reported in Fig. 5. Hence, the gains are still significant. Setting s-MeasureConfig so that 50% of the UEs are performing intra-freq RRM measurements seems like a reasonably conservative setting for the XR Dense Urban scenario, where majority of UEs are only slow moving.

## Network based scheduling restrictions avoidance due to inter-freq meas. gaps

During RAN1 109-e the following agreement was made:

Agreement The following lists the candidate enhancements techniques based on measurement-gap link to improve XR capacity that are proposed by companies RAN1#109-e. - At least the proponents are encouraged to justify the corresponding capacity benefits for XR traffic for considering potential study of these candidate enhancements techniques. - Dynamic L1 based MG activation/deactivation. - Reuse current R16/R17 RRM relaxation condition to allow scheduling in MG to transform the R16/R17 RRM power saving gain into capacity gain. - Follow the common principle for assessment of the candidate capacity enhancement technique.

As discussed in earlier RAN1 meetings, measurement gaps (MG) for performing inter-frequency RRM measurement will also induce scheduling restrictions, having alike negative effects as discussed in Section 5.1, for gap-assisted UEs (see e.g. 3GPP TS 38.300). Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE, the active BWP of the UE and the current operating frequency. For SSB based inter-frequency measurement, if the measurement gaps are required by the UE, a measurement gap configuration will be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following cases: (i) if the UE only supports per-UE measurement gaps, (ii) if the UE supports per-FR measurement gaps and any of the serving cells are in the same frequency range of the measurement object. During inter-frequency measurement gaps, the UE is not schedulable. Typically, the network configures the UE to perform inter-frequency measurements more seldomly as compared to intra-frequency RRM measurements, for instance only if RRM measurement event A2 (serving cell quality is lower than threshold) is triggered, if the UE asked to start measuring cells on other frequencies for potential inter-frequency handovers. The leads to the following observations:

Observation 6: UEs that are configured with gap-assisted inter-frequency measurements are not schedulable during such gaps, and hence will impact the XR performance negatively as also reported in Section 6.1.

Observation 7: Inter-frequency measurement gaps are configured more seldomly for UEs as compared to intra-frequency RRM measurements (SMTC windows), and hence the problems associated with intra-freq RRM measurements (i.e. scheduling restrictions) shall be addressed first as discussed in Section 6.1.

Considering also the XR uplink traffic with Pose information every 4ms, the impact of measurement gaps present even more challenges as compared to those for DL XR traffic with 60 frames per second.

A possible solution to address XR scheduling problems caused by the UE performing gap assisted inter-frequency measurement may encompass introducing on-demand mechanisms where the gNB can signal to the UE to skip the measurement gap shortly before entering such gaps, and hence enable scheduling of the UE during the gap. This would essentially allow the gNB to temporarily de-activate an inter-frequency measurement gap on-demand that would otherwise prevent the gNB to timely schedule the XR payload(s). The gNB would take advantage of such techniques when there are urgent XR data pending for scheduling that can not afford being postponed until after the measurement gap. The gNB-2-UE signaling for this may be realized via a new compact DCI format. We therefore propose:

Proposal 14: For UEs configured with inter-frequency measurement gaps, solutions where the gNB can signal the UE to skip a measurement gap (to avoid scheduling restrictions) shall be captured in the TR. The gNB-2-UE signaling for this may be realized via a compact DCI format to have fast signaling.

As in case of inter-frequency, also the SMTC induced scheduling restrictions in intra-frequency at FR2, discussed in Section 6.1, we also propose to consider the scheme where the gNB can configure a UE to prioritize decoding of potentially critical PDCCH/PDSCH transmissions from its serving cell, even if colliding with SMTC windows where the UE may perform RSRP measurements. This kind of configuration for the UE also serves as instructing to the UE in FR2 to prioritize listening to its current best antenna panel for reception from its serving cell during such time-instances, so the UE does not switch panels at those times for RRM measurements. This would allow the gNB to configure UEs with a time-domain pattern to prioritize PDCCH/PDSCH decoding (even if colliding with SMTC windows) in coherence with the desired XR scheduling (transmission) opportunities. Furthermore, an on-demand solution, as discussed for inter-frequency measurements gaps where the gNB can signal to the UE to disregard scheduling restrictions shortly before entering a window of SMTC scheduling restrictions to de-active the restriction shall be considered. This would essentially allow the gNB to de-active the SMTC window (scheduling restrictions) that would otherwise prevent the gNB to timely schedule the XR payload(s). Given these considerations, we propose the following:

Proposal 15: FR2 solutions for the gNB to instruct the UE to prioritize PDCCH/PDSCH decoding during a sub-set of SMTC windows shall be captured in the TR such that XR payloads can be scheduled timely without unnecessary scheduling restrictions.

Proposal 16: The gNB could use RRC signaling to instruct the UE with a time pattern where it always will have to prioritize PDCCH/PDSCG decoding even if colliding an SMTC window. RAN1 should ask RAN2’s opinion on this.

Secondly, as proposed above for inter-frequency measurement gaps, we should consider a dynamic solution also for FR-2 scheduling restrictions, where the gNB, shortly before (say up to few milliseconds) an SMTC, should be able to signal the UE to prioritize PDCCH/PDSCH decoding during this particular SMTC window. We refer to this as the on-demand solution as it is sent on-demand to prioritize scheduling for a single SMTC window. For this, we propose to use fast layer-1 signaling in the form of PDCCH signaling.

Proposal 17: Fast on-demand signaling from the gNB to UE to instruct it to prioritize PDCCH/PDSCH decoding during the next SMTC window shall be supported by means of PDCCH signaling.

The actual scheduling of the measurements is up to the UE. Thus, for a given measurement gap or scheduling restriction, it is up to the UE determine how and to which measurements the said occasion is used. If the occasion would have been intended by the UE for neighbor cell measurements, and UE could not re-schedule and compensate the loss of L1 RRM measurement sample in a latter occasion, it might be necessary to account this in L3 measurement sample. As is specified in the RRC specifications (3GPP TS 38.331), the layer-1 (L1) RRM measurements are further filtered at Layer-3 (L3) as configured in the mobility measurement object with the purpose of e.g. removing the impact of fast fading. The L3 filtered RRM measurements are those that are typically used to trigger RRM events such as e.g. A3 as commonly used for handovers. The L3 filter is of the type infinite impulse response (IIR), assuming input sample of L1 RRM measurements at a regular interval. For cases proposed above where UE prioritize PDCCH/PDSCH decoding during a SMTC window or measurement gap, and hence is skipping one L1 RRM measurement, it shall therefore be defined how this is handled at the interface to the L3 filter. For such cases we simply propose adopt a simple solution, where the latest L1 RRM measurement is forwarded to the L3 filter, if the anticipated L1 RRM measurement is skipped due to prioritization of PDCCH/PDSCH decoding in an SMTC window. This would essentially mean that no spec changes are needed for the L3 RRM filtering mechanism in the RRC specifications.

Proposal 18: If a L1 RRM measurement in a measurement gap or  SMTC window is skipped due to prioritization of PDCCH/PDSCH, the latest L1 RRM measurement is forwarded to the L3 filter at RRC.

# Conclusion

In this contribution, we discussed the solutions that can further improve the capacity performance and increase the number of satisfied UEs.

The following observations have been made:

Observation 1: Using CG presents attractive savings in DL signaling overhead as much fewer DCI transmissions for UL traffic are needed. This may also materialize in reduced UE power consumption from having to monitor and decode fewer PDCCH transmissions.

Observation 2: Size of HARQ feedback increases as in CBG-based multi-PDSCH transmissions.

Observation 3: The legacy CQI scheme with a recommended MCS to obtain a certain TB BLER, offers no direct mean for the gNB to control the CBG error probability for PDSCH transmissions. Only under certain conditions of i.i.d. CBG errors is this feasible, which is usually not the case.

Observation 4: As the name indicates, sub-band based CQI offers the channel quality per sub-based. However, as the CBG mapping to PHY is not strictly localized in frequency, there is no one-to-one mapping of the sub-band per sub-band to the CBG error probability.

Observation 5: Scheduling XR users with 60 fps according to the agreed QoS constraints in 3GPP TR 38.838 is seriously challenged for FR2 if subject to scheduling restrictions with SMTC windows of 5 ms every 20 ms time-period. System-level performance results confirm that this severely impacts network XR capacity.

Observation 6: UEs that are configured with gap-assisted inter-frequency measurements are not schedulable during such gaps, and hence will impact the XR performance negatively as also reported in Section 6.1.

Observation 7: Inter-frequency measurement gaps are configured more seldomly for UEs as compared to intra-frequency RRM measurements (SMTC windows), and hence the problems associated with intra-freq RRM measurements (i.e. scheduling restrictions) shall be addressed first as discussed in Section 6.1.

The following proposals have been made:

Proposal 1: The options for configuration of the periodicity of CG resources shall match those of the uplink XR traffic models from 3GPP TR 38.838. Among others, this shall include a periodicity of every 16.6 ms (as required for 60 fps uplink video).

Proposal 2: It should be possible to have CG configuration that includes an integer number of TTI transmissions per periodicity, e.g., in the range from 1 to 8.

Proposal 3: RAN1 to study potential CG enhancements, for example, temporarily deactivating selected CG resources within one CG period.

Proposal 4: Study solutions that reduces HARQ ACK/NACK delay for multi-PDSCH/PUSCH scheduling with a single DCI for XR uses cases with large frame size, e.g., early HARQ ACK/NACK feedback.

Proposal 5: Extend the capability of multi-PDSCH scheduling to lower SCS and frequency range in order to better serve XR use cases in various deployments.

Proposal 6: Consider CBG-based transmission to be supported by multi-PxSCH scheduling with single DCI.

Proposal 7: Consider the per CBG HARQ ACK/NACK feedback for the TBs that are in error to reduce the size of the HARQ feedback in case of CBG-based multi-PDSCH scheduling schemes are specified.

Proposal 8: An enhanced CQI (eCQI) shall be further considered, where the UE measures on the CSI reference resources to determine the highest supported MCS, while at most N out of M CBGs are in error with probability P. The reporting of the eCQI is in form of an index to an eCQI table. Other options for eCQI schemes to gain the most for CBG-based XR transmissions are not excluded.

Proposal 9: Note that the proposed eCQI does not include any changes to the CSI-RS transmission, the specification changes are mainly limited to 3GPP TS 388.214 clause 5.2.2.1 as follows:

- The UE shall derive for each CQI value reported in uplink slot n the highest CQI index which satisfies the following condition: A single PDSCH transport block with a combination of modulation scheme, target code rate and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource with M code block groups, could be received with an error probability of at most N failed code block groups not exceeding:

Proposal 10: Signaling of the eCQI configuration parameters M, N, P may be included in CSI-ReportConfig and sent via RRC signaling (needs to be further discussed with RAN2).

Proposal 11: The baseline for comparing the potential performance benefits of soft HARQ feedback schemes (e.g. with quantized DSI/LLR feedback) shall be CBG-based HARQ as this one is attractive for XR cases with large TB sizes.

Proposal 12: For UEs that are configured with s-MeasureConfig, additional UE-to-gNB signaling shall be introduced to make the gNB scheduler aware of when scheduling restrictions apply. The solution may include signaling when the UE starts and stops making intra-freq measurements as per the s-MeasureConfig.

Proposal 13: The UE-to-gNB signaling to make the gNB scheduler aware of when s-MeasureConfig induced scheduling restrictions apply could be realized with higher-layer signaling such MAC CE or RRC signaling. RAN2 shall be asked for further guidance.

Proposal 14: For UEs configured with inter-frequency measurement gaps, solutions where the gNB can signal the UE to skip a measurement gap (to avoid scheduling restrictions) shall be captured in the TR. The gNB-2-UE signaling for this may be realized via a compact DCI format to have fast signaling.

Proposal 15: FR2 solutions for the gNB to instruct the UE to prioritize PDCCH/PDSCH decoding during a sub-set of SMTC windows shall be captured in the TR such that XR payloads can be scheduled timely without unnecessary scheduling restrictions.

Proposal 16: The gNB could use RRC signaling to instruct the UE with a time pattern where it always will have to prioritize PDCCH/PDSCG decoding even if colliding an SMTC window. RAN1 should ask RAN2’s opinion on this.

Proposal 17: Fast on-demand signaling from the gNB to UE to instruct it to prioritize PDCCH/PDSCH decoding during the next SMTC window shall be supported by means of PDCCH signaling.

Proposal 18: If a L1 RRM measurement in a measurement gap or  SMTC window is skipped due to prioritization of PDCCH/PDSCH, the latest L1 RRM measurement is forwarded to the L3 filter at RRC.

# References

[1] RP-220285, New SID “Study on XR Enhancements for NR”, RAN#95.

[2] TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17)

[3] TR 22.847, Study on supporting tactile and multi-modality communication services, v18.2.0

[4] R1- 2206226, XR-specific capacity enhancements, Nokia, NSB, RAN#110

# Appendix – Simulation settings for capacity enhancements

In this Appendix, we summarize the main simulation settings used for capacity enhancements evaluation.

## Indoor Hotspot (InH) scenario

The carrier frequency is set to 4 GHz and 30 GHz for FR1 and FR2, respectively. System bandwidth is assumed to be 100 MHz for FR1 and for FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz and 120 kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1, and 24dBm with (23dBm per 80MHz) in FR2. Table 1 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 1 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |  |
| Channel model | InH |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |
| System bandwidth | FR1: 100 MHz | FR2: 100 MHz |
| BS height | 3 m |  |
| UE height | 1.5 m |  |
| BS noise figure | FR1: 5 dB | FR2: 7 dB |
| UE noise figure | FR1: 9 dB | FR2: 13 dB |
| BS receiver | LMMSE-IRC |  |
| UE receiver | LMMSE-IRC |  |
| Channel estimation | Realistic (with ideal CSI) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | FR1: 31dBm (24dBm per 20MHz) | FR2: 24 dBm (23dBm per 80MHz) |
| UE Tx max power | 23 dBm |  |
| TDD Frame structure | DDDSU |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |  |
|  | Configuration in FR1: - 32 TxRU - (M, N, P, Mg, Ng) = (4,4,2,1,1) - (dH, dV) = (0.5, 0.5)λ - (Mp, Np) = (4,4) | Configuration in FR2: - 2 TxRU - (M, N, P, Mg, Ng) = (16, 8, 2,1,1) - (dH, dV) = (0.5, 0.5) λ - (Mp, Np) = (1,1) Grid of Beams: - Azimuth angles (degrees): {90, 90, 90, 112.5, 112.5, 112.5, 67.5, 67.5, 67.5, 140, 140, 140, 40, 40, 40} - Elevation angles (degrees): {-30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30} |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) | - Pattern: UE radiation pattern model 1 (TR 38.901) - Gain: 5 dBi - Configuration (Option 1): (M, N, P) = (1, 4, 2), (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,1) 3 panels (left, right, top) |
| Scheduler | SU-MIMO, Proportional Fairness |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |

## Dense Urban (DU) Scenario

The carrier frequency is set to 4 GHz and 30GHz for FR1 and FR2, respectively. System bandwidth has been fixed to 100MHz for both FR1 and FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRB) of 12 subcarriers, each of 30kHz and 120kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmissions before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 51 dBm (i.e., 44dBm for 20MHz). Table 2 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 2– Main parameters for Dense Urban deployment

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |  |
| Channel model | Uma |  |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |  |
| System bandwidth | FR1: Option 1: 100 MHz | FR2: Option 1: 100 MHz |  |
| BS height | 25m |  |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |  |
| BS noise figure | FR1: 5 dB |  | FR2: 7 dB |
| UE noise figure | FR1: 9 dB |  | FR2: 13 dB |
| BS receiver | MMSE-IRC |  |  |
| UE receiver | MMSE-IRC |  |  |
| Channel estimation | Realistic (with ideal CSI) |  |  |
| UE speed | 3 km/h |  |  |
| MCS | Up to 256QAM |  |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |  |
| UE Tx max power | FR1: 23 dBm |  | FR2: 23 dBm, maximum EIRP 43 dBm |
| TDD Frame structure | Option 1: DDDSU |  |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |  |
| Cell Selection | RSRP Slow Fading |  |  |
| BS antenna configuration | 3-sector antenna radiation pattern, 8 dBi |  |  |
|  | FR1: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: 2TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1), (dH, dV) = (0.5λ, 0.5λ) Grid of Beams - Azimuth angles: {33.75, 56.25, 78.75, 101.25, 123.75, 146.25, 33.75, 56.25, 78.75, 101.25, 123.75, 146.25} degrees - Elevation angles: {-12.5, -12.5, -12.5, -12.5, -12.5, -12.5, -57.5, -57.5, -57.5, -57.5, -57.5, -57.5} degrees |  |
| UE antenna configuration | FR1: Omni-directional, 0 dBi | FR2: UE antenna radiation pattern model 1, 5dBi |  |
|  | FR1: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, -N/Aλ) | FR2: (M, N, P)=(1, 4, 2), 3 panels (left, right, top) (Mp, Np)=(1, 1) |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |  |
| Scheduler | SU-MIMO, Proportional Fairness |  |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |  |
| PDCCH overhead | Modelled |  |  |
| Target BLER | 10% for first transmission |  |  |
| Max HARQ transmission | 3 |  |  |
| HARQ scheme | Chase combining |  |  |