3GPP TSG-RAN WG1 #110bis-e	R1-2209550

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | DRAFT | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of LI bitwidth and mapping order of CSI fields for Further Enhanced Type II Port Selection CSI feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current TS 38.214, there is no restriction on LI configuration for Further Enhanced Type II Port Selection CSI feedback. The UE shall calculate and report LI when reportQuantity in CSI-ReportConfig contains LI parameter. However, it is not captured in the bitwidth and mapping order of CSI fields for Further Enhanced Type II Port Selection CSI feedback in 212 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in section 6.3.2.1.2 that the bitwidth and mapping order of CSI fields for LI should be captured in 212 specs for Further Enhanced Type II Port Selection CSI feedback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to report LI when reportQuantity contains LI for Further Enhanced Type II Port Selection CSI feedback. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.3	CSI reporting using PUSCH

<Unrelated part omitted>

Table 6.3.2.1.2-5B: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-PortSelection-r17

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | Layer Indicator as in Table 6.3.2.1.2-9, if reported |
|  | PMI fields X1, from left to right as in Tables 6.3.2.1.2-2B, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*K1M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |

<Unrelated part omitted>

The bitwidth for RI/LI/CQI of codebookType=typeII-PortSelection-r17 is provided in Table 6.3.2.1.2-9.

Table 6.3.2.1.2-9: RI, LI and CQI of codebookType=typeII-PortSelection-r17

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Layer Indicator | log2v |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.7 [6, TS 38.214], v is the value of the rank,  K0=K1Mβ, where K1, M, and β are given by Clause 5.2.2.2.7 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.7 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1.

<Unrelated part omitted>