3GPP TSG RAN WG1 #110bis-e		R1-2209559

e-Meeting, October 10th – 19th, 2022

Source: 	Apple Inc.

Title:                     Maintenance of further enhancements on MIMO for NR

Agenda item:	8.1

Document for:	Discussion and Decision

# 1 Introduction

In the RAN1 110 meeting, several CRs were agreed to fix the problems identified for various topics of Rel-17 feMIMO WI, including enhancement for multi-beam operation and enhancement for mTRP operation [1].

In this contribution, we provide further inputs for the remaining open issues to complete the work item.

# 2. Discussions

In RAN1 110 meeting, one controversial issue extensively discussed without consensus is how to determine the TCI state for a ‘CB-based’ and ‘non-CB’ based PUSCH transmission when the Tx beams for a PUSCH and the associated SRS resources are different in [2]. The problem can occur when the SRS for CB or non-CB transmission is configured not to follow the unified TCI state, but the PUSCH always follows the indicated TCI state.

Four alternatives and corresponding TPs were brought up already in last RAN1 meeting:

- Alt.1: Using a spatial domain filter that is same as the spatial domain transmission filter associated with the indicated SRI.

- Alt.2: UE expects that only single-layer PUSCH transmission can be scheduled by DCI format 0_1 or 0_2.

- Alt.3: Deferring the indicated DLorJointTCIState or UL-TCIstate to be applied after most recent SRS transmission applying the indicated DLorJointTCIState or UL-TCIstate.

- Alt.4: A UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are equal to the indicated [TCI-State] with [tci-StateId_r17].

Among these alternatives, Alt.3 has a clear drawback of a larger latency for unified TCI update. Considering the restriction that only one SRS resource set is allowed for CB-based and non-CB based PUSCH transmission and ‘follow unified TCI’ flag is configured per SRS resource set, if an SRS resource set for PUSCH is configured not to follow unified TCI, the SRS transmission applying the indicated TCI may not occur or happen with a large delay.

The Alt.2 is mainly motivated by the consideration that single layer transmission is not sensitive to spatial relation misalignment. On the other hand, the uplink throughput performance is unnecessarily degraded, which is therefore less attractive.

Between Alt.1 and Alt.4, the latter option is preferrable which is inline with the basic design principle of Rel-17 unified framework i.e., unicast PUSCH always follows indicated TCI state without any exception. It is our view that the SRI field for non-CB PUSCH and TPMI field for CB-PUSCH can still be utilized to indicate the precoder for the corresponding PUSCH in Alt.4.

Proposal 1:

- Adopt Alt.4 for TCI determination of PUSCH transmission i.e., A UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are equal to the indicated [TCI-State] with [tci-StateId_r17].

A text proposal implementing Proposal 1 is as follows:

Reason for changes The procedure of antenna port determination for CB-based and non-CB based UL transmission are unclear. Summary of changes State that a UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are equal to the indicated [TCI-State] with [tci-StateId_r17]. Specs/Sections impacted 38.214 Section 6.1.1.1 and Section 6.1.1.2 Consequences if not approved Incomplete specification for CB-based and non-CB based UL transmission. ------------------- Text Proposal (TP#2) for 38.214, Section 6.1.1.1 and Section 6.1.1.2 ---------------------------- ### 6.1.1.1 Codebook based UL transmission *** Unchanged text omitted *** The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3. If an SRS resource set with parameter usage set to 'codebook' is not configured with followUnifiedTCIstate, the UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are always equal to the indicated [TCI-State] with [tci-StateId_r17]. ….. ### 6.1.1.2 Non-Codebook based UL transmission *** Unchanged text omitted *** The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as . If an SRS resource set with parameter usage set to 'nonCodebook' is not configured with followUnifiedTCIstate, the UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are equal to the indicated [TCI-State] with [tci-StateId_r17]. …… ----------------------------------------------------------- End Text Proposal -----------------------------------------------------------

# 3. Conclusion

In this contribution, we have presented our views on the support of two TA values for multi-DCI mTRP operation. Based on the discussions, the following was proposed:

Proposal 1:

- Adopt Alt.4 for TCI determination of PUSCH transmission i.e., A UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are equal to the indicated [TCI-State] with [tci-StateId_r17].

Reason for changes The procedure of antenna port determination for CB-based and non-CB based UL transmission are unclear. Summary of changes State that a UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are equal to the indicated [TCI-State] with [tci-StateId_r17]. Specs/Sections impacted 38.214 Section 6.1.1.1 and Section 6.1.1.2 Consequences if not approved Incomplete specification for CB-based and non-CB based UL transmission. ------------------- Text Proposal (TP#2) for 38.214, Section 6.1.1.1 and Section 6.1.1.2 ---------------------------- ### 6.1.1.1 Codebook based UL transmission *** Unchanged text omitted *** The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3. If an SRS resource set with parameter usage set to 'codebook' is not configured with followUnifiedTCIstate, the UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are always equal to the indicated [TCI-State] with [tci-StateId_r17]. ….. ### 6.1.1.2 Non-Codebook based UL transmission *** Unchanged text omitted *** The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as . If an SRS resource set with parameter usage set to 'nonCodebook' is not configured with followUnifiedTCIstate, the UE shall expect that the configured [TCI-State]s with [tci-StateId_r17] of the SRS resource(s) are equal to the indicated [TCI-State] with [tci-StateId_r17]. …… ----------------------------------------------------------- End Text Proposal -----------------------------------------------------------

# References

- RAN1 110 e-meeting Chairman notes

- R1-2207955	Moderator Summary #1 for Maintenance on Rel-17 Multi-Beam	Moderator (ZTE)