3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209563

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.214 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CPS monitoring length during sidelink DRX inactive time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When a UE performs PBPS and CPS with periodic reservation, the sensing window is M consecutive logical slots earlier than slot ty0'SL till Tproc,0SL+Tproc,1SL slots earlier than ty0'SL, where ty0'SL is the first slot of the selected Y candidate slots of PBPS. When a UE performs CPS with aperiodic reservation, the CPS monitoring window starts at least M logical slots before ty0'SL till Tproc,0SL+Tproc,1SL slots earlier than ty0'SL. In both cases, the total number of logical slots that UE performs CPS sensing can be less than M. However, when a UE performs CPS on the slots in SL DRX inactive time, UE monitors a minimum of M slots. The CPS monitoring window size in case of SL DRX inactive time is even larger than the CPS monitoring window size in case of without SL DRX configuration. This additional sensing is unnecessary and is against the intention of power saving. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE follows the similar CPS monitoring window size in case of SL DRX inactive time as that in case of without SL DRX configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | With processing time of sensing results, the CPS monitoring window starts more than M logical slots before the first slot of the selected candidate slots. This does not help in detecting aperiodic resource reservation and wastes power for sensing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=================================== Start of Text Proposal =================================

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

< Unchanged parts are omitted >

Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given Prsvp_TX, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y or Y' candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic or sl-CPS-WindowAperiodic when Prsvp_TX≠0 or Prsvp_TX=0, respectively; otherwise, M equals 31.

< Unchanged parts are omitted >

=================================== End of Text Proposal ==================================