3GPP TSG RAN WG1 #110bis-e																			R1-2209567

e-Meeting, October 10th – 19th, 2022

Agenda item:	8.16.2

Source: 	Apple Inc.

Title: 	View on Rel-17 UE features

Document for:	Discussion/Decision

- Views on Rel-17 FeMIMO UE features

In the last RAN1#110 meeting, further progress has been made in Rel-17 FeMIMO UE features [1]. In this contribution, we provide our view on a few remaining issues regarding Rel-17 FeMIMO UE features.

## FG 23-1/10-x (beam management)

In Rel-17 FeMIMO, we introduced two inter-cell operation enhancement especially regarding SSB measurement, one for inter-cell beam management (BM), and the other one is for inter-cell multi-TRP operation.

- Inter-cell beam management (BM) is covered by FG23-1-2

- Inter-cell multi-TRP operation is covered by FG23-4

It is important to note that inter-cell BM and inter-cell multi-TRP should be two indepdent UE features since these two features are very likely to be deployed independently. For example, inter-cell BM can be deployed without deploying inter-cell multi-TRP. As results, component 2 and 3 in FG23-4 regarding X1 and X2 should be replicated for inter-cell BM. We proposal to add the following new FG,

| 23. NR_FeMIMO | 23-1-2a | Inter-cell beam measurement and reporting | 1. The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 2. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 | FG23-1-2 | Yes |  |  | per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,3,4,5,6,7} Component 2 candidate values: {0,1,2,3,4,5,6,7} Note: case1 and case2 cannot be enabled simultaneously as any configuration that is not based on Case 1 is defined as Case 2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## FG 23-7-x (mTRP CSI enhancement)

- We propose to introduce FG23-7-6 for the support of CSI-IMR

| 23. NR_FeMIMO | 23-7-6 | Support of CSI-IM for CSI enhancement for multi-TRP | Support CSI-IM for CSI enhancement for Multi-TRP | 23-7-1 | Yes |  |  | Per UE | n/a | Yes | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Reference

[1] R1-2207923, Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday, 3GPP TSG RAN WG1 #110 Toulouse, France, August 22nd – 26th, 2022