3GPP TSG RAN WG1 #110bis-e                                                              R1-2209573

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.1.4.1

Source:	Apple Inc.

Title:	Views on UL precoding indication for multi-panel simultaneous transmissions

Document for:	Discussion/Decision

# Introduction

In Rel-16, multi-TRP based simultaneous PDSCH transmissions using a single DCI was specified for both FDM and SDM PDSCH transmissions. While Rel-17 supports multi-TRP based PUSCH, where different repetitions in time may be associated with different beams, simultaneous PUSCH transmission across different panels is one of the objectives to be studied in R18. In previous RAN1 meetings, some progress toward simultaneous uplink transmission over multi-panel (STxMP) was made. Most importantly, in RAN1#110 it was agreed under a working assumption that in Rel-18, SDM scheme is supported for single-DCI based STxMP PUSCH transmission in mTRP based system. In this contribution, we provide our views on multi-panel simultaneous PUSCH transmissions, including antenna port indication and precoding indication.

# Simultaneous multi-beam PUSCH transmissions

## Indication of PUSCH Resources by a s-DCI

Multi-TRP based simultaneous PDSCH transmissions using a single DCI is achieved in Rel-16 by indicating the PDSCH resources associated to each TCI state (i.e., each beam). More precisely,

- For FDM simultaneous PDSCH transmissions through multi-TRP and by a single DCI indication, a UE is configured by higher layer parameter repetitionScheme set to one of 'fdmSchemeA', or 'fdmSchemeB', and the UE is indicated a TCI codepoint associated with two TCI states, and the indicated DM-RS port(s) are within one CDM group. FDRA bit-field indicates frequency domain resources that will be eventually split between the two TCI states, as part of PRB bundling and depending on the precoding granularity size, such that PDSCH occasions associated to each TCI state has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion, as described in TS 38.214, Clause 5.1.2.3

- For SDM simultaneous PDSCH transmissions through multi-TRP and by a single DCI indication, the UE is indicated a TCI codepoint associated with two TCI states, but the indicated DM-RS ports are within two CDM groups, where the first TCI state corresponds to the CDM group of the first antenna port, and the second TCI state corresponds to the other CDM group (TS 38.214, Clause 5.1.6.2). Here, FDRA bitfield indicates a PDSCH transmission occasion that is used by both TRPs, although different layers of that single TB are transmitted by different TRPs.

In RAN1#110, it was agreed that

Working assumption

Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

- SDM scheme

- In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

For STxMP PUSCH transmissions, a similar design as in Rel-16 PDSCH resource indication can be adopted. On single CW vs 2 CWs for SDM, in our view support of two CWs is not well justified. If the two PUSCHs toward two TRPs are so different for which different MCS is needed, switching back to single TRP transmission seems a better and more robust solution.

Proposal 1: For SDM STxMP, PUSCH with two codewords is not supported.

Proposal 2: For SDM STxMP, FDRA bit-field indicates a PUSCH transmission occasion that is used by both panels, although different layers of that single TB are transmitted by different panels.

## PUSCH Mapping to REs

If UE is indicated to transmit a single TB through simultaneous PUSCH transmissions over multi-panel, current specification can be adoptedto determine the TB size. For mapping of the coded symbols to the REs, different options can be discussed, For example

- Option 1: start with the first allocated symbol, the coded symbols are mapped in frequency first to all resource elements (REs) associated to the 1st PUSCH and next to REs associated to the 2nd PUSCH. Next move to the next symbol.

- Option 2: start with 1st PUSCH, coded bits are mapped in all frequency and time REs associated to 1st PUSCH (frequency next time). Next, move to 2nd PUSCH.

Proposal 3: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

## UL Precoding Indication

In Rel-17, when UE is indicated to perform PUSCH repetition in time with different UL beams associated to each repetition, SRI bit-field, and TPMI for CB based, is extended to indicate beam association to each repetition. The procedure assumes same number of layers, which is indeed the case for R17 TDM based repetitions. For SDM STxMP, it was agreed in RAN1#110 that

Agreement

For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case.

- FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc.

- FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4).

- This applies to SDM with 1 CW at least.

Based on the above agreement, separate/independent SRI/TPMI bit-fields seems a straightforward solution to indicate UL precoding and layer indication per PUSCH. We have the following proposal:

Proposal 4: For s-DCI based STxMP:

- For ‘txConfig = nonCodebook’: the second SRI bit field assumes all possible ranks that can be indicated by the corresponding SRS resource set, without dependency on the indicated rank by the first SRI.

- For ‘txConfig = Codebook’: the second TPMI bit-field indicates all possible number of layers for the corresponding beam/panel

## UL Antenna Port Indication

For single-DCI based mTRP system and DMRS port indication in SDM based scheme of STxMP PUSCH transmission, couple of proposals was discussed in RAN1#110. Given that based on WID description in [1] total number of layers across both panels is up to 4, one proposal is to adopt existing procedure to indicate total DMRS ports and then map the DMRS ports to each PUSCH based on the indicated number of layers. To minimize the specification impact, DMRS ports associated to the same PUSCH may or may not belong to the same CDM group, i.e. separate CDM groups in not a must for STxMP PUSCH.

Proposal 5: Single “Antenna ports” bit-field indicates DMRS ports for the total number of layers r1+r2 across both panels, where r1 represents number of layers of the PUSCH transmitted from the first panel, r2 represents number of layers of the PUSCH transmitted from the second panel

## CG+DG/CG STxMP

In NR R17, UE can transmit over a configured grant (CG) multiple repetitions of the same TB across different UL beams, where repetitions are TDM. More precisely, from 38.214, 6.1.2.3:

When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, precoding information and number of layers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook’) associated with the first and second SRS resource set is provided by precodingAndNumberOfLayers and precodingAndNumberOfLayers2, respectively, and SRS resource indicators associated with the first and second SRS resource sets are provided by srs-ResourceIndicator and srs-ResourceIndicator2, respectively.

For simultaneous transmission of two CG PUSCHs, subject to UE capability, across different panels, we have the following proposal:

Proposal 6: For simultaneous transmission of separate CG configurations

- For Type-1 UL CG configuration, UE is indicated about STxMP once UE is configured with two SRS resource sets, each of which is associated to different srs-ResourceIndicator

- For Type-2 UL CG configuration, UE is indicated about STxMP if coresetPoolIndex associated to each DCI activating the two CG PUSCHs are different

## PHR triggering and reporting

On m-TRP based simultaneous PUSCH transmissions, there are couple of issues related to PHR triggering and reporting that worth further discussion. For example, for PHR triggering conditions, whether PHR may be triggered subject to PLs associated to both TRPs change more than the predefined threshold(s), or PHR may be triggered even if the PL corresponding to any of TRPs changes more than the predefined threshold. In addition, it should be discussed for the case of simultaneous transmission of PUSCH+PUSCH, the PHR calculation is based on actual or virtual PUSCH. For example, for s-DCI based STxMP, if both PUSCHs are fully overlapped in time, then both PUSCHs are considered either actual or both considered virtual based on existing specification. On the other hand, for m-DCI based STxMP, UE may determine virtual vs actual PHR per PUSCH occasion based on existing specification.

Proposal 7: For STxMP PUSCH+PUSCH transmission:

- If both PUSCHs are fully overlapped in time (e.g. for the case of s-DCI), both are considered either actual or both considered virtual based on existing specification

- If PUSCHs are not fully overlapped in time (e.g. for the case of m-TRP), UE determines virtual vs actual PHR per PUSCH occasion based on existing specification

## UCI multiplexing procedures

If a m-TRP based PUSCH indicated for STxMP with FDM-A or SDM scheme overlaps in time with a PUCCH that is associated to single TRP (s-TRP), UCI multiplying procedure may follow the existing specification per PUCCH+PUSCH resources that are associated to the same TRP. In other words, UE may resolve per panel PUCCH+PUSCH collision (multiplex UCI, drop PUCCH, etc) based existing procedures.

Proposal 8: If a mTRP based PUSCH+PUSCH overlaps in time with a PUCCH that is associated to a sTRP, UCI is multiplexed only on the PUSCH resource that is associated to the same TRP/beam/panel as of PUCCH

- FFS: How the code-rate for UCI multiplexing is determined, i.e., based on REs allocated to both PUSCHs, or based on only those REs associated to the PUSCH toward the same TRP as PUCCH

In current specification, TDAI bit-field in uplink DCI indicates number of HARQ-ACK bits that shall be multiplexed on PUSCH. More precisely, when there is overlapping PUSCHs and PUCCH, up to one PUSCH will be selected for HARQ-ACK multiplexing, where no overlapping PUSCHs within the same carrier with the same priority index, is expected. Now in Rel-18, if STxMP for PUSCH+PUSCH is supported for m-DCI based m-TRP, when UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs with two different values of coresetPoolIndex, TDAI indication should be per panel, as shown in the following example. In this case, UE will multiplex HARQ-ACKs to each PUSCH independently, unlike current spec which only one PUSCH with UL-TDAI not equal to 4 carries HARQ-ACK

Figure 3: TDAI indication for dynamic HARQ-ACK codebook and m-DCI based STxMP

Proposal 9: If UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs that are associated to different ControlResourceSets having different values of coresetPoolIndex, TDAI indication is per panel.

# Simultaneous multi-beam PUCCH transmissions

In NR R17, UE can transmit multiple repetitions of a PUCCH resource across different UL beams, where repetitions are TDM:

- Same PUCCH resource with the same UCI is repeated in intra-slot (scheme 3) or inter-slot (scheme 1)

  - two spatial relation info’s (PUCCH-SpatialRelationInfo) can be activated via MAC CE (38.213, Sec. 6.1.3.25), and/or

  - two PUCCH power control parameter sets is activated via MAC-CE

- If number of repetitions is more than 2, PUCCH mapping to beam can be cyclic or sequential

## PUCCH+PUCCH indication

In RAN1#109-e, it has been discussed to study and evaluate the following schemes for STxMP PUCCH transmission in single-DCI based mTRP:

- PUCCH FDM-A scheme: different frequency domain parts of one PUCCH resource are transmitted by two different UE panels.

- PUCCH FDM-B scheme: two FDM-ed PUCCH transmission occasion of the same UCI with the same PUCCH format are transmitted from two different UE panels

- PUCCH SFN scheme: The same PUCCH/PUCCH-DMRS is transmitted from two different UE panels with simultaneously

For simultaneous transmission of PUCCH over multi-panel using a single DCI, the STxMP scheme indication can be as part of RRC PUCCH resource configuration. For example, a PUCCH resource configuration is tagged with FDM-A, or FDM-B, or SFN in RRC configuration. Once that PUCCH resource in selected by DCI (or MAC-CE), UE is indicated to perform STxMP for PUCCH transmission, subject to UE capability, with the corresponding scheme.

Proposal 10: For simultaneous transmission of PUCCH over multi-panel (STxMP), UE will be indicated about the STxMP scheme in a single-DCI, as part of RRC PUCCH resource configuration

- a PUCCH resource configuration is tagged with FDM-A, or FDM-B, or SFN in RRC configuration

- Once that PUCCH resource in selected, UE is indicated to perform STxMP for PUCCH transmission with the corresponding scheme

# Conclusion

In this contribution, we provided some views on multi-panel simultaneous uplink transmissions. Based on what we discussed, the following proposals are made:

Proposal 1: For SDM STxMP, PUSCH with two codewords is not supported.

Proposal 2: For SDM STxMP, FDRA bit-field indicates a PUSCH transmission occasion that is used by both panels, although different layers of that single TB are transmitted by different panels.

Proposal 3: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

Proposal 4: For s-DCI based STxMP:

- For ‘txConfig = nonCodebook’: the second SRI bit field assumes all possible ranks that can be indicated by the corresponding SRS resource set, without dependency on the indicated rank by the first SRI.

- For ‘txConfig = Codebook’: the second TPMI bit-field indicates all possible number of layers for the corresponding beam/panel

Proposal 5: Single “Antenna ports” bit-field indicates DMRS ports for the total number of layers r1+r2 across both panels, where r1 represents number of layers of the PUSCH corresponding to the first SRI bit-field

Proposal 6: For simultaneous transmission of different CGs

- For Type-1 UL CG configuration, UE is indicated about STxMP once UE is configured with two SRS resource sets, each of which is associated to different srs-ResourceIndicator

- For Type-2 UL CG configuration, UE is indicated about STxMP if coresetPoolIndex associated to each DCI activating the two CG PUSCHs are different

Proposal 7: For STxMP PUSCH+PUSCH transmission:

- If both PUSCHs are fully overlapped in time (e.g. for the case of s-DCI), both are considered either actual or both considered virtual based on existing specification

- If PUSCHs are not fully overlapped in time (e.g. for the case of m-TRP), UE determines virtual vs actual PHR per PUSCH occasion based on existing specification

Proposal 8: If a mTRP based PUSCH+PUSCH overlaps in time with a PUCCH that is associated to a sTRP, UCI is multiplexed only on the PUSCH resource that is associated to the same TRP/beam/panel as of PUCCH

- FFS: How the code-rate for UCI multiplexing is determined, i.e., based on REs allocated to both PUSCHs, or based on only those REs associated to the PUSCH toward the same TRP as PUCCH

Proposal 9: If UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs that are associated to different ControlResourceSets having different values of coresetPoolIndex, TDAI indication is per panel.

Proposal 10: For simultaneous transmission of PUCCH over multi-panel (STxMP), UE will be indicated about the STxMP scheme in a single-DCI, as part of RRC PUCCH resource configuration

- a PUCCH resource configuration is tagged with FDM-A, or FDM-B, or SFN in RRC configuration

- Once that PUCCH resource in selected, UE is indicated to perform STxMP for PUCCH transmission with the corresponding scheme

# References

- RP-213517, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung (Moderator), Dec. 2021