3GPP TSG- RAN WG1 Meeting #110bis-e                                               R1- 2209580

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.2.4.1

Source:	Apple Inc.

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion/Decision

# Introduction

In the SID governing the AI/ML study, an initial set of use cases has been decided on for positioning accuracy enhancements in different scenarios including, e.g., those with heavy NLOS conditions, with  the following objective:

- evaluate the performance benefits of AI/ML-based algorithms with a methodology based on statistical models for link and system simulations.

In this contribution, we provide some preliminary performance evaluation results for the direct AI ML positioning method based on the evaluation methodology and parameteres agreed on in RAN1 #109-e  and RAN1 #110.

# Discussion

## Evaluation Methodology

In RAN1 #109-e  and RAN 110 , the following agreements were made on the evaluation methodology [4]:

Agreement Companies are encouraged to provide evaluation results for: - Direct AI/ML positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - AI/ML assisted positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - details of the output of the AI/ML model inference, how the AI/ML model output is used to obtain the UE’s location Agreement When reporting evaluation results with direct AI/ML positioning and/or AI/ML assisted positioning, proponent company is expected to describe if a one-sided model or a two-sided model is used. - If one-sided model (i.e., UE-side model or network-side model), the proponent company report which side the model inference is performed (e.g. UE, network), and any details specific to the side that performs the AI/ML model inference. - If two-sided model, the proponent company report which side (e.g., UE, network) performs the first part of interference, and which side (e.g., network, UE) performs the remaining part of the inference. Agreement As a starting point, the training, validation and testing dataset are from the same large-scale and small-scale propagation parameters setting. Subsequent evaluation can study the performance when the training dataset and testing dataset are from different settings. Agreement For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy. - Companies can optionally report vertical accuracy. Agreement The CDF percentiles to analyse are: {50%, 67%, 80%, 90%}. Agreement For evaluation of AI/ML based positioning, the computational complexity can be reported via the metric of floating point operations (FLOPs). - Note: For AI/ML assisted methods, computational complexity for the AI/ML model is only one component of the overall complexity for estimating the UE’s location. - Note: Other metrics to measure the computational complexity are not precluded. Agreement For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”.

## Data Set Generation

We generate the dataset based on agreements in RAN1 #109-e using the IIoT  InF-DH scenario based on the following agreements with detailed assumptions in the Appendix:

Agreement The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning. Agreement For evaluation of AI/ML based positioning, at least the InF-DH sub-scenario is prioritized in the InF deployment scenario for FR1 and FR2. Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area. Agreement If spatial consistency is enabled for the evaluation, companies model at least one of: large scale parameters, small scale parameters and absolute time of arrival, where - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = for InF (Section 7.6.3.1 of TR 38.901) - the small scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 Agreement If spatial consistency is enabled for the evaluation of AI/ML based positioning, the baseline evaluation does not incorporate spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). - It is optional to implement spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). Agreement When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including: - Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA) - Percentage of training data without label, if incomplete labeling is considered in the evaluation - Imperfection of the ground truth labels, if any Agreement For evaluation of AI/ML based positioning, study the performance impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

## AI Model and Input

We evaluate the direct AI/ML positioning technique in which parameters from multiple gNBs serve as inputs into a neural network that directly estimates the UE position coordinates. Examples of these inputs could include the Channel Impulse Response (CIR), the Power Delay Profile (PDP), and/or the Layer 1 Reference Signal Received Power (L1-RSRP) and what (if any) pre-processing is needed. The specific input  may have an impact on the specification. Figure 1 illustrates a block diagram with the AI model, its input and output

Figure 1: High Level AI model

We assume a one-sided AI-model with model inference at either the UE-side (UE-based positioning) or at the network side (LMF based/UE assisted positioning or LMF based/NG-RAN assisted positioning).  Note that the training can take place at either side with the resulting model used at the side at which it is trained or  transferred to the other side.

The complexity of the AI model is shown in the Table below:

Table 1: Model Complexity

| AI model | FLOPs(×106) | Trainable Parameters (×106) |
| --- | --- | --- |
| CNN | ~5.12 | 2.43 |

## KPIs or Metrics

In RAN1 #109-e, the following agreement was made

Agreement For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy. - Companies can optionally report vertical accuracy. Agreement The CDF percentiles to analyse are: {50%, 67%, 80%, 90%}.

## Generalization

To study the generalization of the AI model, the following agreements have been made

Agreement To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time.

# Evaluation Results

The synthetic dataset used in this contribution is generated with the following options:

Table 2: General Assumptions/Options

| Scenario | InF-DH |
| --- | --- |
| Training/Validation Data Set Size | 47500 |
| Testing Data Set Size | 2500 |
| UE Distribution | uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area |
| Spatial Consistency | We model spatial consistency for the Large Scale Parameters, Small scale Parameters and Absolute Time of Arrival We do not model spatial consistency for UT/BS mobility modeling |
| Input Data | CIR ~ [18 x 256 x 2] ~ [# gNBs x # taps x Real/Imaginary] The # of gNBs indicates the channel from each gNB. The # of taps is truncated if greater than 256 The third dimension captured the real and imaginary values of the channel taps NOTE: CIR for each gNB is the CIR of {1Tx x 1Rx} pair from the {32 Tx x 4 Rx} channel based on the channel assumptions |
| Ground Truth Label | Perfect UE coordinates with 100% of the training data labeled |
| Synchronization | Perfect synchronization |
| Learning Algorithm | Supervised learning |
| Generalization | - Different drops - Different clutter parameters One aspect at a time is evaluated |

The corresponding results are shown in the tables below:

Table 3: Evaluation results for AI/ML model deployed on UE/network-side, without model generalization, with a CNN

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity (x106) | Computational complexity (x106) | AI/ML |
| CIR [18 x 256 x 2] | UE coordinates [1x2] | 0% | {60%, 6, 2} | 47500 | 2500 | 5.12 | 2.43 | 1.1m |

Table 4: Evaluation results for AI/ML model deployed on UE/network-side, with model generalization for Clutter (train {60%,2m,2m}, test {40%, 2m, 2m}) with a CNN

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity (x106) | Computational complexity (x106) | AI/ML |
| CIR [18 x 256 x 2] | UE coordinates [1x2] | 0% | {60%, 6, 2} | 47500 | 2500 | 5.12 | 2.43 | 3.6m |

Table 5: Evaluation results for AI/ML model deployed on UE/network-side, with model generalization for different drops with a CNN

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Training | test | Model complexity (x106) | Computational complexity (x106) | AI/ML |
| CIR [18 x 256 x 2] | UE coordinates [1x2] | 0% | {60%, 6, 2} | 47500 | 2500 | 5.12 | 2.43 | 3.1m |

Figure 2 : 2-D Horizontal Accuracy for baseline and generalization study

Observation 1

- Direct AI based positioning does show good performance in the baseline case

- There is some loss when the training parameters are not exactly matched.

- Proposal 1: Fine tuning may be needed to improve the performance in a genarizalized scenario where the test data  has a different set of parameters from those used to train the network

# Conclusion

In this contribution, we have evaluation methodology for AI_ML for positioning and presented some preliminary results showing the benefits of AI-based positioning in high NLOS scenarios. We also have made the following observations and proposals:

Observation 1

- Direct AI based positioning does show good performance in the baseline case

- There is some loss when the training parameters are not exactly matched.

- Proposal 1: Fine tuning may be needed to improve the performance in a genarizalized scenario where the test data  has a different set of parameters from those used to train the network

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- 3GPP TR 38.857, Study on NR Positioning Enhancements; (Release 17)

- 3GPP TR 398.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

- Chairman’s Notes, RAN1 #109-e, May 2022.

- Chairman’s Notes, RAN1 #110, August 2022

# Simulation Assumptions

|  |  | FR1 Specific Values |
| --- | --- | --- |
| Channel model |  | InF-DH |
| Layout | Hall size | InF-DH: (baseline) 120x60 m |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |
|  | Room height | 10m |
| Total gNB TX power, dBm |  | 24dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation |
| gNB antenna radiation pattern |  | Single sector – Note 1 |
| Penetration loss |  | 0dB |
| Number of floors |  | 1 |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area is the convex hull of the horizontal BS deployment. |
| UE antenna height |  | Baseline: 1.5m |
| UE mobility |  | 3km/h |
| Min gNB-UE distance (2D), m |  | 0m |
| gNB antenna height |  | Baseline: 8m |
| Clutter parameters: {density , height ,size } |  | High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |