3GPP TSG RAN WG1 #110bis-e		R1-2209592

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.7.2

Source:	Apple

Title:	Discussion on network energy saving techniques

Document for:	Discussion/Decision

# Introduction

A study item on network energy savings [1] has been approved for Rel-18, with one of the objectives being:

Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

Various network energy saving techniques have been proposed by companies, and brief descriptions of these techniques are summarized in [2]. The following guideline was provided:

For future meetings:

- Companies are encouraged to review the summary of description of potential techniques of the moderator summary in Section 3 of R1-2208185. It should be noted that the description of the techniques are a draft and additional updates and modification of the description are expected.

- Based on the summary, companies are encouraged to work further to formulate description of potential network energy techniques to be eventually captured into the SI TR.

In this contribution, we provide our comments on these techniques in terms of what additional aspects should be clarified for further discussion.

# Network energy saving techniques

Various techniques have been proposed so far, and it is important that we understand these proposals. We identify here what aspects should be further clarified for each of the proposals. Also note by the end of the study item, we should be able to provide prioritization among these techniques to facilitate the WI scope determination. The prioritization should be determined based on factors including network energy saving gain, the specification impact, the performance impact, the impact on UE complexity/power consumption, and whether legacy UEs can be supported. Given that we have a large number of proposals on the table and there are only two meetings left for the SI, we should start the discussion on prioritization now.

Time domain

- Technique #A-1 Adaptation of common signals and channels - Network energy saving can be realized by flexibly varying the periodicity and/or dynamically changing a transmission pattern (when applicable) of downlink common and broadcast signals, such as SSB/SI/paging/cell common PDCCH, and/or flexibly varying the periodicity of uplink random access opportunities. - This also include introducing light version of downlink common and broadcast signals, where for some periodicity occasion one or more common signals/channels can be skipped. - This is mainly for BS idle/inactive mode, e.g. cell deactivation without DL data transmission. - [Comment] this does not seem to be a complete solution, because mechanisms are necessary to put the BS back to normal operation. It should be clarified what other techniques are needed to make this complete. - Support of burst transmission and reception of common signals and channels with more than one periodicity and/or adaptation of a burst pattern, including periodicity, are expected to potentially provide longer inactivity periods for the gNB and potentially provide higher power saving gains. - [Comment] Is this only applicable to green field deployment? Does this prohibit legacy UEs from accessing this BS? - Support of dynamic adaptation of SSB/SIB transmission or on-demand SSBs/SIB1 transmissions or SSB/SIB1-less operations may also enable long periods of inactivity at the gNB and potentially provide energy savings. - [This may include leveraging SSB-less cell operations and potential enhancements for SSB-less cells, e.g. support SSB-less cell operation for inter-band CA. and/or support offloading system information from one cell to another for inter-band CA.] - This may include support of signals/channels to aid discovery of cells in lieu of SSBs. - This may include support of mechanism for UE to trigger on-demand SSB/SIB1 transmission for fast access/fast cell activation. - It should be noted that use of CA means the technique is only applicable to UEs in connected mode. - [Comment] If the intention is to use it in the context of CA, should this be merged together with technique B-1? Otherwise, sufficient distinction is needed between the two. - [Support of scheduling enhancements for SIB1 along with a long period (rather than the period as the same as the SSB period) adaptation of CORESET 0 (e.g. in a separately configured CORESET) are expected to avoid/reduce redundant DCI transmissions within the CORESET 0 for the gNB and potentially provide higher power saving gains.] - This may include support of a long period (rather than the period as the same as the SSB period) of CORESET 0 - This may include support of scheduling of SIB1 by SSB to avoid transmissions of DCIs within CORESET 0, support of the mechanism to reduce impacts on SSB and overhead - [Comment] It is not clear how much benefit can be achieved by omitting PDCCH if SSB still needs to be transmitted. May be deprioritized in our view. - Adaptation of common channel/signals might have impact to the UE normal access to the network, such as initial access, and legacy UE network access. - [Comment] the exact impact should be further clarified for each of the sub-bullets above. For example, would it prohibit legacy UE from accessing the cell or just introduce longer latency for legacy UE to access the cell? - Technique #A-2: Dynamic adaptation of UE specific signals and channels - Network energy saving opportunities may be restricted by UE specific signals and channels that are semi-statically configured such as periodic or semi-persistent CSI-RS, group-common/UE-specific PDCCH, SPS PDSCH, PUCCH carrying SR, PUCCH/PUSCH carrying CSI reports, PUCCH carrying HARQ-ACK for SPS, CG-PUSCH, SRS, positioning RS (PRS). - Reducing the number of time occasions for or temporarily disabling the following resources during periods of low activity may potentially provide energy saving benefits. - CSI-RS, group-common/UE-specific PDCCH, SPS PDSCH, PUCCH carrying SR, PUCCH/PUSCH carrying CSI reports, PUCCH carrying HARQ-ACK for SPS, CG-PUSCH, SRS, positioning RS (PRS). - This may include report of UE assistance information, e.g., UE buffer status to help gNB make decisions. - Support of enhancements to synchronize the UE specific signal and channel transmission reception such that they provide longer inactivity periods at the gNB can be considered. - Support of configuration signaling of the UE specific signals and channel transmission and reception to be reduced, e.g. by utilizing UE/cell group-level or cell common signaling to allow gNB to minimize configuration overhead and potentially minimize overall gNB activity. - Support of group common signaling that indicates to UEs to temporarily stop the transmission/reception of semi-statically configured channels/signals - The impact to the UE performance by adaptation of UE specific signal/channels should be included along with the network energy saving performance results. - Technique #A-3: wake up signal (WUS) for gNB - Support of wake up of gNB that is in a dormant power state/energy saving state (e.g., SSB-less/SIB1-less/SSB relaxed state), support of wake up signal (WUS) transmitted by the UE/neighboring gNB including UEs to the gNB (e.g. the gNB/cell in dormant state or the anchor gNB/cell). - Whether UE detection of a dormant power state/energy saving state is required before WUS transmission should be identified. - Resource reserved for WUS and the assumption of the gNB receiver should be identified - This may include support of assistance information from the UEs intended to aid wake up operations by the gNBs. - This is mainly for connected mode UEs - Can be used in support of techniques #A-1 techniques #A-2 and other techniques. Exact design may depend on the supported technique. - The power model of receiving WUS is associated with the gNB receiver sensitivity of WUS decoding, which will reflect the results of UE WUS coverage area. - A legacy UE cannot access a gNB in such dormant power state/energy saving state. - Technique #A-4: Adaptation of DTX/DRX - DTX/DRX cycle configuration/pattern at the BS, which can be potentially aligned with the DRX cycle configured for UEs in connected mode or idle mode can potentially provide longer inactivity periods at the gNB. - This may include potential enhancements to UE behavior when both cell-specific DTX/DRX cycle and UE DRX cycle are configured. - An alternative BS DTX with UE C-DRX alignment would be the use of DTX/DRX patterns that are defined by the BS. - [Comment] this sentence seems unclear. - The two techniques/approaches of DTX/DRX alignment can be complementary to each other and they may be beneficial to energy savings both at the network and at the UE side. - [Comment] It is not clear what are complementary to each other. - Reduction of periodically transmitted/semi-static configured channels/signals(e.g. SSB, SIB, CG PUSCH etc.) during the longer inactivity periods (i.e. outside UE’s DRX active time). - Controlling UE DRX on/off periods for multiple DRX cycles with a single indication can potentially provide longer inactivity periods at the gNB. - This may include group level indication for, such as UE-group signaling or cell-specific signaling, UE DRX commend such as DRX enhanced command MAC CE and long DRX commend MAC CE. - Technique #A-5: Adaptation of BS inactive state or DTX/DRX - Support of gNB entering into sleep mode (or DTX/DRX state) for a period of time along with the indication of active/inactive state, e.g., in terms of start time and duration are expected to potentially provide flexible adaptation at the gNB and can potentially provide higher power saving gains. - This may include support of semi-static and/or dynamic gNB active/inactive state (or DTX/DRX state) adaptation. - This may include group common signaling for the indication of adapted active/inactive state (or DTX/DRX state) - This may include defining corresponding UE behaviors when gNB enters inactive state or sleep mode

Frequency domain

- Technique #B-1: Multi-carrier energy savings enhancements - The gNB can achieve potential energy savings from operating SCells without or with reduced transmission and reception of periodic signals and channels such as SSB, SI, and CSI-RS for mobility measurements, PRACH, paging, etc. - This may include support of mechanism for UE to trigger normal SSB/SIB1 transmission on a SCell for fast access if the SCell can not share synchronization with PCell. - This may include leveraging SSB-less cell operations and potential enhancements for SSB-less cells, e.g. support SSB-less cell operation for inter-band CA, and support offloading system information from one cell to another for inter-band CA. - Note that intra-band CA cases are already supported by current specification. - Moreover, regarding cross carrier synchronization and measurement for inter-band CA cases, involvement of RAN4 WG is needed to study the feasibility, and if feasible, identify necessary requirements and guide for future RAN1 work, i.e. about sync. requirement between carriers, frequency distance requirement between carriers, Rx power difference between carriers, QCL assumption requirement across carriers, etc. - [Comment] if we are seriously considering this, we should send an LS to RAN4 for feasibility study. Otherwise, it would not be possible to include it in the future WI. - To facilitate leveraging of lean SCells, potential enhancements to provide time and frequency synchronization, and other measurement sources by another cell can be considered. - Common signaling to a group of the UEs of PCell change - [Comment] This should be and is discussed in RAN2. - Ability to quickly activation and deactivation of CC, for example, based on on-demand RS, aperiodic RS, UE request, and L1 response or dynamically switch PCell is expected to potentially provide energy savings at the network. - Hardware architecture needs to be carefully considered. For shared hardware components among carriers, switching off or disable one of the carriers may not bring benefits to the network energy saving, since the shared hardware components are still utilized by other active carriers. - Technique #B-2: Dynamic adaptation of bandwidth part of UE(s) within a carrier - Enhancements to enable UE group-common or cell-specific BWP configuration and/or switching may lower signaling overhead and operational cost (e.g. signaling overhead) for adaptation of BWPs of UE(s) and potentially improve gNB power consumption. - Reducing the BW adaptation delays for Rel18 UEs - Technique #B-3: Dynamic adaptation of bandwidth of UE(s) within a BWP [and dynamic adaptation of a resource grid in a carrier] - Enhancements to enable group-common signaling to adapt the bandwidth of active BWP and continue operating in same BWP reduces the latency and lowers the signaling overhead.

Spatial domain

- Technique #C-1: Dynamic adaptation of spatial elements - gNB may conserve energy by reducing the number of active transceiver chains or antenna elements. - CSI-RS/reporting re-configuration should be indicated to the UEs for spatial adaptation of gNB/cell power state - Adaptation can be further categorized into two types: - Type 1: enable/disable all spatial elements associated to a logical antenna port, e.g. a subset of ports of a CSI-RS resource. - Type 2: enable/disable of part of spatial elements associated to a logical antenna port(s). This may result in changes to the antenna pattern, gains, TCI states, and/or transmission power of the reference signal or channel that uses the antenna port(s). - Both Type 1 and Type 2 may have impact on measurement operation, so the potential enhancement may include CSI-RS configurations, CSI-RS and PL RS measurements, CSI reporting, beam failure recovery, radio link monitoring, cell (re)selection and handover procedure. - CSI reporting enhancement on muted spatial elements patterns can be considered for assistance information feedback. - [Comment] It is not clear how CSI reporting is done on muted spatial elements and how this assists gNB. - Support enhancements to UE behaviors due to dynamic adaptation of spatial elements, e.g., measurements, CSI feedback, power control, PUSCH/PDSCH repetition, SRS transmission, TCI configuration, beam management, beam failure recovery, radio link monitoring, cell (re)selection, handover, initial access, etc. - The different set of ports such as 64/32/8/4 and their associated CSI-RS configurations may be determined from the hypothesis of TRX On/Off. Spatial configuration for the network energy saving may then be determined by mapping the selected TRX ports setting to an associated configuration index. The configuration index can also be used to select the best of directional beams, NZP-CSI-RS configuration and measurement reporting in reportConfig. Over a certain coherent period, whenever the network enters the energy saving mode, the corresponding spatial domain configuration can then be determined from the configuration index. - [Comment] This description does not seem clear. It seems to be discussing a very specific type of enhancements for CSI-RS configuration/measurement/reporting. If this is to be included, should we also include detailed description of other potential solutions? - Support of light-weight mechanisms such as DCI/MAC-CE-based, that allow fast CSI-RS reconfigurations. - Techniques including conditions/criteria for UE measurements and feedback to gNB for (de)activation of antenna ports. - UE feeding back antenna muting pattern recommendations to the gNB. - Technique #C-2: Dynamic adaptation of TRPs in mTRP - Adaptation is categorized as type 3: - Type 3: activate/deactivate a set of spatial elements corresponding to a TRP, e.g., activating N1-port CSI-RS resource (set) corresponding to one TRP and/or deactivating N2-port CSI-RS resource (set) corresponding to another TRP - Type 3 may have impact on redundant CSI measurement or reporting to a muted TRP, so enhancement may include dynamic signaling for TRP ID (CORESETPollIndex). - It is not clear how dynamic signaling for TRP ID address the issue. - gNB may conserve energy by reducing the number of active TRPs in the mTRP deployment. - This may also include signaling of the adaptation of TRPs in mTRP, e.g. by utilizing group-level or cell common signaling. - Support enhancements to UE behaviors due to dynamic adaptation of TRPs, e.g., measurements, CSI feedback, power control, PUSCH/PDSCH repetition, SRS transmission, TCI configuration, beam management, beam failure recovery, radio link monitoring, cell (re)selection, handover, initial access, etc

Power domain

- Technique #D-1: Adaptation of transmission power of signals and channels - Network energy savings could be potentially obtained by reducing the transmission power or PSD of various signals and channels, e.g SSB, CSI-RS, PDSCH, during specific scenarios or situations. - Support of signaling of modified power ratio between CSI-RS and PDSCH/SSB or between SSB and CSI-RS are expected to provide adaptation of flexible power ratio values and potentially reduce overhead, e.g. by utilizing group-level or cell common signaling. - This may include enhancements on CSI-RS based measurements, such as beam management, beam failure recovery, radio link monitoring, cell (re)selection and handover procedure - The transmission bandwidth may be adapted jointly with transmission power to keep the similar reception performance. - Network energy savings could be potentially obtained by transmission power adaptation with UE feedback information, e.g, CSI reporting, power adjustment indication, etc. - The linear reduction of PAE (power added efficiency) when Tx power reduction should be included in the scaling of the power model. - [Comment] This sentence needs rephrasing. - This will impact legacy UEs if the transmission power of common signals/channels is adapted. - Technique #D-2: enhancements to [gNB digital pre-distortion] and UE post-distortion - Transmission energy efficiency at the network can be potentially improved with use of [enhanced over the air digital pre-distortion at the gNB and/or] post-distortion at the UE. - Whether and how much improvement of the PAE (power-added efficiency) should be disclosed. - In gNB digital pre-distortion over the air, the UEs assist the gNB in reducing nonlinear impairments introduced by the PA, by processing (e.g., calculation of the cross correlation of received signal after applying non-linear kernels) and reporting the information needed for gNB digital pre-distortion, on training signals - In UE post-distortion, the gNB assist the UE in reducing nonlinear impairments introduced by its PA (e.g., non-linear equalization stage that will “invert” the non-linearity), by sending RS signal at low periodically or some signaling to the UE. - [Comment] This should be discussed in RAN4. - Technique #D-3: adaptation of transceiver processing algorithm - Transmission energy efficiency at the network can be potentially improved with use of techniques such as channel aware tone reservation that decrease PAPR. - The UE must be notified of the sub-carriers carrying the TR signal, as using existing patterns (e.g., CSI-RS) is not practical - gNB may opt to use different transceiver processing algorithms, e.g. different receive filtering, different transmitter digital pre-distortion methods, etc,, including some that may favor lower power consumption at the expense of degraded system performance. For example, disabling use of DPD that would potentially increase out of band emissions or tx EVM, but would potentially conserve transmitter power consumption. Different transceiver processing algorithms at the gNB should be transparent to the UE. - Power model for the scaling of different transceiver processing algorithm should be provided with justification. - [Comment] This should be discussed in RAN4. - Technique #D-4: PA Input Power Bias ("input backoff”) Adaptation - Technique(s) allowing to modify/reduce the input power bias (“input power backoff”) in cases of no or very low load in the cell and in neighbor cells. - The PA energy consumption consists around ~70 % of the energy consumed at the BS. - The majority of this energy consumed at the PA is due to the input power bias (“backoff”). - In some cases, especially when the cell and neighbor cells are almost empty, reducing this input power bias (“backoff”) results in significantly lower energy consumption. - This input power bias adaptation results in lower output PAPR, which is translated into some in band and out of band emissions being generated. - With appropriate signal processing techniques, it is possible to “steer” the unwanted emissions either to the in-band signal or out-of-band. - With suitable base station coordination and by steering the unwanted emissions onto carrier frequencies in which their impact can be traced, it is possible to avoid any eventual impact onto UEs in the cell or in neighbor cells. - In general, this technique is activated only in case of zero or very low load in the cells; hence, the expectation is that no UEs will be affected by the generated in-band or out-of-band emissions. - The effect of PAE to the scheme should be disclosed. - [Comment] This should be discussed in RAN4.

Other

- Technique #E-1: UE assistance information or feedback/report to further facilitate gNB network energy saving - Support of PUCCH transmission with negative SR report can be considered to aid gNB’s decision on whether to go into a dormant power state or not. - Support of UE’s mobility status and location can be considered to aid gNB’s perform energy saving techniques - UE assistance information including traffic relation information, such as pattern, volume etc. - UE report of certain measurement, e.g., based on discovery reference signal. - [Comment] This can be merged into A-1. - UE assistance data for gNB to assess whether it can go into a sleeping state, e.g. polling number of idle UEs, polling UEs beyond certain coverage.

# Conclusion

In this contribution, we provided our comments on the descriptions of the proposed techniques for network energy savings.

# Reference

- RP-220297, Revised SI: Study on network energy savings for NR, Huawei, RAN#95e, Mar. 2022.

- R1-2208185, Discussion Summary #2 for energy saving techniques of NW energy saving SI, Moderator (Intel Corporation), RAN1#110, August 2022.