3GPP TSG-RAN WG1 Meeting #110bis-e	R1- 2209627

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.2.3.1

Source:	NVIDIA

Title:	Evaluation of AI and ML for beam management

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

The initial use cases focused in this study include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration, the study aims to evaluate performance benefits of AI/ML based algorithms:

- Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

  - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

  - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

  - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

  - Consider adequate model training strategy, collaboration levels and associated implications

  - Consider agreed-upon base AI model(s) for calibration

  - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

- KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

  - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

  - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

At RAN1#109-e and RAN1#110, a comprehensive set of evaluation assumptions were agreed (see Appendix A.1 and A.2).

In this contribution, we discuss the remaining issues on evaluation assumptions and present initial evaluation results on AI/ML for beam management.

# 2	Methodology

NR can operate at a wide range of frequencies, ranging from sub-6 GHz to millimeter wave frequencies. To support operation over such a wide range of carrier frequencies, NR has been designed to utilize beam-based operation, where gNB and UE may use transmit and receive beamforming for all channels and signals.

Beam management procedures include beam determination, beam measurement and reporting, beam indication and switching, and beam recovery. In beam determination, the gNB and UE find suitable transmit and receive beam directions for communication. In beam measurement and reporting, the UE measures the link quality of multiple beam pairs and reports the measurement results to the gNB. In beam indication and switching, the gNB can indicate to the UE which beam to use for the communication, including switching to a beam different from the current beam. In beam recovery, the UE can attempt to re-establish a transmit-receive beam pair with the gNB from beam failure by using a L1 procedure without declaring radio link failure.

In general, the downlink beam management procedures consist of three steps: Procedure 1 (P-1): SSB-based beam sweeping; Procedure 2 (P-2): CSI-RS based transmit-end beam refinement; Procedure 3 (P-3): CSI-RS based receive-end beam refinement. There are also uplink beam management procedures.

AI/ML based algorithms may find applications in many steps of beam management. It is worthwhile discussing which of the steps should be the focal point for evaluation. Beam prediction in time and/or spatial domain can help reduce overhead and latency, as well as improving beam selection accuracy. They are good candidates for evaluating AI/ML based algorithms for beam management.

As the Rel-18 study on AI/ML for NR air interface is the first one in 3GPP that explores the benefits of augmenting air interface with features enabling improved support of AI/ML based algorithms, it is important to calibrate evaluation results from different companies in order to facilitate drawing observations and making conclusions.

3GPP has established simulation methodology (see, e.g., TR 38.901 [2]), which can be used to generate synthetic data for the study of AI/ML based algorithms for beam management. Beam management is particularly relevant for NR operation in millimeter wave frequencies. The typical millimeter wave deployment is in urban scenarios where there is high-capacity demand. Therefore, the evaluation could focus on UMi-street canyon and UMa scenarios.

However, the simulation layout for UMi-street canyon and UMa scenarios described in TR 38.901 is much simplified compared to real-world urban scenarios. Additional simulation methodology for generating synthetic data, such as digital twins, can be explored. A digital twin is a virtual representation — a true-to-reality simulation of physics and materials — of a real-world physical asset or system, which is continuously updated. Digital twins can help generate synthetic data that are closer to real-world data, compared to the traditional 3GPP statistical simulation methodology.

While synthetic data can be the baseline for evaluating AI/ML based algorithms for beam management, it would be beneficial to identify existing sets of real data and/or build up new sets of real data, as part of the 3GPP Rel-18 AI/ML study for NR air interface. Such efforts would pay off as it is anticipated that AI/ML will become increasingly more integrated into the 3GPP family of technologies from 5G Advanced to 6G.

Proposal 1: Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for beam management.

Model generalization measures the ability of an AI/ML model to perform in new and unseen situations that do not form part of its training experience. It is natural that there is performance difference between how the AI/ML model performs in the training experiences it has seen and in the new unseen situations it is evaluated on. The smaller the performance difference, the better the AI/ML model’s generalization ability.

There are many different types of “unseen situations.” In AI/ML community, the type of generalization commonly investigated is the case where the distribution of the training samples is the same as that of the test samples. This type of generalization is referred to as “in-distribution generalization.”

In practise, there are often distribution shifts between training samples and test samples. Therefore, it is of practical relevance to study out-of-distribution generalization, which refers to generalization that is not in-distribution generalization. Certainly, one cannot expect the AI/ML model to generalize to test data that is arbitrary or unrelated to the training data. Therefore, specific assumptions are needed to address the out-of-distribution generalization for the problem in question.

To investigate the model generalization capability, as discussed in the previous RAN1 meetings, the following aspects can be considered for the evaluation for AI/ML in beam management:

- Different UE parameters: UE speed, UE antenna configuration, UE trajectory, number of Rx beams, UE antenna height, etc.

- Different NW settings: BS antenna configuration (e.g., number of Tx beams, Tx beam width, TX beam boresight directions, etc.), Tx beam pattern, BS antenna height, etc.

- Different Scenarios: UMa, UMi, including UE distribution, etc.

Proposal 2: To investigate the model generalization capability, the following aspects can be considered for the evaluation for AI/ML in beam management:

- Different UE parameters: UE speed, UE antenna configuration, UE trajectory, number of Rx beams, UE antenna height, etc.

- Different NW settings: BS antenna configuration (e.g., number of Tx beams, Tx beam width, TX beam boresight directions, etc.), Tx beam pattern, BS antenna height, etc.

- Different Scenarios: UMa, UMi, including UE distribution, etc.

# 3	KPIs

The study item description lists many dimensions for KPIs, including performance, inference latency, computational complexity, overhead, power consumption, memory storage, hardware requirements, and generalization capability.

Though it is beneficial to have a full characterization of the performance of AI/ML based algorithms for NR air interface, it is important to focus on a few most important KPIs in the initial phase to understand the gains of AI/ML based algorithms.

From beam management perspective, overhead reduction, latency reduction, and accuracy improvement are highlighted in the study item description.

- Overhead may include reference signal overhead and measurement reporting overhead. With beam prediction, less frequent reference signal transmission and/or less measurement reporting may be needed.

- Latency may include reference signal transmission time, UE measurement time, UE measurement reporting time, among others. With beam prediction, the latency may be reduced.

- Beam management based on UE measurement and reporting may not be accurate due to UE measurement error and the feedback delay in UE measurement reporting. Beam prediction may improve the accuracy.

Also, many of the KPIs such as inference latency depend on the used computing platform (such as the GPU model). Therefore, it is important to report the KPIs together with the used computing platform (such as the GPU model).

It was discussed that complexity should be evaluated as a KPI, where complexity include model complexity and computational complexity. For evaluation of AI/ML based beam management, the computational complexity can be reported via the metric of floating point operations (FLOPs), and the model complexity may be measured by memory storage in terms of AI/ML model size and number of AI/ML parameters.

Proposal 3: For evaluation of AI/ML based beam management, the computational complexity can be reported via the metric of floating point operations (FLOPs) for inference.

Proposal 4: For evaluation of AI/ML based beam management, the model complexity may be measured by memory storage in terms of number of AI/ML model parameters.

It is however important to keep in mind that increasing hardware performance can support successively more complex models. For example, Figure 1 shows how single GPU performance has scaled up to meet the demands of deep learning. GPU inference performance has improved by 317x, more than doubling each year. Figure 2 shows single GPU FP64 performance increased by 20x over the decade from 2010 to 2020, an annual growth rate of 35%. In addition to scaling up the performance of individual GPUs, GPUs are also being scaled out to larger clusters for deep learning and high performance computing applications.

Observation 1: Increasing hardware performance can support successively more complex AI/ML models. For example, GPU inference performance has improved by 317x in 8 years (2012-2020), more than doubling each year.

Proposal 5: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for beam management enhancements.

Figure 1: GPU inference performance is more than doubling every year. (Source: Ref. [3])

Figure 2: Single GPU performance scaling. (Source: Ref. [3])

# 4	Evaluation results

In this section, we provide initial evaluation results on beam management using AI/ML based algorithms. The system-level simulation assumption and scenarios are built on the basis of the RAN1#109-e agreements.

## 4.1	Beam prediction in spatial domain

For AI/ML-based beam prediction in spatial domain, we study BM-Case 1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams. In the evaluation, Set A consists of 64 beams, while Set B consists of 16 beams, which is a subset of Set A.

The AI/ML-based beam prediction utilizes a convolutional neural network (CNN). The input to the CNN is the RSRP values of the 16 beams in Set B. The output of the CNN is the predicted best beams in the full Set A.

As a benchmark, we use exhaustive beam sweeping, where the best beam within Set A of 64 beams is determined based on the RSRP values of all the 64 beams in Set A.

Figure 3 shows the beam prediction accuracy (%) for Top-K beams, where K ranges from 1 to 8. The beam prediction accuracy is the percentage of the Top-1 beam from the exhaustive beam sweeping being one of the Top-K predicted beams. It can be seen that the prediction accuracy for Top-4 beams exceeds 95%.

Figure 3:Beam prediction accuracy for Top-K beams in spatial domain.

Figure 4 shows the CDF of RSRP difference for Top-1 predicted beam, where the RSRP difference is the difference between the RSRP of Top-1 predicted beam and the RSRP of the Top-1 beam from the exhaustive beam sweeping. The mean of the RSRP difference for Top-1 predicted beam is 0.27 dB.

Figure 4: CDF of RSRP difference for Top-1 predicted beam in spatial domain.

In summary, the results in Figure 3 and Figure 4 show that the AI/ML-based beam prediction in spatial domain, which measures a subset of 16 beams from a full set of 64 beams, can achieve performance (in terms of RSRP) comparable to that of exhaustive beam search, while the reference signal overhead, measurement effort, reporting overhead, and latency can be much reduced as only 16 beams out of the total of 64 beams need to be measured and reported.

Observation 2: AI/ML-based algorithms for beam prediction in spatial domain can achieve performance comparable to that of exhaustive beam search, while the reference signal overhead, measurement effort, reporting overhead, and latency can be much reduced.

## 4.2	Beam prediction in time domain

For AI/ML-based beam prediction in time domain, we study BM-Case 2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams. In the evaluation, Set A consists of 64 beams, and Set B is the same as Set A.

The AI/ML-based beam prediction utilizes a long short term memory (LSTM) network which is a special kind of recurrent neural network (RNN).

- The optimal beam indices of K=8 latest measurement instances are used for AI/ML model input, where each input of optimal beam index is for each measurement instance.

- The AI/ML model output is up to F=8 predictions for F=8 future time instances, where each prediction of the optimal beam index is for each time instance.

- The sampling interval (i.e., the time interval between two consecutive time instances) is 40 ms.

- The UE speed is 120 km/h. With this high speed, all UEs are assumed to be outdoor.

Figure 5 shows the beam prediction accuracy (%) for Top-K beams, where K ranges from 1 to 8. The beam prediction accuracy is the percentage of the ground-truth Top-1 beam being one of the Top-K predicted beams. It can be seen that the prediction accuracy for Top-8 beams is close to 90% for the farthest future instance of 8 x 40 ms.

Figure 6 shows the average RSRP difference for Top-K predicted beam, where the RSRP difference is the difference between the maximum of RSRPs of Top-K predicted beams and the RSRP of the ground-truth Top-1 beam. It can be seen that the average RSRP difference for Top-2 predicted beam is smaller than 1 dB for the farthest future instance of 8 x 40 ms.

Figure 5: Beam prediction accuracy for Top-K beams in time domain.

Figure 6: Average RSRP difference for Top-K predicted beam in time domain.

The evaluation results show that using a history of the best beam index, the AI/ML model can predict the best beam for future time instances. Such temporal beam prediction can help lower reference signal overhead and reduce UE’s measurement requirement as the prediction can help narrow down candidate beams for UE to measure.

In the evaluation, we use only a history of the best beam index to perform the prediction. Temporal prediction performance may be further improved with additional input of information such as the corresponding RSRP, UE position, etc.

Observation 3: AI/ML-based algorithms for beam prediction in time domain can simply use a history of the best beam index to perform the prediction.

Observation 4: AI/ML-based algorithms for beam prediction in time domain can help lower reference signal overhead and reduce UE’s measurement requirement.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: Increasing hardware performance can support successively more complex AI/ML models. For example, GPU inference performance has improved by 317x in 8 years (2012-2020), more than doubling each year.

Observation 2: AI/ML-based algorithms for beam prediction in spatial domain can achieve performance comparable to that of exhaustive beam search, while the reference signal overhead, measurement effort, reporting overhead, and latency can be much reduced.

Observation 3: AI/ML-based algorithms for beam prediction in time domain can simply use a history of the best beam index to perform the prediction.

Observation 4: AI/ML-based algorithms for beam prediction in time domain can help lower reference signal overhead and reduce UE’s measurement requirement.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for beam management.

Proposal 2: To investigate the model generalization capability, the following aspects can be considered for the evaluation for AI/ML in beam management:

- Different UE parameters: UE speed, UE antenna configuration, UE trajectory, number of Rx beams, UE antenna height, etc.

- Different NW settings: BS antenna configuration (e.g., number of Tx beams, Tx beam width, TX beam boresight directions, etc.), Tx beam pattern, BS antenna height, etc.

- Different Scenarios: UMa, UMi, including UE distribution, etc.

Proposal 3: For evaluation of AI/ML based beam management, the computational complexity can be reported via the metric of floating point operations (FLOPs).

Proposal 4: For evaluation of AI/ML based beam management, the model complexity may be measured by memory storage in terms of number of AI/ML parameters.

Proposal 5: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for beam management enhancements.

# Reference

RP-221348, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #96, June 2022.

TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz.”

W. J. Dally, S. W. Keckler and D. B. Kirk, "Evolution of the Graphics Processing Unit (GPU)," in IEEE Micro, vol. 41, no. 6, pp. 42-51, 1 Nov.-Dec. 2021.

# Appendix

## A.1	RAN1#109-e agreements

Agreement

- For dataset construction and performance evaluation (if applicable) for the AI/ML in beam management, system level simulation approach is adopted as baseline

  - Link level simulation is optionally adopted

Agreement

- At least for temporal beam prediction, companies report the one of spatial consistency procedures:

  - Procedure A in TR38.901

  - Procedure B in TR38.901

Agreement

- At least for temporal beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

- For spatial-domain beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

Agreement

- At least for spatial-domain beam prediction in initial phase of the evaluation, UE trajectory model is not necessarily to be defined.

Agreement

- At least for temporal beam prediction in initial phase of the evaluation, UE trajectory model is defined. FFS on the details.

Agreement

- UE rotation speed is reported by companies.

  - Note: UE rotation speed = 0, i.e., no UE rotation, is not precluded.

Agreement

- For AI/ML in beam management evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline.

Conclusion

- Further study AI/ML model generalization in beam management evaluating the inference performance of beam prediction under multiple different scenarios/configurations.

  - FFS on different scenarios/configurations

  - Companies report the training approach, at least including the dataset assumption for training

Agreement

- For evaluation of AI/ML in BM, the KPI may include the model complexity and computational complexity.

  - FFS: the details of model complexity and computational complexity

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in Table 1.2-1b for Dense Urban scenario for SLS

Table 1.2-1b Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 30km/h (baseline), 60km/h (optional) - Other values are not precluded |
| UE distribution | - FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded. - For spatial domain beam prediction: FFS: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration |  [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline]  [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional]  Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M,N,P) = (1,4,2)]  2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline  Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: - Option 1: Full buffer - Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Agreement

- For temporal beam prediction, the following options can be considered as a starting point for UE trajectory model for further study. Companies report further changes or modifications based on the following options for UE trajectory model. Other options are not precluded.

  - Option #2: Linear trajectory model with random direction change.

    - UE moving trajectory: UE will move straightly along the selected direction to the end of an time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - UE move straightly within the time interval with the fixed speed.

    - FFS on UE orientation

  - Option #3: Linear trajectory model with random and smooth direction change.

    - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N.

      - UE move straightly within the time sub-interval with the fixed speed.

    - FFS on UE orientation

  - Option #4: Random direction straight-line trajectories.

- Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed.

  - The initial UE location should be randomly drop within the following blue area

where d1 is the minimum distance that UE should be away from the BS.

    - Each sector is a cell and that the cell association is geometry based.

    - During the simulation, inter-cell handover or switching should be disabled.

For training data generation

- For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance.

  - The value of T (or D) can be further discussed

  - The trajectory sampling interval granularity depends on UE speed and it can be further discussed.

- UE can move straightly along the entire trajectory, or

- UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length ΔT

  - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]

- If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated.

  - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded.

  - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.

    - FFS on UE orientation

- Generalization issue is FFS

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the following assumption for LLS as optional methodology

| Parameter | Value |
| --- | --- |
| Frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Data allocation | [8 RBs] as baseline, companies can report larger number of RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is oppler) |
| Channel model | FFS: LOS channel: CDL-D extension, DS = 100ns NLOS channel: CDL-A/B/C extension, DS = 100ns Companies explains details of extension methodology considering spatial consistency Other channel models are not precluded. |
| BS antenna configurations | - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline - Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS antenna element radiation pattern | Same as SLS |
| BS antenna height and antenna array downtile angle | 25m, 110° |
| UE antenna configurations | Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE antenna element radiation pattern | Same as SLS |
| UE moving speed | Same as SLS |
| Raw data collection format | Depends on sub-use case and companies’ choice. |

Agreement

- For UE trajectory model, UE orientation can be independent from UE moving trajectory model. FFS on the details.

  - Other UE orientation model is not precluded.

Agreement

- Companies are encouraged to report the following aspects of AI/ML model in RAN 1 #110. FFS on whether some of aspects need be defined or reported.

  - Description of AI/ML model, e.g, NN architecture type

  - Model inputs/outputs (per sub-use case)

  - Training methodology, e.g.

    - Loss function/optimization function

    - Training/ validity /testing dataset:

      - Dataset size, number of training/ validity /test samples

      - Model validity area: e.g., whether model is trained for single sector or multiple sectors

      - Details on Model monitoring and model update, if applicable

  - Others related aspects are not precluded

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

  - System performance related KPIs, may include the following options:

    - UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

    - RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

      - 1-N/M,

      - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where (FFS) M is the total number of beams

      - Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

      - FFS on whether to define a proper value for M for evaluation.

    - Other System performance related KPIs are not precluded and can be reported by companies.

- Other KPIs are not precluded and can be reported by companies, for example:

    - Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

    - Latency reduction:

      - (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

        - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

        - where M is the total number of beams

    - Power consumption reduction: FFS on details

## A.2	RAN1#110 agreements

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE distribution | - FFS 10 UEs per sector/cell for system performance related KPI (if supported) [e.g,, throughput] for full buffer traffic (if supported) evaluation (model inference). - X UEs per sector/cell for system performance related KPI for FTP traffic (if supported) evaluation (model inference). - Other values are not precluded - Number of UEs per/sector per cell during data collection (training/testing) is reported by companies if relevant - More UEs per sector/cell for data generation is not precluded. |
| UE Antenna Configuration | - Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) - [Panel structure: (M,N,P) = (1,4,2)] - panels (left, right) with (Mg, Ng) = (1, 2) as baseline - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 3km/h(optional), 30km/h (baseline), 60km/h (optional), 90km/h (optional), 120km/h (optional) - Other values are not precluded |
| UE distribution | - For spatial domain beam prediction: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |

Agreement

- If UE orientation is modeled, it can be independently modeled from UE moving trajectory model.

  - This is not precluded that UE orientation coupled with UE moving trajectory model.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement

- To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options.

  - FFS: number of UCI reports and UCI payload size