3GPP TSG RAN WG1 #110-bis-e		                                    		   R1-2209657

Electronic, Oct 10 – Oct 19, 2026

Agenda Item:	9.11.1

Source:	InterDigital, Inc.

Title:	Discussion on XR specific power saving techniques

Document for:	Discussion and Decision

# 1	Introduction

In RAN1#109e, the following conclusions and agreements were made on power saving techniques [1]:

Conclude that “SFN wraparound mismatch” is a RAN2 issue. It can be left to RAN2 to address. RAN1 does not further study it. Agreement RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity Conclusion RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA. Conclusion All the proposed PDCCH monitoring adaptation/reduction schemes including those for jitter handling need to be compared against the Rel-17 PDCCH monitoring adaptation which is to be used as performance reference. Conclusion UE transmission and reception alignment for Issue 3-1 is deprioritized for power saving in Rel-18 XR SI. Conclusion RAN1 does not assume dynamic switch of different XR video data rates or frame rates for Rel-18 XR power saving study before further input is provided by SA.

In this contribution, we discuss the potential enhancements and evaluations for power savings for XR traffic.

# 2	Discussion

During Rel-17 SI [2], the traffic models for different XR applications (AR, VR and cloud gaming) were defined and the power savings evaluations when supporting XR traffic in DL and UL were performed.

Given the small device form factor and limited battery capacity, achieving power savings in UE is important when supporting any of the XR applications. The typical XR traffic patterns defined by non-integer/high periodicity, jitter and variable payload sizes allow limited opportunity for operating in low power mode and achieving power savings with legacy techniques (e.g. semi-static CDRX).

Observation 1:	XR traffic patterns (e.g. defined by non-integer/high integer periodicity, jitter, variable data rates and multiple correlated flows), make it extremely challenging for achieving power savings with legacy semi-static techniques

The Rel-17 evaluations also indicate a tradeoff relationship between power savings gain and capacity achievable. This is mainly due to mismatch between the CDRX parameters configured in UE (e.g. cycle duration, ON/active duration) and the supported XR traffic patterns.

The discussions in previous RAN1#110 meeting [1] include identification of a solution for CDRX to align with XR traffic periodicity and several conclusions regarding jitter handling. In this regard, whether and how the XR-specific traffic patterns can be accommodated in any of the power saving enhancements should be further discussed. In our view, the main XR-specific issues related to power savings that should be investigated are:

- Mismatch between CDRX cycle and XR traffic pattern for each flow

- Jitter handling

- Multiple XR traffic flows/patterns handling

In the following sections, we discuss the power saving scheme enhancements for addressing the above issues. We also provide evaluation results showing the performance of the enhancement schemes.

# 3	CDRX enhancements

As discussed during previous RAN1#109e and RAN1#110 meetings, XR traffic consists of PDU sets which are delivered in one or multiple flows. A PDU set, in turn, is composed of one or multiple PDUs carrying the payload of one unit of information generated at the application level (e.g. video frame). The PDU sets may be subject to new QoS requirements such as PDU set delay bound (PSDB) and PDU set error rate (PSER), which need to be met during transmissions.

For power savings, when the UE is configured with legacy CDRX with integer periodicity values (e.g. 8ms, 10ms, 16ms), the non-integer periodicity values associated with the PDU set generation rates (e.g. 16.66ms, 8.33ms) results in a mismatch when receiving DL data. The mismatch between the XR traffic (e.g. PDU sets) and CDRX periodic occasions may occur after a number of cycles, where the earliest cycle at which the mismatch occurs depends on the CDRX periodicity configured for a given PDU set generation rate. In this regard, how to configure and/or adapt the periodicity of CDRX cycles to align with the XR traffic should be discussed.

Observation 2:	The earliest cycle at which the mismatch between XR traffic (e.g. PDU sets) and CDRX occasions occur depends on the CDRX periodicity configured for a given PDU set generation rate

In one approach, since the number of cycles for the mismatch to occur can be predetermined, the CDRX configuration can be readjusted by periodically changing the start offset (e.g. advancing/delaying the start offset) by some fixed duration to realign with the XR traffic. In this case, the number of cycles for readjustment and start offset adjustment duration can be preconfigured in the UE and used when detecting any misalignments. A similar approach would be to configure non-uniform CDRX cycle patterns (e.g. 16ms, 16ms, 17ms) in the UE that may correspond closely with the non-integer XR traffic periodicity.

While such semi-static approaches may address the mismatch issue to some extent, optimizations to CDRX by introducing new parameters (e.g. number of cycles before readjustment, non-unform cycle pattern) that are closely related to the existing video frame generation rates (e.g. 60fps, 120fps) may be less robust against a wide variety of data generation rates and traffic patterns that the application/network may support.

Observation 3:	Applying semi-static approaches to CDRX enhancements for XR (e.g. non-unform cycle pattern) are less robust against a variety of data generation rates and traffic patterns that the application/network may support

In the following, enhancements to CDRX for addressing the above issues are discussed.

- 2.1 Dynamic adaptation of CDRX Active time

The gNB has visibility of the DL XR traffic patterns (e.g. PDU set/data burst payload sizes). In this case, one approach for addressing the mismatch issue is by dynamically signaling to the UE to adjust the CDRX active time in the current CDRX cycle. The adjustment to the active time can be done by increasing/decreasing the ON duration or inactive timer for receiving the PDUs in one or multiple PDU set burst. For example, when the traffic arriving at gNB has high number of PDUs and strict PSDB, the length of the current ON duration applied at UE can be extended or an additional ON durations can triggered for receiving the PDUs in the PDU set. If the PDU set has low number of PDUs which can be delivered using a shorter active time, the current ON duration at UE can be shortened.

For supporting this approach, the UE can be preconfigured with different ON duration length values and/or inactivity timer values when configuring the CDRX configuration. Then a dynamic indication can be sent (e.g. in DCI) during the current active time, for example, when detecting a mismatch between XR traffic pattern and the configured CDRX parameters. For minimizing the amount of signaling, a single indication (e.g. single DCI) can be considered for adapting the length of the active time for multiple CDRX cycles. When receiving the indication, the UE can adjust the CDRX according to the indicated parameters for receiving the PDU sets.

Proposal 1:	Support configuring multiple CDRX parameters (e.g. ON duration, inactivity timer) for handling adjustments to CDRX configuration

Proposal 2:	Support adaptation (e.g. via DCI) for increasing/decreasing the length of CDRX active time per cycle

Proposal 3: 	Study single indication (e.g. single DCI) for dynamically adapting the length of CDRX active time for multiple cycles

- 2.2 Dynamic adaptation of start time offset of ON duration

The presence of jitter during data transport in network may result in the data arrival to deviate from the expected non-integer periodicity associated with the XR traffic. Jitter can cause arrival of PDUs within a PDU set (e.g. intra-PDU set jitter) and across different PDU sets (e.g. inter-PDU set jitter) to be delayed/advanced when arriving at gNB. For intra-PDU set jitter, the PDU set boundary corresponding to the arrival time of the first PDU and last PDU of a PDU set may vary instead of arriving periodically in a burst aligned with the frame generation periodicity. For inter-PDU set jitter, the arrival time of different PDU sets may vary from one another instead of following the frame generation periodicity.

For addressing jitter, the adjustments to the start offset time can be dynamically signaling to the UE to advance or delay the ON duration in a CDRX cycle. The dynamic indication sent to UE (e.g. in DCI) during the ON duration in the current cycle can indicate the preconfigured start offset value to apply for the next cycle. Similar to previous section, a single indication (e.g. single DCI) can be considered for indicating adaptation to the start offset time for multiple CDRX cycles to minimize overhead. Adjusting the start offset can be useful for flexibly sending the PDUs in the presence of jitter especially when configured CDRX at UE has short cycles/ON durations.

Proposal 4:	Support adaptation (e.g. via DCI) for advancing/delaying the start offset time of the CDRX ON duration per cycle

Proposal 5: 	Study single indication (e.g. single DCI) for dynamically adapting the start offset time of CDRX ON duration for multiple cycles

- 2.3 UE-initiated CDRX parameter adjustments

When supporting split rendering applications (e.g. VR, AR), the UE sends pose and/or video traffic in UL, and in response, receives the pre-rendered video traffic in the DL. For ensuring adequate user experience, after sending the UL data, the DL data should be received within a maximum RTT latency (e.g. corresponding to motion-to-render-to-photon latency). Since the UE is both the source and destination of the UL and DL traffic, the higher layers in UE can determine the traffic pattern expected in DL (e.g. payload sizes of DL PDU sets, arrival time of DL traffic) based on the traffic transmitted in UL and knowledge of RTT latency.

In these scenarios, when identifying that the configured CDRX parameters (e.g. ON duration, start offset) are not suitable for handling the traffic pattern expected in DL, the UE can send an indication to gNB to request certain adaptation to the CDRX parameters. For example, the UE can send a dynamic indication (e.g. in UCI or MAC CE) for extending the ON duration or changing the start offset time of the ON duration in next cycle for receiving the DL traffic within the RTT latency.

Proposal 6:	Support UE sending an indication to gNB to request dynamic adaptation to CDRX parameters (e.g. ON duration, start offset) for receiving DL traffic

- 2.4 Multiple active CDRX configurations

During Rel-17 evaluations [2], the power saving performance achievable with CDRX for XR applications with multiple flows indicate only marginal gain compared to the always-on baseline case. Such degradation is due to handling of high traffic load (e.g. for aggregated video) and different traffic patterns associated with the multiple flows, which typically require the use of long ON duration/active time. In this case, how to realize power savings when supporting different traffic patterns/flows should be discussed.

In one approach, it can be beneficial to support multiple active CDRX configurations corresponding to the different flows. For example, when handling a flow with small payload sizes and another flow with large payload sizes, dedicated CDRX configurations with short/long ON durations per cycle can be used instead of a single CDRX configuration with high periodicity and/or long ON duration for supporting multiple flows. The simulation results showing the power saving gain when supporting multiple active CDRX configurations are provided in Section 5.

It is also beneficial to dynamically activate/deactivate multiple CDRX configurations at once for matching with the traffic patterns of different flows and for reducing overhead. Essentially, by using multiple active CDRX configurations that can be flexibly activated/deactivated, it would be possible to realize both power savings gains and meet the QoS corresponding to the different traffic flows/patterns.

Proposal 7:	Support multiple active CDRX configurations for handling multiple flows with different traffic patterns

Proposal 8:	Support dynamic activation/deactivation of multiple CDRX configurations

In another approach, the UE can be configured with multiple CDRX configurations with different parameters (e.g. periodicity, ON duration, start offset) associated with handling of different types of PDU sets (e.g. I-frames or P/B frames). When receiving a DL data (e.g. PDU set carrying I-frame data) with an initial CDRX configuration, the higher layers in UE may determine the traffic pattern for the DL data in next cycle (e.g. based on GOP structure and frame rate). This information can be used by the UE for identifying other preconfigured CDRX configurations that match with the traffic pattern of the next DL data. The UE can send in a dynamic indication to gNB on the identified CDRX configurations that can be used for receiving the next DL data. The gNB can then dynamically activate/deactivate the CDRX configurations in the UE for receiving the DL traffic.

Proposal 9:	Support UE sending an indication to gNB to request activation of preconfigured CDRX configurations

# 4	PDCCH monitoring enhancements

Reducing PDCCH monitoring to improve power savings while supporting XR traffic patterns (e.g. high periodicity, variable PDU set sizes, multiple correlated flows) and stringent QoS is challenging. Rel-17 enhancements for PDCCH monitoring include PDCCH skipping and SSSG switching. Whether and how the PDCCH monitoring schemes can accommodate the XR traffic patterns should be discussed.

Similar to the previous section, dynamic adaptations to PDCCH monitoring behavior can be considered when handling XR traffic patterns and jitter. For example, when delivering the last PDU of a PDU set or a data burst, the DCI can indicate the PDCCH skipping duration until the arrival of the next PDU set/burst. This approach can be useful especially when handling PDU sets with short data bursts, where the UE can be transitioned to sleep mode for longer durations until the arrival of the next PDU set/burst. The skipping durations can be based on the periodicity values supported for delivering the PDU sets in one or multiple traffic flows (e.g. corresponding to different frame rates) and jitter range.

Proposal 10:	Support PDCCH monitoring adaptation for dynamically increasing/decreasing the PDCCH skipping duration

Proposal 11:	Study PDCCH skipping duration values that can be used for handling XR traffic patterns

When considering UE-initiated adaptations to PDCCH monitoring, similar to the discussion in previous section on handling of UL dependent DL traffic, the UE can indicate to gNB the PDCCH skipping duration it intends to use for receiving the next DL transmission. This can be based on the knowledge of RTT latency and association between UL and DL traffic available from higher layers at the UE. Since jitter in network may impact the arrival of DL traffic, the UE can account for the jitter by initiating PDCCH monitoring in sparse SSSG prior to the indicated skipping duration and switch to dense SSSG at the end of the skipping duration.

Proposal 12:	Support UE dynamically indicating the duration for PDCCH skipping

For supporting certain retransmissions after indicating in DCI to trigger PDCCH skipping, the Rel-17 scheme allows the skipping to start only after the next slot. This, however, results in further reduction in the sleep duration available for the UE. When handling XR traffic with high periodicity or data bursts with high number of PDUs/PDU sets, it is possible that PDCCH skipping may not translate to high power saving gains. For improving the sleep duration, the gNB can minimize the possibility for retransmissions by using robust MCS when transmitting the last PDU of PDU set/burst before sending the indication for PDCCH skipping. Alternatively, the PDCCH skipping can be indicated using a non-scheduling DCI so that the UE can enter sleep mode soon after reception of the last PDU of PDU set/burst. Whether this approach can translate to power savings and meet latency requirement for XR traffic, especially when considering variable PDU set/burst sizes and variable transmission duration, should be discussed.

Proposal 13:	Study PDCCH skipping triggered by non-scheduling DCI

# 5	Simulation Results

In this section we provide system level simulation (SLS) results on the performance of multiple active CDRX configurations when handling multiple flows in DL. The SLS are performed using baseline evaluation assumptions and parameters as listed in TR 38.383 (Annex in Section 7).

We consider an Indoor Hotspot (InH) deployment. The UE receives the following flows in DL:

- AR video traffic: 30 Mbps @ 30 fps with a PDB of 10 ms.

- Audio traffic: 0.756 Mbps every 10 ms with a PDB of 30 ms.

The following are the parameters applied for the multi-CDRX schemes:

|  |  | DRX cycle (ms) | On duration timer value (ms) | Inactivity timer value (ms) |
| --- | --- | --- | --- | --- |
| Multi-CDRX (Set 1) | CDRX1 (Video) | 33 | 10 | 23 |
|  | CDRX2 (Audio) | 10 | 2 | 8 |
| Multi-CDRX (Set 2) | CDRX1 (Video) | 33 | 24 | 9 |
|  | CDRX2 (Audio) | 10 | 2 | 8 |

The power saving gain performance of multi-CDRX schemes are compared with respect to the baseline single active CDRX scheme that supports both the DL video and audio traffic. The parameters of the baseline single CDRX are (10, 8, 2).

To simulate the multi-CDRX scheme, each UE is modeled as two virtual UEs with the same channel conditions, where the first virtual UE receives video traffic and uses a CDRX1. Similarly, the second virtual UE receives audio traffic and uses CDRX2.

Using multi-CDRX configurations based on different traffic patterns can help achieve higher PSG as shown in the following table.

| Scenario | Mean PSG | 5% Percentile PSG | 50% Percentile PSG | 95% Percentile PSG |
| --- | --- | --- | --- | --- |
| Single CDRX Config (10,8,2) | N/A | N/A | N/A | N/A |
| Multi-CDRX Config Set 1 | 12.52% | 0% | 3.37% | 36.88% |
| Multi-CDRX Config Set 2 | 3.17% | 0% | 2.87% | 9.59% |

The following figures show the achievable PSGs of Multi-CDRX configuration schemes when compared with respect to the single active CDRX scheme and always ON case (no CDRX).

Figure 1: PSG performance of multiple active CDRX configurations. (a) PSG of multi-CDRX with respect to single active CDRX, (b) PSG of multi-CDRX with respect to always-ON case

Observation 4:	Using multiple active CDRX configurations when handling multiple flows with different traffic patterns helps achieve a higher PSG compared to using single CDRX configuration

Observation 5:	Multi-CDRX config Set 1 achieves a higher PSG than Multi-CDRX config Set 2 due to having a shorter ON/active duration, which is more aligned with the traffic patterns of AR video and audio

# 6	Conclusion

In this contribution, the following observation are made:

Observation 1:	XR traffic patterns (e.g. defined by non-integer/high integer periodicity, jitter, variable data rates and multiple correlated flows), make it extremely challenging for achieving power savings with legacy semi-static techniques

Observation 2:	The earliest cycle at which the mismatch between XR traffic (e.g. PDU sets) and CDRX occasions occur depends on the CDRX periodicity configured for a given PDU set generation rate

Observation 3:	Applying semi-static approaches to CDRX enhancements for XR (e.g. non-unform cycle pattern) are less robust against a variety of data generation rates and traffic patterns that the application/network may support

Observation 4:	Using multiple active CDRX configurations when handling multiple flows with different traffic patterns helps achieve a higher PSG compared to using single CDRX configuration

Observation 5:	Multi-CDRX config Set 1 achieves a higher PSG than Multi-CDRX config Set 2 due to having a shorter ON/active duration, which is more aligned with the traffic patterns of AR video and audio

Based on these observations, the following proposals are made:

Proposal 1:	Support configuring multiple CDRX parameters (e.g. ON duration, inactivity timer) for handling adjustments to CDRX configuration

Proposal 2:	Support adaptation (e.g. via DCI) for increasing/decreasing the length of CDRX active time per cycle

Proposal 3: 	Study single indication (e.g. single DCI) for dynamically adapting the length of CDRX active time for multiple cycles

Proposal 4:	Support adaptation (e.g. via DCI) for advancing/delaying the start offset time of the CDRX ON duration per cycle

Proposal 5: 	Study single indication (e.g. single DCI) for dynamically adapting the start offset time of CDRX ON duration for multiple cycles

Proposal 6:	Support UE sending an indication to gNB to request dynamic adaptation to CDRX parameters (e.g. ON duration, start offset) for receiving DL traffic

Proposal 7:	Support multiple active CDRX configurations for handling multiple flows with different traffic patterns

Proposal 8:	Support dynamic activation/deactivation of multiple CDRX configurations

Proposal 9:	Support UE sending an indication to gNB to request activation of preconfigured CDRX configurations

Proposal 10:	Support PDCCH monitoring adaptation for dynamically increasing/decreasing the PDCCH skipping duration

Proposal 11:	Study PDCCH skipping duration values that can be used for handling XR traffic patterns

Proposal 12:	Support UE dynamically indicating the duration for PDCCH skipping

Proposal 13:	Study PDCCH skipping triggered by non-scheduling DCI

# 7	Annex

The following show the parameters from TR 38.838 [2] used in system level simulations for XR.

Table 1: Assumptions for System-level simulations

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:64 Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,8,2,1,1;4,8), (dH, dV) = (0.5λ, 0.5λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | Reciprocity-based precoding |  |  |
| Scheduling Algorithm | DL: SU-MIMO with PF, FIFO, and resource sharing based scheduling |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

Table 2: DL Traffic models for CG/AR/VR evaluations

|  | CG | VR | AR |
| --- | --- | --- | --- |
| Data Rate | 30Mbps (baseline) @60fps | 30Mbps (baseline), 45Mbps @60fps | 30Mbps (baseline), 45Mbps @60fps |
| FPS | 60 fps (baseline) 120 fps (optional) Other values, e.g., 30, 90 fps can be also optionally evaluated. |  |  |
| Packet Arrival Distribution (single video stream) | Periodic (with periodicity = 1/fps) - Each packet k corresponds to set of IP packets belonging to video frame k - Jitter (with random distribution) is added to arrival slot of each packet k |  |  |
| Packet Size Distribution | Truncated Gaussian distribution - Mean: Derived from average data rate and fps as: (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes] - STD: [10.5% of Mean] - Max packet size: [150% of Mean] - Min packet size: [50% of Mean] |  |  |
| Air Interface PDB | 15ms (baseline) | 10ms (baseline) | 10ms (baseline) |
| Jitter (single video stream) | Arrival time of packet k is k/X x 1000 [ms] + J [ms], where X is the given fps value and J is a random variable (drawn from Truncated Gaussian Distribution) - Mean: [0], STD: [2 ms], Range: [-4, 4]ms] |  |  |

Table 3: UL Traffic models for CG/VR/AR evaluations

|  | Stream 1 (pose/control data) (for CG/VR/AR) | Stream 2 (aggregated video) (for AR) |
| --- | --- | --- |
| Packet Arrival distribution | Periodic: 4ms (no jitter) | Periodic (periodicity: 1/60fps) (no jitter) |
| Data rate | 0.2 Mbps | 10 Mbps |
| Packet Size distribution | 100 bytes | Truncated Gaussian, same parameters as DL (Table 2) |
| Air Interface PDB | 10ms | 30ms |
| Capacity KPI [X, PDB] | [99%,10ms] (baseline) | [99%,30ms] (baseline) |
| Jitter | Periodic: 4ms (no jitter) | Jitter: same model as for DL |

Table 4: DL Audio traffic model parameters

| Parameters | Baseline values for evaluation |
| --- | --- |
| Periodicity P | 10 ms |
| Data rate: R | 0.756 Mbps |
| Packet size | R×1e6 × P /1000 / 8 |
| PDB | 30 ms |
| Packet Success Rate | 99% |

Table 5: Power model for DL

| Power state | Relative Power(1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

# Reference

[1]	RAN1 chairman notes, RAN#110e, Aug 2022

[2] 	3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17), Dec. 2021

[3]	3GPP TR 23.700-60, “Study on XR (Extended Reality) and media services (Release 18), v0.3.0, May 2022