3GPP TSG RAN WG1 #110bis-e								    R1-2209659

e-meeting, October 10th – 19th, 2022

Agenda Item:	9.4.1.1

Source:	InterDigital, Inc.

Title:	SL channel access in unlicensed spectrum

Document for:	Discussion and Decision

# Introduction

In RAN1#110 meeting, different aspects of channel access in sidelink unlicensed operation were discussed and the agreements are captured in [1]. In this contribution we discuss type 1 and type 2 channel access, resource allocation and evaluation of SL-U and WiFi coexistence.

# Discussion

## Type 1 and Type 2 channel access procedures

Type 1 channel access:

In the RAN1#109e meeting, channel access procedures for sidelink unlicensed were discussed. Most companies have similar view that RAN1 should focus on dynamic channel access before discussing the semi-static channel access. The following was agreed on the different types of LBE channel access:

Agreement Type 1 and Type 2 (2A/2B/2C) channel access procedures, transmission gap and LBT sensing idle time requirements specified in TS37.213 for NR-U are taken as baseline for NR sidelink operation in a shared channel. - FFS conditions for the actual channel access type(s) used for each SL channel and signal transmitted, and based on COT sharing conditions (if supported) - FFS whether UL CAPC or DL CAPC or both should be used as the baseline, - FFS how the channel access priority classes apply to each SL channel and signal - FFS sidelink priority levels (PQI or L1 priority), channel and signal mapping to the 4 channel access priority classes. The discussion may involve other WGs.

Type 1 channel access is used when the UE initiate its own COT and can use 4 channel access priority classes depending on the transmission characteristics. The UE selects a random number of additional sensing slots uniformly distributed between 0 and the CW, where CW is depending on the CAPC value. One of the FFS points is about which CAPC table from the existing specification should be used for Type 1 sildelink unlicensed channel access. We think that using DL CAPC table for SL U can give a slight advantage to sidelink UEs over NR U UEs. Based on the values of UL CAPC and DL CAPC tables in the TS 37.213, UL CAPC table have larger values of the allowed contention window sizes compared to the values of DL CAPC table for the same priority. For example, for DL CAPC=3, the set of allowed are {15,31,63} while for UL CAPC=3, the set of allowed are {15,31,63,127,255,511,1023}. Note that the set of allowed CW is used in the CW size adjustment procedure, where CW is increased to the next allowed value under some conditions. Larger values of CW implies that the channel access will take longer time. We therefore propose the following:

Proposal 1: UL CAPC table is re-used for Type 1 SL U channel access mechanism

The CAPC value for a given grant can be indicated by the gNB for dynamic grant or the UE selects the CAPC based on the logical channels available for configured grant transmission. In NR-U, the MAC layer specifies how the UE associates logical channels to certain CAPC value. Similarly, RAN2 should specify how CAPC values are associated with logical channel and the relation with sidelink priority levels.

Apart from sidelink priority level, we think that CAPC selection can be impacted by other factors. For example, the cast type can be considered when selecting channel access priority class. The broadcast transmission can have higher channel access priority compared to unicast transmission with similar sidelink priority level. Another factor that can impact CAPC selection is resource allocation mode i.e., whether Mode 1 or Mode 2 is used. To some extent, similar differentiating is adopted in NR-U, where there are two different CAPC tables one for UL and one for DL transmissions.

Proposal 2: It is up to RAN2 to determine the association between sidelink priority levels and CAPC values.

Type 2 channel access:

With Type 2 channel access procedure, the UE senses the channel for a fixed duration for Type 2A/2B to determine if the channel is busy or not before it can transmit. For Type 2C, the UE does not sense the channel at all and transmits immediately.  In the last RAN1 meeting, the following were agreed:

Agreement - Type 2A/2B/2C SL channel access procedures - Type 2A channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2A is used also for the case of short control signalling transmission - Type 2B channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy - FFS the case when the gap is between 16 and 25us - FFS any other transmission by a UE (e.g., other than COT sharing) - Type 2C channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us. - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2C is used also for the case of short control signalling transmission - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs

One of the use cases of LBT Type 2 usage is used is COT sharing. A transmitter initiates a COT and share it with the receivers so they can transmit on the same COT. Depending on the gap between the transmissions, one of the sub types of LBT type 2 can be used. Another use case of LBT Type 2 is for short control signalling transmission. For example, when the UE transmitting SL-SSB with a duty cycle and total duration that complies with regulatory requirements, can be transmitted using Type 2C channel access. To add more protection to the short control signalling transmission, LBT type 2A can be applied before transmission to avoid interference from other unlicensed transmitters similar to DRS transmission in NR-U with duty cycle less than 1/20 and maximum duration of 1ms. Which LBT type 2A/2C to use prior to SCSt transmission can be (pre-)configured to the UE.

Proposal 3: Type 2A and Type 2C can be used for the case of short control signaling transmission.

For the channels that can qualify for SCSt transmission, we think both SL-SSB and PSFCH can be considered for SCSt. In the case SL-SSB/PSFCH transmissions will exceed the SCSt requirements (e.g., smaller duty cycle), the UE can use instead the Type 1 LBT or Type 2 if COT sharing is available.

Proposal 4: At least SL-SSB and PSFCH can be transmitted as SCSt when the regulatory requirements are satisfied.

In NR-U, the gNB indicates to the UE which LBT type to use within a shared COT based on its scheduling decision and the planned gap to leave between DL and UL transmissions. For SL-U, there are two options: option 1 is the UE initiating a COT to indicate which LBT type the Rx UE can use and option 2 is the UE initiating a COT will stop transmitting and Rx UE decides which LBT type to use depending on the gap between the two transmissions. Option 1 can be used if the UE initiating the COT is planning to use the COT in later occasion and Option 2 can be used if the UE initiating the COT has finished its transmissions and will not be using the COT again.

Proposal 5: The COT initiator UE can indicate which Type 2A/2B/2C SL-U channel access to use by other UEs for transmissions within a shared COT.

Retaining the channel after LBT:

In some cases, the UE can finish the LBT procedure before the start of sidelink transmission. RAN1 agreed to study the possibility to enable a UE retaining the channel after LBT procedure:

Agreement - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access

If the UE does not transmit immediately after the end of LBT procedure, the UE may lose its priority to transmit on the available channel and in that case, the UE should restart the LBT procedure. One option to retain the channel is to transmit a reservation signal after the LBT ends and before the start of SL transmission. Transmitting a cyclic prefix extension to retain the channel can be a good alternative. We think that for a gap between the ends of LBT procedure and the start of SL transmission that is less than one symbol, CPE can be used to retain the channel.

Proposal 6: At least re-use CPE to retain the channel for duration less than one symbol.

Another option could be to increase the possible starting positions within the slot to reduce the gap between the end of LBT and the start of SL transmission. This option combined with CPE transmission can help mitigating the possible loss of the channel after the end of LBT. These multiple starting positions can be reserved for high priority transmission to give them higher probability to access the channel.

Proposal 7: Multiple starting positions can be enabled by applying different CPE.

## COT sharing between sidelink UEs

In the last RAN1 meeting, the following alternatives for UE-to-UE COT sharing was agreed for further consideration:

Agreement - For UE-to-UE COT sharing, continue considering the following alternatives: - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS any additional conditions - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS how to determine a SL UE is a target receiverFFS: details of the channel type of the COT initiating UE’s transmission - FFS any additional conditions - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s). - FFS: details of the channel type of the responding UE’s transmission(s)

Alt1 restrict the responding UE to receive at least PSSCH from the COT initiator whereas Alt2 may include other transmissions. It can be beneficial to consider sharing the COT with responding UE to transmit PSSCH or other transmission(s). Therefore, Alt2 can offer more flexibility compared to Alt1.The responding UE can use the shared COT to at least transmit PSCCH/PSSCH/PSFCH to the COT initiator within the shared COT. The responding UE can then use the COT for other transmissions as well. To avoid uncoordinated COT sharing, the COT initiator can indicate its COT structure and assign time opportunities to responding UE(s) to use the COT.

Proposal 8: A responding UE can use the COT to at least transmit PSFCH/PSCCH/PSSCH to the COT initiator.

## Multiple transmissions within a COT

In the RAN1#110 meeting, it was agreed to support multi-consecutive slots transmission for both mode 1 and mode 2:

Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

Details on how to support the multi-slot transmission were left for further study.

The multi-consecutive slots can be used by one UE or by different UE(s) sharing the same COT. In our view, the consecutive slots can be at least used for transmissions of multiple TBs by one UE and the usage of multi-consecutive slots by different UEs in the case of COT sharing can be discussed after more details agreed on UE to UE COT sharing.

Proposal 9: Allow transmissions for one or more TBs in a COT by one UE.

The maximum COT duration depends on the CAPC used by the UE to initiate the channel. In some cases, the initiated COT can be long enough to potentially contain initial transmission and retransmissions. For example, when the UE performs blind retransmissions without waiting for the sidelink HARQ-ACK, the UE can transmit in a consecutive slot the initial transmission and the retransmissions/repetitions. In another scenario, if SL HARQ-ACK is enabled and the SL HARQ feedback is received prior to the end of the COT, retransmission can occur in the already initiated COT.

Proposal 10: Support initial transmission and re-transmissions of a TB within a COT.

The COT initiated by a sidelink UE may not be enough for transmitting multiple TBs and the corresponding re-transmissions. For example, a TB transmitted at the last slot in a COT will not have time for retransmission in the same COT. In such case, a different COT will be required to retransmit the TB. New approaches to link the initial transmission and re-transmission in different COTs need to be studied.

Proposal 11: Study re-transmissions of a TB in a different COT than the one including the initial transmissions.

With the LBT requirements in unlicensed spectrum, sidelink HARQ transmission can be delayed and can lead to exceed the remaining Packet Delay Budget (PDB). Solutions to minimize the impact of LBT uncertainty are needed to mitigate additional delays due to unlicensed spectrum. One approach could be to have PSFCH transmitted in the same COT as the corresponding PSSCH transmission. For example, PSFCH can be transmitted at the end of the COT where the corresponding PSSCH is received. Another example could be a time relationship between the PSSCH and PSFCH in the ongoing COT. The PSFCH position within a COT can be indicated in the PSSCH. This approach cannot always be applicable as it may not be enough resources available for COT sharing to transmit PSFCH. For example, the UE initiate a short COT that will contain only PSSCH or the remaining time in the ongoing COT is not sufficient for the Rx UE to decode and report PSFCH before the end of the COT. In such case, reporting PSFCH can be done a different COT. This COT can be initiated by Rx UE or Tx UE. In order to report the PSFCH transmission in different COT, HARQ-ACK polling can be used or an association between data transmitted in a first COT and PSFCH occasion in a second COT. To enable flexibility in PSFCH transmission time, fixed relationship between the PSSCH transmission time and the corresponding PSFCH transmission should be avoided.

Proposal 12: Support PSFCH transmission in a different COT than the corresponding PSSCH transmission.

## Resource allocation in SL U

Mode 1 resource allocation:

Regarding the mode 1 resource allocation for SL-U, it was agreed in RAN1#109e meeting to support the existing mode as a baseline and to further study possible enhancements:

Agreement - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 1 resource allocation selection procedure needs to be updated / enhanced due to shared spectrum channel access

The gNB is responsible for scheduling sidelink grants to Mode 1 UEs. gNB can use dynamic grant or configured grants (type 1 and type 2 configured grant are both supported). With dynamic grant, the UE is allocated with frequency assignment as well as the slot where to start the sidelink transmission. As mentioned before in the contribution, it is not guaranteed to access the unlicensed channel due to the dependency on LBT success/failure. The UE can fail to access the channel at the scheduled time and even after receiving a new grant to attempt to access the channel, the UE can fail again to access the channel. To increase the chance of accessing the channel, the gNB can schedule the UE with multiple time and frequency resources to access the sidelink unlicensed resources. For example, the UE can be configured with a time window and a set of frequency resources to access the channel.

Proposal 13: Support configuring Mode 1 UE with time window and set of frequency resources to initiate a COT in SL U.

In RAN1#110 it was agreed to further study the UE to report a COT related information to gNB:

Agreement - gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18 - FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA

In NR U, the gNB can determine if the UE was able to access the channel or not, as the uplink transmissions are intended to the gNB. With Mode 1 scheduling in sidelink unlicensed, the gNB cannot determine if the UE was able to access the channel or not since the scheduled transmission are intended to sidelink UE(s). To assist the gNB for scheduling additional grants or scheduling other sidelink UEs, channel access status can be reported to the gNB. For example, the UE can use new UCI type to report the channel access outcome to the gNB. The new UCI type can be transmitted using PUCCH or PUSCH. The UE can also report the COT related information to assist gNB scheduling in mode 1 RA.

Proposal 14: Study reporting of the channel access outcome and COT related information to the gNB in mode 1 SL U.

Mode 2 resource allocation:

For mode 2 resource allocation, it was agreed in RAN1#109e meeting that the existing mechanism will be supported as baseline and to further study the possible enhancements due to the shared spectrum:

Agreement - The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access

Sidelink supports different type of traffic for different use cases. Periodic type of traffic arriving at deterministic time is supported in NR sidelink Rel-16/Rel-17 and it is expected to be supported also in sidelink unlicensed operation. For this type of traffic, it is beneficial to reserve sidelink resource at the expected time of data arrival to avoid competing with other UEs each time. NR Rel-16/17 sidelink supports resource reservation by having a UE indicating the reservation period in the SCI. The resource reservation in unlicensed spectrum will also be beneficial as it can be applied to reduce the number of SL UEs contending for the channel at a given time. With LBT uncertainty, it is difficult to reserve the sidelink resource at specific time instance due to dependency on the LBT outcome. Instead, a time window can be reserved in unlicensed channels. The time window can consist of a set of slots that occurs periodically and starts approximately before the expected time of data arrival to account for possible LBT failure. The frequency resource reserved during this time window can be a set of interlaces/RBs, e.g., the same as the ones used by the initial transmission (similar to R16/R17 NR SL semi-persistent resource reservation) and not necessarily the entire bandwidth should be reserved.

Proposal 15 Study reservation of a periodic time window for periodic type of traffic in SL unlicensed spectrum.

The baseline mode 2 RA consists of sensing the channel during a sensing window and selects the resources for transmission during the selection window. With the uncertainty of LBT success, the selected resources may not be available when the UE wants to transmit. To reduce the impact of multiple sidelink UEs competing to access the channel at the same time, SL UEs which acquired the channel can indicate their intended COT duration for other SL UEs. The sensing can be done with granularity of time window and a SL UE that is performing sensing and selection can exclude time windows corresponding to these reserved COTs from the set of resources to be selected from.

Proposal 16: In Mode 2 RA, the UE excludes time window(s) corresponding to COT(s) initiated by other SL UEs.

## Wideband operation

In the last RAN1 meeting, channel access procedures were discussed for wideband operation. It was agreed to support the NR-U channel access procedure for multiple channels:

Agreement Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation as defined by TS37.213 for NR-U (wherever applicable) - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation

Wideband operation offers larger bandwidth which will help meeting the service requirements in unlicensed bands. This can help both UEs operating under mode 1 and mode 2 RA. A UE can use the full bandwidth for itself or otherwise, to maximize the usage of the bandwidth, COT sharing can be done in FDM manner. For example, a UE can share frequency resources of its COT with multiple UEs. In mode 1, the gNB can coordinate the COT sharing among multiple UEs. In mode 2, the UE initiate a COT can indicate the available resources to share using an indication in SCI.

Proposal 17: Wideband operation is supported for both mode 1 and mode 2 RA.

## Evaluation results

### Wi-Fi Performance

Figure 1: Average UPT of Wi-Fi for Wi-Fi+SL-U coexistence scenario with same traffic load

To evaluate the impact of SL-U on Wi-Fi, we evaluated the UPT of Wi-Fi. Figure 1 shows the average user throughput of the Wi-Fi operator in Wi-Fi+Wi-Fi scenario and Wi-Fi+SL-U coexistence. To ensure fairness between the two operators, we use the same traffic modeling for both Wi-Fi and SL-U operator (i.e., we set the number of retransmissions of SL-U operator to zero as the Wi-Fi STA). The results show the impact of SL-U operator on Wi-Fi throughput is similar to the impact of another Wi-Fi operator coexisting with the existing Wi-Fi operator.

Observation 1: In indoor scenario with symmetric traffic medium loads in 20MHz bandwidth at 5GHz, the evaluation results of Wi-Fi UPT show that fair coexistence between Wi-Fi and SL-U can be achieved.

Figure 2: Average UPT of Wi-Fi for Wi-Fi+SL-U coexistence scenario with higher SL-U traffic load

Figure 2 shows the Wi-Fi average throughput with SL-U coexistence for different values of EDs. In this scenario, we show the impact of increasing the traffic load of SL-U through increasing the number of blind retransmissions (i.e., enhance SL-U communication reliability) on the Wi-Fi system performance. We set the number of retransmissions for SL-U operator to 2. The figure shows that the fairness between Wi-Fi and SL-U is guaranteed since the performance degradation of Wi-Fi operator in Wi-Fi+SL-U doesn’t exceed 10% even though the SL-U traffic is denser which cause high interference on the Wi-Fi operator. Moreover, we observe that when ED of SL-U operator decreases, there is a slightly performance degradation in Wi-Fi operator. The reason behind that is probability of simultaneous SL-U transmissions decreases; hence, the SL-U interference is not randomized (i.e., each slot will be occupied with SL-U transmission due to the long back-off from previous slots).

Figure 3: Average PPR of Wi-Fi for Wi-Fi+SL-U coexistence scenario with higher SL-U traffic load

Figure 3 shows the PRR of the Wi-Fi operator per AP (left figure) and total average PRR (right figure) with high traffic load of SL-U (i.e., number of blind retransmissions = 2). The figure shows that the fairness between Wi-Fi and SL-U can be achieved since the PRR of Wi-Fi in the Wi-Fi+SLU scenario degrades by less than 10%.

Observation 2: The evaluation results of Wi-Fi UPT and PRR show that fair coexistence between Wi-Fi and SL-U can be achieved even with high SL-U traffic.

### SL-U performance

Figure 4: Average PIR of SL-U for Wi-Fi+SL-U coexistence scenario with higher SL-U traffic load

To evaluate SL-U performance in the presence of Wi-Fi, we present the PIR of SL-U in both Wi-Fi+SL-U and SL-U only scenarios in Figure 4. We can see that the PIR of SL-U UEs degrades by less than 10% for Wi-Fi+SL-U coexistence compared to SL-U only. The reason behind that is Wi-Fi STAs compete SL-U UEs during LBT process to access the channel. However, the result indicates that fair coexistence can be achieved for SL UEs in Wi-Fi+SL-U coexistence.

Observation 3: PIR of SL-U UEs degrades due to the competitiveness of Wi-Fi STAs and SL-U UEs  to access the channel during LBT process.

# Conclusion

In this contribution, we have discussed and propose the following:

Observation 1: In indoor scenario with symmetric traffic medium loads in 20MHz bandwidth at 5GHz, the evaluation results of Wi-Fi UPT show that fair coexistence between Wi-Fi and SL-U can be achieved.

Observation 2: The evaluation results of Wi-Fi UPT and PRR show that fair coexistence between Wi-Fi and SL-U can be achieved even with high SL-U traffic.

Observation 3: PIR of SL-U UEs degrades due to the competitiveness of Wi-Fi STAs and SL-U UEs  to access the channel during LBT process.

Proposal 1: UL CAPC table is re-used for Type 1 SL U channel access mechanism

Proposal 2: It is up to RAN2 to determine the association between sidelink priority levels and CAPC values.

Proposal 3: Type 2A and Type 2C can be used for the case of short control signaling transmission.

Proposal 4: At least SL-SSB and PSFCH can be transmitted as SCSt when the regulatory requirements are satisfied.

Proposal 5: The COT initiator UE can indicate which Type 2A/2B/2C SL-U channel access to use by other UEs for transmissions within a shared COT.

Proposal 6: At least re-use CPE to retain the channel for duration less than one symbol.

Proposal 7: Multiple starting positions can be enabled by applying different CPE.

Proposal 8: A responding UE can use the COT to at least transmit PSFCH/PSCCH/PSSCH to the COT initiator.

Proposal 9: Allow transmissions for one or more TBs in a COT by one UE.

Proposal 10: Support initial transmission and re-transmissions of a TB within a COT.

Proposal 11: Study re-transmissions of a TB in a different COT than the one including the initial transmissions.

Proposal 12: Support PSFCH transmission in a different COT than the corresponding PSSCH transmission.

Proposal 13: Support configuring Mode 1 UE with time window and set of frequency resources to initiate a COT in SL U.

Proposal 14: Study reporting of the channel access outcome and COT related information to the gNB in mode 1 SL U.

Proposal 15 Study reservation of a periodic time window for periodic type of traffic in SL unlicensed spectrum.

Proposal 16: In Mode 2 RA, the UE excludes time window(s) corresponding to COT(s) initiated by other SL UEs.

Proposal 17: Wideband operation is supported for both mode 1 and mode 2 RA.

# References

- “Draft Report of 3GPP TSG RAN WG1 #110 v0.2.0”

# Appendix: Simulation assumption

SL-U simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier Frequency (GHz) | 5 GHz |
| Bandwidth | 20 MHz/Uplink |
| Subcarrier Spacing | 15 kHz |
| UE Tx Power | 23 dBm |
| Channel Model | InH-office pathloss mode defined in TR 38.901 |
| No. Of Tx/Rx pairs | 10 |
| Antenna Configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = 0.5λ |
| Antenna Height | 1.5m |
| UE Dropping | Uniformly distributed |
| SL UEs path loss model | InH-office pathloss mode defined in TR 38.901 |
| UE Noise Figure | 10 dB |
| ED threshold | {-62, -72, -90, -110} dBm |
| Max retransmission | {0 (Wi-Fi Fairness), 2} |

Wi-Fi simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier Frequency (GHz) | 5 GHz |
| Bandwidth | 20 MHz/Uplink |
| Subcarrier Spacing | 15 kHz |
| AP/STA Tx Power | 23 dBm |
| Channel Model | InH-office pathloss mode defined in TR 38.901 |
| No. of AP/ STA per AP | 3/ 5 STA per AP |
| Antenna Configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = 0.5λ |
| UE Antenna Height | 1.5m |
| AP Antenna Height | 5m |
| UE Dropping | Uniformly distributed |
| SL STAs path loss model | InH-office pathloss mode defined in TR 38.901 |
| UE Noise Figure | 10 dB |
| Traffic model | VoIP model Packet size: 200 – 2000 Packet inter-arrival: 30 ms CW= 3/7 DIFS= 32 us |
| Max retransmission | 0 |
| ED threshold | -62 dBm |
| CTS/RTS | ON |