3GPP TSG RAN WG1#110-bis-e			R1-2209716

e-Meeting, October 10th – October 19th, 2022

Agenda item:	9.1.2

Source: 	Samsung

Title: 	Views on CSI enhancements

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s view on the key issues related to Rel.18 CSI enhancements that are prioritized for RAN1#110-bis-e (as announced by the FL). It also discusses other issues that are relevant for further discussions.

# Type II codebook refinement for coherent-JT

## Key issues

    - Issue 1 (TRP selection)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, down-select from the following TRP selection/determination schemes (where N is the number of cooperating TRPs assumed in PMI reporting) by RAN1#110bis-e: - Alt1. N is gNB-configured via higher-layer (RRC) signalling - The N configured TRPs are gNB-configured via higher-layer (RRC) signalling - Note: only one transmission hypothesis is reported - Alt2. N is UE-selected and reported as a part of CSI report where N∈{1,..., NTRP} - N is the number of cooperating TRPs, while NTRP is the maximum number of cooperating TRPs configured by gNB - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme) - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit - Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses. FFS: Whether S-TRP transmission hypothesis is also reported

For the NW deployments with NTRP=3,4, selection of N<NTRP TRPs for CSI reporting can be beneficial, and C-JT operation can be performed across selected N TRPs. The selection of N TRPs is UE-specific (since different UEs are likely to be located at different distances from NTRP TRPs). There are two alternatives for TRP selection. In our view, NW-controlled (Alt1) selection is preferred from NW perspective since the NW can cooperate/determine N TRPs for CSI reporting (e.g. based on UL RSRP measurements), and control the inter-cell interference. The dynamic selection (Alt2) can result into increased flash-light effect due to unpredictable (inter-TRP/cell) interference fluctuations, which is undesired. Alt2 will result in large mismatch between reported SINR (via CQI) and received SINR (via PDSCH), which will lead UPT loss. This is corroborated by the simulation results provided in Section 2.3.

Observation 1: Regarding TRP selection

- Semi-static selection is preferred from NW perspective in order to control the inter-cell interference

- Dynamic selection can lead to dynamic inter-TRP/cell interference fluctuations, resulting in large CQI mismatch

Proposal 1: support gNB-based TRP selection

- support Alt1 (via RRC) as baseline

- study more dynamic gNB-based TRP selection (e.g. via CSI request in DCI)

    - Issue 2 (W2 quantization group + SCI) and Issue 3 (Strongest TRP indicator)

[1] Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, for each layer, down-select one from the following alternatives by RAN1#110bis-e: - Alt1. One group comprises one polarization across all TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2), one (common) SCI across all TRPs/TRP groups - Alt2. One group comprises one polarization for one TRP/TRP-group (Cgroup,phase=N, Cgroup,amp=2N), per-TRP/TRP-group SCI - FFS: Quantization of N strongest coefficients - Alt3. One group comprises one polarization for one TRP/TRP-group with a common phase reference across TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2N) - FFS: SCI, per-TRP/TRP-group vs. one (common) SCI across all TRPs/TRP groups - FFS: Quantization of N strongest coefficients - Alt4. For a selected TRP/TRP-group, one group comprises one polarization, and for remaining N-1 TRPs/TRP-groups, one group comprises one polarization across remaining N-1 TRPs/TRP-groups (Cgroup,amp=2+2=4), with a common phase reference across all of N TRPs/TRP-groups (Cgroup,phase=1) - FFS: The selected TRP/TRP-group FFS: The need for “strongest” TRP/TRP-group indicator in addition to SCI(s)

Regarding the W2 quantization group, there are 4 alternatives to down-select from. A summary of these alternatives is provided in Table 1. Alt2 will incur performance loss especially when applied to mode-2. Alt2 should be precluded. Among the remaining alternatives, in our view, Alt1 is tailored for the case (A) when N TRPs have ‘similar’ power, and Alt3 is beneficial for the case (B) when TRPs have different power levels. Alt4 on the other hand can strike a balance for both cases (A and B), and also for the general case (C) when a subset of TRPs are ‘strong’ and remaining TRPs are ‘weak’. The main idea of Alt4 is that in general (with high probability), one of the N TRPs is likely to be the strongest (i.e. having the max power), and remaining N-1 TRPs will be relatively ‘weak’ (lower power than the strongest TRP), hence they can be grouped together for amplitude quantization. As mentioned, Alt4 is likely to achieve a better performance-overhead trade-off than Alt1 and Alt3 since it includes the key aspects of both Alt1 (optimal when all TRPs have comparable power) and Alt3 (optimal when TRPs have different powers). We verify this via simulation results provided in Section 2.3.

Observation 2:

- Alt2 will incur performance loss especially when applied to mode-2

- Alt1 is tailored for the case (A) when TRPs have similar power

- Alt3 is beneficial for the case (B) when TRPs have different power levels

- Alt4 works for (A) and (B), and for the general case (C) when a subset of TRPs are ‘strong’ and remaining TRPs are ‘weak’

Table 1

|  | #groups for phase | #groups for amplitude | #SCI |
| --- | --- | --- | --- |
| Alt1 | 1 | 2 (per pol, across TRPs) | 1 (across TRPs) |
| Alt2 | N | 2N (per pol, per TRP) | N (per TRP) |
| Alt3 | 1 | 2N (per pol, per TRP) | 1 vs N |
| Alt4 | 1 | 4 (per pol for 1 selected TRP, per pol across remaining N-1 TRPs) | 1 vs >1 |

The SCI is used to indicate the strongest coefficient that normalizes all non-zero coefficients necessary before quantization. For mTRP, this normalization can be common across all N TRPs (i.e. 1 SCI), or per TRP (i.e. N SCIs). For CJT operation, 1 SCI is sufficient. N SCIs is an over-design, and incurs more overhead since it will require reporting of inter-TRP co-phase/co-amplitude information in order to obtain a coherent/joint precoder across TRPs.

Observation 3: N SCIs (per TRP SCI) is not needed for CJT precoding, incur more overhead, and are unlikely to have any performance benefits.

Another issue related with the SCI is the remapping/shifting operation in frequency domain. In Rel.16, the remapping is performed w.r.t. the FD index (fl*) of the strongest coefficient such that fl*=0 after remapping, and the SCI indicates the SD index il*. In Rel.17, on the other hand, there is no remapping, FD basis vectors are rather shifted/rotated such the smallest FD basis index = 0 after shifting; and the SCI indicates both SD and FD indices (il*,fl*). For CJT codebook, we prefer to have a unified (one) solution to minimize the specification work and RAN1 discussion time.

The need for a TRP indicator reporting is another FFS point from the above agreement. In our view, such reporting can be beneficial. For instance, it can be used to facilitate efficient (lower overhead) reporting of per-TRP FD basis selection (e.g. in mode 1). It can also be used as a reference for normalizing per-TRP co-scaling.

Observation 4: A TRP indicator reporting can be beneficial for various use cases (e.g. per-TRP FD basis selection, per-TRP co-scaling reporting).

Proposal 2: Support the following

- Alt4 for the W2 quantization group

- One SCI across all TRPs, including a unified (one) solution regarding remapping/shifting in FD for both modes and for both Rel.16 and Rel.17-based CJT codebooks

- A TRP indicator reporting for components that are reported relative to a reference (e.g. per-TRP FD basis selection, per-TRP co-scaling)

    - Issue 4 (SD and FD basis selection)

[1] Agreement For the Rel-18 Type-II codebook for CJT mTRP based on the Rel-16 Type-II codebook, SD basis and FD basis are separate, each fully reusing the legacy Rel-16 DFT-based design [2] Agreement On the Type-II codebook refinement for CJT mTRP, the resulting codebook(s) are associated with at least the following parameters: - Parameters for basis reporting, including - The number of basis vectors: gNB-configured via higher-layer signaling - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs - Basis selection indicator(s): a part of CSI report - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs - … [OFFLINE EMAIL DISCUSSION] Offline proposal 1.1: On the Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), for a given CSI-RS resource: - SD basis selection is layer-common and polarization-common, with L, N1, N2, O1, O2 defined per Rel-16 specification for refinement based on Rel-16 regular eType-II, and per Rel-17 specification for refinement based on Rel-17 PS FeType-II - FD basis selection is - For refinement based on Rel-16 regular eType-II: per-layer with Mv, pv, N3, and R defined per Rel-16 specification - For refinement based on Rel-17 PS FeType-II: layer-common with M, N3, and R defined per Rel-17 specification - FFS: Details on FD basis selection window Note: The supported value(s) for each of the defined parameters are to be discussed separately (e.g. possibilities of adding new or removing existing value(s) in addition to those supported by legacy specification). Offline proposal 1.2: On the SD basis selection for Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), SD basis selection is per CSI-RS-resource. - Down select from the following alternatives (RAN1#110bis-e): - Alt1. Per-CSI-RS-resource Ln parameter - TBD: Whether {Ln, n=1, ..., N} are higher-layer configured by gNB, or the total n=1NLn is higher-layer configured by gNB while {Ln, n=1, ..., N} are reported by the UE - Alt2. Common L parameter for all N CSI-RS resources

Following legacy design, separate SD and FD bases for CJT codebook were agreed in RAN1#110. Two offline proposals regarding the details of SD and FD basis selections and corresponding parameters were discussed via email. The first (offline proposal 1.1) proposes to reuse/extend the legacy SD and FD basis selection to N TRPs (or CSI-RS resources). We are supportive of this proposal with a small clarification about the SD basis selection parameter L. In Rel.16, L is configured via parameter combination, and in Rel.17, L is given by L=αPCSIRS2, and α is configured via parameter combination. Since Rel.17 codebook is designed for a specific use case (i.e. FDD reciprocity of angle and delay) and the parameter α was introduced to allow selection of all CSI-RS ports as a possible configuration, we prefer to reuse the same design for Rel.17-based CJT port selection codebook, hence use parameter α.

The second (offline proposal 1.2) proposes two alternatives for parameter L for N TRPs (or CSI-RS resources): Alt1 for per CSI-RS resource parameter Ln and Alt2 for one/common L parameter for all CSI-RS resource. We support Alt1 since it allows different number of SD basis vectors for different TRPs, which is needed in realistic scenarios (e.g. non-co-located inter-site scenario). It can also be beneficial to control the overhead of W2 (e.g. bitmap size reduction) reporting. Besides, Alt1 achieves better performance-overhead tradeoff than Alt2, as shown in section 2.3.

Regarding the set of values {Ln}, we prefer to follow legacy and allow NW to configure them. There are two ways NW can provide these values to the UE. In example 1, NW can configure one value Ln for each n. In example 2, NW can configure one value L and Ln for each n is derived based on the configured L. For instance, NW can configure L=4, and the UE can use L1=L=4 for TRP 1, and Ln=L2=2 for TRP n>1.

Proposal 3:

- Support offline proposal 1.1 with the following modification regarding SD basis selection parameter

  - Reusing parameter α (defined per Rel.17 specification) for Rel.17-based CJT port selection codebook

- Support Alt1 in offline proposal 1.2 with the following examples included in TBD

  - Ex1: Each Ln is configured

  - Ex2: One L is configured, and {Ln} is determined based on the configured L value

    - Issue 5 (NNZC and bitmap design)

[2] Agreement On the Type-II codebook refinement for CJT mTRP, the resulting codebook(s) are associated with at least the following parameters: - … - Number of non-zero coefficients and bitmap to indicate non-zero coefficients, including whether it is per TRP/TRP-group (separate) or across all TRPs/TRP-groups (joint): a part of CSI report - … [OFFLINE EMAIL DISCUSSION] Offline proposal 1.3: On the Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), regarding the location of non-zero coefficients (NZCs) indicated by bitmap (following legacy mechanism), for each layer, support separate bitmaps for all N CSI-RS resources - Total size = n=1NBn where is the bitmap size for CSI-RS resource n - TBD: Whether Bn=2LnMv,n (Mv,n=Mv for mode 2) analogous to legacy, or further reduction of bitmap size is supported. - FFS: Depending on the outcome of other issues, whether Mv,n=Mv or Ln=L - FFS: Per-CSI-RS-resource NNZC (number of NZCs) constraint vs. joint NNZC constraint across N CSI-RS-resources

We support offline proposal 1.3 on the bitmap to indicate NZ coefficients. Separate bitmap for each CSI-RS resource (TRP) is needed since the each TRP is likely to experience different channel, implying different SD/FD basis vectors and associated coefficients. Regarding the TBD on bitmap size, we prefer reusing legacy, i.e., Bn=2LnMυ,n.

Regarding the FFS on the constraint on NNZC, we prefer reuse/extend legacy to N CSI-RS resources, i.e., per-CSI-RS resource NNZC constraint for rank 1-2, and joint NNZC constraint for rank 3-4. The parameter β can also be the same as in legacy, i.e., one β value is configured via parameter combination. The supported β values can CJT can be discussed together with the parameter combinations.

Proposal 4: Support offline proposal 1.3 and reuse/extend legacy solution to N CSI-RS resources

- bitmap size Bn=2LnMυ,n

- NNZC constraint

  - when rank 1-2, per-CSI-RS resource NNZC constraint

  - when rank 3-4, joint NNZC constraint across N CSI-RS resources

- one β value configured via parameter combination

## Other issues

We provide our views regarding the following additional issues.

Parameter combinations

As seen in [3], it has been identified that using the Rel-16 parameter combination table for CJT codebooks imposes large feedback overhead compared to sTRP case. One potential area where some further optimization can be beneficial is the supported parameter combinations to ensure not only competitive UPT-overhead trade-off, but also to prevent excessive increase in PMI payload. Based on simulation results (cf. section 2.3), we observed that low-overhead regime can be achievable with sufficiently better throughput than the sTRP case, when we allow L=1 and low values of pv . We therefore suggest to study this area and a new parameter combination table including L=1 for CJT in low-overhead regime. Additionally, to ensure continuity with Rel-16/17 Type-II codebooks and manageable scope in Rel-18, maximum reuse of the Rel-16/17 detailed designs (e.g. SD/FD basis designs, UCI parameters) should also be the goal.

FD basis reporting window

A window-based FD basis selection reporting is supported in legacy (Rel.16/17) codebooks. A summary is provided in Table 2. In Rel.16, when N3>19, the FD basis selection/reporting is per layer from a FD basis window: mod(Minit+n,N3), n=0,1,..,N-1, where Minit is reported by the UE. In Rel.17, when N=2,4 and M=2, the FD basis selection/reporting is layer-common from a FD basis window: mod(Minit+n,N3), n=0,1,..,N-1, where Minit=0 is fixed. Such a window-based FD basis reporting is adopted due to the fact that the ‘relative’ FD indices of the ‘dominant’ FD basis vectors (corresponding to dominant delay profile of the channel) are likely to reside within a window. As mentioned above, there are two different remapping/shifting operations for Rel.16 and Rel.17 codebooks, respectively, and the SCI reporting is also different.

Table 2

|  | Rel.16 | Rel.17 |
| --- | --- | --- |
| Minit | Reported | Fixed, Minit=0 |
| N | 2Mυ | N∈{2,4} |
| Remapping/shifting | w.r.t. FD index fl* of the strongest coef. s.t. fl*=0 after remapping | w.r.t. lowest FD index n3(0) s.t. n3(0)=0 after remapping |
| SCI | SCI=il* | SCI=2Lfl*+il* |

For CJT, the window for the FD basis selection/reporting should be studied. The legacy FD basis window can be baseline. However, unlike legacy, it is preferable for spec simplicity to unify and have one common FD basis design for both Rel.16-based and Rel.17-based CJT codebooks.

Proposal 5:

- Support new parameter combination(s) including L=1, targeting low-overhead regime

- Study the need for FD basis window for FD basis reporting

  - Legacy (Rel.16 or Rel.17) FD basis window design is baseline

  - If supported, the same FD basis window design for both Rel.16 and Rel.17-based CJT codebooks is preferable

## Simulation results

We provide system-level simulation (SLS) results on (1) performance comparison w.r.t. TRP selection methods (gNB-based vs UE-based), (2) performance comparison for different reference grouping methods, i.e., Alt1, Alt2, Alt3, and Alt4 of Issue 2 (Table 1), (3) performance comparison for a) TRP-independent SD beam selection with different L values, b) TRP-independent SD beam selection with common L value, and c) TRP-common L SD beam selection, (4) performance comparison for the cases of common-bitmap and separate-bitmap across TRPs, and (5) performance in a low-overhead regime with new parameter combination(s). The simulation assumptions are summarized in Appendix A. The absolute UPT values for the figures in this subsection are provided in Appendix B.

Evaluation 1: performance comparison w.r.t. TRP selection methods (gNB-based vs UE-based)

Figure 1: Average UPT gain vs overhead w.r.t. TRP selection methods in the Outdoor1 scenario

Figure 2: Average UPT gain vs overhead w.r.t. TRP selection methods in the Outdoor2-OptA scenario

To evaluate UE-based vs gNB-based TRP selection methods, we have considered the following three methods, 1) semi-static TRP selection, 2) UE-based dynamic TRP selection, and 3) gNB-based dynamic TRP selection. In the semi-static TRP selection method, once a subset of TRPs are selected for each UE, the set of TRPs is not changed during the whole simulation period (semi-static) as described in EVM, whereas in the UE-based/gNB-based dynamic TRP selection methods, the set of TRPs can be changed based on each method’s own criteria. For the UE-based dynamic TRP selection method, it is considered that each UE selects a subset of TRPs and reports CSI corresponding to the selected subset of TRPs for every CSI reporting. For the gNB-based dynamic TRP selection method, it is considered that gNB collects RSRP for each UE over time to average out and selects a subset of TRPs for the UE using the averaged RSRP, where the selection is performed in a longer periodicity (e.g., 40ms, 100ms, or when needed based on a criterion). Note that the results in Figure 1 and Figure 2 use the UPT for the semi-static TRP selection case with paraComb=1 as the reference UPT value (which we regard as 100%).

As shown in Figure 1 and Figure 2, it is observed that UE-based dynamic TRP selection degrades the performance of UPT vs overhead (4% UPT loss) especially in the intra-cell scenario due to the unpredictable interference fluctuation issue. Additionally, it is observed that the gNB-based dynamic TRP selection method outperforms (2~4% UPT gain) the other two methods in both of the intra-/inter-cell scenarios. This is because the gNB can be aware of desired TRP selection occasion for a UE based on available information only at the gNB side (e.g., average RSRP over time, inter-TRP interference info).

Observation 5: 1) UE-based dynamic TRP selection degrades the performance of UPT vs overhead (4% avg. UPT loss) especially in the intra-cell scenario due to unpredictable interference fluctuation, and 2) the gNB-based dynamic TRP selection method outperforms (2~4% avg. UPT gain) the other two methods in both of the intra-/inter-cell scenarios.

Evaluation 2: performance comparison for different reference grouping methods, Alt1, Alt2, Alt3, and Alt4 of Issue 2

Figure 3: Average UPT gain vs overhead w.r.t. reference grouping methods, Alts 1, 2, 3, and 4, in the Outdoor2-OptA scenario

As shown in Figure 3, it is observed that the UPT performance vs overhead trade-off is in the following order: Alt4 > Alt1 > Alt3 ≈ Alt2. Here, we use the UPT for the Alt1 with paraComb=1 as the reference UPT value. Among the four methods, it is shown that Alt4 achieves the best UPT vs overhead trade-off and yields 2~4% UPT gain over the other methods. One additional observation is that the Alt2/Alt3 (#. Ref Groups for amp = 2N) imposes not only additional overhead but also yields worse UPT performance than the other two methods (Alt1 and Alt4). It implies that the finer resolution using 2N reference amplitude groups trailered for different ranges of channel magnitudes across N TRPs can be over-optimized. As an example, for the case of TRPs having different ranges of channel magnitudes (which is the case that Alt2/3 can only allow a finer coefficient reporting among the 4 Alts), a weakest TRP contributes to marginal CJT operation but rather increases the probability of interference leakage to neighbourhood cells, which turns out performance degradation for Alt2/3.

Observation 6: Alt4 (#. Ref Groups for amp = 4) yields the best UPT vs overhead trade-off and 2~4% avg. UPT gain over the other methods, Alt1, 2, and 3.

Evaluation 3: performance comparison for Alt1) TRP-independent SD beam selection with different L values, Alt2) TRP-independent SD beam selection with common L value, and Alt3) TRP-common L SD beam selection

Figure 4: Average UPT gain vs overhead for 1) per-TRP SD beam selection, per-TRP L parameter, 2) per-TRP SD beam selection, common L parameter for all TRPs, and 3) TRP-common L SD beam selection in the Outdoor2-OptA scenario

To simulate Alt1 (the case of different L values), we considered that one L value is configured (which follows the Rel-16 parameter combination table) but one L value is applied for a reference TRP and L/2 is applied for the remaining N-1 TRPs. As shown in Figure 4, it is observed that the case of different L values (Alt1) yields the best UPT vs overhead trade-off, which has 5%~10% UPT gain over the other methods. The main benefit of Alt1 comes from overhead saving by reducing the size of W2 for weaker TRPs whereas the UPT loss is minimized by maintaining the number of SD beams for a stronger TRP to capture most of the stronger coefficients. As seen in the result, TRP-common SD beam selection yields the worst UPT vs overhead trade-off because of the constraint of the common SD beam selection across TRPs that degrades UPT performance significantly. Also, common SD beam selection does not much save the overhead since the W2 size imposing most of the overhead is still the same for all TRPs. Here, we use the UPT for the Alt2 with paraComb=1 as the reference UPT value.

Observation 7: multiple (or different) L values (Alt1) can be beneficial as showing ~5% avg. UPT gain over the same L value case (Alt2). TRP-common SD beam selection (Alt3) yields the worst UPT vs overhead trade-off performance.

Evaluation 4: performance comparison for the cases of common-bitmap and separate-bitmap across TRPs.

Figure 5: Average UPT gain vs overhead for the cases of common-bitmap and separate-bitmap in the Outdoor2-OptA scenario

As shown in Figure 5, it is observed that TRP-common bitmap incurs large UPT loss that cannot be compensated for the overhead saving, which turns out 2~3% UPT loss. Here, we use the UPT for the case of common-bitmap with paraComb=1 as the reference UPT value.

Observation 8: TRP-common bitmap incurs large avg. UPT loss that cannot be compensated for the overhead saving (it turns out 2~3% avg. UPT loss).

Evaluation 5: performance in a low-overhead regime with new parameter combination

Figure 6: Average UPT gain vs overhead with new parameter combination for NTRP=3 and 16 ports per TRP, in the Outdoor2-OptA scenario

To evaluate the CJT performance in a low-overhead regime, we have used the following parameters for new parameter combinations.

| New ParaComb | L | pv | β |
| --- | --- | --- | --- |
| 1 | 1 | 1 (pv=1/16) | 0.25 |
| 2 | 1 | 2 (pv=1/8) | 0.25 |
| 3 | 1 | 2 (pv=1/8) | 0.50 |
| 4 | 2 | 2 (pv=1/8) | 0.50 |

As shown in Figure 6, it is identified that low-overhead regime can be achievable with sufficiently better throughput (70% - 100% gain) than that of the sTRP case, when L=1 and/or low values of pv are allowed. Here, we use the UPT for the sTRP case with paraComb=1 as the reference UPT value.

Observation 9: a sufficient performance gain (70% - 100%) can be obtained in a low-overhead regime that is comparable to the overhead of sTRP case, when L=1 and/or low values of pv (e.g., 1/8) are allowed.

Type II codebook refinement for high/medium velocities

## Key issues

    - Issue 6 (Rel-16-based vs Rel-17-based Doppler)

[2] Agreement The work scope of Type-II codebook refinement for high/medium velocities includes refinement of the following codebooks, based on a common design framework: - Rel-16 eType-II regular codebook - Rel-17 FeType-II port selection (PS) codebook FFS: Whether to prioritize/down-select from the two

Given limited time/resource constraints and large number of potential Rel.18 enhancements, refining both Rel-16-based and Rel-17-based Type-II codebooks for high/medium velocities is quite challenging, considering substantial simulation work required for codebook design. Between the two, Rel-16-based design therefore should be prioritized.

Proposal 6: prioritize Rel-16-based design for Type II codebook refinement for high/medium UE velocities.

    - Issue 7 (supported RI values)

Regarding supported rank values, at least rank 1-2 should be supported. It is unclear whether rank 3-4 can be beneficial for high/medium UE velocity UEs, and perhaps should be studied. However, considering legacy (Rel.16/17) codebooks support up to rank 4, we can support up to rank 4 for Doppler codebook also.

Proposal 7: support rank 1-4 for Type II codebook refinement for high/medium UE velocities

    - Issue 8 (Codebook structure and DD basis)

[OFFLINE EMAIL DISCUSSION] Offline proposal 2.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, down-select at least one from the following codebooks structures (by RAN1#110bis-e): - Alt1: Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - Alt2: Doppler-domain orthogonal DFT basis independently selected for different SD/FD bases reusing the legacy W1 and Wf - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - Alt3. Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf⨂I)H In addition: - Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification - FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report. - FFS: Whether Doppler-/time-domain (DD/TD) basis vector length (N4) is RRC-configured or reported by the UE - FFS: Whether the number of selected DD/TD basis vectors (for Alt1 and Alt2) is RRC-configured or reported by the UE Offline proposal 2.2: For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling: - For N4=[1], Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf⨂I)H - For N4>[1], Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components

The following aspects can be compared among the three alternatives in offline proposal 2.1.

- Compression: Alt1 and Alt2 include DD/TD compression; whereas Alt3 is uncompressed.

- Overhead: Alt1 and Alt2 can reduce overhead due to compression using DD/TD basis; whereas Alt3 will incur large overhead due to multiple (uncompressed)  W2 reporting, especially when N4 is large.

- Inter-dependency and spec-impact: if (W1,Wf) and multiple W2 are reported in different slots, then it will introduce inter-dependency across reports, the performance will be impacted by error propagation, and it will also require specifying dropping rules. Such a design should be avoided.

We support Alt1 since it is simple, has similarity with legacy codebooks, and is a general formulation which includes Alt2 and Alt3.

- Alt2 can be included if the common selected TD/DD basis in Alt1 is a ‘larger’ pool including TD/DD bases for different SD/FD bases. This can be accomplished via Doppler domain compression parameter (say D) being set to a value which can allow different DD basis selection for different SD/FD bases (e.g. via bitmap).

- Alt3 can be included by allowing identity matrix (no Doppler compression) as a possible configuration.

Observation 10: regarding offline proposal 2.1

- Alt1 and Alt2 includes TD/DD basis for compression; whereas Alt3 does not (i.e. uncompressed)

- Alt3 can incur large overhead due to multiple W2s

- If (W1,Wf) and multiple W2s are reported in different slots, this will lead to inter-dependency, error propagation, and will require specifying dropping rules.

- Alt1 can accommodate Alt2 and Alt3

An open issue in Alt1 is the rotation factor. In our view, it’s needed in order to acquire more accurate or ‘higher-resolution’ Doppler components, especially when the length of Doppler basis vectors (N4) is relatively small. Whether this rotation factor is common across SD basis vectors or per-SD basis vector requires further study.

There are two FFS point in offline proposal 2.1 regarding the DD basis vector:

- Doppler-/time-domain (DD/TD) basis vector length (N4): following legacy, N4 can be configured by the NW. UE reporting N4 will amount to more UE complexity, which is already larger than legacy codebooks. Besides, it is unclear what benefits we can avail with this.

- Number of selected DD/TD basis vectors: this number depends on number of ‘strong’ DD components across all SD basis vectors. In practice, this number can be small, e.g. {1,2} when the ‘strong’ DD components are the same for all SD basis vectors. So, small values {1,2} should be supported. For the case when ‘strong’ DD components are different across SD basis vectors, we may either consider larger values or consider different rotation factor for each SD basis vector assuming that {1,2} DD basis vectors can just be rotated across SD basis vectors.

Proposal 8: Regarding codebook structure

- Support Alt1 in offline proposal 2.1 and proposal 2.2

- Support at least one rotation factor, and study whether additional (e.g. per-SD basis vector) rotation factor is needed

- Length of the DD/TD basis vector (N4) is RRC-configured

- Number of DD/TD basis vectors: support {1,2} and study the need for value(s) > 2

    - Issue 9 (UE-side prediction)

[OFFLINE EMAIL DISCUSSION] Offline proposal 2.3: On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, support UE “predicting” channel/CSI after the slot with a reference resource (l ≥ nref) where the location of CSI reference resource is configured (from multiple candidate values) by gNB via higher-layer signalling - Candidates of CSI reference resource location include the legacy slot location and slot n - FFS: Possible value(s) of WCSI

There are two alternatives on the UE-side prediction and WCSI.

- Alt1.B: UE “predicting” channel/CSI after the slot with a reference resource

- Alt2.B: UE “predicting” channel/CSI after slot n (where the CSI is reported)

We provide a technical analysis on these alternatives. A more details can be found in Appendix C.

- First, both Wmeas and WCSI depend on UE speed in general. For low speed, they can be longer, and for high speed, both need to be shorter.

- The prediction accuracy reduces (or prediction error increases) as we move to future slots.

- The starting slot (l) of the prediction should be as close to the measurement window as possible.

- The end slot (l+WCSI-1) should be based on (a) UE complexity (how far UE can predict), and (b) impact of prediction error on performance (at some time in future, prediction error starts to dominate resulting in UPT loss).

- This implies that there is an ‘optimal’ prediction window for a given UE speed. And there is a scenario (e.g. high UE speed), this ‘optimal’ window can be within [nref,n], which can’t be supported with Alt2.B.

Observation 11: there is an ‘optimal’ prediction window for a given UE speed, whose starting slot is desired to be as close to the measurement window as possible, and ending slot is such that the prediction error doesn’t affect the UPT performance.

We therefore prefer Alt1.B. Besides, as shown in Section 3.3, we observe that Alt1.B is better or no worse than Alt2.B for both low as well as high UE speeds.

Regarding the offline proposal 2.3, we can accept it since it includes Alt1.B. We however would like to clarify the meaning of the ‘new’ CSI reference resource. In legacy, the CSI reference resource lies in a slot nref=n-nCSI,ref and is used for (A) the latest channel and interference measurement occasion; and (B) CQI calculation meeting BLER requirement. If we enhance the definition of the CSI reference resource (as in the proposal), then there seems to some impact of both (A) and (B) as well, which require some enhancements. In particular, for (A), how the CSI-RS occasion is defined w.r.t. to the ‘new’ CSI reference resource need to be specified, and for (B), since the reported CSI needs to be valid for a window WCSI, the CQI BLER requirement need to be defined over the window.

Proposal 9: support offline proposal 2.3 only if Alt1.B (i.e. legacy slot location of the CSI reference resource) is included, and include the following enhancements

- CSI-RS occasion is defined w.r.t. ‘new CSI reference resource’

- CQI BLER requirement over the window WCSI

    - Issue 10 (spec-support for gNB-side prediction)

[2] Agreement The work scope of Type-II codebook refinement for high/medium velocities includes the following CSI measurement and calculation aspects: - Potential refinement on Resource setting configuration on CSI-RS (for CSI and/or tracking) for measuring a burst of CSI-RS, including the applicable time-domain behaviors - Whether/how UE-side or gNB-side prediction is assumed for CQI/PMI/RI calculation - Potential enhancements on CQI definition and calculation procedure in relation to the PMI of Rel-18 Type-II codebook for high/medium velocities - Potential enhancement on definition of CSI reference resource

Similar to legacy CSI reporting, whether gNB-side prediction is needed or not should be left up to gNB implementation, i.e., the Type II Doppler codebook should not assume/require gNB to perform any prediction. It should be left up to gNB’s discretion.

Proposal 10: do not support any specification requiring gNB-side prediction

    - Issue 11 (CSI-RS)

Agreement On potential refinement of Resource setting configuration associated with Type-II codebook refinement for high/medium velocities, study the following options to assess whether/how the legacy Resource setting configuration needs to be enhanced for “burst” measurement: - Periodic (P) CSI-RS: periodicity and offset - Semi-persistent (SP) CSI-RS: activation/deactivation, periodicity, and offset - Aperiodic (AP) CSI-RS: triggering, offset of a group of AP CSI-RS resources FFS: Support for K>1 NZP CSI-RS resources association with Type-II codebook refinement for high/medium velocities FFS: Whether specification support for jointly utilizing two types of CSI-RS time-domain behaviors is needed

Figure 7

For Doppler-domain (DD) compression, a UE can measure a burst of NZP CSI-RS resource(s), referred to as CSI-RS burst, in B>1 time slots (where B is configured such that the measurement “window” is well within the channel coherence time). There are two examples of a CSI-RS burst, as shown in Figure 7.

- Ex1: the B time slots are evenly/uniformly separated with an inter-slot spacing d.

- Ex2: the B time slots can be non-uniformly separated with inter-slot spacing e1=d1, e2=d2-d1, e3=d3-d2,…, so on, where ei≠ej for at least one pair (i,j) with i≠j.

The CSI-RS burst can be configured based on one SP CSI-RS resource, where the first and last slots of the CSI-RS burst are determined based on the MAC CE based activation and deactivation commands, respectively. This, however, can be too restrictive in some cases, e.g.:

- It can only be configured for uniformly separated CSI-RS bursts.

- According to the current specification, the minimum periodicity value of a SP CSI-RS resource is 4 slots, which may not be sufficient for high UE velocities.

One way to facilitate faster CSI-RS burst measurement for lower periodicity (or separation) is configuring multiple SP CSI-RS resources with the same periodicity but different offsets

The CSI-RS burst can also be configured based on a group of AP CSI-RS resources, which can be activated by a MAC CE based activation command (e.g. sub-selection step for AP CSI-RS resources in Rel-15) or a DCI based triggering. The slot offsets of the group of AP CSI-RS resources can determine the spacing (di) between AP CSI-RS resources. The AP CSI-RS resource based CSI-RS burst can be more flexible for CSI-RS burst configuration. It can address the two limitations with SP CSI-RS resource mentioned above. It can be used for both uniformly and non-uniformly separated CSI-RS bursts, and it can facilitate measurement in consecutive slots.

Finally, the time restriction for channel measurement (RRC parameter) should be set to ‘nonConfigured’ to facilitate CSI-RS burst measurement.

Observation 12: Regarding CSI-RS burst configuration

- SP CSI-RS resource can be restrictive due to (1) uniformly separated burst, and (2) the min periodicity = 4 slots.

- Multiple SP CSI-RS resources and a group of AP CSI-RS resources can be used to address these limitations.

Based on this observation, we propose to support both SP CSI-RS resource and AP CSI-RS resources for CSI-RS burst configuration for wider range of medium/high UE velocity.

Proposal 11: support the following regarding the CSI-RS “burst/occasion”

- both SP and AP CSI-RS resources

  - multiple SP CSI-RS resources in one CSI-RS resource set (e.g. same periodicity, and different offset)

  - a group of AP CSI-RS resources activated/triggered together

- time restriction = ‘notConfigured’

## Other issues

DD unit and DD unit size

[1] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support DD/TD (compression) unit (analogous to PMI sub-band for Rel-16 codebook) as a codebook parameter. - FFS: whether this parameter is defined as a function of another parameter - FFS: whether this is used for PMI only, or PMI/CQI

Figure 8

The notion of FD unit for FD compression in Rel-16/17 Type II codebook can be extended to DD. In particular, a total of B slots comprising the CSI reporting window: T0, T1,…,TB-1 can be partitioned into sub-time (ST) units ST0,ST1,…, each ST unit comprises NST measurement time instances. An example is shown in Figure 8 where NST=2. The number of DD units (say N4, which is also the length of the DD basis vectors) can be determined based on B and NST. Similar to Rel-16/17, the number of DD basis vectors (say, D) for DD compression is configured.

Similar to legacy, the DD/TD unit should be used for both PMI and CQI. Whether the DD/TD unit size is the same or different for PMI and CQI require further study.

Proposal 12: Regarding DD/TD unit, support the following

- a DD/TD unit comprises NST consecutive time slots, where NST is configured via RRC

- NST determines N4, i.e., the number of DD units or length of DD basis vectors

- Number of DD basis vectors (D) for DD compression is configured

- DD/TD unit for both PMI and CQI

  - FFS: whether DD/TD unit is the same or different for PMI and CQI

CQI calculation/reporting

[2] Agreement The work scope of Type-II codebook refinement for high/medium velocities includes the following CSI measurement and calculation aspects: - Potential refinement on Resource setting configuration on CSI-RS (for CSI and/or tracking) for measuring a burst of CSI-RS, including the applicable time-domain behaviors - Whether/how UE-side or gNB-side prediction is assumed for CQI/PMI/RI calculation - Potential enhancements on CQI definition and calculation procedure in relation to the PMI of Rel-18 Type-II codebook for high/medium velocities - Potential enhancement on definition of CSI reference resource

The CQI reporting based on Type II Doppler codebook requires enhancement.

- Number of CQIs in time domain: depending on the DD/TD unit size, the number of CQIs in time domain can be one or more than one.

- BLER requirement: since the CSI report needs to be valid for N4 units, if N4>1, CQI BLER requirement can be defined either for (i) a slot in a DD/TD unit, or (ii) over all slots in a DD/TD unit.

Proposal 13: support CQI enhancements for Type II Doppler codebook based CSI reporting,

- One or multiple CQIs in time domain, e.g. 1 CQI per DD/TD unit

- CQI BLER requirement, e.g. at one slot in a DD/TD unit or over all slots in a DD/TD unit

Overhead reduction

The coefficient matrix ( W2) comprises 2LMυD coefficients. The Rel-16/17 based W2 reporting mechanism (i.e., SCI, bitmap to indicate indices of NZ coefficients, and amplitude/phase of the NZ coefficients) should be reused. However, the total number of coefficients increases to D times when compared to Rel-16/17, which can be too large (for reporting via two-part UCI). Thus, mechanisms to limit the increase in W2 reporting payload may need to be considered, e.g. via parameter combination or bitmap.

Proposal 14: Regarding W2 reporting for Type II Doppler codebook

- Rel-16/17 based W2 reporting mechanism is reused,

- Study the need for mechanisms to limit the payload increase of W2 reporting, e.g. via parameter combination or bitmap

## SLS results

The SLS results based on the agreed EVM assumptions (Table 4 in Appendix A) are provided in this section.

Evaluation 1: codebook structure

The simulation setting for comparing Alt1 and Alt2 of the codebook structure is summarized as follows:

- CSI report setting

  - Periodicity P = 20 slots

  - WCSI=24

- CSI-RS measurement:

  - Wmeas=4 (period=1, per slot measurement)

  - Measurement overhead modelled

- UE speed = 10 kmph

The UPT vs overhead tradeoffs is shown Figure 9. We can make the following observation.

Observation 13: Alt1 and Alt2 achieve similar performance vs overhead trade-off

Figure 9

Evaluation 2: UE-side prediction

Figure 10

The simulation setting for the eTypeII (Rel-16-based Type II) Doppler is illustrated in Figure 10 and summarized as follows:

- CSI report periodicity P = 20 slots

- CSI-RS measurement:

  - Wmeas=4 (period=1, per slot measurement)

  - Measurement overhead modelled

- UE speed = 10, 60 kmph

- UE-side prediction: LMMSE based prediction of channel

  - Alt1B

    - [n_ref, n+P/2]  WCSI=14

    - [n_ref, n+P]  WCSI=24

  - Alt2B

    - [n, n+P/2]  WCSI=10

    - [n, n+P]  WCSI=20

The UPT vs overhead tradeoffs for UE speed 10 and 60 kmph are shown in Figure 11 and Figure 12, respectively. We can make the following observation.

Observation 14:

- Alt1B outperforms Alt2B

- There is an ‘optimal’ WCSI (predicting beyond this window does not help)

  - Alt1B with CSI window [nref,n+X] is the best among the considered CSI windows

  - In general, the value of X depends on UE speed (cf. Appendix C)

Figure 11

Figure 12

TDCP reporting

## Key issues

    - Issue 12 (TDCP parameter)

[1] Agreement For the Rel-18 TRS-based TDCP reporting, down select one of the following alternatives by RAN1#110bis-e: - AltA. Based on Doppler profile - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc - AltB. Based on time-domain correlation profile - E.g. Correlation within one TRS resource, correlation across multiple TRS resources - Note: The correlation over one or more lags of TRS resource may be considered. The lags may be within one TRS burst or different TRS bursts - AltC: CSI-RS resource and/or CSI reporting setting configuration parameter(s) to assist network - E.g. gNB configures UE with multiple choices on what to assist (e.g. two or more CSI-RS/report periodicities, or precoding schemes depending mainly on UE velocity), then UE report according to configuration; parameters correspond to CSI reporting periodicity, codebook type, etc. Note: Different alternatives may or may not apply to different use cases

Regarding the TDCP parameter, we have slight preference for the (auto-)correlation based parameter since it is feasible with current UE implementations. Besides, some information about the Doppler profile can also be extracted (e.g. taking FFT) from (auto-)correlation values corresponding to ‘dominant’ lag values. This has been shown vis simulation in Section 4.2.

There are a few open points with AltB that require further discussion. For instance,

- Number and indices of lag values: whether NW-configured or UE-reported

- Number of TRS resources: whether one TRS resource is sufficient considering that the min periodicity of a TRS resource is 4 slots, implying the possible lag value can be {0,1,4,5,…}. If the lag values = 2,3 are ‘dominant’, they can be supported with one TRS resource. So, number of TRS resources for TDCP reporting requires further study

- Number of TRS resource sets: the current spec allows up to 4 TRS resources in one TRS resource set, 2 resources in 2 consecutive slots. Multiple TRS resource sets may be needed for (1) reporting different lag values and (2) for mTRP scenarios.

Proposal 15:

- support AltB and AltA (2nd preference) for TDCP parameter

- support TDCP reporting across multiple TRS resources

- study TDCP reporting across multiple TRS resource sets

    - Other issues

[1] Agreement The Rel-18 TRS-based TDCP reporting comprises stand-alone auxiliary feedback information to enable refinement of CSI reporting configuration, and/or codebook configuration parameters, and/or (to be confirmed in RAN1#110) gNB-side CSI prediction - Not conditioned on other UCI parameters - Not reported together with CQI/PMI/RI/(CRI) associated with a codebook - Note: This does not prevent TDCP reporting from being multiplexed with other UCI parameters on PUCCH and/or PUSCH - Note: Aperiodic reporting is supported (per agreed Alt1 in RAN1#109-e) [2] Agreement The TRS-based TDCP reporting is down selected from the following alternatives: - Alt1 (stand-alone): TDCP reporting comprises auxiliary feedback information to enable refinement of CSI reporting configuration, and/or codebook configuration parameters, and/or (to be confirmed in RAN1#110) gNB-side CSI prediction - Aperiodic reporting is supported - FFS: Whether periodic, semi-persistent and/or event-triggered (UE-initiated) reporting are supported - Alt2 (non-stand-alone): TDCP reporting corresponds to a subset of the UCI parameters associated with a codebook/PMI for high/medium velocities, reported by the UE and measured via TRS - FFS: The associated codebook(s)/PMI(s)

Regarding the FFS on other reporting format (i.e. periodic, semi-persistent, event-triggered), we support event-triggered (UE-initiated) TDCP reporting on UL MAC CE, e.g. based on an event. The event can correspond to a condition on UE speed > threshold, or Doppler shift > threshold etc. Such reporting can be beneficial for system operation since the NW is usually unaware of the need for such reporting (e.g. when the UE speed is low, such reporting is not needed). The UE on the other hand is aware of its speed, hence can assist the NW by triggering/initiating this reporting.

Proposal 16: support event-triggered/UE-initiated TDCP reporting on UL MAC CE

## Simulation results

We consider CDL-A channel model, 30 kHz subcarrier spacing, 3.5GHz carrier frequency, 52 RBs, a sampling periodicity of 5 slots, 32 Tx antennas and 4 Rx antennas. In our simulations, there is one TRS resource in a slot, so correlation lags of 1, 2, 3…correspond to 5, 10, 15… slots.

The Doppler frequency is denoted by fD in Hertz and sampling time is denoted by T in seconds. The normalized frequency is fDT which should be less than 0.5 to avoid aliasing or to satisfy the Nyquist sampling theorem. The power spectral density (PSD), which is the FFT of correlation lag values with a squared and absolute operation, lies between -fDT + foT  and fDT + foT, where foT is due to frequency offset or any Doppler shift. The Doppler spread is 2fDT, which is proportional to the UE speed (due to fD being proportional to the UE speed).

Figure 13 depicts the PSD for various UE speeds. Since Doppler information changes very slowly, an observation window of 1 second was used to calculate the correlation values. Correlation lags were averaged across subcarriers carrying the TRS resource.

Figure 13: PSD for various UE speeds computed from the feedback of correlation lags.

Next, we look at the number of correlation lag values that need to be reported. Note that the PSD, which involves the FFT of correlation lag values has an issue due to windowing. When a signal is windowed and the FFT is taken, the effective bandwidth of the signal increases due to windowing. This increase is more prominent for smaller windows. So, the perceived Doppler spread of the PSD increases as the number of reported correlation lags is reduced. Figure 14 depicts the perceived PSD by the gNB for various number of reported correlation lags from the UE. It can be seen that as the number of reported lags decreases, the effective Doppler spread increases. For a given sampling periodicity of the TRS resource, an increase in effective Doppler spread means a reduction in the maximum UE speed supported, as the maximum Doppler spread should be less than 0.5. Figure 15 shows that for small number of correlation lags, the effective Doppler spread is high and settles down to a final value at around 10 correlation reported lags. So it becomes important for the gNB to inform the UE of the number of correlation lags that needs to be reported.

Observation 15:

- The perceived Doppler spread increases as the number of reported correlation lags decreases due to windowing before FFT operation.

- For a given UE speed, there is a minimum number of reported correlation lags that can represent the Doppler spread accurately.

Figure 14: Effective PSD seen by the gNB for various number of reported correlation lags for a UE speed = 40 km/hr.

Figure 15: Doppler spread variation vs. number of correlation lags for 40 km/hr.

Conclusion

In this contribution, the following observations and proposals are made:

Type II C-JT

Observation 1: Regarding TRP selection

- Semi-static selection is preferred from NW perspective in order to control the inter-cell interference

- Dynamic selection can lead to dynamic inter-TRP/cell interference fluctuations, resulting in large CQI mismatch

Observation 2:

- Alt2 will incur large performance loss due to the absence of inter-TRP co-phase information

- Alt1 is tailored for the case (A) when TRPs have similar power

- Alt3 is beneficial for the case (B) when TRPs have different power levels

- Alt4 works for (A) and (B), and for the general case (C) when a subset of TRPs are ‘strong’ and remaining TRPs are ‘weak’

Observation 3: N SCIs (per TRP SCI) is not needed for CJT precoding, incur more overhead, and are unlikely to have any performance benefits.

Observation 4: A TRP indicator reporting can be beneficial for various use cases (e.g. per-TRP FD basis selection, per-TRP co-scaling reporting).

Observation 5: 1) UE-based dynamic TRP selection degrades the performance of UPT vs overhead (4% avg. UPT loss) especially in the intra-cell scenario due to unpredictable interference fluctuation, and 2) the gNB-based dynamic TRP selection method outperforms (2~4% avg. UPT gain) the other two methods in both of the intra-/inter-cell scenarios.

Observation 6: Alt4 (#. Ref Groups for amp = 4) yields the best UPT vs overhead trade-off and 2~4% avg. UPT gain over the other methods, Alt1, 2, and 3.

Observation 7: multiple (or different) L values (Alt1) can be beneficial as showing ~5% avg. UPT gain over the same L value case (Alt2). TRP-common SD beam selection (Alt3) yields the worst UPT vs overhead trade-off performance.

Observation 8: TRP-common bitmap incurs large avg. UPT loss that cannot be compensated for the overhead saving (it turns out 2~3% avg. UPT loss).

Observation 9: a sufficient performance gain (70% - 100%) can be obtained in a low-overhead regime that is comparable to the overhead of sTRP case, when L=1 and/or low values of pv (e.g., 1/8) are allowed.

Proposal 1: support gNB-based TRP selection

- support Alt1 (via RRC) as baseline

- study more dynamic gNB-based TRP selection (e.g. via CSI request in DCI)

Proposal 2: Support the following

- Alt4 for the W2 quantization group

- One SCI across all TRPs, including a unified (one) solution regarding remapping/shifting in FD for both modes and for both Rel.16 and Rel.17-based CJT codebooks

- A TRP indicator reporting for components that are reported relative to a reference (e.g. per-TRP FD basis selection, per-TRP co-scaling)

Proposal 3:

- Support offline proposal 1.1 with the following modification regarding SD basis selection parameter

  - Reusing parameter α (defined per Rel.17 specification) for Rel.17-based CJT port selection codebook

- Support Alt1 in offline proposal 1.2 with the following examples included in TBD

  - Ex1: Each Ln is configured

  - Ex2: One L is configured, and {Ln} is determined based on the configured L value

Proposal 4: Support offline proposal 1.3 and reuse/extend legacy solution to N CSI-RS resources

- bitmap size Bn=2LnMυ,n

- NNZC constraint

  - when rank 1-2, per-CSI-RS resource NNZC constraint

  - when rank 3-4, joint NNZC constraint across N CSI-RS resources

- one β value configured via parameter combination

Proposal 5:

- Support new parameter combination(s) including L=1, targeting low-overhead regime

- Study the need for FD basis window for FD basis reporting

  - Legacy (Rel.16 or Rel.17) FD basis window design is baseline

  - If supported, the same FD basis window design for both Rel.16 and Rel.17-based CJT codebooks is preferable

Type II Doppler

Observation 10: regarding offline proposal 2.1

- Alt1 and Alt2 includes TD/DD basis for compression; whereas Alt3 does not (i.e. uncompressed)

- Alt3 can incur large overhead due to multiple W2s

- If (W1,Wf) and multiple W2s are reported in different slots, this will lead to inter-dependency, error propagation, and will require specifying dropping rules.

- Alt1 can accommodate Alt2 and Alt3

Observation 11: there is an ‘optimal’ prediction window for a given UE speed, whose starting slot is desired to be as close to the measurement window as possible, and ending slot is such that the prediction error doesn’t affect the UPT performance.

Observation 12: Regarding CSI-RS burst configuration

- SP CSI-RS resource can be restrictive due to (1) uniformly separated burst, and (2) the min periodicity = 4 slots.

- Multiple SP CSI-RS resources and a group of AP CSI-RS resources can be used to address these limitations.

Observation 13: Alt1 and Alt2 achieve similar performance vs overhead trade-off

Observation 14:

- Alt1B outperforms Alt2B

- There is an ‘optimal’ WCSI (predicting beyond this window does not help)

  - Alt1B with CSI window [nref,n+X] is the best among the considered CSI windows

  - In general, the value of X depends on UE speed (cf. Appendix C)

Proposal 6: prioritize Rel-16-based design for Type II codebook refinement for high/medium UE velocities.

Proposal 7: support rank 1-4 for Type II codebook refinement for high/medium UE velocities

Proposal 8: Regarding codebook structure

- Support Alt1 in offline proposal 2.1 and proposal 2.2

- Support at least one rotation factor, and study whether additional (e.g. per-SD basis vector) rotation factor is needed

- Length of the DD/TD basis vector (N4) is RRC-configured

- Number of DD/TD basis vectors: support {1,2} and study the need for value(s) > 2

Proposal 9: support offline proposal 2.3 only if Alt1.B (i.e. legacy slot location of the CSI reference resource) is included, and include the following enhancements

- CSI-RS occasion is defined w.r.t. ‘new CSI reference resource’

- CQI BLER requirement over the window WCSI

Proposal 10: do not support any specification requiring gNB-side prediction

Proposal 11: support the following regarding the CSI-RS “burst/occasion”

- both SP and AP CSI-RS resources

  - multiple SP CSI-RS resources in one CSI-RS resource set (e.g. same periodicity, and different offset)

  - a group of AP CSI-RS resources activated/triggered together

  - time restriction = ‘notConfigured’

Proposal 12: Regarding DD/TD unit, support the following

- a DD/TD unit comprises NST consecutive time slots, where NST is configured via RRC

- NST determines N4, i.e., the number of DD units or length of DD basis vectors

- Number of DD basis vectors (D) for DD compression is configured

- DD/TD unit for both PMI and CQI

  - FFS: whether DD/TD unit is the same or different for PMI and CQI

Proposal 13: support CQI enhancements for Type II Doppler codebook based CSI reporting,

- One or multiple CQIs in time domain, e.g. 1 CQI per DD/TD unit

- CQI BLER requirement, e.g. at one slot in a DD/TD unit or over all slots in a DD/TD unit

Proposal 14: Regarding W2 reporting for Type II Doppler codebook

- Rel-16/17 based W2 reporting mechanism is reused,

- Study the need for mechanisms to limit the payload increase of W2 reporting, e.g. via parameter combination or bitmap

TDCP reporting

Observation 15:

- The perceived Doppler spread increases as the number of reported correlation lags decreases due to windowing before FFT operation.

- For a given UE speed, there is a minimum number of reported correlation lags that can represent the Doppler spread accurately.

Proposal 15:

- support AltB and AltA (2nd preference) for TDCP parameter

- support TDCP reporting across multiple TRS resources

- study TDCP reporting across multiple TRS resource sets

Proposal 16: support event-triggered/UE-initiated TDCP reporting on UL MAC CE

# References

- Chairman’s notes, RAN1#110

- Chairman’s notes, RAN1#109-e

- R1-2203895, “Initial SLS results on Type-II CSI enhancements for CJT”, Samsung

# Appendix A

Table 3: EVM for Type II C-JT

| Parameter |  | Value (Intra-cell scenario) | Value (Inter-cell scenario) |
| --- | --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM | FDD, OFDM |
| Multiple access |  | OFDMA | OFDMA |
| Scenario |  | RMa (Rural Macro) N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP) for each UE Outdoor1 | Dense Urban (Macro only) N_TRP (#TRPs): 2, 3, (N_TRP is semi-statically chosen based on, e.g. RSRP) Outdoor2 OptA |
| Frequency Range |  | FR1, 700Hz | FR1, 2GHz |
| Inter-BS distance |  | 1.7km | 200m |
| Channel model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT |
| Number of Rings |  | 2 rings (57 sectors) - Each sector has N TRP as a cooperating mTRP set. | 2 rings (57 sectors): - The three sectors of each site is a cooperating mTRP set. |
| Number of UEs per sector |  | 30 | 30 |
| Antenna setup and port layouts at gNB |  | For each TRP, - 4 ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ - 8 ports: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ Total #ports in mTRP = N TRP x {4,8} | For each TRP, - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32} |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 46 dBm for 10 MHz | 41 dBm per TRP for 10 MHz |
| BS antenna height |  | 25m | 25m |
| UE antenna height & gain |  | Follow TR36.873 | Follow TR36.873 |
| UE receiver noise figure |  | 9dB | 9dB |
| Modulation |  | Up to 256QAM | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS | 15kHz | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots | Slot Format 0 (all downlink) for all slots |
| MIMO and scheduling scheme |  | MU-MIMO PF scheduling (User Rank 1 or 2) | MU-MIMO PF scheduling (User Rank 1 or 2) |
| MIMO layers |  | Up to 4 | Up to 12 |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Based on Alt1A/B, Alt2, Rel-16 eType-II | Based on Alt1/B, Alt2B, Rel-16 eType-II |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | RU 30~40% or 70~80% | RU 30~40% or 70~80% |
| UE distribution |  | 50% indoor (3km/h), 50% outdoor (120km/h) | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC | MMSE-IRC |
| Feedback assumption |  | Realistic | Realistic |
| Channel estimation |  | Realistic | Realistic |
| Evaluation Metric |  | User perceived throughput and CSI feedback overhead | User perceived throughput and CSI feedback overhead |

Table 4: EVM for Type-II Doppler

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. UE speed: 10 kmph Mobility model: - Spatial consistency procedure not modeled - No trajectory is assumed - Doppler spectrum model is not needed Single TRP |
| Frequency Range |  | FR1 only, 2GHz |
| Duplexing gap (b/w DL and UL) |  | 200MHz |
| Inter-BS (site) distance |  | 200m |
| Channel generation model |  | According to the TR 38.901 O2I car penetration loss per TS 38.901 can be assumed |
| Reciprocity model |  | Not applicable |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 20 MHz (10 MHz DL + 10 MHz UL) for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | Dynamic SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Baseline scheme: CSI feedback periodicity (full CSI feedback): p1=5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling): D=4 ms eType II Doppler: CSI feedback periodicity : W=p2=10 ms CSI-RS burst: window of B measurement instances with separation d - B=2 - d=1 CSI reporting window: WCSI=B+D+p2=16 Prediction: - gNB-side prediction: DFT-based - UE-side prediction: LMMSE CQI: - gNB-side prediction: 1 CQI (legacy) - UE-side prediction: 1 or 2 CQIs over WCSI |
| Overhead |  | DMRS, CSI-RS (including CSI-RS burst overhead) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% for SU/MU-MIMO with dynamic rank 1-2 adaptation |
| UE distribution |  | 100% outdoor |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 regular eTypeII with CSI feedback periodicity 5 ms |

# Appendix B

We used the Rel-16 parameter combination table.

|  | ParaComb=1 | ParaComb=2 | ParaComb=3 | ParaComb=4 | ParaComb=5 | ParaComb=6 |
| --- | --- | --- | --- | --- | --- | --- |
| Semi-Static TRP Sel. | 40.11 | 40.99 | 40.69 | 41.67 | 41.1 | 42.03 |
| UE-based Dynamic TRP Sel. | 40.19 | 40.49 | 40.4 | 40.66 | 40.97 | 40.83 |
| gNB-based TRP Sel. | 39.71 | 41.12 | 40.97 | 42.04 | 41.58 | 41.97 |

Table B1: Absolute average UPT values (Mbps) for Figure 1.

|  | ParaComb=1 | ParaComb=2 | ParaComb=3 | ParaComb=4 | ParaComb=5 | ParaComb=6 |
| --- | --- | --- | --- | --- | --- | --- |
| Semi-Static TRP Sel. | 46.14 | 49.01 | 49.85 | 51.55 | 53.43 | 54.4 |
| UE-based Dynamic TRP Sel. | 46.93 | 49.05 | 49.91 | 52.59 | 54.58 | 53.82 |
| gNB-based TRP Sel. | 46.33 | 48.96 | 50.52 | 52.79 | 54.08 | 53.71 |

Table B2: Absolute average UPT values (Mbps) for Figure 2.

|  | ParaComb=1 | ParaComb=2 | ParaComb=3 | ParaComb=4 | ParaComb=5 | ParaComb=6 |
| --- | --- | --- | --- | --- | --- | --- |
| #. Ref groups=2 & 1 SCI | 36.58 | 36.98 | 39.28 | 39.6 | 39.52 | 41.9 |
| #. Ref groups=2N & 4 SCIs | 36.88 | 36.41 | 39.20 | 39.56 | 39.83 | 41.82 |
| #. Ref groups=2N & 1 SCI | 36.55 | 36.46 | 38.92 | 39.38 | 39.07 | 41.64 |
| #. Ref groups=4 & 1 SCI | 36.35 | 37.35 | 39.65 | 39.98 | 39.92 | 42.04 |

Table B3: Absolute average UPT values (Mbps) for Figure 3.

|  | ParaComb=1 | ParaComb=2 | ParaComb=3 | ParaComb=4 | ParaComb=5 | ParaComb=6 |
| --- | --- | --- | --- | --- | --- | --- |
| Per-TRP SD beam selection w/ different L values | 46.19 | 48.02 | 48.65 | 52.34 | 52.75 | 53.31 |
| Per-TRP SD beam selection w/ same L value | 46.14 | 49.01 | 49.85 | 51.55 | 53.43 | 54.4 |
| TRP-common L SD beam selection | 44.69 | 48.31 | 48.52 | 50.71 | 52.6 | 52.93 |

Table B4: Absolute average UPT values (Mbps) for Figure 4.

|  | ParaComb=1 | ParaComb=2 | ParaComb=3 | ParaComb=4 | ParaComb=5 | ParaComb=6 |
| --- | --- | --- | --- | --- | --- | --- |
| Common-bitmap case | 46.03 | 48.63 | 49.09 | 51.95 | 51.93 | 54.65 |
| Separate-bitmap case | 50.03 | 49.77 | 52.62 | 52.58 | 53.86 | 55.01 |

Table B5: Absolute average UPT values (Mbps) for Figure 5.

|  | ParaComb=1 or NewParaComb=1 | ParaComb=2 or NewParaComb=2 | ParaComb=3 or NewParaComb=3 | ParaComb=4 or NewParaComb=4 | ParaComb=5 | ParaComb=6 |
| --- | --- | --- | --- | --- | --- | --- |
| Alt2 w/ New ParaComb | 38.87 | 39.55 | 40.95 | 45.55 | - | - |
| Alt2 w/ ParaComb | 46.14 | 49.01 | 49.85 | 51.55 | 53.43 | 54.4 |
| sTRP | 23.71 | 25.48 | 25.6 | 27.8 | 28.39 | 29.22 |

Table B6: Absolute average UPT values (Mbps) for Figure 6.

# Appendix C (Alt1.B vs Alt2.B and W_CSI)

# Measurement: 10 slots

# Normalized MSE b/w channel HT and predicted channel Ht' where t≤T

# T=1,…,40

# t=1,…,T

# Observation

# MSE period (P) decreases w/ UE speed

# CSI window (W_CSI) should depend on P, e.g. W_CSI=P/2

# If P is small (e.g. @higher speed), W_CSI can be within [n_ref, n]

- W_CSI

  - Alt2.B works @ low speed

  - Alt1.B is needed @higher speed

Figure 16

Figure 17

Figure 18