3GPP TSG RAN WG1 #110bis-e			R1-2209727

e-Meeting, October 10th – 19th, 2022

Agenda Item:		9.2.4.2

Source:				Samsung

Title:					Representative sub use cases for Positioning

Document for:	Discussion

Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. The objective of the study item is as follows.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we will provide our view on AI/ML for positioning, including the general consideration for applying AI in positioning and also the preference of representative sub use cases and the corresponding specification impact.

General consideration on using AI in Positioning

As discussed in companion contribution, artificial intelligence (AI)/ machine learning (ML) have two phases in general, training/learning and inference phase. During the training phase, In the consideration of AI/ML structure, an AI model will be trained based on (massive) training input data and the training output corresponding to these training input data. Thus, the updated/trained weighting/offset will be obtained after the training phase is completed.  Further, the trained model could be tested/validated so that it could be actually applied to inference phase for a particular purpose, i.e., the positioning in our discussion.

Fig.1 general AI/ML structure

On the other hand, in the existing framework of RAT-dependent positioning schemes from R15 to latest R17, the following steps are generally required for the positioning procedure:

Fig.2 general positioning structure

- A service requiring positioning location information activated;

- The positioning request is activated;

- LMF determines the suitable methods, RS configuration etc, and corresponding signalling exchange;

- UE will conduct the RS measurement (i.e., DL based methods) and/or RS transmission (i.e., UL based methods),

- UE/gNB will obtain the measurement results (i.e., RSRP related and/or timing related);

- With potential measurement results exchange, the positioning location is derived eventually.

In the study of RAT-dependent positioning methods, no matter timing based and/or angle based, they are highly dependent on the environment. For example, in the heavy NLoS situation, even the estimation of the timing of arrival (TOA) is ideal. The information can barely help the final positioning estimation since the true LoS distance is missing. Thus in such scenario where the legacy methods are merely useful, the AI based method are expecting to jump in. This is a direction that using philosophy of math cannot achieve.

Observation 1: the use cases in which legacy positioning methods cannot work well could be prioritized to check whether AI based methods could work.

Candidate sub use cases

Based on the discussion in above, the more attractive part for applying AI mode in PHY for positioning purpose is to use AI contributing to step 4,5,6. More specifically,

Sub use case 1: channel information  Positioning location estimation

Motivation: one typical usage of AI is that, with input information as channel information (e.g., the CIR for both LOS/NLOS environment) and output information directly as the positioning location. Assuming if there is a well trained AI model which have “seen” massive channel characters and their corresponding locations, then such AI model is able to identify the location according to the newly input channel information even though such channel is NLOS. The logic behind this that, the channel information is actually including all the information related to timing delay and RSRP which are dependent on the distance between base station and UE.

Sub use case 2: channel information  Measurement estimation

Motivation: One reason why legacy RAT dependent method cannot work well in NLOS environment is the intermediate measurement output are incorrect. For example, the TOA cannot be recovered even assuming estimation is error-free. Thus, another typical usage is that, still based on the channel information (which could be NLOS cases), the output now is the intermediate measurement e.g., TOA, TDOA or AOA/AOD etc. After that, applying legacy algorithm could derive the positioning location information.

Potential spec impact

in order to make AI based method work, several potential specification supports are considered:

- Training data type/size: CIR related information (e.g., full dimension CIR or truncated CIR) and corresponding location information

- Determination of source (device) providing input data: whether a device is qualified to provide training data, e.g., a device with known(correct) location information;

- Measurement data (for training input and/or inference input) exchange signalling/procedure, if any;

- AI model related configuration exchange signalling/procedure, if any.

Proposal 1: both positioning location estimation and intermediate measurement estimation can be considered as candidate sub use-cases;

Proposal 2: studying the potential spec impact with consideration on training data type/size, determination of source providing input data, measurement data or AI model related configuration exchange if any.

AI/ML model management

In last meeting discussion, there were quite a lot of aspects proposed to be studied in AI/ML model management. In the following, several interesting aspects are discussed for which we think are important for applying the AI/ML model in actual usage.

Training data collection/update

One essential aspect is to obtain a qualified training data, as it could not only be beneficial for the original model training  but it could also help the model update/finetuning if there is. The qualified training data means the accurate input data (e.g., the CIR measurement etc.) and/or the accurate label (e.g., the UE location information). But how to obtain this information is a question. One possible direction is that using whatever offline method/equipment to derive in the actual scenarios, e.g., using the PRU like device to contribute. But if this is purely relying on the implementation, then there will be no control on the training data quality, it could jeopardize the utilization of AI/ML model. So it’s preferable for RAN1 to study the training device (which is the training data provider) about their criteria, e.g., how/which/when a device could be training device. For a simple example, probably when a UE holds a good channel condition and holds a confident accurate location information could be a training device. And responding to the request, such training device could provide training data under certain conditions.

In addition, given some needed training data type is aligned with existing measurement outcome of RAT-dependent positioning method, such as the RSRP, TOA of the received signal or even a particular path of the received signal. Some extension of the reporting could be fit into the data collection, e.g., enlarge the report size. In that sense, the current signaling framework of the measurement-report could be used to training data collection, at least for a starting point.

Proposal 3: RAN1 to study the training data collection criteria, e.g., the qualified training device determination.

Proposal 4: Current signaling framework of the measurement-report could be used as starting point to enable training data collection

Model validation

Once a trained model is generated, or obtained from other side instead of the model user itself. One important thing is to ensure such model is indeed useful, or to say, valid for current usage with such model user. Unlike the computer science scenarios that the AI model may be used to handle the image/video/text which are not impacted by the timing varying environment. The channel condition are vital for the actual usage of the AI model especially for positioning purpose. Thus, a validation process for a trained/obtained model should be studied. It is noticed that the test/verify step could be implemented within the model itself. However, that step could only be related to verify the effectiveness of the model under the character of the training data type. For example, the full size of CIR, or truncated CIR, or even single CIR of first path could be used a training input, the testing/verifying inside the model are done in the training process under certain CIR choice. But the model validation mentioned here is aside the training process, it is preferable to know whether or not the trained model obtained a certain time ago are still valid for coming usage.

Proposal 5: RAN1 to study the validation of the trained/obtained AI/ML model before actually apply it.

Model recovery/update

Assuming a (valid) AI/ML model is putting in use. Still according to the environment (channel condition, location etc) change, the current used model might be out of date. The performance of the model will be of course degraded. Then what the UE should do? One choice is to directly terminate such model. This is a simple but not constructive choice. The UE is under the situation of hoping using a AI/ML model for positioning. So instead, the more reasonable choice is to do a model recovery or update. To make the model get recovered/updated, and become more suitable for current environment, such model recovery or update should be studied for aspects like how to determine a model need to be recovered or updated, and then how to recovery/update.

For condition to update the model, there could be two types of condition, one is the event based condition, the other is the timer/counter based condition. The former one is based on the monitoring of changing on a target subject, for example, if the channel condition (e.g., measured RSRP, pathloss) has varied larger than a threshold, or, the collected input data is not less than the required amount or required quality for the existing model, it can trigger the update of the existing model. The latter one is more like periodically updating process. Regardless the change of environment, the model will run into the update procedure if the timer or counter has reached a certain amount, and such timer or counter will be (re)set when the model has been enabled or updated.

Proposal 6: RAN1 to study the condition/methods to recovery/update a AI/ML model for positioning, e.g., event based condition or timer/counter based condition.

Conclusion

This contribution discusses the UE features for NR positioning enhancements. The proposals are summarized as follows:

Observation 1: the use cases in which legacy positioning methods cannot work well could be prioritized to check whether AI based methods could work.

Proposal 1: both positioning location estimation and intermediate measurement estimation can be considered as candidate sub use-cases;

Proposal 2: studying the potential spec impact with consideration on training data type/size, determination of source providing input data, measurement data or AI model related configuration exchange if any.

Proposal 3: RAN1 to study the training data collection criteria, e.g., the qualified training device determination.

Proposal 4: Current signaling framework of the measurement-report could be used as starting point to enable training data collection

Proposal 5: RAN1 to study the validation of the trained/obtained AI/ML model before actually apply it.

Proposal 6: RAN1 to study the condition/methods to recovery/update a AI/ML model for positioning, e.g., event based condition or timer/counter based condition.

- References

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 108-e, Moderator, RAN1#108e.

- Annex – RAN1#109e~110 agreement

RAN1#109e agreement

Agreement

Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels.

- Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2.

- Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement.

- Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case

Agreement

For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered.

- Direct AI/ML positioning: the output of AI/ML model inference is UE location

  - E.g., fingerprinting based on channel observation as the input of AI/ML model

  - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation

  - FFS: applicable scenario(s) and AI/ML model generalization aspect(s)

- AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement

  - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement

  - FFS the details of input and output for corresponding AI/ML model(s)

  - FFS: applicable scenario(s) and AI/ML model generalization aspect(s)

- Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement

Agreement

Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement.

- AI/ML model training

  - training data type/size

  - training data source determination (e.g., UE/PRU/TRP)

  - assistance signalling and procedure for training data collection

- AI/ML model indication/configuration

  - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection)

- AI/ML model monitoring and update

  - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning)

- AI/ML model inference input

  - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference)

  - model input acquisition and pre-processing

  - type/definition of model input

- AI/ML model inference output

  - report/feedback of model inference output

  - post-processing of model inference output

- UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring)

- Other aspects are not precluded

- Note: not all aspects may apply to an AI/ML approach in a sub use case

- Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

RAN1#110 agreement

Agreement

For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected.

- Direct AI/ML positioning

- AI/ML assisted positioning

- Note 1: the selection does not intend to provide any indication of the prospects of any future normative project.

- Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results

Conclusion

Defer the discussion of prioritization of AI/ML positioning based on collaboration level until more progress on collaboration level discussion in agenda 9.2.1.

Agreement

Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement

- Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report)

  - Partial and/or noisy ground truth label

- Signaling for data collection

- Other aspects are not precluded

Agreement

Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement

- AI/ML model monitoring performance metrics

- Condition of AI/ML model update

- Reference signals and measurement feedback/report

- Other aspects are not precluded

Agreement

Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least

- UE-side or Network-side training

- UE-side or Network-side inference

  - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches

Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity

Conclusion

To use the following terminology defined in TS 38.305 when describe their proposed positioning methods

- UE-based

- UE-assisted/LMF-based

- NG-RAN node assisted

Note: companies are required to clarify their positioning method(s) when their approaches do not fall in one of the above