3GPP TSG RAN WG1 #110bis-e	    			            R1-2209738

e-Meeting, October 10th – October 19th, 2022

Agenda item:	9.5.2.2

Source: 	Samsung

Title: 	Discussion on NR Carrier Phase Positioning Measurements

Document for:	Discussion and Decision

# Introduction

A new study item has been approved to expand and improve NR positioning in Rel-18 [1]. One of the objectives of the Rel-18 positioning SI is to improve accuracy, integrity and power efficiency of positioning techniques. To this end, carrier-phase method is one of the promising techniques to improve the accuracy of positioning.

- Improved accuracy, integrity, and power efficiency:

  - …

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

- Improved accuracy, integrity, and power efficiency:

  - …

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

In this document, we present Samsung’s views on the carrier-phase method.

# Discussion

Carrier phase (CP) positioning relies on measuring a carrier phase at the RF frequency of a signal transmitted from one device (e.g., device A) and received by another device (e.g., device B). The carrier phase measured at device B is a function of the propagation time, and consequently the propagation distance, from transmitter of device A to the receiver of device B. Device A and device B can be a gNB and a UE respectively or vice versa.

The propagation time Tp from the transmitter of device A to the receiver of device B can be expressed as a sum of an integer number of cycles at the carrier frequency, N Tc, where Tc is the carrier frequency period, and a fraction part of a cycle Tf, where Tf<Tc. Therefore,

Tp= N Tc+Tf  	                                                                                 (1)

The measured carrier phase, (phase difference between device A and device B) is given by φp=2πfcTp=2πfcTf. The accuracy of the carrier phase measurement can be in the range of 0.01 to 0.05 cycles [2]. For a carrier frequency of 3 GHz, the wavelength is 10 cm, this corresponds to 1 mm to 5 mm accuracy, which is well within cm-level accuracy. However, the carrier phase method has its own design consideration which are further discussed and analysed in this document.

In section 2.1, we further discuss the carrier phase method and the challenges associated with the carrier phase method when measuring the phase of the signal transmitted from device A to device B.

If device A transmits from multiple antenna elements, and the transmitted signals are received by a single receiver at device B, the propagation path from each of the transmitters of device A to the receiver of device B undergoes a different propagation delay, the difference in propagation delay and hence the difference in phase at the receiver of device B can be used to determine the angle of departure from device A. Alternatively, if device A transmits from a single antenna, and the signal is received by multiple antenna elements at device B, the propagation path from the transmitter of device A to each receiver of device B undergoes a different propagation delay, the difference in propagation delay and hence the difference in phase at each antenna element receiver of device B can be used to determine the angle of arrival at device B. Using differential phase measurements and the associated challenges is discussed in section 2.2.

## Carrier-phase measurements

Consider a transmitter transmits a signal at time t=0 from device A. As illustrated in Figure 1, the signal travels to the destination receiver at device B and arrives at time t=Tp, wherein Tp is the time of arrive of the signal at the receiver and equals the propagation delay from device A to device B as described in equation (1)

To use the carrier phase measurement (φp) to measure the propagation delay using equation (1), there are a few aspects to consider:

- The signal transmitted from device A to device B is a single carrier frequency with period Tc (frequency fc). In practice of 3GPP networks, the signal is transmitted more than one sub-carrier modulating an RF carrier. This is further discussed in section 2.1.1.

- Device A and device B have the same time reference, i.e. at the same time the phase of the signal transmitted from device A and the phase of the reference signal at device B are known. In general, this is not the case, at least within the level of accuracy expected from using the carrier phase method. Not only do device A and device B have a different time references, but their clocks can also drift relative to each other. Furthermore, the offset between the initial phase of the transmitter and the initial phase of the receiver can be random. This issue is further discussed in section 2.1.2.

- The carrier phase method measures fractional propagation delay Tf (φp=2πfcTf). The integer number of cycles N Tc, cannot be measured with a single carrier frequency. This gives rise to the well-known, integer ambiguity problem when using the carrier phase method. This is further discussed in section 2.1.3.

- Based on the agreement from RAN1#110, the carrier phase centre offset can be model as:

dPCO =  a * dPhi + w

where, dPhi is the difference between two directions, and dPCO is the corresponding phase center offset difference. The impact of this on the carrier phase measurement is further discussed in section 2.1.4.

- Equation (1) measures the propagation delay between device A and device B. The propagation delay depends on the path travelled from device A to device B. Having a line-of-sight path gives one propagation delay, but having a non-line-of-sight path gives a different propagation delay. Therefore, to use the propagation delay to find a distance between device A and device B we need to determine the line of sight condition. This is further discussed in section 2.1.5.

Figure 1: Carrier phase method to refine TOA measurement

### Positioning Reference Signal

Carrier-phase method has been used in satellite-based systems for positioning (e.g., GPS)[2]. In such systems a single carrier frequency is used – For example, GPS uses a frequency fL1=1575.42 MHz in the L-band (GPS can use a second frequency fL2=1227.60 MHz, but not for civil applications). In 3GPP for positioning, 3GPP defined two positioning reference signals one for downlink and one for uplink based on CP-OFDM.

- DL positioning reference signal (PRS). This is described in TS 38.211 clause 7.4.1.7

- Positioning sounding reference signal (Pos-SRS). This is described in TS 38.211 clause 6.4.1.4

A common aspect of these reference signals is that they have multiple sub-carriers in the same OFDM symbol. Each sub-carrier when modulated to the RF frequency will have its own RF frequency, and hence its own phase. The phase of carrier is related to its frequency and the distance of the travel path d between device A and device B as follows

φ=2πfcTp mod 2π=2πfcdc mod 2π

Where, c is the speed of light, hence each subcarrier can undergo a different phase shift due to propagation. This fact is further exploited in section 2.1.3 to eliminate the integer ambiguity.

Another aspect of the 3GPP positioning reference is the Comb structure and sub-carrier offset hopping from one OFDM symbol to the next, as provided by Table 7.4.1.7.3-1 of TS 38.211 for DL PRS and Table 6.4.1.4.3-2 of TS 38.211 for positioning SRS. Having a Comb structure reduces the number of sub-carriers that can be used for positioning, hence can impact the accuracy of the carrier phase method as fewer sub-carriers would be used to estimate the phase. Therefore,

Proposal 1: Study the use of Rel-16 DL and UL positioning reference signals with no or small Comb for the carrier phase method.

Having a different sub-carrier offset (k') between symbols creates gaps when measuring the phase of a sub-carrier, the same sub-carrier is not allocated to all symbols. This impacts the performance of the carrier phase method. By having the same sub-carrier allocated to multiple symbols, the sub-carrier phase can be calculated by averaging over multiple consecutive symbols to improve the accuracy of the carrier phase method.

Proposal 2: Study the use of Rel-16 DL and UL positioning reference signals using the same frequency offset (k') across OFDM symbols of a slot for the carrier phase measurement.

### Synchronization Mismatch between devices used for positioning

In equation (1), it has been assumed that device A and device B have the same time reference, i.e., t=0 refers to the same time point in both devices and the carrier phases are synchronized at this point. In practice this is not the case, at least within the desired accuracy when using the carrier phase method. In 3GPP networks, a UE can do DL synchronization by measuring DL reference signals e.g., PSS/SSS and synchronizes its uplink transmission time by getting gNB feedback based on timing advance commands. However, this synchronization targets having the UL received signal at the gNB’s antenna interface to be within a CP to avoid inter-user interference. This level of synchronization is not acceptable to do carrier phase detection. For example, for a carrier frequency fc=3 GHz, an offset of 1 ns between the clocks of device A and device B corresponds to a 3 carrier cycles (6π radians), which is well beyond the accuracy of the carrier phase measurement. Thus, having an improved synchronization between gNB and UE, or cancelling out the synchronization errors, could be a way to improve the effectiveness of using the carrier phase measurement to have high positioning accuracy.

In this section we study two ways to overcome synchronization mismatch between gNB and UE:

- First, by having a reference UE (positioning reference unit).

- Second by using round-trip carrier phase measurement.

#### PRU based method

Figure 2: Impact of clock biases on carrier phase method.

Figure 2 illustrates the impact of clock biases on the carrier phase measurements. Device A is a gNB transmitting to device B which is a UE. The gNB has a bias in its clock relative to a common reference time of δtb1 The UE has a bias in its clock relative to the common reference time of δtu. The phase of the reference signal at the UE’s reference time (tu=0) is ϕu0. The gNB transmits DL PRS n1, the DL PRS is transmitted after time Tn1 from the gNB’s reference time. We assume that the phase of the DL PRS after the CP is φb1=0. The UE receives symbol n1 (DL PRS symbol) after a propagation delay of Tp=τ. As illustrated in Figure 2,  symbol n1 (DL PRS symbol) is received after time Tn1+τ+δtb1-δtu from the UE’s reference time. The UE can measure the phase difference between the UE’s reference signal and the received signal. This phase difference is:

Δφue1=ϕu0+Tn1+τ1+δtb1-δtu2πfc-ϕb1 mod 2π

To eliminate the clock biases, we can use the single difference method and the double difference method. For the single difference method, a second gNB transmits a DL PRS at time Tn2. The second gNB can have a different clock bias δtb2 and a phase for the DL PRS at Tn2 of φb2=0. Therefore, the measured phase of the second gNB is:

Δφue2=ϕu0+Tn2+τ2+δtb2-δtu2πfc-ϕb2 mod 2π

Subtracting Δφue2 from Δφue1 and assuming ϕb1=ϕb2=0, we get the single difference carrier phase measurement, which eliminates the UE’s clock bias, but we still have the gNB clock biases (δtb1 and δtb2):

Δφue-sd=Δφue1-Δφue2=Tn1-Tn2+τ1-τ2+δtb1-δtb22πfc mod 2π

In the last equation, Tn1 and Tn2 are known, τ1-τ2 is the time difference of arrival, which is the target of the measurement. δtb1-δtb2 is the difference in clock biases between gNB1 and gNB2, this is unknown.

Consider now a second reference UE, also known as positioning reference unit (PRU), with a known location. The single difference carrier phase measurement for that UE using the same two gNBs (gNB1 and gNB2) is:

Δφpru-sd=Tn1-Tn2+τ1pru-τ2pru+δtb1-δtb22πfc mod 2π

Where, τ1pru is the propagation delay between the reference UE (PRU) and gNB1. τ2pru is the propagation delay between the reference UE (PRU) and gNB2. Subtracting Δφpru-sd from Δφue-sd, we get the double difference carrier phase measurement, which eliminates the gNBs’ clock biases:

Δφue-dd=Δφue-sd-Δφpru-sd=τ1-τ2-τ1pru-τ2pru2πfc mod 2π

In the last equation τ1pru and τ2pru for the reference UE (PRU), measured for gNB1 and gNB2, are known (the position of the reference UE (PRU) is known, hence the only unknown in the equation is τ1-τ2, the difference in time of arrival between signals transmitted from gNB1 and gNB2 at the UE, which can be determined within an integer ambiguity, the handling of the integer ambiguity is further discussed in section 2.1.3. While, using a PRU can be considered for eliminating the clock biases of the UE and the gNBs, it may introduce additional complexity in the network topology if the PRU is regarded as a new type of node. It would be desirable to achieve the PRU’s function, which is eliminating clock biases, without having to introduce a new node type. Thus, it is beneficial for RAN1 to study how to provide PRU or PRU-like functionality under existing system, e.g., how to determine a PRU and apply the PRU during the positioning procedure.

Observation 1: PRU can be considered for elimination of gNB and UE clock biases, but at the expense of additional network topology complexity.

Proposal 3: RAN1 to study how to provide PRU or PRU-like functionality under existing system.

#### Round-trip carrier phase measurement based method

Now let’s consider the round trip carrier phase measurement, the round trip carrier phase measurement is the sum of the DL carrier phase measurement at the UE and the UL carrier phase measurement at the gNB. Using the round trip carrier phase method, the UE measures the carrier phase of the DL PRS, and the gNB measures the carrier phase of the positioning SRS. Adding the two carrier phase measurements together eliminates the clock biases of the gNB and the UE as these cancel out.

In Figure 3, device A is a gNB transmitting to device B which is a UE. The gNB has a bias in its clock relative to a common reference time of δtb The UE has a bias in its clock relative to the common reference time of δtu. The phase of the reference signal at the gNB’s reference time (tb=0) is ϕb0.  The phase of the reference signal at the UE’s reference time (tu=0) is ϕu0. The gNB transmits DL PRS n1, the DL PRS is transmitted after time Tn1 from the gNB’s reference time. We assume that the phase of the DL PRS after the CP is φb1=0. The UE receives symbol n1 (DL PRS symbol) after a propagation delay of Tp=τ. As illustrated in Figure 3,  symbol n1 (DL PRS symbol) is received after time Tn1+τ+δtb-δtu from the UE’s reference time. The UE can measure the phase difference between the UE’s reference signal and the received signal. This phase difference is:

Δφue1=ϕu0+Tn1+τ+δtb-δtu2πfc-ϕb1 mod 2π

The UE transmits positioning SRS n2, the pos-SRS is transmitted after time Tn2 from the UE’s reference time. We assume that the phase of the pos-SRS after the CP is φu2=0. The gNB receives symbol n2 (pos-SRS symbol) after a propagation delay of Tp=τ. As illustrated in Figure 3,  symbol n2 (pos-SRS symbol) is received after time Tn2+τ+δtu-δtb from the gNB’s reference time. The gNB can measure the phase difference between the gNB’s reference signal and the received signal. This phase difference is:

Δφbs2=ϕb0+Tn2+τ+δtu-δtb2πfc-ϕu2 mod 2π

Adding the two carrier phase measurements, we get the round-trip carrier phase which is given by:

Δφue1+Δφbs2=ϕu0+ϕb0+Tn1+Tn2+2τ2πfc-ϕb1-ϕu2mod 2π

In the last equation the clock biases have been eliminated. The carrier phases at the reference time ϕu0 and ϕb0 can be set to 0. Similarly, the carrier phases at the transmission of the OFDM symbols, ϕu2 and ϕb1 can be set to 0. Tn1 and Tn2 are known. Hence, the propagation delay τ can be determined within an integer ambiguity, the handling of the integer ambiguity is further discussed in section 2.1.3

Figure 3: Clock biases using round-trip carrier phase method.

A simplified block diagram of a transceiver is shown in Figure 4. A single oscillator can be used for the transmitter and the receiver. This makes the initial phase of the transmitter and receiver within the same device the same or have a fixed offset, where the fixed offset accounts for any phase shift introduced by the amplifiers, filters and antenna arrays. This fixed offset can be determined by calibration of the device. In the above equations, this fixed offset can be reflected as an additional fixed offset in ϕb1 for the base station and an additional fixed offset in ϕu2 for the UE.

Figure 4: Simplified block diagram of transceiver.

Proposal 4: Study the use of round-trip carrier phase measurement to eliminate UE and gNB clock biases. The round trip carrier phase measurement is the sum of the DL carrier phase measurement at the UE and the UL carrier phase measurement at the gNB.

### Integer Ambiguity

A well know problem with the carrier phase measurement is integer ambiguity. The carrier phase method measures the carrier phase modulo 2π, where the carrier phase is in the range of 0,2π or -π,π. Increments of 2π provide the same phase measurement, i.e., phase values  φ±2πN are all measured as the same phase value, wherein N=0, 1,2,…. In this section we study three ways to resolve the integer ambiguity:

- Coarse measurement using other positioning methods to estimate N

- Using multiple carriers to increase the period (periodic time) of a virtual carrier frequency used to measure the carrier phase.

- Using the slope of the carrier phase measurement relative to frequency with multiple sub-carrier measurements.

#### Carrier phase measurement combining with existing methods

The UE’s position can be estimated using Rel-16/17 positioning techniques, which can provide a coarse accuracy of the UE’s position (e.g., an accuracy in the range of 1 to 3 meters). If the carrier has a frequency fc and a periodicity Tc=1fc. The propagation time from device A to device B as measured using legacy positioning methods can be expressed as:

Tp-mlegacy=N.Tc+Tf+e1

Wherein, N is the number of complete cycles the signal traversed when travelling from the transmitter to the receiver, N can’t be measured directly by the carrier phase measurement due to the integer ambiguity.  Tf corresponds to a time of a partial cycle Tf <Tc and e1 is the time of positioning measurement error by legacy-based positioning methods, if accuracy of the positioning measurement is 1 to 3 meters, e1=3.3-10 ns. When using the carrier-phase method, the number of cycles could be estimated from the legacy-based positioning measurement, the carrier-phase method can more accurately estimate Tf. Using the carrier-phase method the time of arrival has a refined accuracy of

Tp-mCP=N.P+Tf+e2

Where, e2≪e1, thanks to higher accuracy of the carrier phase measurement – as mention earlier, the carrier phase measurement can have an accuracy of 0.01 to 0.05 carrier cycles. For this method to work well evenly in a single measurement basis, e1 should be less than Tc2, this would imply a carrier frequency of 50 MHz or less. This is well below the carrier frequencies used in cellular systems. Generally speaking, if the coarse estimate of Tp is accurate enough to resolve the integer ambiguity the carrier phase method can be further used to improve the accuracy of Tp.

Observation 2: If the coarse position estimate of the legacy-based positioning methods is accurate enough, the carrier phase measurement can be used to further improve the positioning accuracy.

Proposal 5: Study using legacy-based positioning measurements to estimate the number of integer cycles between the gNB and the UE.

#### Multiple carrier frequencies

Now consider using two carriers for the carrier phase measurement. For the first carrier at frequency fc1, the measured carrier phase is with integer ambiguity N1, and propagation delay Tp is

φ1=2πfc1Tp-2πN1

For the  second carrier at frequency fc2, the measured carrier phase is with integer ambiguity N2, and propagation delay Tp is

φ2=2πfc2Tp-2πN2

Subtracting φ2 from φ1 we get

φ1-φ2=2πfc1-fc2TP-2πN1-N2

This creates a virtual carrier frequency with frequency fcv=fc1-fc2, and virtual phase φv=φ1-φ2.  If fc1 and fc2 are close in value, fcv≪fc1 and fcv≪fc2. Hence, the tolerance (error) of the legacy-based positioning methods can be less than half the period of the virtual carrier allowing the traditional positioning method to estimate the number of cycles for the virtual carrier.

Proposal 6: Study using multiple carriers to determine a phase of a virtual carrier with a lower virtual frequency.

Alternatively, the multiple carriers used are in different frequency bands or frequency ranges. By using a long-wavelength frequency, the carrier-phase method could be more robust to the phase jumping and interruption while traveling. Thus the integer number of complete wavelength obtained could be more accurate. However, the accuracy of the partial wavelength estimation is worse due to the larger distance corresponding to a phase unit. By using another short-wavelength frequency, we can accomplish a finer partial wavelength estimation. Thus with a combination of both long wavelength and short wavelength, we get a better carrier phase measurement result.

Observation 3: Using a combination of both long wavelength and short wavelength, we get a better carrier phase measurement result.

#### Slope of the carrier phase measurement relative to frequency

The positioning reference signals, defined in Rel-16, e.g., DL PRS (clause 7.4.1.7 of TS 38.211) and pos-SRS (clause 6.4.1.4 of TS 38.211) have multiple sub-carrier. The phase measurement across the multiple sub-carriers can be exploited to eliminate the integer ambiguity. The DL carrier phase measurement at the UE can be given by:

Δφue=ϕu0+Tn1+Tp+δtb-δtu2πfc-ϕb1 mod 2π

Δφue=ϕu0+Tn1+Tp+δtb-δtu2πfc-ϕb1+2πN1

Taking the derivative with respect to the carrier frequency eliminates the integer ambiguity.

ddfcΔφue=Tn1+Tp+δtb-δtu2π

Tn1 is the time between the reference time and the DL PRS symbol. Tp is the propagation delay to be estimated. δtb1 and δtu are the clock biases of the gNB and UE respectively. These are unknown and can be eliminated using one of the methods described in section 2.1.2.

To measure ddfcΔφue, the phase for each subcarrier can be calculated and then the slope of the best line that fits the data can be estimated. It should be noted that the phase should be unwrapped to avoid any 2π discontinuities. Figure 5 illustrates an example of getting the carrier phase slope by measuring the sub-carrier phase for each sub-carrier of the DL positioning reference signal or the positioning sounding reference signal.

Figure 5: Carrier phase slope.

Proposal 7: Study using the slope of the carrier phase measurement relative to frequency with multiple sub-carrier measurements to eliminate the integer ambiguity.

### Phase Centre Offset

Based on the agreement from RAN1#110, the carrier phase centre offset can be model as:

dPCO =  a * dPhi + w

where, dPhi is the difference between two directions, and dPCO is the corresponding phase center offset difference. “a” is a scale factor that can be 0, 1 or 3. “w” is random variable, that is 0 in the idea case.

The phase center offset in general is not a linear function as illustrated in Figure 6. Therefore, the model provided by the last equation is an approximate model when dPhi is small enough and the value of “a” is not constant, but depends on the direction.

Figure 6: Example of phase centre offset.

To illustrate the impact and mitigation of the phase centre offset, let’s revisit the double difference method described in second 2.1.2.1, as explained the phase difference measured by the UE for gNB1 is given by:

Δφue1=ϕu0+Tn1+τ1+δtb1-δtu2πfc-ϕb1 mod 2π

This equation assumed no phase centre offset. Now, we add the phase centre offset, for array arriving at angle θ1 close to a reference direction θ0, the phase centre offset is model as aθ1-θ0+w. Hence, the phase difference of gNB1 measured at the UE becomes:

Δφue1=ϕu0+Tn1+τ1+δtb1-δtu2πfc-ϕb1 mod 2π+aθ1-θ0+w

Similarly, for a base station gNB2 with a ray arriving at angle θ2 that is close to the reference direction θ0, the measured phase difference of gNB2 at the UE taking into account the phase centre offset is given by:

Δφue2=ϕu0+Tn2+τ2+δtb2-δtu2πfc-ϕb2 mod 2π+aθ2-θ0+w

To calculate the single difference carrier phase between gNB1 and gNB2, we get:

Δφue12-sd=Δφue1-Δφue2=Tn1-Tn2+τ1-τ2+δtb1-δtb22πfc-ϕb1+ϕb2 mod 2π+aθ1-θ2

The impact of centre phase offset on the single difference carrier phase is evident in the term: aθ1-θ2

Now we want to eliminate the effect of the centre phase offset. Consider a third base station gNB3 with a ray arriving at angle θ3 that is close to the reference direction θ0, the measured phase difference of gNB3 at the UE taking into account the phase centre offset is given by:

Δφue3=ϕu0+Tn3+τ3+δtb3-δtu2πfc-ϕb3 mod 2π+aθ3-θ0+w

Now let’s calculate the single difference carrier phase between gNB3 and gNB2, we get:

Δφue32-sd=Δφue3-Δφue2=Tn3-Tn2+τ3-τ2+δtb3-δtb22πfc-ϕb3+ϕb2 mod 2π+aθ3-θ2

This equation too has the effect of the centre phase offset evident in the term: aθ3-θ2

To eliminate, the effect of the centre phase offset, we calculate a second single difference carrier phase as follows:

Δφue-2sd=Δφue12-sdθ1-θ2-Δφue32-sdθ3-θ2

Therefore,

Δφue-2sd=Tn1-Tn2+τ1-τ2+δtb1-δtb22πfc-ϕb1+ϕb2 mod 2πθ1-θ2-Tn3-Tn2+τ3-τ2+δtb3-δtb22πfc-ϕb3+ϕb2 mod 2πθ3-θ2

The effect of the phase centre offset has been eliminated from the last equation. The condition for this is that rays from gNB1, gNB2 and gNB3 are sufficiently close in the spatial domain, to allow the phase centre offset to be modelled linearly. The UE can select gNBs that satisfy this criterion.

A second single difference carrier phase can be calculated for a PRU (using the angles of the UE). A second double difference carrier phase is calculated between the UE and the PRU, that eliminates the clock biases of the gNBs that are used to calculate the position of the UE using the carrier phase measurement to get a more accurate measurement.

The carrier phase method can also be used to calculate the angle of rays coming from each gNB as described in section 2.2.

Proposal 8: Study methods to mitigate the effect of the phase centre offset, e.g., measuring carrier phase of 3 gNBs with rays that are sufficiently close in the spatial domain at the UE.

### NLOS/multi-path condition

Multi-path reflections and NLOS conditions are a challenge to positioning methods and measurement. This is especially true for carrier phase method where a small change in the propagation path (e.g., 5 cm for 3 GHz carrier) causes a phase shift of π radians. Cellular networks, especially indoor cases have a rich multi-path environment which would degrade the positioning accuracy.

Observation 4: NLOS conditions and multi-path reflections impact the accuracy and usability of the carrier-phase method.

Since the carrier phase based methods/measurements are sensitive to the NLOS or multiple path environment, the criteria under which the carrier phase method can be used should be carefully studied.

One possible criteria is when a NLOS condition or multiple path condition exists, such methods/measurements (i.e., based on the carrier phase method) shall not be used. In Rel-17 positioning enhancement, the NLOS indicator is introduced for UE to help gNB identify current channel condition is LOS or NLOS. Both hard value (0 or 1) and soft value [0:0.1:1] is introduced. So we could further decide whether or not to use the CP method based on such indicator. Alternatively, the indicator for multipath could also be considered.

Another possible criteria is that, even under multipath environment, there is still some possibility to use CP methods. It is well known that, the estimated phase value (e.g., via the subcarrier after IFFT) is highly dependent on the strongest path (in RSRP perspective). Thus, if the first detected path is also the strongest path, and/or much higher than the second (strongest) path, the estimated phase value could still represent the propagation delay properly. As shown in the simulation results, when applying multi path mitigation, the phase estimation results could be improved in some cases, thus the accuracy of determining the first path is quite important to get good performance.

Proposal 9: Study the condition for applying CPP method based on the NLOS and multi-path including first path detection.

## Using carrier phase difference for angle estimation

A single ray of a radio wave propagating in space can be described by the wave equation:

ψt,r=Acoswt-k∙r+φ

Where,

- t is time of the wave propagated.

- r is the position vector.

- w is the angular frequency, related to wave frequency f by; w=2πf.

- k is the wave vector, which is related to the wave number k by k=k. k is related to the wavelength of the wave, λ, by k=2π/λ.

- A is the amplitude of the wave, in general can be a function of time t and position.

- φ is the initial phase given at t=0 and r=0.

- φ-k∙r can be consider as the initial phase (at t=0) at position r.

By measuring phase of the wave at a time t, it is possible to determine distance related information (i.e., k∙r). In this section we explore phase difference between two rays and how they can be used to determine positioning information.

- If two signals are transmitted from two antenna elements of an antenna array separated by distance L, as illustrated in Figure 5. In the far field the phase difference between the two rays depends on the angle of departure, i.e., angle between the rays and the antenna array (or the angle between the rays and normal to the antenna arrays). If the angle between the normal to the antenna array and the ray is θ, the phase difference between the two rays is 2πLλsinθ. By measuring the phase difference between the two transmitted rays the angle of departure can be calculated.

Figure 7: Phase difference between two transmitted rays.

- If a signal is transmitted from a transmit antenna is received by the two antenna elements of an antenna array, as illustrated in Figure 6, the phase difference between the two rays depends on the angle arrival θ.  Angle θ is the angle between the ray and the normal to the antenna array. When travelling far from the transmit antenna, radio waves that travel parallel to each other, as shown in Figure 6, the wave front (plane of equal phase) is a plane perpendicular to the direction of the wave. If the radio waves are incident on two elements (Rx0 and Rx1) as shown in Figure 6, the radio wave first reaches Rx0, then Rx1. To reach Rx1, the wave travels an extra distance Lsinθ. Hence, there is a phase lag at Rx1 compared to Rx0 of  2πλLsinθ. If the signal S0 at Rx0 is S0=A0cosφ0, the signal at Rx1 is S1=A1cosφ1=A1cosφ0-2πλLsinθ. Therefore, by measuring the phase of the received signal at Rx0 and comparing to the phase of the received signal at Rx1 at the same time, we can get a phase difference that depends on the direction of travel θ

θ=arcsinφ0-φ1λ2πL

Figure 8: Incidence of radio waves on a two-element antenna area.

Using the carrier phase difference measurement as described above, even if there is synchronization phase mismatch, it is cancelled when subtracting the carrier phase measurement of the first ray from the carrier phase measurement of the second ray, as both rays are transmitted from the same device and received by the same device.

Using the carrier phase difference measurement as described above, and with separation between the antenna elements of L≤λ/2, there is no integer ambiguity issue as the maximum phase between rays of two antenna elements is ±π.

Using the carrier phase difference measurement as described above, the same consideration for the link condition (LOS vs NLOS) as described in section 2.1.4 apply.

Proposal 10: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of arrival at the UE.

Proposal 11: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of departure at the UE or the gNB.

As a result of the UEs mobility, the position of the UE changes over time as shown in Figure 7. While this can cause a challenge when trying of find the positioning of the UE, it can also be leveraged to aid in positioning. If the positioning reference signal occurs every T seconds, and the UE is moving at a speed v, the distance between two carrier phase measurements is d=v∙T. For example, a UE moving at a speed of 108 km/h (about 67.5 mph), or 30 m/s, with a subcarrier spacing of 30 kHz, i.e., the slot duration is Ts=0.5 ms, and if the number of slots between consecutive positioning reference signal transmissions is 4 slots, the distance the UE moves between corresponding measurements is d=30∙4∙0.5=60 mm, this represents λ/2 for a wave with frequency 2.5 GHz. The presence of the UE at different positions over time creates a virtual antenna array that can be leveraged for phase difference measurements and accordingly calculating the direction of travel of the radio wave.

Figure 9: Virtual antenna array due to UE's mobility.

Proposal 12: Study and evaluate the use of the UEs mobility to create a virtual antenna array.

# Evaluation Methodology

In this section we present evaluation results for the round trip carrier phase method using the carrier phase measurement slop to eliminate the integer ambiguity. Three cases are simulated:

- Continuous sub-carrier allocation (comb of 1) with 24 PRBs (Comb=1). The same sub-carrier is used in all 12 OFDM symbols allocated to the positioning reference signal (DL PRS or pos-SRS).

- Comb of 4 with 24 PRBs. No sub-carrier offset hopping between symbols (Comb=4A). The same sub-carrier is used in all 12 OFDM symbols allocated to the positioning reference signal (DL PRS or pos-SRS).

- Comb of 4 with 24 PRBs (Comb=4B). With sub-carrier offset hopping between symbols as described in 38.211

Figure 10:CDF of Distance Error

Figure 8 illustrates the benefit of continuous sub-carrier allocation over a comb structure. By having a continuous comb allocation, we have more sub-carriers allowing for a more accurate calculation of the slop. Figure 8 illustrates the benefit of not having frequency offset hopping between symbols to have more accumulation for the same sub-carrier. Based on these results, it would beneficial to have continuous allocation of sub-carriers for carrier phase measurement.

Observation 5: Round-trip carrier phase derivative with respect to carrier frequency can provide centimetre level accuracy.

In addition, the performance of phase estimation by using time domain, frequency domain, and multipath mitigation methods is evaluated, and the simulations are performed in InF-SH scenario.

Figure 11: illustration of phase estimation method

| Phase estimation | CDF@70% | CDF@80% | CDF@90% |
| --- | --- | --- | --- |
| Frequency domain | 0.33 radian | 0.56 radian | 2.3 radian |
| Time domain | 0.02 radian | 0.06 radian | 1.9 radian |
| Multipath mitigation | 0.07 radian | 0.14 radian | 0.45 radian |

As we can see, the performance of frequency domain method is the worst this is because the multiple path has strong distortion in the phase estimation in the F domain as shown in following figure, thus even with some smooth or averaging, the estimated phase is still worse than the other two. However, the other two (time domain/mitigation method) are highly dependent on the first path detection. In case LOS path exists, the time domain method could outperform others and mitigation methods could provide outstanding performance when the LOS path possibility is lower.

Fig. 12 the distortion caused by multiple path effect.

Observation 6: The phase estimation in frequency domain is seriously degraded by multi path effect.

Observation 7: Multi-path mitigation method could provide better phase estimation when LoS path probability is low.

For performance evaluation considering different error sources, the following simulations are presented for

- Case 1: phase error (offset) between TRP and UE is with 1% or 10% of 2*pi as error;

- Case 2: with ARP error with +/-2 cm in each of the x,y,z;

(a) 1% of phase error                                             (b) 10% of phase error

Fig13: performance of phase error with 1% and 10% of 2*pi;

Fig14: performance of ARP error at +/-2cm

As we can see, the 1% error can still be tolerable for up to 65% of UEs, and the 10% error is even worse than the DL-TDOA method, which means the error of the carrier phase positioning is so bad that CPP is worsening the location estimate of legacy methods instead of improving it. For ARP errors, given the wavelength is around 8cm at the 3.5Ghz then 2 cm is actually 25% of 2pi, so worse performance could be expected.

Observation 8: With larger phase error and ARP error, the performance is degraded severely.

# Conclusions

The following observations and proposals have been made regarding the carrier phase method

Proposal 1: Study the use of Rel-16 DL and UL positioning reference signals with no or small Comb for the carrier phase method.

Proposal 2: Study the use of Rel-16 DL and UL positioning reference signals using the same frequency offset (k') across OFDM symbols of a slot for the carrier phase measurement.

Observation 1: PRU can be considered for elimination of gNB and UE clock biases, but at the expense of additional network topology complexity.

Proposal 3: RAN1 to study how to provide PRU or PRU-like functionality under existing system.

Proposal 4: Study the use of round-trip carrier phase measurement to eliminate UE and gNB clock biases. The round trip carrier phase measurement is the sum of the DL carrier phase measurement at the UE and the UL carrier phase measurement at the gNB.

Observation 2: If the coarse position estimate of the legacy-based positioning methods is accurate enough, the carrier phase measurement can be used to further improve the positioning accuracy.

Proposal 5: Study using legacy-based positioning measurements to estimate the number of integer cycles between the gNB and the UE.

Proposal 6: Study using multiple carriers to determine a phase of a virtual carrier with a lower virtual frequency.

Observation 3: Using a combination of both long wavelength and short wavelength, we get a better carrier phase measurement result.

Proposal 7: Study using the slope of the carrier phase measurement relative to frequency with multiple sub-carrier measurements to eliminate the integer ambiguity.

Proposal 8: Study methods to mitigate the effect of the phase centre offset, e.g., measuring carrier phase of 3 gNBs with rays that are sufficiently close in the spatial domain at the UE.

Observation 4: NLOS conditions and multi-path reflections impact the accuracy and usability of the carrier-phase method.

Proposal 9: Study the condition for applying CPP method based on the NLOS and multi-path including first path detection.

Proposal 10: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of arrival at the UE.

Proposal 11: Study and evaluate the performance of carrier-phase difference measurement for estimating the angle of departure at the UE or the gNB.

Proposal 12: Study and evaluate the use of the UEs mobility to create a virtual antenna array.

Observation 5: Round-trip carrier phase derivative with respect to carrier frequency can provide centimetre level accuracy.

Observation 6: The phase estimation in frequency domain is seriously degraded by multi path effect.

Observation 7: Multi-path mitigation method could provide better phase estimation when LoS path probability is low.

Observation 8: With larger phase error and ARP error, the performance is degraded severely.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, 3GPP RAN#94e, e-Meeting, December, 2021.

- Pratap Misra, and Per Enge, Global Positioning System: Signals, Measurements, and performance, Ganga-Jamuna Press, 2012.

# Agreements from prior meetings

## A.1	Agreements from RAN1#109-e

Agreement

NR carrier phase positioning performance will be evaluated at least with the carrier phase measurements of a single measurement instance.

Agreement

The impact of integer ambiguity on NR carrier phase positioning and potential solutions to resolve the integer ambiguity will be studied in the SI.

Agreement

The study of the accuracy improvement based on NR carrier phase measurements in Rel-18 SI may include:

- UE-based and UE-assisted carrier phase positioning,

- UL carrier phase positioning and DL carrier phase positioning.

- NR carrier phase positioning with the carrier phase measurements of one carrier frequency or multiple frequencies

- Combination of NR carrier phase positioning with another standardized Rel. 17 positioning method, e.g., DL-TDOA, UL-TDOA, Multi-RTT, etc.

- Note: The use of “carrier phase positioning” does not necessarily mean it is a standalone positioning method

- FFS: whether SL carrier phase positioning is to be discussed in Rel-18 SI

Agreement

- The impact of multipath for the carrier phase positioning will be evaluated during the SI

- The methods of mitigating the impact of multipath for the carrier phase positioning will be studied during the SI, if it is considered to be necessary after the evaluation.

R1-2205164	FL Summary for improved accuracy based on NR carrier phase measurement	Moderator (CATT)

Agreement

- Reuse the simulation assumptions of NR Rel-16/17 for carrier phase positioning

  - Note: Optional modification of the simulation assumptions defined in NR Rel-16/17 are allowed only if needed.

- The evaluation scenarios:

  - Baseline: InF-SH, InF-DH

  - Optional: IOO, Umi, Highway

    - Note 1: Other evaluation scenarios are not precluded.

    - Note 2: Existing Rel-17 DL/UL reference signals in Uu interface is to be used for the Highway scenario.

- Frequency range:

  - Baseline: FR1

  - Optional: FR2

Agreement

- In addition to the evaluation assumptions of NR Rel-16/17, the following error sources may also be considered during the evaluation:

  - Phase noise (FR2)

  - CFO/Doppler

  - Oscillator-drift

  - Transmitter/receiver antenna reference point location errors

  - Transmitter/receiver initial phase error

  - Phase center offset

- Note: Other error sources are not precluded

- Note: UE mobility can be considered in the evaluations

- Note: one or more error sources can be evaluated jointly

- Note: companies should provide the error sources model with their evaluations

Agreement

- For the purposes of discussion, for NR downlink and/or uplink carrier phase positioning, the carrier phase (CP) at a RF frequency at a receiver is a phase that is a function of the signal propagation time from an Tx antenna reference point of a transmitter (e.g., a TRP or a UE) to a Rx antenna reference point of the receiver (e.g., a UE or a TRP).

  - The propagation time can be expressed in a fractional part of a cycle of the RF frequency and a number of integer cycles, but the CP may be independent of the number of integer cycles.

Agreement

The use of PRUs to facilitate NR carrier phase positioning can be evaluated in the SI by RAN1.

## A.2 Agreements from RAN1#110

Agreement

Endorse the templates in section 17 under (H)(Round 1) Proposal 17-1 in R1-2207690 to collect carrier-phase based positioning simulation results, with the following notes:

- The TR editor can adjust the sections/sub-sections arrangement

- Adjust the titles of the tables to refer to NR carrier-phase based positioning

- The detailed rows of the tables can be further discussed

R1-2207691	FL Summary #2 for improved accuracy based on NR carrier phase measurements	Moderator (CATT)

Agreement

In the evaluation of NR carrier phase positioning, the following frequency errors can be considered, which are modeled independently for each UE and each TRP:

  - Initial Residual CFO (is the same for one measurement instances [or multiple phase measurement instances]):

    - Ideal: 0 (UE/TRP)

    - Practical: uniform distribution within

        - [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE),

        - [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE),

        - [-10, +10] Hz (for each TRP, FR1),

        - [-40, +40] Hz (for each TRP, FR2).

  - Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix):

    - Ideal: 0 (UE/TRP)

    - Practical: uniform distribution within [-0.1, 0.1] ppm (UE), [-0.02, +0.02] ppm (each TRP) within measurement duration

- Note: The Doppler frequency can be determined based on the UE speed in the evaluation assumption.

Agreement

In the evaluation of NR carrier phase positioning, the offset between the initial phase of the transmitter and the initial phase of the receiver can be modeled as a random variable uniformly distributed within [0, X].

- Possible values of X: 2pi

  - Other values FFS

Agreement

In the evaluation of NR carrier phase positioning, the antenna reference point (ARP) location error of a TRP can be modeled as follows:

  - Ideal: no ARP error

  - Practical: a zero-mean, truncated Gaussian distribution with zero mean and standard deviation of T=[1, 5] cm truncated to 2T in each of (x, y, z) direction

R1-2208206	FL Summary #3 for improved accuracy based on NR carrier phase measurements	Moderator (CATT)

Agreement

In the evaluation of NR carrier phase positioning, the following the UE/TRP antenna phase center offset (PCO) model can be considered as the starting point:

dPCO =  a * dPhi + w

where

  - a is the scale factor, a=[0, 1, 3]

    - FFS: other values

  - dPhi is the direction difference (in degrees):

    - Example 1, dPhi is the difference between the true and the calculated (or measured) directions between a transmitter (UE/TRP) and a receiver (TRP/UE).

    - Example 2: dPhi is the direction difference between one UE to two TRPs, or between one TRP to two UEs.

  - w is 0 or a random variable uniformly distributed within [-2, +2], or [-5, +5], or [-X, +X] degrees

    - FFS: value of X or left up to companies

  - Note: the above model is valid only when absolute value of dPhi < Y degrees

    - FFS: value of Y or left up to companies

Agreement

For the evaluation of NR carrier phase positioning, UE position can be calculated by the use of the carrier phase measurements obtained at the M sequential time instances, where

  - Baseline:

    - M=1

  - Optional :

    - M=4

    - Other values of M

  - Companies should report their assumptions on UE mobility (e.g. speed)

Agreement

Further evaluate the following multipath mitigation methods for the carrier phase positioning, which include, but are not limited to, the following:

  - The methods of estimating the carrier phase of the first path

    - Note: Both time-domain and frequency-domain methods can be considered

  - LOS/NLOS/ Multi-path indication for the carrier phase measurements for improving the accuracy of the position calculation

    - Rel-17 LOS/NLOS indicator can be used as the starting point

  - measurements of the first path and additional paths

    - E.g. carrier phase measurements, timing measurements

  - other channel information, such as RSRP/RSRPP, CIR/CFR, etc.