3GPP TSG RAN WG1 #110bis-e	        	           R1-2209742

e-Meeting, October 10th – 19th, 2022

Agenda item:		9.7.1

Source:			Samsung

Title:			NW Energy Savings Performance Evaluation

Document for:	Discussion and decision

- Introduction

This contribution discusses aspects related to energy consumption modelling, evaluation methodology, and evaluation results for NW energy saving techniques.

- Energy Consumption Modelling

  - BS power consumption model

During the RAN1#110 meeting [1], the following agreements for energy consumption modelling have been agreed for evaluation of NW energy saving techniques.

Agreement For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set. - FFS: One or multiple values for relative power and transition time. Power state; Characteristic; Relative Power; Additional transition energy3; Total transition time. Deep sleep1; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state.; P1=1; E1; T1. Light sleep; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1); P2; E2; T2. Micro sleep; There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state.; P3; 0; 0. Active DL; There is only DL transmission.; P4; NA; NA. Active UL; There is only UL reception. FFS: Whether multiple P5 values are needed to address low power UL mode; P5; NA; NA. Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined.. - For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study. - FFS: Optionally, a state machine where BS may transit between sleep modes without entering non-sleep mode can be considered. Companies are to report the involved sleep modes and the assumptions for inter-sleep mode transition time used in their evaluations. - FFS: Details on how to use the above table for low power uplink reception (e.g. for WUS).

As stated in the above agreement, several power states were supported, and the BS power consumption aspects of each power states, like relative power, total transition time and additional transition energy, were further investigated over different reference configurations. In terms of relative power and total transition time, the following values have been discussed for reference configuration set 1, set 2 and set 3.

Agreement For non-sleep mode, the relative power value in power model table for UL reception and/or DL transmission is provided based on reference configuration. Working Assumption For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1: 50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A.. Agreement - The total transition time for set 2 and set 3 is the same as that for set 1. Companies are encouraged to check the input and values provided in section 2.1.4.2 of R1-2208312 for further determination. Proposed working assumption 2.1.4.2-1 - FFS: The relative power values for reference configuration set 2 and set 3 is Power state; Relative Power P for Category 1; Relative Power P for Category 2. Set 2; Set 3; Set 2; Set 3. Deep sleep; 1; 1; 1; 1. Light sleep; 23; 20; 2.6; 1.8. Micro sleep; 50; 38; 5; 3. Active DL; 240; 152; 40; 8.4. Active UL; 90; 80; 5.8; 4.2. The total transition time for set 2 and set 3 is the same as that for set 1.

Given the above relative power values, both categories have difference tendency between the relative power of micro sleep and the relative power of active UL. In general, the difference between micro sleep and active UL is whether to include the power consumption of low-noise amplifier (LNA), which takes small portion of total power consumption. In order to consider practical BS power consumption of active UL, the relative power and total transition time of category 2 were reasonable, but the category 1’s relative power gap between micro sleep and active UL seems too high. Therefore, for evaluation, we support the relative powers of category 2 for all sets and total transition times of both category 1 and category 2 for all sets. Regarding the relative powers of category 1, we would like to further clarify the relative power of active UL for all sets, other state’s relative powers in category 1 seems appropriate.

Observation 1: Relative powers of active UL in Category 1 should be further investigated in order to reflect the proper BS power consumption.

Proposal 1: Support the relative powers for Category 2 and total transition time for Category 1 and Category 2 for reference configuration set 1.

- For reference configuration set 1, the values are provided as below.

| Power state | Relative Power P | Total transition time T |  |
| --- | --- | --- | --- |
| Deep sleep | 1 | Cat 1: 50ms | Cat 2: 10s |
| Light sleep | Cat 2: 2.1 | Cat 1: 6 ms | Cat 2: 640 ms |
| Micro sleep | Cat 2: 5.5 | 0 | 0 |
| Active DL | Cat 2: 32 | N.A. | N.A. |
| Active UL | Cat 2: 6.5 | N.A. | N.A. |

Proposal 2: Support the relative powers for Category 2 for reference configuration set 2 and set 3 in R1-2208312.

- For reference configuration set 2 and set 3, the values are provided as below.

| Power state | Relative Power P for Category 2 |  |
| --- | --- | --- |
|  | Set 2 | Set 3 |
| Deep sleep | 1 | 1 |
| Light sleep | 2.6 | 1.8 |
| Micro sleep | 5 | 3 |
| Active DL | 40 | 8.4 |
| Active UL | 5.8 | 4.2 |

In addition, there were some discussions to determine the proper values for the additional transition energy in [2] as below.

Proposal 2.1.3-1: The additional transition energy Ei represents the energy that BS enters from non-sleep mode to a sleep mode i and BS leaves the same sleep mode to non-sleep mode. For evaluation purpose, it is calculated as - Ei≈12×∆×Ti where - ∆ is the difference of the relative power between sleep mode i and micro sleep - Ti is the corresponding total transition time of sleep mode i, which is a two-way time, assuming no inter-sleep state transition. Proposal 2.1.3.2-1-rev1: - During the transition time period, relative power of sleep mode i is assumed to be consumed. Additional transition energy and total transition time also include energy and time for both ramping down and ramping up spent in two-way (ramping down and up) during the transition period is considered. - (Working Assumption) for set 1, the additional energy (unit in relative power*(duration in ms)) is Power state; Additional transition energy Ei. Category 1; Category 2. Deep sleep; 1350; 22500. Light sleep; 90; 1088. Agreement During the transition time period, relative power of sleep mode i iis assumed to be consumed. Additional transition energy and total transition time also include energy and time for both ramping down and ramping up

During the discussions, some approaches have been proposed to calculate the additional transition energy. For evaluation purpose, we believe it would be better to simplify the calculation method, so it can be calculated with difference of the relative powers between sleep mode and non-sleep mode and total transition time used in UE power saving. Hence, we suggest the following additional transition energy for each set.

Proposal 3: Define the additional transition energy for set1, set2 and set3 as follow:

- For set 1, the additional energy (unit in relative power*(duration in ms)) is

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 |
| Light sleep | 90 | 1088 |

- For set 2, the additional energy (unit in relative power*(duration in ms)) is

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1225 | 20000 |
| Light sleep | 81 | 768 |

- For set 3, the additional energy (unit in relative power*(duration in ms)) is

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 925 | 10000 |
| Light sleep | 54 | 384 |

  - BS power consumption scaling for adaptation.

For evaluation of network energy saving, another aspect for BS power consumption modeling is the scaling of average power when considering adaptation of resources in different domains. In the previous post-email discussion, the details for scaling have been discussed as follow.

Proposed conclusion: Using the below as a base for further discussion/determination of scaling details in the next meeting. Other alts may still be considered. Revised Alt 1-update: At least for FR1 TDD, - the BS power consumption for active DL is provided by - P= Pstatic+Pdynamic - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on - Category 1: - [55, Huawei] - [140, CATT] - Category 2: - [5.5, Huawei] - [1.79, Nokia] - [16, CATT] - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on sa*Pdyn,ante+sf*spηsf, sp*Pdyn,joint, where - Pdyn,ante is - Category 1: - [0, Samsung] - [57, Huawei] - [110, Intel] - Category 2: - [0, Samsung] - [7.3, Huawei] - [1.62, Nokia] - [12, CATT] - Pdyn,joint is - Category 1: - [225, Samsung] - [84, Huawei] - [115, Intel] - [30, CATT] - Category 2: - [26.5, Samsung] - [9.6, Huawei] - [10.6, Nokia] - [4, CATT] - ηsf, sp is the PA efficiency - For initial evaluations, η= - [0.34, Samsung, Nokia, CATT] - [0.5, Huawei] - [1, Intel, CATT] - The Pdyn,ante and Pdyn,joint should be reported along with η, which may not be perfectly the candidate values in the current list - FFS whether/how to use a non-linear function to represent η. - sa, sf, sp is the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively. - FFS: the BS power consumption for active UL is provided by - P=PstaticUL+sa*PdynamicUL - PstaticUL=Pstatic Revised Alt 3 - The BS power consumption for active DL is provided by Psa,sf,sp=P3+sasfspηsf, spP4-P3 - P3 and P4 are relative power values of micro sleep and active DL transmission, respectively - sf is the resource usage ratio in frequency domain - sp is ratio of simulated DL power level per TxRU between the DL transmission and reference configuration - sa is percentage of active TxRUs with respect to the reference configuration - ηsf, sp is the ratio between a reference PA efficiency and actual PA efficiency depending on the actual transmit power and actual frequency domain usage. - Companies report how to model ηsf, sp. - Note: modelling ηsf, sp as a single value for all different power and frequency resource allocation values is not realistic. - FFS: BS power consumption for active UL - Notes, - In time domain, - when slot level model is provided, a time domain scaling factor is linearly applied on Pdynamic, if applicable, or on P, according to - (1-alpha)*P3 + alpha*P4 where alpha represents the ratio of the number of active DL symbols within a slot to the number of symbols within a slot - The symbol without active DL is to be treated as micro sleep. - Companies to describe how to scale for symbols with different frequency domain allocations. - If an explicit symbol level model is provided, scaling is not applied - (Already agreed) system simulation evaluations can be per slot regardless of detailed approach for calculating symbol-level power consumption - In frequency domain, for at least inter-band CA, the total power consumption of BS is calculated as the sum of the power consumption of each cell - For intra-band CA, a scaling factor of [0.75] is assumed on Pdynamic - In spatial domain, for M-TRP at least with separate RF chains, the total power consumption of BS is assumed as the sum of the power consumption of each TRP.

For evaluation of network energy saving, the potential solution will be evaluated on all domains in [3]. To acquire the energy saving gain, parameters in the draft proposal, like bandwidth, the number of active TxRUs, the number of symbols and PSD, can be adapted and reduced, and the BS power consumption changed by the adaptations should be defined to calculate the energy saving gain. To reflect the BS power consumption with the practical network architecture, it’s necessary to define the power consumption of a static part, which is no affected by adaptation, and the power consumption of a dynamic part, which is scaled by adaptation. Hence, for the scaling aspects, we can go with revised Alt 1 in proposed conslusion.

For power consumption of the static part Pstatic, it would be same as power consumption in micro sleep (i.e., P3). Based on the definition of micro sleep which is non-sleep mode without any traffic, P3 should be the one correctly representing the power consumption of the static part. So, we propose Pstatic is same as P3.

For power consumption of the dynamic part Pdynamic, the total power consumption of Pstatic and Pdynamic should be equal to the power consumption of active DL or UL. Therefore, Pdynamic can be calculated as P4 – P3 for DL or P5 – P3 for UL. Regarding the details of Pdynamic, it may consist of sa*Pdyn,ante+sf*spηsf,  sp*Pdyn,joint to consider each scaling parameters in all domain. Based on the measured power consumption of practical network with 64T64R, the following concrete values and scaling formula were suggested:

The scaling of BS power consumption for set 1, - the BS power consumption for active DL is provided by - P= Pstatic+Pdynamic - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on - Category 1: 55 - Category 2: 5.5 - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on sa*Pdyn,ante+sf*spηsf, sp*Pdyn,joint, where - Pdyn,ante is - Category 1: 15 - Category 2: 1.5 - Pdyn,joint is - Category 1: 71 - Category 2: 8.5 - ηsf, sp is the PA efficiency - For initial evaluations, η= 0.34 - sa, sf, sp is the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively. - The percentage of active TRxRUs sa: {1 for 64 TRxRUs, 0.67 for 32 TRxRUs, 0.67^2 for 16 TRxRUs, 0.67^3 for 8 TRxRUs} - The ratio of RF bandwidth sf: 0.35+0.65* Xf /100MHz, Xf [MHz]= {10, 20, 40, 50, 80, 100} - The ratio of PSD per TxRU between the DL transmission and reference configuration sp: 0.67^(Xp /3[dB]), Xp = {0, 3[dB], 6[dB]} - the BS power consumption for active UL is provided by - P=PstaticUL+sa*PdynamicUL - PstaticUL=Pstatic - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on - Category 1: 55 - Category 2: 5.5 - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on - Category 1: FFS - Category 2: 1 - The percentage of active TRxRUs sa: {1 for 64 TRxRUs, 0.67 for 32 TRxRUs, 0.67^2 for 16 TRxRUs, 0.67^3 for 8 TRxRUs}

As stated in the above, the details can be according to the following:

- For the ratio between the RF bandwidth and the maximum system BW (i.e., sf), the scaling for adaptation on BW (e.g., Xf MHz) can be modeled as a linear interpolation from intermediate bandwidths, such as the scaling for the BW of Xf MHz can be given by 0.35 + 0.65 * (Xf) / 100, wherein the valid BW can be Xf = 10, 20, 40, 50, 80, and 100.

- For the percentage of active TRxRUs (i.e., sa), the scaling of average power in terms of the number of DL/UL antenna ports and the number of TxRU can be considered. The scaling factor for each applicable number of TX/RX antenna ports or number of active TxRUs based on 64T64R can be modeled as {1 for 64 TRxRUs, 0.67 for 32 TRxRUs, 0.67^2 for 16 TRxRUs, 0.67^3 for 8 TRxRUs}.

- For the adaptation in power domain, the scaling for adaptation on Tx power (i.e., sp) can be modeled as 0.67^(Xp/3[dB]), where the reduced PSD Xp can be 6 dB, 3 dB, and 0.

- In addition, regarding the PA efficiency ηsf,  sp, η= 0.34 is the reasonable PAE value based on practical products.

- For the adaptation in time domain, scaling of average power in terms of the number of symbols occupied per slot can be considered.

For the scaling values of set 2 and set 3 would be further discussed. However, to be simple, the above sa, sf, sp and ηsf,  sp can be reused for evaluation.

Proposal 4: Support the scaling of BS power consumption as below:

- The scaling of BS power consumption,

- the BS power consumption for active DL is provided by

  - P= Pstatic+Pdynamic

    - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on

      - Category 1: 55

      - Category 2: 5.5

    - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on sa*Pdyn,ante+sf*spηsf,  sp*Pdyn,joint, where

        - Pdyn,ante is

          - Category 1: 15

          - Category 2: 1.5

        - Pdyn,joint is

          - Category 1: 71

          - Category 2: 8.5

        - ηsf,  sp is the PA efficiency

          - For initial evaluations, η= 0.34

        - sa, sf, sp is the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively.

        - The percentage of active TRxRUs sa: {1 for 64 TRxRUs, 0.67 for 32 TRxRUs, 0.67^2 for 16 TRxRUs, 0.67^3 for 8 TRxRUs}

        - The ratio of RF bandwidth sf: 0.35+0.65* Xf /100MHz, Xf [MHz] = {10, 20, 40, 50, 80, 100}

        - The ratio of PSD per TxRU between the DL transmission and reference configuration sp: 0.67^(Xp /3[dB]), Xp = {0, 3[dB], 6[dB]}

- the BS power consumption for active UL is provided by

  - P=PstaticUL+sa*PdynamicUL

    - PstaticUL=Pstatic

    - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on

      - Category 1: 55

      - Category 2: 5.5

    - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on

      - Category 1: [FFS]

      - Category 2: 1

        - The percentage of active TRxRUs sa: {1 for 64 TRxRUs, 0.67 for 32 TRxRUs, 0.67^2 for 16 TRxRUs, 0.67^3 for 8 TRxRUs}

- Remaining Issues for Evaluation Methodology

  - KPIs

The following agreements were made for KPIs and requirement/QoS target of KPIs to identify the performance gain or loss by applying the potential network energy saving techniques.

Agreement#11 - FFS whether to set exact requirements/QoS target for UPT and/or latency impact - Other KPIs can be optionally reported, conditioned with clear definition/descriptions provided. - Note for potential new channel/signals, e.g. WUS from UE, the assumption for detection reliability at BS side is reported (performance and complexity impact would subject to results and further discussion).

During the previous meeting [1], numerous possible KPIs were considered, including energy saving gains, as well as assessing/balancing impact to user performance (e.g., UPT, latency, handover performance, initial access performance, spectral efficiency, capacity, etc.), network coverage, energy efficiency, UE power consumption, and complexity. At least, we consider the energy saving gain, latency and coverage as the most critical KPIs for fundamental network operation.

Among all the possible KPIs, we think energy saving gain (ESG) should be prioritized for evaluating the performance for an energy saving technique. The ESG can be defined as

ESG = (B-A)/B×100%,

where A is the energy consumption of gNB corresponding to a given energy saving scheme, and B is the energy consumption of gNB corresponding to the baseline operation. The baseline can be defined as always-on case, which the gNB is always available for scheduling and/or traffic processing.

In addition to ESG, two kinds of latency, which are packet latency and scheduling latency, should be considered. The increase in the packet latency can be modeled as

Packet latency increase = Lenhancement - LbaselineLbaseline×100%, and L = tdecoded - tarrival,

where the packet latency, L, is calculated as the delay between the time when a packet arrivals and the time when the packet is decoded similar as user plane latency in [4]. It provides the performance of network service in terms of how long it takes to deliver a packet. The increase in the scheduling latency can be modeled as

Scheduling latency increase = Lenhancement' - Lbaseline'Lbaseline'×100%, and L' = tscheduled - tarrival,

where the scheduling latency, L', is calculated as the delay between the time when a first packet arrivals and the time when the packet is scheduled. The scheduling latency had been used during SI for UE power saving [5]. It provides the performance impact on scheduling by applying the network energy saving techniques. Both latencies should be considered to evaluate the performance impact of network energy saving techniques. In terms of latency increase, both latency increases can be compared with range of the target/requirement to identify acceptable network energy saving techniques.

Meanwhile, a user perceived throughput loss, i.e., UPT loss, can also be considered to reflect the impact on system performance such as [6]

UPT loss = UPTenhancement - UPTbaselineUPTbaseline×100%, UPT = amount of data (file size) / time needed to download data,

where the UPT is calculated as a packet size division time needed to download data, between the time when the packet is received in the transmit buffer and the time when the last bit of the packet is correctly delivered to the receiver. Also, user throughput loss can be compared to the range of target required UPT.

Last but not least, coverage and UE power consumption should be further evaluated. The coverage is one of key factors for network deployment, so it should be involved into evaluation metrics. Coverage loss can be provided by 5-percentile UE’s latency increase or user throughput loss. Also, an adoption of network energy saving techniques should not result in a non-negligible increase in the UE power consumption.

Given above, we propose the following KPIs for evaluating NW energy saving techniques.

Proposal 5: Use the following KPIs for evaluating NW energy saving techniques:

- Energy saving gain

- Latency (packet latency and scheduling latency)

- UPT

- Coverage

- UE power consumption

Furthermore, when it comes to identify the system reliability and performance loss according to the application of network energy saving techniques, it should be compared with requirements of latency increase and UPT loss. For example, if the degraded performance is still within the requirement range, then the techniques can be identified as possible network energy saving techniques. Therefore, we suggest to further determine the range of requirements on latency increase and UPT loss for network energy saving.

Proposal 6: Define acceptable range of requirements on latency increase and UPT loss, like less than 10 %, less than 20 % and less than 30 %, for the evaluation of network energy saving techniques.

  - Simulation scenarios

The following agreement was made in the previous meeting for reference SLS configuration.

Agreement - For FR1, adopt the Reference SLS configurations in Annex-A in R1-2208312 as baseline SLS assumptions. - Other carrier frequencies can be optionally considered. - FFS For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption. - BS antenna configurations - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ) - Traffic model & UE density - Follow previous agreements with adjusted UE density - Total transmit power per TRxP - Value scaled from that in set 3 reference configuration considering BW - Further adjustment/clarification can be discussed in the next meeting.

Based on the previous agreements, FR1 reference configuration has been adopted for initial evaluation. Regarding FR2, as the FFS in the agreement, it seems reasonable to reuse the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with updated antenna configuration and Tx power. In addition, we support to use BS antenna configuration and total transmit power in set 3 reference configuration as in the agreement.

Proposal 7: Support to use the reference SLS configuration in the above agreement for FR2 evaluation.

- For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption.

  - BS antenna configurations

    - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

    - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ)

  - Traffic model & UE density

    - Follow previous agreements with adjusted UE density

  - Total transmit power per TRxP

    - Value scaled from that in set 3 reference configuration considering BW

- Preliminary Evaluation Results

  - Potential techniques in time domain solution

In this section, we provide the preliminary result of SSB adaptation. For evaluation of SSB adaptation, the following cases described in Figure 1, including multiple SSB configurations, have been assumed in conjunction with multiple sleep modes in Table 1 and Table 2.

Figure 1: Illustration of baseline scenarios for evaluation on SSB adaptation

Table 1: Evaluation assumption for SSB adaptation with relative power and total transition time for Cat 1

|  | Baseline | Reduced SSB transmission |
| --- | --- | --- |
| Scenario 0. | SSB configuration: {8 SSBs in burst, ssb-periodicity: 20 ms, relative power for a SSB: 45} Sleep mode: light sleep {relative power: 25, transition time: 6 ms, transition energy: 90} | SSB configuration: {2 SSBs in burst, ssb-periodicity: 20 ms, relative power for a SSB: 45} Sleep mode: light sleep {relative power: 25, transition time: 6 ms, transition energy: 90} |
| Scenario 1. | SSB configuration: {8 SSBs in burst, ssb-periodicity: 40 ms, relative power for a SSB: 45} Sleep mode: light sleep {relative power: 25, transition time: 6 ms, transition energy: 90} | SSB configuration: {2 SSBs in burst, ssb-periodicity: 40 ms, relative power for a SSB: 45} Sleep mode: light sleep {relative power: 25, transition time: 6 ms, transition energy: 90} |
| Scenario 2. | SSB configuration: {8 SSBs in burst, ssb-periodicity: 160 ms, relative power for a SSB: 45} Sleep mode: deep sleep {relative power: 1, transition time: 50 ms, transition energy: 1350} | SSB configuration: {2 SSBs in burst, ssb-periodicity: 160 ms, relative power for a SSB: 45} Sleep mode: deep sleep {relative power: 1, transition time: 50 ms, transition energy: 1350} |

Table 2: Evaluation assumption for SSB adaptation with relative power total transition time for Cat 2

|  | Baseline | Reduced SSB transmission |
| --- | --- | --- |
| Scenario 3. | SSB configuration: {8 SSBs in burst, ssb-periodicity: 20 ms, relative power for a SSB: 5} Sleep mode: micro sleep {relative power: 5.5, transition time: 0 ms, transition energy: 0} | SSB configuration: {2 SSBs in burst, ssb-periodicity: 20 ms, relative power for a SSB: 5} Sleep mode: micro sleep {relative power: 5.5, transition time: 0 ms, transition energy: 0} |
| Scenario 4. | SSB configuration: {8 SSBs in burst, ssb-periodicity: 40 ms, relative power for a SSB: 5} Sleep mode: micro sleep {relative power: 5.5, transition time: 0 ms, transition energy: 0} | SSB configuration: {2 SSBs in burst, ssb-periodicity: 40 ms, relative power for a SSB: 5} Sleep mode: micro sleep {relative power: 5.5, transition time: 0 ms, transition energy: 0} |
| Scenario 5. | SSB configuration: {8 SSBs in burst, ssb-periodicity: 160 ms, relative power for a SSB: 5} Sleep mode: micro sleep {relative power: 5.5, transition time: 0 ms, transition energy: 0} | SSB configuration: {2 SSBs in burst, ssb-periodicity: 160 ms, relative power for a SSB: 5} Sleep mode: micro sleep {relative power: 5.5, transition time: 0 ms, transition energy: 0} |

To calculate the amount of network energy saving, the PDCCH+PDSCH, SSB transmission and sleep mode are considered, and the duration and relative power of each channels and mode has been used. For PDCCH+PDSCH, the duration was derived by resource utilizations acquired on multiple traffic loads (e.g. 28.56 % RU, 17.42 % RU and 6.03 % RU) from system level simulation, given reference configuration in Annex, and relative power was assumed as 280 for Cat 1 and 32 for Cat 2 in section 2.1. For SSB transmission, we assumed 8 SSB transmissions over 4 slot with 45 and 5 relative power per a SSB for Cat 1 and Cat 2, given reference configuration set 1 and the scaling in section 2.2. By adapting the number of SSB transmission, we have taken into account 2 SSB transmission over 2 slot. For sleep modes, we determined the suitable sleep mode, which could have longer sleep duration than transition time, depending on SSB-periodicity, and assumed relative power, total transition time and energy shown in Table 1 and Table 2.

Table 3: Preliminary evaluation results of SSB adaptation

| NW energy saving scheme | ESG for each traffic loads |  | Evaluation methodology/baseline assumption | Note |
| --- | --- | --- | --- | --- |
|  | Category 1 | Category 2 |  |  |
| SSB adaptation From 8 SSBs to 2 SSBs | Light_0 load: 5.20 % Light_1 load: 7.25 % Low load: 10.60 % | Light_0 load: 3.61 % Light_1 load: 4.71 % Low load: 6.24 % | Baseline: {8 SSBs in burst, ssb-periodicity: 20 ms} Reduced: {2 SSBs in burst, ssb-periodicity: 20 ms} | Traffic loads [RU]: {28.56%, 17.42%, 6.03%} |
|  | Light_0 load: 2.82 % Light_1 load: 3.89 % Low load: 6.30 % | Light_0 load: 1.89 % Light_1 load: 2.43 % Low load: 3.40 % | Baseline: {8 SSBs in burst, ssb-periodicity: 40 ms} Reduced: {2 SSBs in burst, ssb-periodicity: 40 ms} | Traffic loads [RU]: {28.56%, 17.42%, 6.03%} |
|  | Light_0 load: 1.01 % Light_1 load: 1.58 % Low load: 3.66 % | Light_0 load: 0.48 % Light_1 load: 0.62 % Low load: 0.88 % | Baseline: {8 SSBs in burst, ssb-periodicity: 160 ms} Reduced: {2 SSBs in burst, ssb-periodicity: 160 ms} | Traffic loads [RU]: {28.56%, 17.42%, 6.03%} |

As analyzed in Table 3, the preliminary results of SSB adaptation show 0.48 % ~ 10.6% energy saving gain. As decreasing traffic loads (i.e. % RUs) for PDCCH + PDSCH, the ESG of SSB adaptation becomes more increasing. Also, the ESG of SSB adaptation are depending on ssb-periodicity.

Observation 2: Adapting the number of SSBs from 8 to 2 provides energy saving gain up to 10.6 %, given SSB periodicity, in preliminary results.

  - Potential techniques in frequency domain

In this section, BWP adaptation has been evaluated for network energy saving. In the preliminary evaluation result, the ESG of BWP adaptation are calculated by resource utilization for PDCCH+PDSCHs and deep sleep mode for Cat 1 or micro sleep mode for Cat 2, and relative power and total transition time with the scaling model in section 2, as follow: the scaling for the BW of Xf MHz = 0.35 + 0.65 * (Xf) / 100, over 160 ms duration. The UPT and packet latency, which acquired with reference configuration with traffic load, have been taken into account as baseline performance of UPT and packet latency. In terms of scheduling latency, it was compared between full bandwidth and reduced bandwidth to show the scheduling impact, which may be affected by traffic loads. Figure 2 depicts the BWP adaptation with fixed PSD value for keeping the coverage. BW are adapted from 100 MHz with 55 dBm to 60 MHz with 53 dBm with by changing the traffic loads from 28.56 % RU traffic load, which is calculated by light_0 traffic model in Annex, to 20.42 % RU traffic load, which is calculated by light_1 traffic model in Annex.

Figure 2: Illustration of BWP adaptation scenario for evaluation

Table 4: Preliminary evaluation results of BWP adaptation

| NW energy saving scheme | ESG | UPT loss | Scheduling latency | Packet latency | Evaluation methodology/baseline assumption | Note |
| --- | --- | --- | --- | --- | --- | --- |
| BWP adaptation from 100 MHz to 60 MHz. | Cat 1 with deep sleep: 39.41 % | 29.93 % | No increase | 41.34% | Baseline: 100MHz with 55 dBm Reduced: 60 MHz with 53 dBm. | Baseline traffic load: 28.56 % RU Reduced BW traffic load: 20.42 % RU |
|  | Cat 2 with micro sleep: 26.46 % |  |  |  |  |  |

As analyzed in Table 4, BWP adaptation between full bandwidth and reduced narrow bandwidth achieves the ESG up to 39.41 %. Under same traffic load, the BWP adaptation show 29.93 % UPT loss and packet latency 41.34 %.

Observation 3: Adapting bandwidth from 100MHz to 60 MHz achieves up to 39.41 % energy saving gain in preliminary results.

Observation 4: Adapting bandwidth from 100MHz to 60 MHz shows up to 29.93 % UPT loss and packet latency increase up to 41.34 % in preliminary results.

- Conclusions

This contribution discusses aspects related to power consumption modelling and evaluation methodology for evaluating NW energy saving techniques. The proposals and observations made in this contribution are summarized as below:

Observation 1: Relative powers of active UL in Category 1 should be further investigated in order to reflect the proper BS power consumption.

Observation 2: Adapting the number of SSBs from 8 to 2 provides energy saving gain up to 10.6 %, given SSB periodicity, in preliminary results.

Observation 3: Adapting bandwidth from 100MHz to 60 MHz achieves up to 39.41 % energy saving gain in preliminary results.

Observation 4: Adapting bandwidth from 100MHz to 60 MHz shows up to 29.93 % UPT loss and packet latency increase up to 41.34 % in preliminary results.

Proposal 1: Support the relative powers for Category 2 and total transition time for Category 1 and Category 2 for reference configuration set 1.

- For reference configuration set 1, the values are provided as below.

| Power state | Relative Power P | Total transition time T |  |
| --- | --- | --- | --- |
| Deep sleep | 1 | Cat 1: 50ms | Cat 2: 10s |
| Light sleep | Cat 2: 2.1 | Cat 1: 6 ms | Cat 2: 640 ms |
| Micro sleep | Cat 2: 5.5 | 0 | 0 |
| Active DL | Cat 2: 32 | N.A. | N.A. |
| Active UL | Cat 2: 6.5 | N.A. | N.A. |

Proposal 2: Support the relative powers for Category 2 for reference configuration set 2 and set 3 in R1-2208312.

- For reference configuration set 2 and set 3, the values are provided as below.

| Power state | Relative Power P for Category 2 |  |
| --- | --- | --- |
|  | Set 2 | Set 3 |
| Deep sleep | 1 | 1 |
| Light sleep | 2.6 | 1.8 |
| Micro sleep | 5 | 3 |
| Active DL | 40 | 8.4 |
| Active UL | 5.8 | 4.2 |

Proposal 3: Define the additional transition energy for set1, set2 and set3 as follow:

- For set 1, the additional energy (unit in relative power*(duration in ms)) is

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 |
| Light sleep | 90 | 1088 |

- For set 2, the additional energy (unit in relative power*(duration in ms)) is

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1225 | 20000 |
| Light sleep | 81 | 768 |

- For set 3, the additional energy (unit in relative power*(duration in ms)) is

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 925 | 10000 |
| Light sleep | 54 | 384 |

Proposal 4: Support the scaling of BS power consumption as below:

- The scaling of BS power consumption,

- the BS power consumption for active DL is provided by

  - P= Pstatic+Pdynamic

    - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on

      - Category 1: 55

      - Category 2: 5.5

    - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on sa*Pdyn,ante+sf*spηsf,  sp*Pdyn,joint, where

        - Pdyn,ante is

          - Category 1: 15

          - Category 2: 1.5

        - Pdyn,joint is

          - Category 1: 71

          - Category 2: 8.5

        - ηsf,  sp is the PA efficiency

          - For initial evaluations, η= 0.34

        - sa, sf, sp is the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively.

        - The percentage of active TRxRUs sa: {1 for 64 TRxRUs, 0.67 for 32 TRxRUs, 0.67^2 for 16 TRxRUs, 0.67^3 for 8 TRxRUs}

        - The ratio of RF bandwidth sf: 0.35+0.65* Xf /100MHz, Xf [MHz] = {10, 20, 40, 50, 80, 100}

        - The ratio of PSD per TxRU between the DL transmission and reference configuration sp: 0.67^(Xp /3[dB]), Xp = {0, 3[dB], 6[dB]}

- the BS power consumption for active UL is provided by

  - P=PstaticUL+sa*PdynamicUL

    - PstaticUL=Pstatic

    - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on

      - Category 1: 55

      - Category 2: 5.5

    - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on

      - Category 1: [FFS]

      - Category 2: 1

        - The percentage of active TRxRUs sa: {1 for 64 TRxRUs, 0.67 for 32 TRxRUs, 0.67^2 for 16 TRxRUs, 0.67^3 for 8 TRxRUs}

Proposal 5: Use the following KPIs for evaluating NW energy saving techniques:

- Energy saving gain

- Latency (packet latency and scheduling latency)

- UPT

- Coverage

- UE power consumption

Proposal 6: Define acceptable range of requirements on latency increase and UPT loss, like less than 10 %, less than 20 % and less than 30 %, for the evaluation of network energy saving techniques.

Proposal 7: Support to use the reference SLS configuration in the above agreement for FR2 evaluation.

- For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption.

  - BS antenna configurations

    - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

    - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ)

  - Traffic model & UE density

    - Follow previous agreements with adjusted UE density

  - Total transmit power per TRxP

    - Value scaled from that in set 3 reference configuration considering BW

- References

[1] Chairman’s notes, RAN1#110, August 2022.

[2] R1-2208312, FL summary for Post-110-R18- NW_ES2, Moderator (Huawei).

[3] R1-2209743, Discussion on network saving techniques, Samsung.

[4] Report ITU-R M.2410, "Minimum requirements related to technical performance for IMT-2020 radio interface(s)".

[5] R1-1812048, “Summary for UE Power Saving Evaluation Methodology”, Qualcomm, RAN1#94bis, October 2018.

[6] 3GPP TR 36.814, “Further advancements for E-UTRA physical layer aspects”, Release 9

Annex:

Table A. The evaluation assumption for BS power consumption model for FR1

| Basic parameters | Channel model | 3D/HF-Uma based on TR 38.901 (low-loss O2I penetration model) |
| --- | --- | --- |
|  | Device deployment | 80% indoor, 20% outdoor |
|  | Inter-site distance | 500m |
|  | Network Topology | 7*3 Sector |
|  | Carrier Frequency | 4.0GHz |
|  | Multiple access | OFDMA |
|  | Duplexing | TDD (for set 1 ref. config.) |
|  | Numerology | 30kHz, 14 OFDM symbol slot |
|  | Guard band ratio on simulation bandwidth | TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth) |
|  | Simulation bandwidth | Follow reference configuration |
|  | Frame structure | DDDSU |
|  | UT attachment | Based on RSRP |
|  | Wrapping around method | Geographical distance based wrapping |
|  | Traffic model | Follow previous RAN1 agreements Light_0 traffic load: FTP3 (0.5MB as packet size, 50ms as mean inter-arrival time) Light_1 traffic load: FTP3 (0.5MB as packet size, 100ms as mean inter-arrival time) Low traffic load: FTP3 (0.5MB as packet size, 400ms as mean inter-arrival time) |
|  | HARQ retransmission | No retransmission |
| BS parameters | BS antenna height | 25 m |
|  | BS noise figure | 5 dB |
|  | BS antenna element gain | 8 dBi |
|  | Antenna configuration at TRxP | For 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8). based on 38.802 |
| UE parameters | UE power class | 23dBm |
|  | UE noise figure | 9 dB |
|  | UE antenna element gain | 0 dBi |
|  | UE antenna height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height |
|  | Antenna configuration at UE | For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Transmission parameters | Modulation | Up to 256 QAM |
|  | Transmission scheme | SU-MIMO |
|  | SU dimension | For 4Rx: Up to 4 layers |
|  | DL CSI measurement | Precoded CSI-RS based |
|  | DL codebook | non-PMI transmission |
|  | SRS transmission | For UE 4 Tx ports: Non-precoded SRS |
|  | Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
|  | Scheduling | PF |
|  | Receiver | MMSE-IRC |
|  | Channel estimation | Non-ideal |
| Common RS | SSB/SIB1 period | 20ms |

(M, N, P, Mg, Ng; Mp, Np)

- M: Number of vertical antenna elements within a panel, on one polarization

- N: Number of horizontal antenna elements within a panel, on one polarization

- P: Number of polarizations

- Mg: Number of panels in a column;

- Ng: Number of panels in a row;

- Mp: Number of vertical TXRUs within a panel, on one polarization

- Np: Number of horizontal TXRUs within a panel, on one polarization