3GPP TSG RAN WG1 #110-bis-e    	                                              R1-2209757

e-Meeting, October 10th – 19th, 2022

Agenda Item:		9.13.2

Source:				Samsung

Title:					Receiver architecture for LP-WUS

Document for:	Discussion

# Introduction

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

    - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In the approved revised SI for low-power Wake-up Signal and Receiver for NR [1], one important direction is to study and evaluate low-power wake-up receiver architectures.

This contribution discusses aspects related to low-power wake-up receiver architectures.

# The design criteria for receiver architecture

According to the SID shown as above, new signals will be introduced to be used as LP-WUS to further reduce the power consumption compared to the existing Rel-15/16/17 UE power saving mechanisms. To further prolong the battery life, a separate receiver will be designed to monitor new wake-up signal with ultra-low power consumption. When the WUR detects the wake-up signal, it will trigger the main radio on in order for the main radio to receive signals and/or channels. Otherwise, the main radio will maintain off state (from Rel-16) or off as a new state which can be further discussed during SI.

When new receiver architecture is taken into acount in NR for LP-WUS, the coverage should be evaluated. For increasing the application of LP-WUS, the coverage of LP-WUS should be consistent with the legacy signal of the main receiver. That is to say, the UE in the edge of the cell can also receive the LP-WUS to reduce their power consumption. Therefore, amplifier (like low noise amplifier) can be adopted when designing the WUR.

Proposal 1: The coverage of LP-WUS should be consistent with the legacy signal of the main receiver.

To match the coverage of LP-WUS with one for legacy signal of the main receiver, the WUR architecture maybe not achieve maximum energy efficiency. However, considering that the WUR is assumed to be always on to monitor LP-WUS when the main receiver is off, the power consumption should still be much lower than the main radio.

Proposal 2: The power consumption of the separate WUR should be reduced dramatically compared with main radio.

Since out of synchronization has a great impact on OOK signal detection, the synchronization issue should be further considered when designing the WUR. Besides, when LP-WUS is transmitted based on OFDM symbol by OOK modulation, the interference between LP-WUS and other signals cannot be ignored. In this case, Phase-locked loop (PLL) and/or Low pass filter (LPF) and/or band-pass filter (BPF) can be essential at the wake-up receiving by the WUR.

Proposal 3: Study synchronization and interference issue in LP-WUS reception.

According to the survey [2]-[16], we found that the data rate of LP-WUS can range from 0.3 to 200 kbps. In addition, the sensitivity can reach - 80 dBm in most papers, and the power consumption ranges from 4.5 uw to 578 mw. It should be noticed that there is a tradeoff between sensitivity, data rate and power consumption in the process of WUR designing. If we pursue higher sensitivity and data rate, it will lead to higher power consumption then.

Proposal 4: Study the impact of the tradeoff between sensitivity, data rate and power consumption in the process of WUR designing.

# The basic structure for WUR

For Hardware module of the wake-up receiver used for signal detecting and processing, a separate receiver for reception of LP-WUS in addition to main NR radio transceiver can be a basic UE architecture for the study. Referring to the architecture of WUR, the structure can be divided into RF end and BB end.

The LP-WUS is first received by the RF end via the antenna and then passes through the matching network that filters and boosts the incoming LP-WUS. After input matching, the signal passes through the amplifiers, often the low noise amplifier (LNA) for increasing the sensitivity and the coverage of the receiver by amplifying weak signals while meeting noise requirements. The LNA dominates in terms of power consumption. Then, an envelope detector (ED) or heterodyne detector can be consider for signal detection and conversion to baseband signal to make the circuit simpler and energy efficient. In addition, if ED is replaced by mixer, the power consumption on Oscillators, phase locked loop and mixer, may be further considered. Therefore, we think RAN1 should study various types of receivers (e.g., envelop detector, heterodyne detector) for LP-WUS reception in RF end.

To further reduce the interference between other signals and/or channels and LP-WUS in OFDM symbol and decode the data carried by LP-WUS, BB end should also be included in the separate WUR in our view. After that, the voltage multiplier rectifies the LP-WUS energy and converts this input signal into a direct current (DC) signal. The DC signal at the rectifier output is detected by a comparator then. When the comparator is triggered, an interrupt is generated to trigger the main radio on for data reception.

Proposal 5: Study various types of receivers (e.g., envelop detector, heterodyne detector) for LP-WUS reception in RF end.

Proposal 6: Considering the coverage and the decoding issue of LP-WUS, RF+BB architecture seems more suitable for receiving LP-WUS signals.

# Conclusion

This contribution discusses the architectures for low-power wake-up receiver. Observations and proposals are summarized as follows:

Proposal 1: The coverage of LP-WUS should be consistent with the legacy signal of the main receiver.

Proposal 2: The power consumption of the separate WUR should be reduced dramatically compared with main radio.

Proposal 3: Study synchronization and interference issue in LP-WUS reception.

Proposal4: Study the impact of the tradeoff between sensitivity, data rate and power consumption in the process of WUR designing.

Proposal 5: Study various types of receivers (e.g., envelop detector, heterodyne detector) for LPWUS reception in RF end.

Proposal 6: Considering the coverage and the decoding issue of LP-WUS, RF+BB architecture seems more suitable for receiving LP-WUS signals.

# References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, Sep, 2022.

- J. Im, J. Breiholz, S. Li, B. Calhoun and D. D. Wenzloff, "A Fully Integrated 0.2V 802.11ba Wake-Up Receiver with -91.5dBm Sensitivity," 2020 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), 2020, pp. 339-342, doi: 10.1109/RFIC49505.2020.9218383.

- R. Liu et al., "An 802.11ba-Based Wake-Up Radio Receiver With Wi-Fi Transceiver Integration," in IEEE Journal of Solid-State Circuits, vol. 55, no. 5, pp. 1151-1164, May 2020, doi: 10.1109/JSSC.2019.2957651.

- R. Dorrance et al., "An Ultra-Low Power, Fully Integrated Wake-Up Receiver and Digital Baseband with All-Digital Impairment Correction and -92.4dBm Sensitivity for 802.11ba," 2019 Symposium on VLSI Circuits, 2019, pp. C80-C81, doi: 10.23919/VLSIC.2019.8778013.

- J. Im, H. Kim and D. D. Wentzloff, "A 220-uW −83-dBm 5.8-GHz Third-Harmonic Passive Mixer-First LP-WUR for IEEE 802.11ba," in IEEE Transactions on Microwave Theory and Techniques, vol. 67, no. 7, pp. 2537-2545, July 2019, doi: 10.1109/TMTT.2019.2913654.

- X. Huang, P. Harpe, G. Dolmans, H. de Groot and J. R. Long, "A 780–950 MHz, 64–146 µW Power-Scalable Synchronized-Switching OOK Receiver for Wireless Event-Driven Applications," in IEEE Journal of Solid-State Circuits, vol. 49, no. 5, pp. 1135-1147, May 2014, doi: 10.1109/JSSC.2014.2307056.

- K. -W. Cheng and S. -E. Chen, "An Ultralow-Power OOK/BFSK/DBPSK Wake-Up Receiver Based on Injection-Locked Oscillator," in IEEE Transactions on Very Large Scale Integration (VLSI) Systems, vol. 29, no. 7, pp. 1379-1391, July 2021, doi: 10.1109/TVLSI.2021.3073166.

- X. Huang, S. Rampu, X. Wang, G. Dolmans and H. de Groot, "A 2.4GHz/915MHz 51µW wake-up receiver with offset and noise suppression," 2010 IEEE International Solid-State Circuits Conference - (ISSCC), 2010, pp. 222-223, doi: 10.1109/ISSCC.2010.5433958.

- A. Dissanayake, H. L. Bishop, S. M. Bowers and B. H. Calhoun, "A 2.4 GHz-91.5 dBm Sensitivity Within-Packet Duty-Cycled Wake-Up Receiver," in IEEE Journal of Solid-State Circuits, vol. 57, no. 3, pp. 917-931, March 2022, doi: 10.1109/JSSC.2021.3104540.

- Oller, J. , et al. "IEEE 802.11-enabled wake-up radio system: Design and performance evaluation." Electronics Letters, 50.20(2014):1484-1486, doi: 10.1049/EL.2014.2468.

- D. Yoon et al., "A New Approach to Low-Power and Low-Latency Wake-Up Receiver System for Wireless Sensor Nodes," in IEEE Journal of Solid-State Circuits, vol. 47, no. 10, pp. 2405-2419, Oct. 2012, doi: 10.1109/JSSC.2012.2209778.

- E. Nilsson and C. Svensson, "Ultra Low Power Wake-Up Radio Using Envelope Detector and Transmission Line Voltage Transformer," in IEEE Journal on Emerging and Selected Topics in Circuits and Systems, vol. 3, no. 1, pp. 5-12, March 2013, doi: 10.1109/JETCAS.2013.2242777.

- M. Magno and L. Benini, "An ultra low power high sensitivity wake-up radio receiver with addressing capability," 2014 IEEE 10th International Conference on Wireless and Mobile Computing, Networking and Communications (WiMob), 2014, pp. 92-99, doi: 10.1109/WiMOB.2014.6962155.

- S. J. Marinkovic and E. M. Popovici, "Nano-Power Wireless Wake-Up Receiver With Serial Peripheral Interface," in IEEE Journal on Selected Areas in Communications, vol. 29, no. 8, pp. 1641-1647, September 2011, doi: 10.1109/JSAC.2011.110913.

- P. -H. P. Wang et al., "A 6.1-nW Wake-Up Receiver Achieving −80.5-dBm Sensitivity Via a Passive Pseudo-Balun Envelope Detector," in IEEE Solid-State Circuits Letters, vol. 1, no. 5, pp. 134-137, May 2018, doi: 10.1109/LSSC.2018.2875826.

- H. Jiang et al., "24.5 A 4.5nW wake-up radio with −69dBm sensitivity," 2017 IEEE International Solid-State Circuits Conference (ISSCC), 2017, pp. 416-417, doi: 10.1109/ISSCC.2017.7870438.