3GPP TSG RAN WG1 #110bis-e		                                                                 R1-2209765

e-Meeting, October 10th – 19th, 2022

Title:	Draft reply LS on Per-FS L1 feature for NR sidelink discovery BC-list

Response to:	R1-2208727/R2-2208867

Release:	Rel-17

Work Item:	NR_SL_enh-Core, NR_SL_Relay-Core

Source:	ZTE, Sanechips [to be RAN1]

To:	RAN2

Cc:

Attachment:	-

Contact Person:

Name:	Yuzhou Hu

E-mail Address:	hu.yuzhou@zte.com.cn

Overall description

RAN1 thanks RAN2 for the LS (R1-2208727/R2-2208867) on Per-FS L1 feature for NR sidelink discovery BC-list. With the conclusion that discovery message is carried by PSSCH, in the same way as for communication, RAN2 assumed the following Rel-16 per-FS L1 features and Rel-17 per-FS L1 features also apply to band combinations supporting discovery.

| 15-25 | Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier | 1) UE can monitor DCI format 3_0 on a different carrier from sidelink for NR sidelink dynamic scheduling and configured grant type 2 |
| --- | --- | --- |

| 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu |
| --- | --- | --- |
| 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set. |

Based on above assumption, the following questions were raised by RAN2, and RAN1 would provide response as follows:

Q1: Can RAN1 confirm the RAN2 assumption above, i.e., these per-FS L1 features above also apply to the band-combination supporting discovery? A1: Yes. Q2: If Yes to Q1, in case a band combination supporting both NR Sidelink communication and NR Sidelink discovery, whether the UE capability of these per-FS L1 feature would be different for NR Sidelink communication and NR sidelink discovery, or it would be the same? A2: In case a band combination supporting both NR Sidelink communication and NR Sidelink discovery, the defined UE capability for NR SL communication can be reused for NR SL discovery, so it would be the same.

Actions:

To RAN2:

RAN1 kindly asks RAN2 to take above responses into account for the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#111			14th -18th Nov 2022				Toulouse, France

RAN1#112			27 February to 3 March 2023			Athens, Greece