3GPP TSG RAN WG1 #110bis-e			R1-2209773

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.4.1.2

Source: 	ITL

Title: 	On Physical Channel Design framework for SL-U

Document for:	Discussion and decision

Introduction

In RAN1#110, a Rel-18 SI for NR SL on unlicensed spectrum (NR SL-U) has been discussed and following agreements are made on SL-U physical channel design framework [1]:

Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior Agreement Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded - FFS details of above alternatives Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

In this contribution, we discuss technical aspects related to the physical channel design frameworks to support NR SL operations on FR1 unlicensed spectrum.

Discussion

Interlaced based resource pool

As discussed in previous RAN1 meeting, interlaced RB based transmission specified in UL for Rel-16 NR-U has been considered for Rel-18 SL-U resource units (e.g., sub-channel and resource pool) to meet the OCB and PSD requirements on unlicensed spectrum. Considering NR-U system operated in same FR1 unlicensed spectrum, we think that it is natural to support the interlaced based resource pool and subchannel for SL-U. In detail, the subchannel can be constructed with at least K interlaced RBs. The interlaced RB structure can be based on Rel-16 NR-U design. Additionally, one resource pool can be defined with subchannel unit similar with legacy NR SL. Accordingly, the interlaced based resource pool and subchannel for SL-U can be used to meet the OCB and PSD requirements and as seen Figure 1, it can allow more than one resource pools within one LBT BW (RBS) so that resource utilization and flexibility of resource pool configuration on unlicesed spectrum would be improved.

Figure 1 Illustration of interlace based sub-channel and resource pool

Proposal 1: It is beneficial to support interlace based resource pool for NR SL-U

- One SL resource pool can be configured with subset of interlaced RBs within one RBS.

- Further study the how to associate subchannels with a resource pool across RBSs, in case multiple RBS is configured

In time domain, SL resource pool configurations are based on bitmap on cell-specific SL slots except SL-SSB’s slots. That is, S-SSB slots, slots without enough UL symbols, and slots indicated as 0 by bitmap are excluded from the resource pool. However, if legacy SL resource pool configurations are simply reused for SL-U, there may be slot level gap between SL slots and then, Tx SL UE may lose the occupied channel by other RAT and UEs. To avoide such problem, resource pool configuration in time domain for SL-U can be operated without SL slot configuration that is, all slots can be potentially SL transmission slots depending on the LBT result, similar to NR-U. If NR-U and SL-U are operated in same unlicensed spectrum, TDD-UL-DL configurations should be provided to SL-U UEs to identify the potential-U SL slots except DL/UL slots in a frame. Among the potential SL-U slots, SL-U UE can perform LBT procedure and if successed the SL slots within a COT can be occupied by the SL-U UE and transmit SL channel/signals during the occupied SL slots. Alternatively, with slot-level bitmap configuration but, set to all “1”s can be adopted for resource pool configuiration in time domain. The one of  mechanisms related to SL resource pool in time domain can be considered for SL-U.

Proposal 2: It is beneficial to consider the resource pool configuration in time domain without slot-level bitmap configuration (i.e. all SL slots can be potencially used for SL-U transmission)

SL Slot structure

In last RAN1 meeting, for slot structure in SL-U, it was agreed that at least legacy NR SL slot-based PSCCH/PSSCH transmission is supported including usage of CP extension (CPE) in a PSSCH. If the slot-based SL transmission is non-contiguous in time domain and the time gap between consecutive SL transmissions is larger than 16/25us, the other UEs may transmit their channel/signal in the time gap. So, it is not guaranteed to maintain the COT of Tx UE if the time gap is allowed during the consecutive SL transmissions by the Tx UE. It will result in additional LBT overhead (e.g. type 1 LBT) and thus, overall SL system performance may be degraded. Accordingly, in order to avoid such the additional LBT operation, it seems to consider additional signalings to indicate whether AGC/Gap symbol are used or not. For example, as seen Figure 2, in case one Tx UE performs SL data burst transmissions(PSCCH/PSSCH) to respective Rx UE (i.e. Rx UE#1 and Rx UE#2) on contiguous SL slots during COT after a LBT is succeed, the AGC symbol is trained in the beginning of the SL data burst so the following AGC symbol in subsequent slot can be used for PSSCH transmission instead. Same priciple can be applied to the Tx-Rx gap symbol on the SL data burst transmission. Moreover, it is beneficial to consider the usage of CPE in the beginning of slot which affects to SL slot structure and the usage of SL symbol gap, in order to efficiently use the SL time domain resources for the PSSCH/PSCCH transmission maintaining the COT duration.

In addition, the other approach to handle the above technical issue is referred in Figure 2 where a Tx UE transmits SL data burst to Rx UE#1 and #2, respectively. That is, the Tx UE may schedule SL data burst transmission to one Rx UE by one SCI signaling on multiple contiguous SL slots. As discussed in this case there would be no tinme gap between SL slots or very small gap is allowed for one-shot sensing to transmit PSSCH/PSCCH transmissions. The such kind of mechnisms to eleminate the time gap between SL slots can be relying on dynamic SCI signaling by indicating the presence of AGC, Gap and/or usage of CPE and so on… Therefore, it should be discussed how to handle the time gap to support SL data burst transmission on the contiguous SL slots within a COT duration, in order to efficiently use SL resources and reduce the LBT overhead.

In order to maintain competitive channel access procedure to SL-U compared with other RATs (e.g. Wi-Fi, LAA, NR-U) on unlicesend spectum, multiple starting locations for PSSCH/PSCCH transmission within a slot can be investigated. The additional starting positions in the middle of SL slot can reduce the SL-U system latency and system performance may be improved by competitively performing LBT than other RATs. The mechnism might be also beneficial for other RATs to cause less LBT overhead from SL-U Ues so that overall usages of unlicensed spectrum would be efficient among the all RATs. However, it is expected that the tradeoff between the number of starting location in a slot and the complexity in monitoring the SL channel/signals within the slot should be considered because it will lead to much higher complexity and further specification works.

Figure 2. Illustration of contiguous SL-U data burst transmission

Proposal 3: It should be considered to support mechanisms to reduce the channel access procedure for SL-U. At least following schemes can be discusssed as a starting point:

- Signaling on the presence of AGC/Gap symbol, CPE and/or symbol repetition between the SL data burst transmission during a COT

- Multi-slot based PSSCH/PSCCH transmission by one SCI signaling

- Mini-slot based PSSCH/PSCCH transmission within a SL slot

- Impacts on PSSCH rate matching if such mechanisms are applied

- Resource allocation of PSSCH, PSCCH, PSFCH and SL RS in the new type of SL slots if supported

PSCCH/PSSCH

As discussed in section 2.1, the interlaced based SL transmission including PSCCH/PSSCH has been discussed to meet the OCB and PSD requirements on unlicensed spectrum, leverging from the NR-U UL waveforms. For 15/30KHz SCS, PSCCH/PSSCH has 10/5 interlaces for resource allocation, respectively.

In Rel-16/17 NR-U, interlace and RBS based resource allocation is basic resource unit for data/control transmission while it can be defined for SL-U using both interlace/RBS from NR-U and subchannel from NR SL. Considering SL-U design guidance in SI description that the legacy designs from both NR-U and SL should be reused as much as possible, it is beneficial to fully reuse the interlace/RBS and subchannel units for PSCCH/PSSCH and so on in SL-U.

PSCCH resource mapping

Assuming the interlace and RBS resource units for SL-U, one PSCCH will occupy at least 1 interlace within one subchannel. In Figure 3, if a Tx UE successes to occupy two adjacent RBSs after LBT procedure to transmit PSCCH/PSSCH, it can be determined to transmit the PSCCH only one RBS (e.g. lowest RBS index) among the two adjacent RBSs, while the PSCCH can be dupplicated on both two adjacent RBSs to improve the PSCCH reception performance. Both PSCCH resource mapping are assumed to be allocated on the frequency resources only within a RBS although PSSCH resource allocation is allowed to be mapped on the guardband between the adjacent RBSs given that the LBT is successed on the adjacent RBSs. Because other SL UEs can’t recognize which RBS (common for all SL UEs) are used for PSCCH/PSSCH in advanced when they are performing PSCCH blind detection in the configured RBSs.

Figure 3 PSCCH resource mapping only within one LBT BW(RBS)

Figure 4 PSCCH resource mapping within more than one LBT BW(RBS)

Proposal 4: It is benefitial to consider PSCCH resource mapping with following design:

- PSCCH resource is allocated only within a LBT BW (RBS)

- PSCCH resource can be allocated within more than one LBT BW (RBSs) except guard band between adjacent RBSs

In Rel-16/17 SL PSCCH time/frequency resource configuration by higher layer signaling,

- In time domain: two or three OFDM symbols (starting to second OFDM symbol after AGC symbol)

- In frequency domain: {10, 12, 15, 20, 25} PRBs, the configured RB is confined in one subchannel

The following tables shows the potential resource allocation units to use PSCCH/PSSCH transmission within a LBT BW (RBS) for SL-U, when considering legacy NR-U resource units e.g. RBS and interlace, as well as SL resource unit e.g., subchannel.

SCS 15kHz, LBT BW(RBS) = 100 or 110 RBs @20MHz (LBT BW)

| Number of subchannel per RBS | Number of interlaces per subchannel (K) | Number of interlaces (M) | Number of RBs per interlace (N) |
| --- | --- | --- | --- |
| 10 | K=1 | 10 | 10 or 11 |
| 5 | K=2 | 10 | 10 or 11 |
| 2 | K=5 | 10 | 10 or 11 |
| 1 | K=10 | 10 | 10 or 11 |

SCS 30kHz, LBT BW(RBS) = 50 or 55 RBs @20MHz (LBT BW)

| Number of subchannel per RBS | Number of interlaces per subchannel (K) | Number of interlaces (M) | Number of RBs per interlace (N) |
| --- | --- | --- | --- |
| 5 | K=1 | 5 | 10 or 11 |
| 1 | K=5 | 5 | 10 or 11 |

If NR-U resource allocation and structures within a LBT BW are considered for SL-U as well, it can be summarized as seen in the above tables. The number of interlaces (M) and number of RBs per interlace can be determined by a SL carrier in a unlicensed spectrum, similar to the NR-U. In addition, the number of interlaces per subchannel (K) and number of subchannel per RBS can be determined by higher layer signaling in SL-U. So, the parameters and system configurations can be determined by various system aspects e.g., resource allocation granularity, scheduling flexibility, and so on.

On the other hand, the number of PRBs for PSCCH is determined by PSCCH reception performance and target coding rate of PSCCH. Accordingly, there can be following two cases, depending on the resource configurations in above for SL-U:

- Case 1: Number of interlaces per subchannel (K) >= Number of RB for PSCCH

- Case 2: Number of interlaces per subchannel (K) < Number of RB for PSCCH

Depending on which case(s) are considered for PSCCH resource mapping in SL-U, the PSCCH resource mapping scheme can be affected. Therefore, further details need to be studied in RAN1.

Proposal 5: it is necessary to discuss how to handle PSCCH mapping case where the number of RBs configured for PSCCH are larger than that of subchannel with interlaces

If PSCCH resource allocation is supported based on the interlace and RBS, how to allote the PSCCH RBs into the interlaces in one RBS should be discussed. Figure 5 shows for example two PSCCH resource mapping schemes that include the contiguous PSCCH RB mapping in one interlace (left) and the non-contiguous PSCCH RB mapping in one interlace (right). Particularly, the non-contiguous PSCCH RB mapping can be differently mapped on either the equally-spaced RBs (Alt 1) or both end of the frequency in a RBS (Alt 2).

There would be trade-off between the example schemes in terms of reliability and complexity. So, it should be studied to identify what PSCCH allocation scheme is used for SL-U.

Figure 5 PSCCH resource mapping with contiguous interlace RBs (left) and non-contiguous interlace RBs (right)

Proposal 6: Study the resource mapping of PSCCH with either contiguous or non-contiguous interlace RBs

PSSCH resource mapping

In RAN1#109-e, it was agreed that the intra-cell guardband between adjacent RBSs is used for the PSSCH transmission. Based on the agreement, it can be seen that PSCCH and PSSCH is multiplexed by TDM and/or FDM manner as in NR SL. For the PSSCH resource indication, 1st SCI (PSCCH) can indicate information of the RBS and the subchannel containing associated interlaces to schedule the PSSCH transmission, similar to NR-U. On the other hand, conventional PSSCH resources can be reserved for maximum 2 or 3 slots by using TRIV and FRIV in SCI signaling. Such PSSCH resource allocation scheme needs to be also maintained for SL-U, together with RBS and interlace based subchannels.

For 2nd SCI on PSSCH, it can be also transmitted in the starting subchannel/interlace similar with the PSCCH. Such resource allocation of 2nd SCI leads to natural and simple receiver design as in NR SL.

Proposal 7: Study the resource mapping of PSSCH/2nd SCI for SL-U

PSFCH

Similar to the discussion on PSSCH/PSCCH for SL-U, PSFCH should be designed to meet PSD limit and minimum OCB requirements. So, interlace based PSFCH just as PUCCH in NR-U can be fully reused. One A/N bit is carrier via one interlace and one CS-pair and also, CS shifting can be applied across interlaced RBs to reduce the PAPR.

As discussed in section  2.2, it can be supported that the contiguous SL-U data burst transmission is performed during COT to allow better resource utilization and efficiency and reduce the LBT overhead. In this case, it would be more efficient to feedback more than one A/N bits by new PSFCH format in addition to the PSFCH format 0 because multiple and simultaneous PSFCH format 0 transmission is dependent on UE capabilities. Therefore, in order to allow more flexible SL HARQ feedback and operation for SL-U, new PSFCH format containing multi-bits can be considered.

Proposal 8: It can be beneficial to consider new PSFCH format that supports more than one HARQ-ACK bit

Considering LBT failure in case of PSFCH Tx, it should be discussed how to compensate PSFCH transmission failure due to LBT result. There can be variuos schemes to handle the problem for SL-U as followings:

- Alt 1: One PSSCH transmission has 1 or multiple related PSFCH occasions

  - Time and/or frequency domain

- Alt 2: PSFCH occasions are dynamically indicated

- Alt 3: Combinations of the Alt 1 and Alt 2

The more PSFCH occasions provided by various ways can be indicated by semi-static signaling and/or dynamic signaling (SCI) according to LBT result. Basically, Alt 1 is the expansion of current NR SL’s PSFCH resource configuration while Alt 2 is required to design new dynamic signaling via SCI for the dynamic PSFCH occasion indication according to the LBT result. It is considered that Alt 2 should be of course designed to introduce the new signaling of PSFCH occasions and potentially may result in PSFCH sensing operation or PSFCH transmission collisions due to no coordinations of PSFCH occasions. Also, for Alt 1, it would lead to inefficient PSFCH resource configurations having exessive PSFCH occasions and larger resource overhead from the PSFCH occasions.

Proposal 9: It is preferred to firstly support one PSSCH transmission associated with one or more of PSFCH occasions and then, FFS PSFCH occasions are dynamically indicated for better PSFCH resource utilization

Conclusion

In this section, we summarize the our proposals on physical channel design framework for SL-U as follows:

Proposal 1: It is beneficial to support interlace based resource pool for NR SL-U

- One SL resource pool can be configured with subset of interlaced RBs within one RBS.

- Further study the how to associate subchannels with a resource pool across RBSs, in case multiple RBS is configured

Proposal 2: It is beneficial to consider the resource pool configuration in time domain without slot-level bitmap configuration (i.e. all SL slots can be potencially used for SL-U transmission)

Proposal 3: It should be considered to support mechanisms to reduce the channel access procedure for SL-U. At least following schemes can be discusssed as a starting point:

- Signaling on the presence of AGC/Gap symbol, CPE and/or symbol repetition between the SL data burst transmission during a COT

- Multi-slot based PSSCH/PSCCH transmission by one SCI signaling

- Mini-slot based PSSCH/PSCCH transmission within a SL slot

- Impacts on PSSCH rate matching if such mechanisms are applied

- Resource allocation of PSSCH, PSCCH, PSFCH and SL RS in the new type of SL slots if supported

Proposal 4: It is benefitial to consider PSCCH resource mapping with following design:

- PSCCH resource is allocated only within a LBT BW (RBS)

- PSCCH resource can be allocated within more than one LBT BW (RBSs) except guard band between adjacent RBSs

Proposal 5: it is necessary to discuss how to handle PSCCH mapping case where the number of RBs configured for PSCCH are larger than that of subchannel with interlaces

Proposal 6: Study the resource mapping of PSCCH with either contiguous or non-contiguous interlace RBs

Proposal 7: Study the resource mapping of PSSCH/2nd SCI for SL-U

Proposal 8: It can be beneficial to consider new PSFCH format that supports more than one HARQ-ACK bit Proposal 9: It is preferred to firstly support one PSSCH transmission associated with one or more of PSFCH occasions and then, FFS PSFCH occasions are dynamically indicated for better PSFCH resource utilization

Reference

- RAN#109e-Meeting Chairman’s Note