3GPP TSG RAN WG1 #110bis-e	R1-2209790

e-meeting, October 10th – 19th, 2022

Source:	Sharp

Title:	Dynamic switching between DFT-S-OFDM and CP-OFDM for Rel-18 CovEnh

Agenda Item:	9.14.3

Document for:	Discussion and Decision

Introduction

In RAN#96-e, revised WID on Further NR coverage enhancements was endorsed [1]. One of the objective of this work item is to specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM as the following:

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, we share the view on Dynamic switching between DFT-S-OFDM and CP-OFDM for Rel-18 coverage enhancement.

Discussions

In order to support dynamic switching between DFT-S-OFDM and CP-OFDM, dynamic signaling from the network to the UE needs to be enhanced. There are two possible types of dynamic signaling, DCI and MAC-CE. As 1-bit indication would be sufficient for indication of whether DFT-S-OFDM or CP-OFDM is used for PUSCH transmission, carrying it via MAC-CE leads to unnecessary overhead. In contrast, adding 1-bit to DCI format is much more reasonable, as increased overhead is at most 1 bit.

Proposal 1: DCI is used for the indication of dynamic switching between DFT-S-OFDM and CP-OFDM.

As for the DCI format, at least DCI format 0_1 should support the indication of dynamic switching between DFT-S-OFDM and CP-OFDM. In addition, DCI format 0_2 is also used by cell-edge UEs, since it supports Rel-17 CovEnh features. Therefore, both DCI format 0_1 and 0_2 should support the indication of dynamic switching.

Proposal 2: DCI format 0_1 and 0_2 support the indication of dynamic switching between DFT-S-OFDM and CP-OFDM.

Another aspect to be discussed for support of dynamic switching between DFT-S-OFDM and CP-OFDM is the direction of switching. According to the WID, the problem of RRC based switching is “a large barrier to switch over to over to DFT-S-OFDM waveform for cell-edge UEs.” If this is the expected benefit, at least dynamic switching from CP-OFDM to DFT-S-OFDM needs to be introduced. On the other hand, it is not so clear if dynamic switching from DFT-S-OFDM to CP-OFDM provides some benefit in terms of the coverage enhancement. Therefore, it is suggested discussing whether to support dynamic switching from DFT-S-OFDM to CP-OFDM.

Proposal 3: Discuss whether support of dynamic switching from DFT-S-OFDM to CP-OFDM is necessary or not.

Conclusion

In this contribution, we have the following observation and proposal:

Proposal 1: DCI is used for the indication of dynamic switching between DFT-S-OFDM and CP-OFDM.

Proposal 2: DCI format 0_1 and 0_2 support the indication of dynamic switching between DFT-S-OFDM and CP-OFDM.

Proposal 3: Discuss whether support of dynamic switching from DFT-S-OFDM to CP-OFDM is necessary or not.

References

RP-221858, ”Revised WID on Further NR coverage enhancements,” China Telecom, RAN#96, June 2022.