3GPP TSG RAN WG1 #110bis-e		R1-2209813

e-Meeting, October 10–19, 2022

Source:	Panasonic

Title: 	Discussion on general aspects of AI/ML framework

Agenda Item:		9.2.1

Document for:	Discussion

Introduction

A new study item “Study on Artificial Intelligence and Machine Learning for NR air-interface” has been approved for Rel.18 [1]. This document discusses general aspects of AI/ML framework.

The agreement in the past meeting is described in Annex.

Discussion

TR skeleton

TR skeleton was discussed in the RAN1#109, but no specific conclusion was reached. Our concern was the original structure was not so clear, e.g., what section is handled by what agenda. With the latest version of R1-2205476, we are ok to the proposed TR skeleton.

Proposal 1: TR skeleton proposed in R1-2205476 should be agreeable.

Terminologies

On two-sided (AI/ML) model, the description of "gNB" would be typo from the network as the network internal mapping can be conducted outside of RAN1. So, we propose to revise as following.

| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by the networkgNB, or vice versa. |
| --- | --- |

Proposal 2: In the definition of Two-sided (AI/ML) model, "gNB" should be updated to "the network".

Network-UE collaboration levels

Following was agreed for network-UE collaboration levels for defining collaboration levels in RAN1#109.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

RAN1#110 had extensive discussion on the more accurate boundary on level x-y and level y-z. We further present our views in the following.

Level y-z boundary

Related to y-z boundary, the meaning of "model transfer" was discussed. We think following two aspects need to be discussed.

- Delivery method: Whether delivery is C plane (RRC, NAS) or U plane (OTT as the user traffic)

- Inference model: Whether the inference model is 3gpp standardized or not

The combination of two criteria can be summarized in the following table.

Table 1: The combinations of delivery method and inference model

| Delivery method | Inference model | Character |
| --- | --- | --- |
| C plane | Standardized | - This requires the most of 3gpp standardization in delivery method and inference model. - To deliver inference model over C plane can be inefficient if the traffic handling priority is similar to RRC or NAS message. How these are delivered would be transparent to RAN1 discussion. |
| U plane | Standardized | - From RAN1 perspective, inference model is standardized or not makes the big difference on the workload. - The inference model is delivered over U plane, but the inference model itself is standardized. The situation could be said similar to pre-configuration of sidelink is delivered as the user traffic, but the actual contents is specified. - To deliver inference model over U plane can be more efficient for large size of inference model data. The server to store inference model can be proprietary. |
| C plane | Proprietary | - Inference model is proprietary makes the amount of RAN1 work less. On the other hand, the indexes and characteristics of inference model still need to be commonly identified between UE and the network. - To deliver inference model over C plane can be inefficient if the traffic handling priority is similar to RRC or NAS message. How these are delivered would be transparent to RAN1 discussion. |
| U plane | Proprietary | - Inference model is proprietary makes the amount of RAN1 work less. On the other hand, the indexes and characteristics of inference model still need to be commonly identified between UE and the network. - To deliver inference model over U plane can be more efficient for large size of inference model data. The server to store inference model can be proprietary. |

Based on the above combination, there is not so much relation between delivery method and inference model. Delivery method is more of RAN2 and/or SA2 protocol design and mainly transparent to RAN1 discussion. RAN1 (and RAN4) is more influenced by inference model is standardized or not. The boundary discussion aimed to clarify the RAN1 discussion level. Therefore, the boundary should be based on the inference model is standardized or not and not based on the delivery is C plane or U plane. We propose following modification to level y and z. Instead of interpreting "model transfer", it should be interpreted that "3gpp based model" is transferred or not.

Proposal 3: To modify level y and z as following.

Level y: Signaling-based collaboration without "3gpp based model" transfer

Level z: Signaling-based collaboration with "3gpp based model" transfer

Note that the protocol layer to deliver 3gpp based model is up to RAN2/SA2 discussion.

Level x-y boundary

Related to x-y boundary, what "signalling" is the boundary was discussed. As discussed in level y-z boundary, our view is level y means the index and characteristics of inference model is identified commonly between UE and the network via 3gpp signalling. In level x, there is no common understanding on the index and characteristics of inference model between UE and the network. Therefore, we propose following.

Proposal 4: In Level x, the index and characteristics of inference model are not common understanding between UE and the network using 3gpp signalling. In Level y, the index and characteristics of inference model are common understanding between UE and the network using 3gpp signalling.

View on the model trained at UE

The AI/ML model trained by UE has the merit to optimize specific UE model, but it may not consider the overall network efficiency. Therefore, some test or verification would be necessary for the reasonableness of the AI/ML model before the deployment regardless of whether the model is conducted in UE or in the network. In case of online training, how these are carried is FFS. The test and verification of the AI/ML model trained by the network is up to the network implementation.

Proposal 5: The model trained at UE always needs some test or verification before the deployment. FFS on online training case.

View on the model download

In spite that AI/ML inference requires less number of algorithmic operations compared to the AI/ML training, power consumption and HW complexity are big concerns when AI/ML inference is conducted in UE. As reported in [2], power efficiency of AI/ML accelerator is 80 times to 200 times more efficient compared to a generic CPU. In order to reduce the complexity, instead of the floating number based operation, fixed point number based operation with specific bit-width is also necessary. To have reduced power consumption with reduced complexity, we think some reference model would be required for the model download. In speech codec, the specification is written based on fixed point reference model [3][4]. Although the same model with speech codec would not be able to be used, the way of the standardization may be useful to look.

Proposal 6: The model download would require some reference model to allow the HW accelerator with fixed point calculation to reduce the power consumption and to reduce the complexity in UE.

Relation between model activation and switching

The model activation would be used to enable the specific model ON. The model switching would be used to switch from current enabled model to the other model. The difference of two is whether model is currently enabled or not. For the newly enabled model, we do not expect the difference whether the other model was previously enabled or not. In 3gpp, such case was integrated to "activation" as often used for SPS/CG operation. Therefore, we propose following.

Proposal 7: The model activation can be used regardless some other model was activated or not. There is no need to define model switching.

Conclusion

This document discussed general aspects of AI/ML framework. We propose following.

Proposal 1: TR skeleton proposed in R1-2205476 should be agreeable.

Proposal 2: In the definition of Two-sided (AI/ML) model, "gNB" should be updated to "the network".

Proposal 3: To modify level y and z as following.

Level y: Signaling-based collaboration without "3gpp based model" transfer

Level z: Signaling-based collaboration with "3gpp based model" transfer

Note that the protocol layer to deliver 3gpp based model is up to RAN2/SA2 discussion.

Proposal 4: In Level x, the index and characteristics of inference model are not common understanding between UE and the network using 3gpp signalling. In Level y, the index and characteristics of inference model are common understanding between UE and the network using 3gpp signalling.

Proposal 5: The model trained at UE always needs some test or verification before the deployment. FFS on online training case.

Proposal 6: The model download would require some reference model to allow the HW accelerator with fixed point calculation to reduce the power consumption and to reduce the complexity in UE.

Proposal 7: The model activation can be used regardless some other model was activated or not. There is no need to define model switching.

- Reference

[1]	RP-213599, “New SI: Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface,”	Qualcomm (Moderator), RAN#94e.

[2] N. Jouppi et al., “In-Datacenter Performance Analysis of a Tensor Processing Unit,” Proc. 44th Annual International Symposium on Computer Architecture (ISCA), 2017

[3] ITU-T G.191 : Software tools for speech and audio coding standardization

[4] ITU-T Software Tool Library (STL) https://github.com/openitu/STL

- Past agreements

Agreements in RAN1#109:

Agreement

Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Table: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Agreements in RAN1#110:

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIsNote: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note:

Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.