3GPP TSG-RAN WG1 Meeting #110-bis-e	R1-2209818

e-Meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Type 1 HARQ codebook generation in TS38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - When multiple PDSCHs scheduling by DCI 1-1 is configured and time domain bundling is not configured and slot aggregation factor is larger than 1 for PDSCH scheduled by for DCI 1-2, the pruning of row r only belongs to TDRA for DCI 1-1 is still based on the UL collsion status for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, which is not align with the agreement. - The pruning rules of row r is based on UE processing capability in current specification, which is not aligned with the agreement that the pruning rule is based on the TDRA table, K1 set and TDD configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Restrict the usage of the pruning rule of “ for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD” not to the row r belongs to DCI 1-1 PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is configured. - Update the pruning rule for row r only belongs to TDRA of DCI 1-1 when PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not aligned with the agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

*** Unchanged text omitted ***

Set j=0 - index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update

Set B=∅

Set MA,c=∅

Set CK1 to the cardinality of set K1

Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 for serving cell c

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and enableTimeDomainHARQ-Bundling are provided for serving cell c

R=RT;

R'=RT';

elseif PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and enableTimeDomainHARQ-Bundling is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R not only belongs to time domain resource allocation table configured for DCI format 1_1 or PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is not provided, where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and at least one symbol of the PDSCH time resource derived by row r in slot n0,k+nD nUis configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R only belongs to time domain resource allocation table configured for DCI format 1_1

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R

while R≠∅

Set r=0

while r<CR

if S≤m for start OFDM symbol index S for row r

br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r

R=R\r;

B=B∪br,k,nD;

else

r=r+1;

end if

end while

MA,c=MA,c∪j

j=j+1;

Set m to the smallest last OFDM symbol index among all rows of R;

end while

end if

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

while k<CK1

if modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and enableTimeDomainHARQ-Bundling are provided for serving cell c

R=RT;

R'=RT';

elseif PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and enableTimeDomainHARQ-Bundling is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2, and slot  n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R not only belongs to time domain resource allocation table configured for DCI format 1_1 or PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is not provided, where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1 or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and  at least one symbol of thePDSCH time resource derived by row r in slot n0,k+nD nUis configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R only belongs to time domain resource allocation table configured for DCI format 1_1

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R

while R≠∅

Set r=0

while r<CR

if S≤m for start OFDM symbol index S for row r

br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r

R=R\r;

B=B∪br,k,nD;

else

r=r+1;

end if

end while

MA,c=MA,c∪j;

j=j+1;

Set m to the smallest last OFDM symbol index among all rows of R;

end while

end if

nD=nD+1;

end if

end while

end if

k=k+1;

end while

end if

*** Unchanged text omitted ***