3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2209841

e-Meeting, October 10 – 19, 2022

Agenda Item:	7.2

Source:	Huawei, HiSilicon

Title:	Discussion on the PRS search window

Document for:	Discussion and decision

# Introduction

In RAN2#119-e, the issue related to PRS search window was discussed with the decision documented as below [1].

[AT119-e][407][POS] Rel-15/16 LPP (Qualcomm) Scope: Evaluate the CRs in R2-2207103/R2-2207104/R2-2207870/R2-2207871/R2-2207872/R2-2208069/R2-2208070/R2-2208071/R2-2208121/R2-2208123 and check for agreeability (discussion document in R2-2208119 can be taken into account). Intended outcome: Agreed CRs, and report in R2-2208962 Deadline: Tuesday 2022-08-23 1200 UTC R2-2208962 Summary of [AT119-e][407][POS] Rel-15/16 LPP (Qualcomm) Qualcomm Incorporated discussion - Noted (email discussion [AT119-e][407]) R2-2208119 Issues with DL-PRS Search Window Definitions Qualcomm Incorporated discussion - Noted (email discussion [AT119-e][407]) R2-2208121 Correction to DL-PRS Search Window calculation Qualcomm Incorporated CR Rel-16 37.355 16.8.0 0375 - F NR_pos-Core - Not pursued (email discussion [AT119-e][407]) R2-2208123 Correction to DL-PRS Search Window calculation Qualcomm Incorporated CR Rel-17 37.355 17.1.0 0376 - A NR_pos-Core - Not pursued (email discussion [AT119-e][407])

The issue of PRS search window was discovered in R2-2208119 [2], where the problem was originated with the following field description of NR-DL-PRS-SFN0-Offset with a negative expected RSTD value.

NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9),

...}

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequent radio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset is counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP, rounded down to multiples of subframes.

In addition, how the search window is determined based on expected RSTD and expected RSTD uncertainty is captured as below.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

# Recap on R2-2208119

In [2], the following problematic cases are described. For the sake of simplicity, we only consider the search window centre of the subframe 0 of SFN 0 for the target neighbouring TRP, since the remaining PRS position within is relatively straightforward.

## Case 1: Geometry RSTD = 0

Figure 3: Calculation of search window centre. [2]

For this case, sfn-Offset is 0, and integerSubframeOffset is 3 where the actual subframe offset is 3.75ms.

The expected RSTD = -0.25ms because the subframe boundary of the neighbouring TRP is 0.25ms ahead of that of the reference TRP.

The discussion in [2] implied that for this case when the expected RSTD is negative, the search window centre of the subframe 0 of SFN 0 of the target neighbouring TRP should add another 1 millisecond as calculated by

Tref + 10sfn-Offset + integerSubframeOffset + 1 + nr-DL-PRS-ExpectedRSTD4Ts

However, later it was also proposed that when the geometry RSTD is not zero, this may not entirely be the solution.

## Case 2: Geometry RSTD not equal to 0 with negative RSTD and positive RTD

Figure 4: Calculation of search window centre. [2]

For this case, sfn-Offset is 0, and integerSubframeOffset is 3 where the actual subframe offset is 3.1ms.

The geometry RSTD is -0.2ms, which results in that the expected RSTD = -0.1ms.

The discussion in [2] implied that for this case when the expected RSTD is negative, the search window centre of the subframe 0 of SFN 0 of the target neighbouring TRP should not add another 1 millisecond as calculated by

Tref + 10sfn-Offset + integerSubframeOffset + nr-DL-PRS-ExpectedRSTD4Ts

## Case 3: Geometry RSTD not equal to 0 with negative RSTD and negative RTD

Figure 5: Calculation of search window centre. [2]

For this case, sfn-Offset is 0, and integerSubframeOffset is 0 where the actual subframe offset is 0.9ms.

The geometry RSTD is 0.05ms, which results in that the expected RSTD = -0.05ms.

The discussion in [2] implied that for this case when the expected RSTD is negative, the search window centre of the subframe 0 of SFN 0 of the target neighbouring TRP should add another 1 millisecond as calculated by

Tref + 10sfn-Offset + integerSubframeOffset + 1 + nr-DL-PRS-ExpectedRSTD4Ts

## Case 4: Geometry RSTD not equal to 0 with negative RSTD and positive RTD

Figure 6: Calculation of search window centre. [2]

For this case, sfn-Offset is 0, and integerSubframeOffset is 0 where the actual subframe offset is 0.05ms.

The geometry RSTD is -0.1ms, which results in that the expected RSTD = -0.05ms.

The discussion in [2] implied that for this case when the expected RSTD is negative, the search window centre of the subframe 0 of SFN 0 of the target neighbouring TRP should not add another 1 millisecond as calculated by

Tref + 10sfn-Offset + integerSubframeOffset + nr-DL-PRS-ExpectedRSTD4Ts

## Case 5: Geometry RSTD not equal to 0 with 1 subframe offset for the closest Tx subframe boundary and the closest Rx subframe boundary

Figure 7: Calculation of search window centre with rounding the integer subframe offset to closest subframe boundary. [2]

For this case, sfn-Offset is 0, and integerSubframeOffset is 0 where the actual subframe offset is 0.55ms.

The geometry RSTD is -0.1ms, which results in that the expected RSTD = 0.45ms.

The discussion in [2] implied that for this case when rounding of closest subframe in Tx and rounding of closest subframe in Rx wraps at 0.5ms boundary, the search window centre of the subframe 0 of SFN 0 of the target neighbouring TRP should not add another 1 millisecond as calculated by

Tref + 10sfn-Offset + integerSubframeOffset + nr-DL-PRS-ExpectedRSTD4Ts

## Summary

It is shown in [2] that because of misalignment between rounding down when it comes to SFN offset and subframe offset, and rounding to the closest subframe when it comes to expected RSTD, it is very complicated either from system or from UE procedure to specify a simple solution, and thus [2] proposed to include a new signalling from LMF to indicate whether additional one subframe offset is needed.

The discussion in RAN2 was as summarized in [3], where most comments believed that this issue should be discussed in RAN1/RAN4 first.

| Company | Essential Correction Yes/No | Comments |
| --- | --- | --- |
| Intel |  | The contribution raised a good point. It is worth to dig into the details. However should not this be discussed in RAN1/4 first? |
| Qualcomm | Yes (proponent) | To Intel: This was introduced by RAN2 via R2-2007836 at RAN2#111-e. The issue was (partly) already discovered in R2-2007836: "In this version of specification, only non-negative values shall be signalled." The above sentence, however, has been deleted in the agreed CR since it covers only part of the problem. The issue has then been "forgotten". I don't think this can be fixed in RAN1/4. |
| Huawei, HiSilicon |  | Agree with Intel that this can be discussed in R1/4 first |
| ZTE |  | Agree with Intel and HW |
| vivo |  | Agree with above that RAN1/4 should discuss first. |
| CATT |  | RAN2 can send an LS to RAN1 to confirm this issue and corrections since uncertainty is defined by RAN1. |
| Ericsson |  |  |
| Xiaomi |  | We think the issue can be discussed directly in RAN1/4. |

We would also like to comment to Qualcomm’s argument that R2-2007836 covers only part of the problem by citing “only non-negative values shall be signalled”. The so-called non-negative values in the CR R2-2007836 actually refers to expected RSTD uncertainty, instead of expected RSTD.

# Discussion

In our understanding, there does exist a simple solution that can work.

[2] discussed the possibility of changing the definition of the integerSubframeOffset to rounded towards the closest subframe boundary would be able to resolve most of the issues.

One possible solution may be to change the definition of the integerSubframeOffset: Instead of "rounding down to multiples of subframes", the subframe offset could be rounded towards the closest subframe boundary (i.e., round() operation instead of a fix() operation). For the examples in Figures 3 and 5, this would result in a correct integerSubframeOffset for the N calculations.

For this potential solution, [2] also had some doubts

This integerSubframeOffset would be incompatible with the fine RTD for UE-based assistance data

It also fails for the case where the transmit timing difference and receive timing difference have one leap subframe at the 0.5 boundary (case 5 in section 2.5 of the paper).

In our view, there is no need to align integerSubframeOffset with the fine RTD.

First, RTD in the UE-based assistance data is for the purpose of location calculation, which is used to express the transmission timing difference.

Second, NR-DL-PRS-SFN0-Offset that includes sfn-Offset and integerSubframeOffset is for the purpose of receiving DL-PRS for measurement.

A workaround is to relax NR-DL-PRS-SFN0-Offset as the one that can be jointly used used with expected RSTD for the search window determination. With this, the location server shall set a proper value so that the search window calculation based on the following equation is correct.

The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

- This is general aligned with the handling of network behaviour in the specification that RAN1/RAN2 specifications mainly specify procedures from UE perspective, leave most network implementation in accordance with the UE procedure.

- Observation 1: There is no such need to specify how LMF sets the specific value of nr-DL-PRS-SFN0-Offset, e.g. rounding-down or rounding-to-the-closest, but only to ensure that the value that LMF sets shall is correct following UE’s assumption in the procedure text.

For case 1, integerSubframeOffset can be set by the location server to 4.

- For case 2, integerSubframeOffset can be set by the location server to 3.

- For case 3, integerSubframeOffset can be set by the location server to 1.

- For case 4, integerSubframeOffset can be set by the location server to 0.

- For case 5, integerSubframeOffset can be set by the location server to 0

- Based on the discussion above, we have the following proposal.

- Proposal 1: It is RAN1 understanding that sfn-Offset and integerSubframeOffset in NR-DL-PRS-SFN0-Offset is not required to be specified as round-down or round-to-the-closest, but should be set a value by the network to ensure that the UE assumption is correct when to determine the search window centre.

The definition of sfn-Offset and integerSubframeOffset could be changed to reflect that intention.

We could also suggest the following change to RAN2 for their consideration.

- Proposal 2: Include the following text proposal in the LS to RAN2.

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequentradio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset corresponds to the number of full subframes counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP. NOTE: Network shall set the value in accordance with the defined search window for the target device using nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

# Conclusion

In this contribution, we analysed the issue of PRS search window that was initially discussed in RAN2 and the analysis leads to the following proposals:

- Proposal 1: It is RAN1 understanding that sfn-Offset and integerSubframeOffset in NR-DL-PRS-SFN0-Offset is not required to be specified as round-down or round-to-the-closest, but should be set a value by the network to ensure that the UE assumption is correct when to determine the search window centre.

The definition of sfn-Offset and integerSubframeOffset could be changed to reflect that intention.

- Proposal 2: Include the following text proposal in the LS to RAN2.

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequentradio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset corresponds to the number of full subframes counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP. NOTE: Network shall set the value in accordance with the defined search window for the target device using nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

# Reference

- R2-2208704, Report from session on positioning and sidelink relay, Session chair (MediaTek), RAN2#119-e, Online, 17-26 August, 2022.

- R2-2208119, Issues with DL-PRS Search Window Definitions, Qualcomm, RAN2#119-e, Online, 17-26 August, 2022.

- R2-2208962, Summary of [AT119-e][407][POS] Rel-15/16 LPP (Qualcomm), Qualcomm, , RAN2#119-e, Online, 17-26 August, 2022.