3GPP TSG RAN WG1 #110bis-e			     R1-2209906

e-meeting, October 10th – 19th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on co-channel coexistence of LTE-SL and NR-SL

Agenda Item:	9.4.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#110 meeting [1], there was discussion on co-channel coexistence of LTE-SL and NR-SL. In this contribution, we share our further views on co-channel coexistence of LTE-SL and NR-SL.

- Discussion

  - Subcarrier spacing in dynamic resource pool sharing

Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools

NR V2X services are not intended to replace the LTE V2X services but competent them for advanced services. NR V2X performance requirements are stricter than LTE V2X to support advanced services, especially from the perspective of latency and reliability. To achieve strict performance requirements in NR V2X, SCS other than 15kHz and HARQ feedback is newly introduced in NR sidelink. The Rel 16/17 NR SL devices have 30kHz SCS capability as mandatory and are intended to be typically operated with 30kHz SCS in ITS band in our understanding and we think that NR sidelink functional degradation should be minimized also in dynamic sharing scenario.

- RAN1 should study how to support dynamic resource pool sharing with 30kHz SCS.

  - PSFCH transmission handling in dynamic resource pool sharing

Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR PSFCH For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set.

At the last meeting, RAN1 discussed how to handle PSFCH transmission overlapped with LTE SL transmission in dynamic resource pool sharing and the above two alternatives were identified. In this section, we will discuss whether each alternative should be supported or not and the details.

    - Avoiding PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions.

We believe that Alt 1 is achieved by the simple method based on the existing resource exclusion/selection functions using shared LTE information. And from the point of view of keeping LTE SL reliability without LTE specifications changed, we should consider what NR SL systems possibly can do as much as possible.

- RAN1 should support avoiding PSFCH transmission in time slots that overlap with subframes used for LTE transmissions (Alt1).

Then, Alt 1 includes two sub-alternatives. We believe that behavior performed by UE transmitting PSSCH should be supported.

In case of avoiding PSFCH transmission performed by UE transmitting PSSCH, there are some merits such as lower complexity of UE processing and less impact on spec. (compared to UE transmitting PSFCH). For example, for NR transmission requiring HARQ feedback, the UE can perform resource exclusion/selection considering whether PSFCH slots corresponding to the PSSCH resources is used for LTE transmissions. This procedure would be aligned with UE behavior for NR transmission not requiring HARQ feedback, where the UE may select PSSCH resources considering LTE reservation.

The general processes are identical so the UE can work with common process to reduce specification and implementation complexity.

Figure 1: The image of resource exclusion/selection considering PSFCH resources in Alt 1

In case of avoiding PSFCH transmission performed by UE transmitting PSFCH, there are some demerits such as possible resource inefficiency as higher complexity of UE processing and much impact on spec. (compared to UE transmitting PSSCH):

one straight way is that the UE can perform following steps:

(1)NR SL module requests LTE module to provide LTE reservation information when NR module receives PSSCH with PSFCH configured.

(2)LTE SL module provides all reservation information immediately.

(3)NR module drops PSFCH transmission if the PSFCH slot is used for LTE transmission.

There seems to be some issues. One is that processing time required for sharing LTE information, interpreting it and determining whether to transmit PSFCH cannot be in time for the timing of HARQ feedback slots (minimum offset between PSSCH reception and HARQ feedback is 2 slots). Second is that if Rx UE determines not to transmit HARQ feedback, Tx UE is going to re-transmit same data, that causes inefficient use of limited resources.

And other ideas are possible, for example, Rx UE transmits HARQ feedback using the next available PSFCH slot. However, these ideas are undesirable because they increase UE complexity due to the addition of new UE procedure and cause delays.

- The way to avoid PSFCH transmissions performed by the UE transmitting PSSCH has an advantage in terms of complexity or resource efficiency over by UE transmitting PSFCH.

- In Alt 1 for handling NR PSFCH slots, support at least avoiding by UE transmitting PSSCH.

  - Whether to support avoiding PSFCH transmission by UE transmitting PSFCH is further studied.

    - NR SL UEs use a periodically repeating set of PSFCH slots.

Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR PSFCH For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set.

As some companies mentioned in RAN1#110, it is expected that LTE transmission is suppressed by LTE S-RSSI measurement of the slots that is periodically occupied by NR PSFCH and/or NR PSCCH/PSSCH transmissions. This is Alt 2 above.

Here, considering the case that one LTE resource pool and one NR resource pool are partially overlapped in frequency domain (but fully overlapped in time domain) as shown in Figure 2. The overlapped region is used by both LTE SL UEs and NR SL UEs but LTE resource exclusion is performed based on S-RSSI measurement only for sub-channels belonging to the LTE resource pool. Then, the LTE RSSI measurements of NR-PSFCH slots may not be calculated higher than other slots and it is impossible to properly exclude NR-PSFCH slot on the LTE module/device, that cases AGC issue on NR-PSFCH slots.

Figure 2: LTE resource exclusion based on LTE RSSI measurement in Alt 2

- If LTE and NR resource pools are misaligned in frequency domain, LTE module/devices will not exclude NR-PSFCH slots properly.

Even if LTE resource pool and NR resource pool are fully overlapped in time and frequency domain, since the step of resource exclusion based on LTE SCI is the previous step before the step of S-RSSI based exclusion, this way (Alt2) will not work properly when LTE resource pool is congested, and resources are not remained until the step (8) as specified [TS 36.213 Clause 14.1.1.6].

- In situation where LTE resource pool is congested, Alt 2 for handling NR PSFCH slots is possible not to work properly.

- On how to handle NR-PSFCH slot overlapped with LTE-SL transmission, deprioritize Alt 2 for handling NR PSFCH slots.

  - Inter-UE coordination at device Type A

Given the concept of inter-UE coordination in Rel17 SL, it is desirable for SL UEs to coordinate with each other, using as much information as possible that their own devices have. Therefore, Rel18 type A devices that have both LTE module and NR module that can get LTE reservation information should be able to use LTE reservation information in IUC as well as NR information.

Concretely in IUC scheme2 when Rel18 type A device detected that the resources indicated by LTE information shared by own LTE module and the resources detected by own NR module are overlapped, the collision is notified to the other Rel18 SL UE reserving overlapping resources. In IUC scheme1, when Rel18 SL UE transmits data to another Rel18 SL UE or is requested by another Rel18 SL UE, LTE reservation and/or LTE half-duplex information shared from its own LTE module is also taken into account to inform preferred/non-preferred resources. These coordination schemes will be useful to mitigate the collision due to hidden-node issue and/or half-duplex issue same as Rel17 IUC schemes to increase LTE/NR reliability in dynamic sharing resource pool.

Figure 3: The image of IUC schme2 using LTE reservation info.

- For dynamic resource pool sharing, LTE-SL reservation information is used for IUC scheme 1 and scheme 2.

  - Details of dynamic resource sharing at device type A

- Which information is shared

As mentioned, no spec impact on LTE-SL is assumed. From this point, complicated information selection at LTE-SL module would not be preferred. Simply, LTE-SL module shares all reservation information obtained from a window [n−10×Pstep, n−1] to NR-SL module when the information sharing is triggered at subframe n, and whether each reservation information is considered or not should be handled at NR-SL module. Besides, information relevant to half-duplex slot can be shared so that corresponding slots are excluded.

- For device type A in dynamic resource sharing, when information sharing is triggered at subframe n,

  - LTE-SL module shares all reservation information with RSRP/PPPP and all half-duplex slots within a window [n−10×Pstep, n−1] to NR-SL module.

- Resource exclusion

Reservation information shared from LTE-SL includes reservations with any RSRP and any priority (PPPP). Whether corresponding NR-SL resources are excluded or not should be handled by using the RSRP/priority information. That is, resource exclusion from a set SA should be performed as the current step 6 of 8.1.4 in 38.214, unlike non-preferred resources of step 6b. In addition, resource exclusion from a set SA based on LTE half-duplex slot should also be performed as the current step 5.

- For device type A in dynamic resource sharing,

  - NR-SL module performs resource exclusion as in step 5 and as in step 6 based on information shared from LTE-SL module.

- Conclusion

In this contribution, we discussed co-channel coexistence of LTE-SL and NR-SL. Observations/Proposals are summarized as following:

- RAN1 should study how to support dynamic resource pool sharing with 30kHz SCS.

- RAN1 should support avoiding PSFCH transmission in time slots that overlap with subframes used for LTE transmissions (Alt1).

- The way to avoid PSFCH transmissions performed by the UE transmitting PSSCH has an advantage in terms of complexity or resource efficiency over by UE transmitting PSFCH.

- In Alt 1 for handling NR PSFCH slots, support at least avoiding by UE transmitting PSSCH.

  - Whether to support avoiding PSFCH transmission by UE transmitting PSFCH is further studied.

- If LTE and NR resource pools are misaligned in frequency domain, LTE module/devices will not exclude NR-PSFCH slots properly.

- In situation where LTE resource pool is congested, Alt 2 for handling NR PSFCH slots is possible not to work properly.

- On how to handle NR-PSFCH slot overlapped with LTE-SL transmission, deprioritize Alt 2 for handling NR PSFCH slots.

- For dynamic resource pool sharing, LTE-SL reservation information is used for IUC scheme 1 and scheme 2.

- For device type A in dynamic resource sharing, when information sharing is triggered at subframe n,

  - LTE-SL module shares all reservation information with RSRP/PPPP and all half-duplex slots within a window [n−10×Pstep, n−1] to NR-SL module.

- For device type A in dynamic resource sharing,

  - NR-SL module performs resource exclusion as in step 5 and as in step 6 based on information shared from LTE-SL module.

References

- 3GPP RAN1#110 meeting,	chair’s notes