3GPP TSG RAN WG1 Meeting #110bis-e    	            R1-2209962

e-Meeting, October 10th – 19th, 2022

Source:	Qualcomm Incorporated

Title:	Discussion on clarification for cross-carrier scheduling from SCell to P(S)Cell

Agenda Item:	8.13

Document for: 	Discussion and Decision

# Discussion

According to TS 38.331 [1], the IE CSI-MeasConfig is used to configure channel state information reports to be transmitted on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. CSI-MeasConfig includes a list of CSI-ReportConfig, a reportTriggerSize, an aperiodicTriggerStateList, and a semiPersistentOnPUSCH-TriggerStateList. In other words, for cross-carrier scheduling, these RRC parameters necessary for A-/SP-CSI report on PUSCH are provided on the scheduling cell, not on each scheduled cell.

When a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, the UE monitors PDCCH on P(S)Cell and sSCell for PDSCH/PUSCH on P(S)Cell. A DCI format 0_1/0_2 on P(S)Cell or sSCell can trigger A-CSI on PUSCH or activate SP-CSI on PUSCH on P(S)Cell.

The current specification can be interpreted such that if a DCI format 0_1/0_2 on P(S)Cell triggers A-CSI report on PUSCH on P(S)Cell, the CSI report related configurations for A-CSI on PUSCH provided on the P(S)Cell is used, and if that on sSCell triggers A-CSI report on PUSCH on P(S)Cell, the CSI report related configurations for A-CSI on PUSCH provided on the sSCell is used.

However, this creates an issue for a UE. Consider the following case:

- A UE indicates the UE capability signalling maxNumberAperiodicCSI-triggeringStatePerCC = 128 for both bands of P(S)Cell and sSCell, respectively.

- CSI-MeasConfig on both P(S)Cell and sSCell configure reportTriggerSize = 6 and the number of entries in CSI-AperiodicTriggerStateList = 128, respectively.

- Different CSI report configurations are provided to each entry of CSI-AperiodicTriggerStateList for P(S)Cell and sCell, i.e., a codepoint of the CSI request field of a DCI format points to different CSI aperiodic trigger states depending on whether the DCI format is received on P(S)Cell or sSCell.

Eventually, it looks that a UE would need to be able to handle up to 128 + 128 = 256 CSI aperiodic trigger states for A-CSI on PUSCH scheduled on P(S)Cell.

We propose to discuss and select one of the following clarifications for cross-carrier scheduling from sSCell to P(S)Cell:

- Opt.1: Regardless of whether the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell or sSCell, the UE refers to CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell for A-/SP-CSI on PUSCH on P(S)Cell

- Opt.2: Depending on whether the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell or sSCell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell or sSCell. Network ensures that the parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that joint set of these parameters across P(S)Cell and sSCell does not result in exceeding any of the UE’s capabilities

- Opt.3: The CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell and sSCell are identical such that the UE can refer to the parameters provided on either of the P(S)Cell and sSCell for A-/SP-CSI on PUSCH on P(S)Cell

RAN1/RAN2 spec impact can be further discussed once consensus is achieved.

Proposal: RAN1 to clarify either of the following:

- Opt.1: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell, regardless of whether the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell or sSCell.

- Opt.2: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, , for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell, and refers to those provided in CSI-MeasConfig of sSCell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on sSCell. Network ensures that the parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that joint set of these parameters across P(S)Cell and sSCell does not result in exceeding any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell, such as:

  - maxNumberAperiodicCSI-triggeringStatePerCC

  - maxNumberAperiodicCSI-PerBWP-ForCSI-Report

  - maxNumberAperiodicCSI-PerBWP-ForBeamReport

  - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report

  - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport

- Opt.3: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell and sSCell are identical such that the UE can refer to the parameters provided on either of the P(S)Cell and sSCell for A-/SP-CSI on PUSCH on P(S)Cell.

# Conclusion

We propose to discuss a clarification for Rel-17 DSS cross-carrier scheduling from sSCell to P(S)Cell as follows.

Proposal: RAN1 to clarify either of the following:

- Opt.1: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell, regardless of whether the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell or sSCell.

- Opt.2: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, , for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell, and refers to those provided in CSI-MeasConfig of sSCell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on sSCell. Network ensures that the parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that joint set of these parameters across P(S)Cell and sSCell does not result in exceeding any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell, such as:

  - maxNumberAperiodicCSI-triggeringStatePerCC

  - maxNumberAperiodicCSI-PerBWP-ForCSI-Report

  - maxNumberAperiodicCSI-PerBWP-ForBeamReport

  - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report

  - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport

- Opt.3: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell and sSCell are identical such that the UE can refer to the parameters provided on either of the P(S)Cell and sSCell for A-/SP-CSI on PUSCH on P(S)Cell.

# Reference

- 3GPP TS 38.331