3GPP TSG RAN WG1, Meeting #110bis-e		    R1- 2209984

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.3.3

Source: 	Qualcomm Incorporated

Title: 	On potential enhancements on dynamic/flexible TDD

Document for:	Discussion/Decision

# Introduction

RAN plenary #94e, the study item on evolution of NR duplex operation was approved [1]. The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In RAN1 #109e, the discussion on dynamic/flexible TDD schemes and solutions were kicked off and few agreements were made in [3] addressing different issues. In this contribution, we continue the discussion on the feasibility and potential enhancement for dynamic/flexible TDD.

High level agreements made in RAN1 #109e for agenda 9.3.3 are listed below. Based on the guideline below, agenda 9.3.3 will handle the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic TDD.  In this contribution, we will discuss the potential solutions for inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic TDD. The potential solutions that are specific for SBFD will be discussed in agenda 9.3.2.

Agreement - For discussion in AI 9.3.3, consider the deployment scenarios for dynamic/flexible TDD which are agreed for evaluation purpose under AI 9.3.1 in RAN1#109-e. - Under AI 9.3.3., no more discussion about the deployment scenario for potential enhancement on dynamic/flexible TDD Agreement At least, following interference scenarios can be considered for study of dynamic/flexible TDD: - gNB-to-gNB inter-cell co-channel interference - UE-to-UE inter-cell co-channel interference Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD. Conclusion The following self-interference scenario and inter-subband CLI scenarios are not considered under AI 9.3.3 (Potential enhancements on dynamic/flexible TDD). - gNB self-interference - UE-to-UE intra-cell co-channel inter-subband CLI - UE-to-UE inter-cell co-channel inter-subband CLI - gNB-to-gNB inter-cell co-channel inter-subband CLI

In RAN1 #110, the discussion on dynamic/flexible TDD schemes and solutions continued, based on the agreed list in RAN1 #109e of candidates of potential enhancement method of gNB-to-gNB and UE-to-UE CLI handling. In RAN1 #110, companies further discussed and focused on a subset of the candidate schemes for study. A couple of agreements were made in this meeting. In this contribution, we continue the discussion on the feasibility and potential enhancement for dynamic/flexible TDD and focus on this subset of candidate schemes.

# Summary of Rel-16 study on dynamic TDD

In Rel-16, coexistence study for the dynamic TDD deployment across adjacent channels were conducted and outcome was captured in TR 38.828 [2]. The main focus of the study was the on co-existence of CLI among different operators in adjacent channels. The outcome of this study showed that there are deployment scenarios where performance is degraded and dynamic TDD is not suitable. While there are some deployment scenarios where little to no performance degradation was observed. The next is summary of the simulations results and recommendations for different scenarios.

## Deployment Scenarios with performance degradation

In FR1, performance degradation due to BS-to-BS interference is observed in macro-to-macro deployments. For indoor-to-indoor scenarios, some performance degradation is observed under high BS transmit power. For macro-to-indoor, performance degradation exists in some scenarios due to UE-to-UE interference. In FR2, BS-to-BS interference leads to some performance degradation in macro-to-macro deployments and some scenarios in indoor-to-indoor and micro-to-micro deployments. Note, in this study that interference from co-located aggressor BS leads to Rx blocking at victim BS in all scenarios and no simulation results were provided for this case.

## Deployment Scenarios without performance degradation

In FR1, no performance degradation is observed due to BS-to-BS interference for in indoor-to-macro and macro-to-indoor deployments. UE-to-UE interference has no impact on performance in macro-to-macro, indoor-to-macro, and indoor-to-indoor deployments. In FR2, no performance degradation is observed due to BS-to-BS interference in indoor-to-macro and macro-to-indoor deployments. UE-to-UE interference has no impact on performance in macro-to-macro, indoor-to-macro, indoor-to-indoor and micro-to-micro deployments.

## Outcome and Recommendation

The following recommendations are made regarding feasibility of dynamic TDD in FR1. Dynamic TDD is not suitable for macro-to-macro deployments because of performance degradation caused by BS-to-BS interference. No performance degradation is expected when operating dynamic TDD between an indoor network and a macro network and vice versa assuming sufficient isolation. For indoor-to-indoor deployments, dynamic TDD is feasible if some conditions are met.

In FR2, for macro-to-macro deployments, dynamic TDD is feasible in certain deployments with coordination between operators to avoid performance degradation caused by BS-to-BS interference. Dynamic TDD is feasible in indoor-to-macro and macro-to-indoor deployments with sufficient isolation. For Indoor-to-Indoor deployments, dynamic TDD is feasible under careful layout planning and parameterization. Finally, dynamic TDD can also be used in certain micro-to-micro deployments with careful planning.

Observation 1: For FR1, deployments scenario with large Tx Power BS suffers from inter-gNB interference.

- In general, inter-UE CLI is not an issue except for macro-to-indoor deployment.

Observation 2: For FR2, Dynamic TDD is possible under careful assumption of layout and power parameterization to avoid inter-gNB interference.

# Scope of the potential enhancement of dynamic TDD

The study item description [1] has a clear note that for potential enhancements on dynamic/flexible TDD, RAN1 should utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. Given Rel-16 dynamic TDD study on inter-operator adjacent channel coexistence, this work should not be repeated. The focus of Rel-18 SI on potential enhancement on dynamic TDD should be limited to co-channel intra-operator deployment.

Observation 3: Rel-18 study on potential enhancement of dynamic TDD suggests utilizing the outcome of Rel-15 and Rel-16 studies outcome to avoid repetition of same discussion, e.g., inter-operator Dynamic TDD coexistence study.

Proposal 1: The focus of Rel-18 study on potential enhancement for dynamic TDD should be limited to co-channel intra-operator deployment.

# Feasibility of Dynamic TDD

## Dynamic TDD using non-overlapping subbands across gNB

To enable dynamic TDD deployment in FR1 macro cell deployment with flexile adaptation of slot format based on traffic, we considered subband half-duplex (SBHD) deployment. In asynchronous slots, where gNBs have different traffic direction, the frequency resources of this slot could be split into DL subband and UL subband as shown in Figure 4-1. This subband split provides frequency isolation between aggressor and victim gNBs which helps in mitigating inter-gNB CLI.

Figure 4-1: subband isolation to enable dynamic TDD

In the scenario where all slots are considered flexible subband as shown in Figure 4-2, it provides flexibility to adapt to traffic demand and addresses new hybrid topologies where use cases dictate use of different slot structures. In addition, the ability to select direction based on traffic profile, and reducing blocking delay. Also, the availabity of fore UL TX opportunities for cell-edge UEs helps improving UL coverage. Finally, this deployment can be achieved by utilizing existing radio HW. This can prove subband operation as a viable approach to mitigate cross-link interference. The only drawback of using all slots as flexible subband is potential loss of frequency resources that may leads to some performance loss for heavily loaded system. However, SBHD could be enabled for asynchronous slots.

Figure 4-2: SBHD slots structure across cells

Observation 4: SBHD can enable dynamic TDD and mitigate the impact of inter-gNB CLI.

Observation 5: SBHD-based dynamic TDD enables flexible adaption of slots direction based on traffic which leads to reduced latency and improved UL coverage.

To study the feasibility of macro-to-macro dynamic TDD using subband half-duplex mode, the following link-budget consider a macro base station with 45 dBm Tx Power and BS array gain of 20 dBi which gives 68 dBm of EIRP. The thermal noise power assuming 7 dB noise figure and 100 MHz system BW is -87 dBm.

|  | Out of subband blocker | subband jammer |
| --- | --- | --- |
| EIRP | 68 dBm |  |
| Inter-gNB Path Loss (500 m) | 98 dB | 98 dB |
| Subband Freq. isolation | 0 | 45 dB |
| shadowing | 5 dB | 5 dB |
| Tx/Rx beam nulling or beam isolation | 10~15 dB | 10~15 dB |
| Rx Power | - 45 ~ 50 dBm | -90 to -95 dBm |

Assuming sufficient Tx and Rx beamform nulling of 10 dB or higher, the residual interference within the UL subband is lower than thermal noise floor and amount of IoT is negligible.

Observation 6: Link budget analysis shows that SB-based dynamic TDD is feasible for macro-cell deployment.

Proposal 2: Support subband half-duplex as solution to enable dynamic TDD at least for FR1

## Qualcomm OTA SBHD Demonstration

It was demonstrated a 5G NR test network supporting subband half-duplex which enables Dynamic TDD and allows for more flexible service multiplexing as well as improve latency and coverage. In this OTA network operates in the 3.5 GHz spectrum with 100 MHz bandwidth, supporting two gNB with advanced multi-user massive MIMO and 5G NextGen Core. Two commercial devices, device 1 is a cell edge UE driving UL heavy bursty traffic served by gNB1 and device 2 is driving downlink heavy burst traffic served by gNB2. The Baseline scenario is fixed TDD with DDDU slot format. While in the SBHD network, different cells have different slots structure which allow adaptation of traffic needs and reduce switch latency. gNB1 has D’U’D’U’ slot format to deliver more UL opportunity while gNB2 has D’D’D’U’ slot format to serve the DL heavy user slot formant where D’ and U’ are DL and UL slots with subband mode. In slot 1 and 3 and 4 gNB1 and gNB2 are either transmitting on downlink or receiving on uplink and there is no cross-link interference. In slot 2, gNBs have different direction and the DL-to-UL interference is mitigated through the subband operation.

Compared to Device 1 uplink perceived throughput and latency is significantly improved due to increased UL duty cycle. Device 2 still gets 75% DL duty cycle from gNB2, and its downlink perceived throughput and latency are mostly not impacted.

Observation 7: A prototype test network validated the feasibility of dynamic TDD in macro-cell deployment using subband half-duplex.

## Dynamic TDD using fully overlapping UL and DL

At least for FR2, fully overlapping UL and DL for dynamic TDD shall be considered as illustrated in Figure 4-3. There could be aligned and misaligned slots across cells. For the misaligned dynamic TDD slots, there will be inter-cell inter-gNB and inter-UE CLI, which could be mitigated with beam isolation, proper beam-pair selection, lower Tx Power for FR2 deployment scenarios.

Figure 4-3 Dynamic TDD with aligned and misaligned slots

Observation 8: In FR2, Dynamic TDD with misaligned slots format is possible where CLI could be mitigated with proper beam-pair selection and lower Tx power.

# Inter-UE Cross-link interference mitigation techniques

An agreement related to inter-UE CLI was made in RAN1 #109e for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed on a list of candidates of potential enhancement method of UE-to-UE CLI handling where further prioritization/down-scoping of candidate schemes for study can be done in this meeting.

Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of UE-to-UE CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - Potential enhancements to UE-to-UE CLI measurement/reporting - Coordinated scheduling - Spatial domain enhancements, - Advanced Receiver - UE and gNB transmission and reception timing - Power control based solution - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancement specific for SBFD will be discussed in 9.3.2

In general, we support to study of potential enhancement to dynamic/flexible TDD and/or SBFD of candidates of potential enhancement method of UE-to-UE CLI handling: potential enhancements to UE-to-UE CLI measurement/reporting, coordinated scheduling, spatial domain enhancements, UE and gNB transmission and reception timing, power control-based solution, OTA or backhaul information exchange for some particular schemes. Benefits per candidate of potential enhancement will be addressed in following sections separately.

In our view, we prefer no further study on advanced receiver, and it could be up to UE implementation and the performance gain is unclear without simulation results. We also prefer no further study on sensing-based mechanism. Its potential benefit is not clear to us, considering the sensing complexity, accuracy, and possible delay.

To summarize, we support most of the candidates in the list highlighted in the agreement above other than “Advanced receiver” and “Sensing based mechanism”.

Proposal 3: Support to study the candidate’s solution for inter-UE CLI other than “Advanced receiver” and “Sensing based mechanism”.

In RAN1 #110, the discussion on dynamic/flexible TDD schemes and solutions continued, based on the agreed list in RAN1 #109e of candidates of potential enhancement method of UE-to-UE CLI handling. In RAN1 #110, companies further discussed and focused on a subset of the candidate schemes for study. A couple of agreements were made in this meeting and in this section, we continue the discussion on the UE-to-UE CLI handling especially on the subset of the candidate schemes agreed in RAN1 #110.

## Potential enhancements to UE-to-UE CLI measurement/reporting

An agreement related to potential enhancements to UE-to-UE CLI measurement/reporting for inter-UE CLI was made in RAN1 #110 for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed on a list of candidate areas of potential enhancement method of UE-to-UE CLI handling which will be further studied and discussed in this meeting.

We will discuss the proposals in separate sub-sections corresponding to each candidate areas: measurement resource/reporting configuration, measurement/reporting details (including UE processing delay), relevant information exchange (between gNBs) if needed, and usage of measurement at gNB.

Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB

### Measurement resource/reporting configuration

Rel-16 CLI framework is based on layer 3 reporting which inherently has limited flexibility. L3 reporting is carried out on PUSCH channel and is collected by gNB-CU first then communicated to the gNB-DU. This will introduce some latency on the CLI report availability to the gNB-DU. However, L1 or L2 based CLI reporting doesn’t suffer from such increased latency.  In addition, L3 reporting is based on periodic CLI measurement resources with L3 filtering. This is not suitable for enabling fast beam selection in response to interference variation as compared to L1 based beam selection. Furthermore, RRC reconfiguration is needed to update the configuration of the CLI measurement resource which is typically in order of tens of millisecond.

Compared with Rel-16 CLI reporting which has limited flexibility and slow adaptability, L1/L2 report can be obtained by gNB-DU with much lower latency, and hence can better reflect current CLI. In addition, L1 report can be sent on-demand to facilitate fast CLI mitigation.

Proposal 4: Support L1/L2 based inter-UE CLI measurement/reporting to increase flexibility and reduce reporting latency compared to Rel-16 L3 based framework.

In addition, L1-CLI report can reuse similar framework as CSI report as a baseline. L1 report can better reflect current CLI, and request CLI for intended beam with low latency when needed, e.g., upon traffic arrival. Similar to CSI report, L1-CLI report can be associated with periodic, semi-persistent, or aperiodic CLI resources and report types. L1 CLI report can also be configured as event triggered, for timely beam update in response to fast channel/interference variation. In particular, AP CLI reporting and AP SRS transmission for CLI measurement can be triggered by a UE-dedicated DCI or a new group-common (GC) DCI.

The CLI resource can be configured to measure the interference based on some reference signal. The CLI resource can be configured as an SRS resource, like Rel-16 framework or based on CSI-RS resource or CSI-IM resource. In addition, support to introduce L1-CLI report with P/SP/AP CLI resource types and P/SP/AP/event-triggered report types.

Proposal 5: L1 CLI measurement and report framework supports the following features

- Support L1-CLI report with P/SP/AP CLI resources and P/SP/AP/event triggered report types

- CLI resource can be configured similar to Rel-16 based on SRS or based on CSI-RS or CSI-IM.

An alternative to L1 CLI is the L2 CLI framework, where UE is configured with SP/P CLI measurement resources. UE measures CLI and sends CLI report via an UL MAC-CE. L2 CLI reporting can be based on SP reporting activated by a MAC-CE or triggered by an L2-event, e.g. measured interference exceeds a configured CLI threshold. Figure 5-1 shows an example on the signaling for L2 CLI. As compared to current L3 reporting, L2 CLI framework reduces much of the report latency and requires minimum specification impact.

Figure 5-1 L2 CLI framework

Proposal 6: Support L2 CLI reporting with at least event triggered. Report can be sent on UL MAC-CE.

### Measurement/reporting details

#### L1 measurement/reporting details

Rel-16 CLI framework supports CLI RSRP/RSSI measurement using SRS-RSRP and CLI-RSSI measurement resources, respectively. In addition, CLI can be incorporated into other metrics, e.g. SINR, CQI, where CLI can be considered as the interference source. For those cases, CLI measurement can be based on an interference measurement resource (IMR). The IMR can be linked to a CSI-IM resource. For more accurate CLI measurement, we can consider an enhanced CSI-IM resource, or a new resource dedicated for CLI measurement. For example, the enhanced CSI-IM resource or the new CLI measurement resource can be defined to have a comb pattern to better reflect CLI-RSRP based on SRS transmissions.

As for signalling, we can consider the following two design options for reporting L1 CLI.

- Separate L1 CLI reporting: L1 CLI reporting is configured as a new reporting quantity which indicates only the CLI metric to be reported, e.g., reportQuantity can be CLI-RSSI, CLI-RSRP, CLI-SINR, …, etc

- Joint L1 CSI and CLI reporting: L1 CLI reporting is configured as part of a new CSI reportQuantity indicating both CSI and CLI metrics to be included in the report, e.g., reportQuantity can be cri-RI-CQI-cli-RSSI

Proposal 7: L1 CLI measurement and report framework supports the following features

- CLI measurement metrics can be RSSI, RSRP

- CLI measurement can be incorporated into other CSI report metrics as the interference part, e.g. SINR, CQI

- CLI measurement can be based on CSI-IM (with enhanced patterns), or dedicated CLI measurement resource

- Separate or joint reporting of CSI and CLI

Current spec specifies timing requirements for CSI processing to guarantee that a UE has enough time to generate CSI report. Current spec defines three latency classes: low latency class (Z1), high latency class (Z2), and latency for beam-reporting (Z3). When UE is configured to report CLI, the computation delay requirement of the CLI report shall be studied to guarantee a UE has enough time to generate CLI report. Processing time may be clarified for L1-CLI, at least for separate CLI reporting. E.g. metrics similar to (Z, Z’) for CSI may be extended to L1-CLI as either fixed value in spec or up to UE capability. RAN1 to study UE CLI processing timeline and L1-CSI timeline can be used as baseline, e.g. whether to follow CSI computation delay requirement low-latency class (Z1) or latency for beam-reporting class (Z3) or a new separate latency class for CLI.

For example, AP CLI can be triggered by a DCI to perform measurement of AP CLI resources and perform aperiodic reporting on L1 based on CLI processing timeline. Support to reuse AP CSI timeline as baseline with different value for timeline of L1-CLI.

Figure 5-2 Example L1-CLI timeline

Proposal 8: RAN1 to study UE CLI processing timeline at least for separate CLI reporting starting with L1-CSI timeline as a baseline.

- e.g. reuse AP CSI timeline as baseline with different value for timeline of L1-CLI.

L1 report includes CSI report and L1-CLI report, and the priority is used when PUCCH is insufficient to transmit all L1 reports overlapped in time, or when total CPU consumed by all L1 reports on same symbol exceeds UE capability. A simple way could be considering L1-CLI as a special CSI report and define corresponding CSI report priority.

Proposal 9: RAN1 to study L1-CLI report priority and multiplexing when reported as UCI.

#### Common L1/L2/L3 measurement/reporting details

In R16, up to maxReportCLI most interfering CLI resources are reported in either periodic or event triggered report. However, for gNB SBFD, gNB may be more interested in which UE can be paired with the reporting UE with negligible CLI. For example, in the figure below, among the two measured CLI resources, UE1 only reports CLI resource #2 with least CLI caused by UE2. The CLI values can be sorted and ranked to select the least CLI caused by neighbor UE.

Proposal 10: In addition to most interfering CLI resources, UE can be configured to report top X least interfering CLI resources for CLI report.

Figure 5-3 Least interfering CLI resources for CLI report

Rel-16 CLI framework does not support subband CLI reporting, i.e., reporting CLI for one or more subbands in the measurement bandwidth. It supports wideband CLI reporting which may not be suitable for measuring CLI leakage from an adjacent subbands in SBFD, since CLI leakage to adjacent subbands is non-uniform over the measurement bandwidth.

In dynamic/flexible TDD operation, CLI leakage is non-uniform over the DL subbands and may require CLI reporting with increased frequency granularity. The UL and DL could be either fully overlapped, partially overlapped or non-overlapping as the case for subband half-duplex. This motivates that subband reporting could be also used for dynamic TDD e.g. for frequency resource selection on low CLI RB sets cross cells.

Proposal 11: Support subband-based CLI reporting to facilitate subband based scheduling for both SBFD and dynamic TDD in which CLI could be non-uniform across the DL RBs.

In our companion paper [4], we discuss in more details the subband-based CLI measurement and reporting.

### Relevant information exchange between gNBs

At least for inter-CU/vendor/operator scenarios, inter-gNB coordination is beneficial to support inter-UE CLI measurement across gNBs. Inter-UE CLI measurement resource configuration per measured UE and inter-UE CLI reporting could be exchanged via OTA or backhaul signalling. OTA signalling could be useful when there is no ideal backhaul, or when backhaul signalling has longer latency.

Proposal 12: Support to study OTA or backhaul information exchange between gNBs for inter-UE CLI mitigation at least for inter-CU/inter-vendor at least for intra-operator scenarios e.g.

- UE-to-UE CLI measurement resource configuration and OTA reporting triggering criteria between gNBs including time/frequency resources and beam indication for inter-UE CLI measurements between gNBs

- UE-to-UE CLI reporting contents including CLI metric per CLI resource

### Usage of measurement at gNB

In our view, this could be up to gNB implementation. gNB can use UE-to-UE CLI measurement and reporting for scheduling UE(s) in the cell to help mitigate intra-cell inter-UE CLI, and also can use the coordinated and exchanged information between gNBs for gNB coordinated UE scheduling to help mitigate inter-cell inter-UE CLI.

Observation 9: UE-to-UE CLI measurement and reporting can be used for gNB scheduling UE in the cell and for gNB coordinated UE scheduling between gNBs.

## Coordinated Scheduling

An agreement related to potential coordinated scheduling enhancements for inter-UE CLI was made in RAN1 #110 for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed on details of coordinated scheduling for time/frequency resources and relevant information exchange for UE-to-UE CLI handling which will be further studied and discussed in this meeting.

Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange (if needed)

In general, gNB can exchange scheduling info for time/frequency/spatial domains to mitigate inter-UE CLI. For example, with coordinated scheduling, different cells can exchange scheduling information on different frequency resources cross cells, so that one cell can avoid scheduling on the subband that overlaps with its neighbour cell to achieve frequency isolation for dynamic TDD. Such frequency isolation could be beneficial for inter-UE CLI mitigation in FR1, e.g. via SBHD. Therefore, we support coordinated scheduling for restricted resources including time/frequency/spatial domains.

Proposal 13: Coordinated scheduling information for time/frequency resources and corresponding UE information can be exchanged via OTA or BH signalling for inter-UE CLI mitigation.

## Spatial domain enhancements

An agreement related to potential spatial domain enhancements for inter-UE CLI was made in RAN1 #110 for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed on details of spatial domain coordination by gNB and relevant information exchange for UE-to-UE CLI handling which will be further studied and discussed in this meeting.

Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI handling based on spatial domain coordination method which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic /flexible TDD, at least includes: - Details for spatial domain coordination by gNB - Relevant information exchange (if needed) Note1: Study can include method for FR1 and FR2

### Details for spatial domain enhancements

Rel-16 CLI framework does not support signalling/configuration of Rx beam (QCL-D) for CLI measurement. Specifically, Rx beam for CLI measurement is up to UE implementation as QCL-D follows one of the latest received PDSCH and the latest monitored CORESET. In turn, gNB cannot characterize CLI for different Rx beams in order to enable CLI-aware beam management.

Proposal 14: Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware beam management.

In R17 IAB, to mitigate self-interference between IAB-DU and IAB-MT, beam coordination is introduced between IAB-MT link and IAB-DU link. Specifically, IAB node can indicate preferred IAB-MT DL/UL beams to its parent node, while its parent can indicate restricted IAB-DU DL/UL beams to the IAB node. The R17 IAB framework can be extended to SBFD for UE to dynamically report a set of recommended beams, not preferred beams, or both, to mitigate inter-UE CLI.

Figure 5-4 Beam coordination between IAB node and parent node in R17 IAB

Proposal 15: UE can dynamically report to the gNB a set of recommended beams, not preferred beams, or both.

- gNB configures multiple Rx (QCL-D) beams for UE to measure

- UE determines the recommended and/or not preferred beams based on measurement of inter-UE CLI using different RX beams (QCL-D)

In addition, similar to R17 IAB, gNB in SBFD can indicate restricted UE beam/panel to mitigate inter-UE CLI. As illustrated in the figure below, gNB can inform UE that when a particular SSB is used as QCL source to indicate UE Rx/Tx beam, UE shall not use Rx/Tx beam on a particular panel to mitigate inter-UE CLI. Note that which panel to use is currently up to UE implementation.

Figure 5-5 gNB indicates restricted UE Rx/Tx panels to mitigate inter-UE CLI

Proposal 16: IAB framework can be extended to gNB SBFD/D-TDD for gNB to indicate restricted UE beam/panel.

To mitigate the inter-UE CLI, gNB can configure slot-specific UE DL/UL spatial parameters for SBFD slots than non-SBFD slot, e.g. beam or precoding codebook (e.g. codebook restriction). Similarly for potential enhancement on dynamic TDD, different UE beams can be applied to slots where two cells have same or different traffic directions.

Proposal 17: Inter-UE CLI can be mitigated by configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding codebook

- For SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

### Details for spatial domain coordination by gNB and relevant information exchange

Propose to have spatial domain enhancements at least for inter-CU/vendor coordination. e.g. CU2 can tell CU1 with least CLI resource ID - which associated to a CU1’s beam or associated to a RS ID on certain T/F resources, so that scheduled data/control can happen on the least CLI beam.

Proposal 18: Support inter-CU/vendor coordination on exchanging scheduled data/control UE beams to mitigate inter-UE CLI.

E.g. CU2 can indicate to CU1 of N RS IDs to measure, CU1 by itself wants to measure M RS IDs, then CU1 can configure NxM CLI resources with repetitions to sweep on multiple Tx and Rx beams.

Proposal 19: Support inter-CU/vendor coordination exchange information on number of required CLI resources, e.g. the total number could be # of measured Tx beams of UL UE multiply # of Rx beams of DL UE.

## UE and gNB transmission and reception timing

In R17 IAB, different timing modes (Case 6 & 7) are introduced for timing coordination between IAB-MT link and IAB-DU link to support simultaneous operation, e.g. Rx+Rx, Tx+Tx, Rx+Tx. Moreover, IAB node can feed back to its parent node that a particular timing mode is required, e.g. Case 6 timing mode.

Similar to the preferred timing mode feedback in R17 IAB, UE in SBFD or flexible TDD may indicate the preferred neighbor UE TA adjustment or report the Rx timing of neighbor UE. This is to align the Rx timing from gNB and from neighbor UE for inter-UE CLI mitigation, as illustrated in the figure below.

Figure 5-6 UE indicates preferred neighbor UE TA adjustment to mitigate inter-UE CLI

Another use case for UE to indicate preferred neighbor UE TA adjustment is described below. In Rel-16, when the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. The assumption of the time alignment of the CLI measurement depends on the assumption that the aggressor and victim UEs have similar UL timing (e.g TA) due to proximity. However, the two UEs could be served by different gNB and have different TA (e.g. HetNet deployment where two indoor UEs one severed by Macro cell and other is served by the indoor TRP). It is beneficial for the victim UE to indicate to the gNB a recommended timing adjustment of aggressor UE.

Proposal 20: The CLI measurement UE can recommend TA adjustment for aggressor UE corresponding to a particular CLI resource transmission.

Alternatively, the CLI measurement UE can report Rx timing difference between UE DL arrival timing and CLI RS arrival timing to help align the timing at the DL UE for inter-UE CLI reduction.

Proposal 21: The CLI measurement UE can report Rx timing difference between UE DL arrival timing and CLI RS arrival timing to help align the timing at the DL UE for inter-UE CLI reduction.

To mitigate the inter-UE CLI, gNB can configure slot-specific TA for SBFD slots than non-SBFD slot. Similarly for potential enhancement on flexible TDD, different TAs can be applied to slots where two cells have same or different traffic directions.

Proposal 22: : Inter-UE CLI can be mitigated by configuring slot-specific TA.

- For SBFD, TA configured for SBFD slots can be different from those configured for HD slots.

- For dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

## Power control based solution

In R17 IAB, to mitigate self interference from IAB-MT to IAB-DU, IAB node can feed back the desired IAB-MT PSD range to its parent node. Furthermore, to handle self interference from IAB-DU to IAB-MT or CLI from neighbor UE/MT, IAB node can feed back the desired parent node DL Tx power adjustment to its parent node.

Similar coordination schemes can be extended to gNB FD for inter-UE CLI mitigation. For example, UE can indicate to gNB the desired UL power backoff for a neighbor UL UE to mitigate corresponding CLI. Alternatively, UE can also indicate the desired DL power boost to cope with the CLI cause by a neighbor UL UE. In both cases, the neighbor UL UE can be identified by the corresponding CLI measurement resource.

Proposal 23: CLI measurement UE can recommend UL power backoff for neighbor UL UE corresponding to a particular CLI resource.

gNB may indicate UL power limit for certain interfering UE to ensure caused CLI is always under limit e.g. for UEs not affecting the UL coverage. The power limit can be in terms of PSD, power backoff, max absolute power, and applied in resources for gNB SBFD symbols/slots.

Proposal 24: gNB may indicate UL power limit for certain interfering UE to ensure caused CLI is always under limit.

Proposal 25: CLI measurement UE can recommend DL power boost to cope with the CLI from neighbor UL UE corresponding to a particular CLI resource.

In addition, if R16 inter-UE CLI framework is reused in SBFD or flexible TDD, DL UE may report gNB when high inter-UE CLI is detected. Based on the report, gNB may take action to mitigate CLI, e.g. reduce UL Tx power, increase guard band, or terminate FD. However, the action may experience certain latency from the time when the high inter-UE CLI is detected.

To reduce CLI mitigation latency, UL UE can autonomously adjust Tx power to limit CLI, e.g. based on pathloss measurement via the pathloss RS transmitted from the DL UE. Through the pathloss measurement, UL UE can determine max Tx power to limit caused CLI to be below a threshold. The pathloss RS from DL UE can be SRS transmitted periodically and the Tx power of SRS may have to be fixed and known to the UL UE.

Figure 5-7 CLI Aware UE autonomous UL power control

Proposal 26: Investigate UL UE autonomously adjust Tx power to limit inter-UE CLI caused to DL UE based on inter-UE pathloss measurement.

To mitigate the inter-UE CLI, gNB can configure slot-specific power control parameters for SBFD slots than non-SBFD slot. Similarly for potential enhancement on flexible TDD, different power control parameters can be applied to slots where two cells have same or different traffic directions.

Proposal 27: Inter-UE CLI can be mitigated by configuring slot-specific power control parameters

- For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

# Inter-gNB Cross-link interference mitigation techniques

An agreement related to inter-gNB CLI was made in RAN1 #109e for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed a list of candidates of potential enhancement methods of gNB-to-gNB CLI handling where further prioritization/down-scoping of candidate schemes for study can be done in this meeting.

Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of gNB-to-gNB CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - gNB-to-gNB CLI measurement and reporting - Coordinated scheduling - Spatial domain enhancements - Advanced receiver - UE and gNB transmission and reception timing - Power control based solution - Potential enhancements to Rel-16 RIM - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancements specific for SBFD will be discussed in 9.3.2

In general, we support to study most of the candidate potential inter-gNB CLI enhancements for dynamic/flexible TDD and/or SBFD, which include gNB-to-gNB CLI measurement and reporting, coordinated scheduling, spatial domain enhancements, UE and gNB transmission and reception timing, power control based solution, potential enhancements to Rel-16 RIM, whether or not a particular scheme requires OTA or backhaul information exchange. Benefits per candidate of potential enhancement will be addressed in following sections separately.

In addition, we prefer not to further study advanced receiver, which could be up to gNB implementation, and corresponding performance gain is unclear without simulation results. We also prefer not to further study on sensing-based mechanism. Its potential benefit is not clear to us, considering the sensing complexity, accuracy, and possible delay.

To summarize, we support most of the candidates in the list highlighted in the agreement other than “Advanced receiver” and “Sensing based mechanism”.

Proposal 28: Support to study the candidate’s solution for inter-gNB CLI other than “Advanced receiver” and “Sensing based mechanism”.

In RAN1 #110, the discussion on dynamic/flexible TDD schemes and solutions continued, based on the agreed list in RAN1 #109e of candidates of potential enhancement method of gNB-to-gNB CLI handling. In RAN1 #110, companies further discussed and focused on a subset of the candidate schemes for study. A couple of agreements were made in this meeting and in this section, we continue the discussion on the gNB-to-gNB CLI handling especially on the subset of the candidate schemes agreed in RAN1 #110.

## Potential enhancements to gNB-to-gNB CLI measurement and reporting

An agreement related to potential enhancements to gNB-to-gNB CLI measurement and reporting for inter-gNB CLI was made in RAN1 #110 for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed on a list of candidate areas of potential enhancement method of gNB-to-gNB CLI handling which will be further studied and discussed in this meeting.

We will discuss the proposals in separate sub-sections corresponding to each candidate areas: measurement resource configuration, measurement details, relevant information exchange, and usage of measurement.

Agreement Study the feasibility and potential benefits of gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource configuration - Measurement details - Relevant information exchange - Usage of measurement

### Measurement resource configuration

#### Reference signal configuration

Existing DL reference signal (e.g., SSB or CSI-RS) can be reused as measurement resource RS for gNB-to-gNB co-channel CLI measurement or CLI channel measurement. In addition, RIM-RS can be enhanced (e.g.  support FR2 SCS, reduce overhead, etc) for inter-gNB CLI purposes.

Proposal 29: Existing DL RS (e.g. SSB, CSI-RS) can be used as starting point as CLI-RS.

- RAN1 to further study possibility to enhance RIM-RS for inter-gNB CLI measurement.

#### Measurement and reporting configuration

As baseline, half duplex can be assumed for gNB CLI measurement and RS transmission. In this case, CLI measurement and RS transmission should be coordinated such that each cell can measure every intended neighbour cell. To achieve this, for example, a central NW unit can configure inter-gNB CLI RS Tx and Rx windows separately for each cell for inter-gNB CLI measurement. The central NW unit can be gNB-CU or OAM. As illustrated in Figure 6-1, Each period of the Tx windows could contain multiple cells’ Tx windows and each period each cell shall have multiple Rx windows corresponding to the Tx windows of multiplex cells. In addition, each cell can sweep different gNB Tx/Rx beams across multiple Tx/Rx windows to measure per Tx/Rx beam pair inter-gNB CLI.

Figure 6-1 Inter-gNB CLI RS Tx/Rx Configuration

Proposal 30: Investigate inter-gNB CLI measurement RS Tx and Rx time window configuration per cell.

Moreover, gNB HD or FD capability can be considered in the inter-gNB CLI measurement RS Tx and Rx time window configuration. For example, for HD only cell, TDMed Tx & Rx windows should be configured, while for FD capable cell, Tx & Rx windows can be overlapped.

Proposal 31: Consider gNB HD/FD capability in the inter-gNB CLI RS Tx and Rx time window configuration.

A 1st DU-cell measures interference from a neighboring 2nd DU-cell, where they may belong to same or different CUs. For measurement by 1st DU, CU may provide the measurement configuration including the resources over which 1st DU should measure CLI from a neighboring 2nd DU, intended TDD configuration of the 2nd DU. 1st DU may itself attempt to mitigate CLI or report the results to CU/OAM.

Figure 6-2 Inter-gNB/DU CLI Measurement

Proposal 32: RAN1 to study inter-gNB/DU CLI measurement, where the gNBs/DUs may belong to same or different CUs and CU may provide DU the measurement configuration.

A 1st DU-cell reports to an entity about interference measured from a neighboring 2nd DU-cell and the DU may be configured by CU/OAM with a report configuration. The report may include

- CLI measured metrics (e.g. RSRP, RSSI, SNR, SINR)

- ID of a neighboring DU-cell or ID of a configured measurement resource

- Beam information (e.g Rx beam of DU 1 and Tx beam of DU 2)

- Estimated propagation delay or RTT

The report may further include suggestion for 2nd DU (or its own) to mitigate CLI

- Power adjustment (on some resources and/or directions)

- Avoiding communicating on some time/frequency resources/directions

- RS configuration (e.g. to orthogonalize RSs of the two cells)

- Timing adjustment (such that the interfering signal is Rxed within CP)

Figure 6-3 Inter-gNB/DU CLI Report

Proposal 33: RAN1 to study inter-gNB CLI report contents.

### Measurement details

There are two possible measurements using the CLI-RS. The first option is to measure the inter-gNB channel using the CL-RS. This option is beneficial for FR1 deployment to enable proper Tx and Rx nulling between the gNB and reduce the CLI.

Figure 6-4 CLI channel measurement and reporting facilitate inter-gNB Tx/Rx beamforming/nulling

In order to get accurate CLI channel measurement, victim gNB sends UL-muting indication to its connected UEs to prevent UL transmissions in the inter-gNB CLI-channel measurement resources. Additionally, victim gNB can report inter-gNB CLI-channel measurements to the aggressor gNBs. One possible enhancement for inter-gNB CLI mitigation in dynamic/flexible TDD and subband full duplex is to optimize Tx beamforming and Rx nulling at the gNBs based on knowledge of inter-gNB CLI channel. In order to guarantee the accuracy of CLI channel measurement at the victim gNB. Victim gNB can send to its connected UEs an UL-muting indication (semi-static patterns) to prevent UEs from transmitting UL signals that can interfere with CLI channel measurement. Other alternatives to semi-static patterns include ULCI-based and slot-format based muting

Proposal 34: Support of inter-gNB CLI channel measurement and reporting to neighbouring gNBs for enabling Tx/Rx beamforming or nulling.

- Semi-static UL-muting patterns are configured to prevent UL transmissions from interfering with the inter-gNB CLI channel measurement.

For CLI measurement, CLI-RS is sent from aggressor gNB to victim gNB to measure actual interference per candidate DL/UL beam pair. Based on measurement results, victim gNB (or central coordinator) signals the non-compatible or compatible DL beam(s) per aggressor gNB and potentially other operation parameters, e.g. applied time/frequency resource, power backoff, etc. Report can be sent in OTA or via BH, e.g. via gNB-CU or OAM.

Figure 6-5 Inter-gNB CLI Measurement and Report

Alternatively, instead of report based scheme (closed loop), inter-gNB beam pair identification/signaling can be report free (open loop). Victim gNB transmits beams to be used for aggressor gNB to measure and decide by itself whether to transmit UL overlapping with aggressor gNB’s DL transmission.

Proposal 35: RAN1 will study report based inter-gNB CLI measurement and report free inter-gNB CLI measurement.

Proposal 36: Inter-gNB CLI measurement RS can be transmitted by aggressor gNB and measured by victim gNB, which will provide measurement results or DL Tx restriction info to aggressor gNB.

Proposal 37: Inter-gNB CLI measurement RS can be transmitted by victim gNB and measured by aggressor gNB, which will derive caused CLI to victim gNB and corresponding DL Tx decision.

Proposal 38: Inter-gNB CLI measurement RS can include transmitting cell ID and can be CDMed across multiple transmitting gNBs to save resource.

Proposal 39: OAM or CU can configure the inter-gNB CLI transmission parameters, including time/frequency location, sequence ID, beam info, periodicity.

Proposal 40: OAM or CU can configure the inter-gNB CLI monitoring parameters, including monitoring window location, beam info and periodicity.

### Relevant information exchange

As for the inter-gNB CLI report, it can be sent either via OTA or backhaul, and the report can be periodic or event triggered with possible contents as inter-gNB CLI metric per Tx/Rx beam pair, allowed/disallowed beams, etc. For example, the victim gNB can measure the strength of the CLI (e.g. RSSI or RSRP) at the configured CLI resources. This is useful to assess the CLI for different Tx/Rx beam pair and helps further coordination between the gNB on the preferred/restricted beams.

Proposal 41: Support to study OTA or backhaul information exchange for inter-gNB CLI reporting contents including inter-gNB CLI metric per Tx/Rx beam pair, allowed/disallowed beams, etc.

### Usage of measurement

The design target is to reduce or eliminate inter-gNB CLI.

- The CLI measurements using the different Tx/Rx beams is used to avoid beam direction that cause excessive CLI at other gNBs.

- The CLI channels is used to facilitate inter-gNB Tx/Rx beamforming/nulling to reduce inter-gNB CLI.

Observation 10: gNB-to-gNB CLI measurement and reporting can be used for gNB coordinated scheduling between gNBs and also can be used to facilitate inter-gNB Tx/Rx beamforming/nulling to reduce inter-gNB CLI.

## Coordinated Scheduling

An agreement related to potential coordinated scheduling enhancements for inter-gNB CLI was made in RAN1 #110 for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed on details of coordinated scheduling for time/frequency resources and relevant information exchange for gNB-to-gNB CLI handling which will be further studied and discussed in this meeting.

Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange

In general, gNB can exchange scheduling info for time/frequency/spatial domains to mitigate inter-UE CLI. For example, with coordinated scheduling, different cells can exchange scheduling information on different frequency resources cross cells, so that one cell can avoid scheduling on the subband that overlaps with its neighbour cell to achieve frequency isolation for dynamic TDD. Such frequency isolation could be beneficial for inter-gNB CLI mitigation in FR1, e.g. via SBHD. Therefore, we support coordinated scheduling for restricted resources including time/frequency/spatial domains.

Proposal 42: Coordinated scheduling information for time/frequency/spatial domain can be exchanged via OTA or BH signalling for inter-gNB CLI mitigation.

In addition, victim cell can signal neighbor cells whether DL transmission is allowed on a pre-determined candidate UL resource of the victim cell, e.g. a set of RRC UL symbols for victim cell to protect its high priority UL transmission. For example, the determination can be based on whether the total inter-gNB CLI measured on that UL resource exceeds a certain threshold or not, and can be indicated via OTA or BH to neighbor cells.

Proposal 43: Support coordinated scheduling on DL Tx restriction on UL resources between cells.

The coordinated scheduling can be achieved, either semi statically frequency domain partitioning of resources or event triggered/dynamically. One example for dynamic coordination can be when one of cells opportunistically uses one or more the SBFD slots as downlink slot only for some period of time and some frequency resources, it can exchange this information e.g. by event triggered report to avoid direct inter-gNB CLI at these resources for that period of time.

Proposal 44: RAN 1 study semi-static or dynamic coordinated scheduling for inter-gNB CLI mitigation.

## Spatial domain enhancements

An agreement related to potential spatial domain enhancements for inter-gNB CLI was made in RAN1 #110 for agenda 9.3.3 which is listed below. Based on the agreement below, RAN1 agreed on details of spatial domain coordination by gNB and relevant information exchange for gNB-to-gNB CLI handling which will be further studied and discussed in this meeting.

Agreement Study the feasibility and potential benefits of spatial domain coordination method for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details for spatial domain coordination - Relevant information exchange Note1: Study can include method for FR1 and FR2

Similar to existing beam management in FR2-1, inter-gNB beam management for inter-gNB CLI mitigation can be based on inter-gNB CLI RS measurement. Based on the measurement results, gNB (or central coordinator) should further identify compatible DL beam(s) per aggressor gNB that will cause negligible interference to used UL beam(s) of victim gNB.

Figure 6-6 Tx/Rx inter-gNB beam-pairs

Proposal 45: Support to investigate schemes for inter-gNB CLI mitigation in dynamic/flexible TDD and SBFD to identify compatible inter-gNB beam pairs, which can be based on inter-gNB CLI measurement and reporting per candidate DL/UL beam pair.

In codebook-based DL operation, an aggressor gNB may need to restrict DL precoding in slots where CLI/CLI-leakage occurs at the victim gNB. In particular, aggressor gNB configures the UE with PMI codebook subset restriction, however, this restriction is only needed for DL transmissions in slots where gNBs have conflicting traffic directions. That is, gNB keeps track of two PMIs for each UE, one PMI for HD slots and one PMI for slots with conflicting traffic directions.

Proposal 46: gNB adopts a slot-specific DL codebook restrictions, where a subset of PMI codebook is restricted in slots where a neighboring gNB has a conflicting traffic direction.

To mitigate the inter-gNB CLI, gNB can coordinate with neighbor gNB and configure slot-specific UE DL/UL spatial parameters for SBFD slots than non-SBFD slot, e.g. beam or precoding matrix. Similarly for potential enhancement on dynamic TDD, different UE beams can be applied to slots where two cells have same or different traffic directions.

Proposal 47: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding matrix

- For SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

For example, the victim gNB can measure the strength of the CLI (e.g. RSSI or RSRP) at the configured CLI resources associated with candidate DL/UL beam pairs. Beam related coordination information can be sent between victim gNB and aggressor gNB for CLI mitigation.

Proposal 48: Beam related coordination info can be sent between victim gNB and aggressor gNB

- If the inter-gNB CLI RS is transmitted from aggressor gNB and measured by victim gNB, the coordination info can include allowed/disallowed aggressor gNB DL beam(s), corresponding Tx power backoff and time/frequency resources.

- If the inter-gNB CLI RS is transmitted from victim gNB and measured by aggressor gNB, the coordination info can include the intended victim gNB UL beam(s), corresponding intended time/frequency resources and max allowed caused interference level.

## UE and gNB transmission and reception timing

Another issue is how to determine inter-gNB CLI RS Tx/Rx timing w.r.t. gNB’s own DL/UL symbol timing for traffic. One possibility is that gNB transmits RS based on its own DL symbol timing, while receiving individual RS based on detected RS arrival time, where the reception is in a dedicated Rx window. By using DL symbol timing, gNB can simultaneously transmit DL traffic via different panels or beams if applicable. It is assumed that Rx gNB can detect Rx timing per sequence with unknown arrival time, e.g. via correlation detection, which can also measure the RSRP. Moreover, gNB can distinguish RSs from multiple Tx gNBs overlapped in time and frequency if low-correlation CLI sequences are used.

Proposal 49: Investigate how to determine inter-gNB CLI RS Tx/Rx timing for accurate inter-gNB CLI measurement.

To mitigate the inter-gNB CLI, gNB can coordinate and configure slot-specific TA for SBFD slots than non-SBFD slot. Similarly for potential enhancement on flexible TDD, different TAs can be applied to slots where two cells have same or different traffic directions. For example, gNB can match the UL TA to the strongest jamming neighbor gNB to better align the reception of UL signal and neighbor gNB interference at the gNB for inter-gNB CLI mitigation, in the case that e.g. victim gNB is U symbol, while aggressor gNB is D symbol.

Proposal 50: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific TA.

- For SBFD, TA configured for SBFD slots can be different from those configured for HD slots.

- For dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

To reduce timing offset between UL Rx and CLI channel measurement RS, gNB can configure UEs with different timing advance (TA) values for CLI measurement occasions in order to reduce ICI between ULs and CLI channel measurement RS. In current spec, UE needs minimum number of symbols before applying new TA value. The applicability of this proposal will depend on UEs’ capability of how fast it can apply this TA adjustment. UE may need to be configured with zero or negative TA values to reduce the timing offset, which may require a restriction on scheduling the UE on consecutive UL and DL symbols, i.e., a guard symbol between D and U may need to be configured for these UEs. Additionally, if UE needs to fallback to default TA, then a guard period is needed to allow UE to adjust its TA.

Proposal 51: Simultaneous UL reception and inter-gNB CLI measurement can be achieved by configuring UE with zero or negative TA.

Figure 6-7 Example: zero TA for reducing timing offset

## Power control based solution

One possible power based enhancement for inter-gNB CLI mitigation in dynamic/flexible TDD is that based on the inter-gNB CLI measurements per DL/UL beam pair, one gNB could request another gNB to have X dB power backoff on time/frequency/spatial resources to mitigate inter-gNB CLI. The power adjustment is needed for slots where inter-gNB CLI persist (e.g. SBFD slot and asynchronous slots in dynamic TDD).

Proposal 52: Support of gNB requesting another gNB to have X dB power backoff on time/frequency/spatial resources to mitigate inter-gNB CLI.

To mitigate the inter-gNB CLI, gNB can coordinate and configure slot-specific power control parameters for SBFD slots than non-SBFD slot. Similarly for potential enhancement on flexible TDD, different power control parameters can be applied to slots where two cells have same or different traffic directions.

Proposal 53: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific power control parameters

- For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

## Potential enhancements to Rel-16 RIM

In Rel-16, remote interference management (RIM) was introduced to facilitate interference detection and mitigation from a remote gNB caused by the atmospheric ducting effect. The DL transmission from a remote gNB interferes with UL reception at victim gNB, as propagation delay experienced by a transmission from the remote gNB is larger than the guard period between DL and UL.

In Rel-16 RIM framework, RIM-RS is transmitted by the gNBs which carries the set ID of the transmitting gNB. Additionally, RIM-RS is used for conveying information about presence of ducting phenomenon and whether the applied interference mitigation is sufficient or not.

In dynamic/flexible TDD and SBFD operation, gNBs may experience CLI from neighbouring gNBs and this inter-gNB CLI will dependent on Tx/Rx beams used at the gNBs, especially in FR2. In Rel-16 RIM framework, there is no support for beam-based interference detection. In turn, Rel-16 RIM will have limited utility for interference management in dynamic/flexible TDD which may require advance coordination for beam management between neighboring gNBs.

The described proposals in above sections could be potential enhancements to Rel-16 RIM, inter-gNB coordination schemes for inter-gNB CLI mitigation in dynamic/flexible TDD and SBFD to identify compatible inter-gNB beam pairs, which is enabled by inter-gNB CLI measurement and reporting per candidate DL/UL beam pair. gNB (or central coordinator) can signal the non-compatible or compatible beam pairs and potentially other operation parameters such as power backoff and the report can be sent in OTA or via BH.

Proposal 54: Support to study enhancements to Rel-16 RIM for inter-gNB CLI mitigation to identify usable beam pair and/or restricted beam of aggressor and victim gNBs.

# Conclusion

In this contribution, we discussed the feasibility and potential enhancement for dynamic/flexible TDD. Below are the summary of the observations and proposals.

Observation 1: For FR1, deployments scenario with large Tx Power BS suffers from inter-gNB interference.

- In general, inter-UE CLI is not an issue except for macro-to-indoor deployment.

Observation 2: For FR2, Dynamic TDD is possible under careful assumption of layout and power parameterization to avoid inter-gNB interference.

Observation 3: Rel-18 study on potential enhancement of dynamic TDD suggests utilizing the outcome of Rel-15 and Rel-16 studies outcome avoid repetition of same discussion, e.g., inter-operator Dynamic TDD coexistence study.

Observation 4: SBHD can enable dynamic TDD and mitigate the impact of inter-gNB CLI.

Observation 5: SBHD-based dynamic enables flexible adaption of slots direction based on traffic which leads to reduced latency and improved UL coverage.

Observation 6: Link budget analysis shows that SB-based dynamic TDD is feasible for macro-cell deployment.

Observation 7: A prototype test network validated the feasibility of dynamic TDD in macro-cell deployment using subband half-duplex

Observation 8: In FR2, Dynamic TDD with misaligned slots format is possible where CLI could be mitigated with proper beam-pair selection and lower Tx power.

Observation 9: UE-to-UE CLI measurement and reporting can be used for gNB scheduling UE in the cell and for gNB coordinated UE scheduling between gNBs.

Observation 10: gNB-to-gNB CLI measurement and reporting can be used for gNB coordinated scheduling between gNBs and also can be used to facilitate inter-gNB Tx/Rx beamforming/nulling to reduce inter-gNB CLI.

Proposal 1: The focus of Rel-18 study on potential enhancement for dynamic TDD should be limited to co-channel intra-operator deployment.

Proposal 2: Support subband half-duplex as solution to enable dynamic TDD at least for FR1

Proposal 3: Support to study the candidate’s solution for inter-UE CLI other than “Advanced receiver” and “Sensing based mechanism”.

Proposal 4: Support L1/L2 based inter-UE CLI measurement/reporting to increase flexibility and reduce reporting latency compared to Rel-16 L3 based framework.

Proposal 5: L1 CLI measurement and report framework supports the following features

- Support L1-CLI report with P/SP/AP CLI resources and P/SP/AP/event triggered report types

- CLI resource can be configured similar to Rel-16 based on SRS or based on CSI-RS or CSI-IM.

Proposal 6: Support L2 CLI reporting with at least event triggered. Report can be sent on UL MAC-CE.

Proposal 7: L1 CLI measurement and report framework supports the following features

- CLI measurement metrics can be RSSI, RSRP

- CLI measurement can be incorporated into other CSI report metrics as the interference part, e.g. SINR, CQI

- CLI measurement can be based on CSI-IM (with enhanced patterns), or dedicated CLI measurement resource

Separate or joint reporting of CSI and CLI

Proposal 8: RAN1 to study UE CLI processing timeline at least for separate CLI reporting starting with L1-CSI timeline as a baseline.

- e.g. reuse AP CSI timeline as baseline with different value for timeline of L1-CLI.

Proposal 9: RAN1 to study L1-CLI report priority and multiplexing when reported as UCI.

Proposal 10: In addition to most interfering CLI resources, UE can be configured to report top X least interfering CLI resources for CLI report.

Proposal 11: Support subband-based CLI reporting to facilitate subband based scheduling for both SBFD and dynamic TDD in which CLI could be non-uniform across the DL RBs.

Proposal 12: Support to study OTA or backhaul information exchange between gNBs for inter-UE CLI mitigation at least for inter-CU/inter-vendor at least for intra-operator scenarios e.g.

- UE-to-UE CLI measurement resource configuration and OTA reporting triggering criteria between gNBs including time/frequency resources and beam indication for inter-UE CLI measurements between gNBs

- UE-to-UE CLI reporting contents including CLI metric per CLI resource

Proposal 13: Coordinated scheduling information for time/frequency resources and corresponding UE information can be exchanged via OTA or BH signalling for inter-UE CLI mitigation.

Proposal 14: Support UE Rx beam (QCL-D) configuration and indication per CLI measurement resource for enabling CLI-aware beam management.

Proposal 15: UE can dynamically report to the gNB a set of recommended beams, not preferred beams, or both.

- gNB configures multiple Rx (QCL-D) beams for UE to measure

Proposal 16: IAB framework can be extended to gNB SBFD/D-TDD for gNB to indicate restricted UE beam/panel.

Proposal 17: Inter-UE CLI can be mitigated by configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding codebook

- For SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

Proposal 18: Support inter-CU/vendor coordination on exchanging scheduled data/control UE beams to mitigate inter-UE CLI.

Proposal 19: Support inter-CU/vendor coordination exchange information on number of required CLI resources, e.g. the total number could be # of measured Tx beams of UL UE multiply # of Rx beams of DL UE.

Proposal 20: The CLI measurement UE can recommend TA adjustment for aggressor UE corresponding to a particular CLI resource transmission.

Proposal 21: The CLI measurement UE can report Rx timing difference between UE DL arrival timing and CLI RS arrival timing to help align the timing at the DL UE for inter-UE CLI reduction.

Proposal 22:  Inter-UE CLI can be mitigated by configuring slot-specific TA.

- For SBFD, TA configured for SBFD slots can be different from those configured for HD slots.

- For dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

Proposal 23: CLI measurement UE can recommend UL power backoff for neighbor UL UE corresponding to a particular CLI resource.

Proposal 24: gNB may indicate UL power limit for certain interfering UE to ensure caused CLI is always under limit.

Proposal 25: CLI measurement UE can recommend DL power boost to cope with the CLI from neighbor UL UE corresponding to a particular CLI resource.

Proposal 26: Investigate UL UE autonomously adjust Tx power to limit inter-UE CLI caused to DL UE based on inter-UE pathloss measurement.

Proposal 27: Inter-UE CLI can be mitigated by configuring slot-specific power control parameters

- For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

Proposal 28: Support to study the candidate’s solution for inter-gNB CLI other than “Advanced receiver” and “Sensing based mechanism”.

Proposal 29: Existing DL RS (e.g. SSB, CSI-RS) can be used as starting point as CLI-RS.

- RAN1 to further study possibility to enhance RIM-RS for inter-gNB CLI measurement.

Proposal 30: Investigate inter-gNB CLI measurement RS Tx and Rx time window configuration per cell.

Proposal 31: Consider gNB HD/FD capability in the inter-gNB CLI RS Tx and Rx time window configuration.

Proposal 32: RAN1 to study inter-gNB/DU CLI measurement, where the gNBs/DUs may belong to same or different CUs and CU may provide DU the measurement configuration.

Proposal 33: RAN1 to study inter-gNB CLI report contents.

Proposal 34: Support of inter-gNB CLI channel measurement and reporting to neighbouring gNBs for enabling Tx/Rx beamforming or nulling.

- Semi-static UL-muting patterns are configured to prevent UL transmissions from interfering with the inter-gNB CLI channel measurement.

Proposal 35: RAN1 will study report based inter-gNB CLI measurement and report free inter-gNB CLI measurement.

Proposal 36: Inter-gNB CLI measurement RS can be transmitted by aggressor gNB and measured by victim gNB, which will provide measurement results or DL Tx restriction info to aggressor gNB.

Proposal 37: Inter-gNB CLI measurement RS can be transmitted by victim gNB and measured by aggressor gNB, which will derive caused CLI to victim gNB and corresponding DL Tx decision.

Proposal 38: Inter-gNB CLI measurement RS can include transmitting cell ID and can be CDMed across multiple transmitting gNBs to save resource.

Proposal 39: OAM or CU can configure the inter-gNB CLI transmission parameters, including time/frequency location, sequence ID, beam info, periodicity.

Proposal 40: OAM or CU can configure the inter-gNB CLI monitoring parameters, including monitoring window location, beam info and periodicity.

Proposal 41: Support to study OTA or backhaul information exchange for inter-gNB CLI reporting contents including inter-gNB CLI metric per Tx/Rx beam pair, allowed/disallowed beams, etc.

Proposal 42: Coordinated scheduling information for time/frequency/spatial domain can be exchanged via OTA or BH signalling for inter-gNB CLI mitigation.

Proposal 43: Support coordinated scheduling on DL Tx restriction on UL resources between cells.

Proposal 44: RAN 1 study semi-static or dynamic coordinated scheduling for inter-gNB CLI mitigation.

Proposal 45: Support to investigate schemes for inter-gNB CLI mitigation in dynamic/flexible TDD and SBFD to identify compatible inter-gNB beam pairs, which can be based on inter-gNB CLI measurement and reporting per candidate DL/UL beam pair.

Proposal 46: gNB adopts a slot-specific DL codebook restrictions, where a subset of PMI codebook is restricted in slots where a neighboring gNB has a conflicting traffic direction.

Proposal 47: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific DL/UL spatial parameters, e.g. beam or precoding matrix

- For SBFD, spatial parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, spatial parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

Proposal 48: Beam related coordination info can be sent between victim gNB and aggressor gNB

- If the inter-gNB CLI RS is transmitted from aggressor gNB and measured by victim gNB, the coordination info can include allowed/disallowed aggressor gNB DL beam(s), corresponding Tx power backoff and time/frequency resources.

- If the inter-gNB CLI RS is transmitted from victim gNB and measured by aggressor gNB, the coordination info can include the intended victim gNB UL beam(s), corresponding intended time/frequency resources and max allowed caused interference level.

Proposal 49: Investigate how to determine inter-gNB CLI RS Tx/Rx timing for accurate inter-gNB CLI measurement.

Proposal 50: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific TA.

- For SBFD, TA configured for SBFD slots can be different from those configured for HD slots.

- For dynamic TDD, TA configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

Proposal 51: Simultaneous UL reception and inter-gNB CLI measurement can be achieved by configuring UE with zero or negative TA.

Proposal 52: Support of gNB requesting another gNB to have X dB power backoff on time/frequency/spatial resources to mitigate inter-gNB CLI.

Proposal 53: Inter-gNB CLI can be mitigated by coordinating and configuring slot-specific power control parameters

- For SBFD, power control parameters configured for SBFD slots can be different from those configured for HD slots

- For dynamic TDD, power control parameters configured for slots where the two cells have different traffic direction can be different from those configured for slots with aligned traffic directions in the two cells.

Proposal 54: Support to study enhancements to Rel-16 RIM for inter-gNB CLI mitigation to identify usable beam pair and/or restricted beam of aggressor and victim gNBs.

# References

- 3GPP TSG RAN Meeting #94e, RP-213591 “New SI: Study on evolution of NR duplex operation”, CMCC, Electronic Meeting, Dec. 6 - 17, 2021.

- 3GPP TR 38.828, “Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”

- 3GPP chair’s Notes, RAN1 #109e

- R1-2209983, Feasibility and techniques for subband non-overlapping full duplex, RAN1 #110bis-e.