3GPP TSG RAN WG1 #110bis-e																R1-2209993

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.5.2.3

Source: 	Qualcomm Incorporated

Title: 	Requirements, Evaluations, Potential Enhancements for Low Power High Accuracy Positioning

Document for:		Discussion and Decision

# Introduction

In RAN1#110-e several agreements were reached on LPHAP Positioning, especially with regards to the requirements and the evaluation assumptions of the power consumption modelling:

Agreement In the LPHAP evaluation, adopt the following model to convert the relative power unit to the battery life: - Alt. 1: battery life is used as the metric to identify the gap - K is an implementation factor, K = 1 (baseline); K = 0.5, 2, 4 (optional) - Note: The definition of the notations will be captured in the updates of TR. - Note: The voltage is assumed to be the same for the reference device and the LPHAP device. Agreement - In the LPHAP evaluation, adopt the following example parameter values in the conversion model to evaluate the battery life: - For the reference device in the conversion model: C1 (mAh); T1 (hour); X; reference traffic type. 4500; 12; 20%; FTP (model 3). - For the LPHAP device, consider 2 types in the conversion model: LPHAP device; C2 (mAh); T2req (month). Type A (baseline); 800; 6~12. Type B (optional); 4500; 6~12. - Note: As the reference device and LPHAP device characteristics, and therefore the parameter values of the model for determining battery life, is dependent on implementation factors, manufacturer, design options and cost options, it is up to individual company to evaluate the optional K values, and report the corresponding parameter values. Agreement In the LPHAP evaluation, adopt the example value of relative power unit of the reference device P1 = 50 to further align the battery life among companies. Agreement For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed: - Option 1: - The relative power unit: 0.015 - Additional transition energy: 2000 - Total transition time: 400ms - Option 2: - The relative power unit: 0.01 - Additional transition energy: 450; - Total transition time: 25ms - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state - Notes: the values above can be further discussed Agreement For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value. Agreement For the purpose of LPHAP evaluation, the following assumptions on eDRX configuration and/or paging reception can be optionally considered: - The eDRX cycle to evaluate: 20.48s; 30.72s; - For paging reception: - 1 paging occasion is included in one eDRX cycle - 10% paging rate - No paging reception can be optionally evaluated; - 1 DL PRS and/or UL SRS for positioning occasion per 1 eDRX cycle - Minimizing the gap between PRS measurement, SRS transmission and/or measurement reporting with paging monitoring in time domain can be evaluated. Agreement The tables to collect evaluation results from each source in section 3.3.2 of R1-2207993 are endorsed. Agreement Capture the following in TR as an observation: - Evaluations of baseline Rel-17 RRC_INACTIVE state positioning with the evaluation assumptions agreed for the study show that the power consumption on deep sleep state accounts for the highest proportion in the total power.

In this paper, we provide our power consumption analysis on LPHAP, together with our views on and potential enhancements within RRC Inactive and/or RRC Idle positioning.

# Potential Enhancements

## 2.1 RRC Idle Positioning

An easy and useful extension with regards to RRC Idle and Inactive Positioning is to extend the UE’s measurements in RRC Idle state. Up to NR Rel-17, all the measurements can happen in RRC connected or RRC Inactive state.

Proposal 1: Support Positioning measurements in RRC Idle state.

## RRC Inactive Positioning Enhancements

With regards to DL+UL RRC inactive positioning, we make the following observations:

- The procedure requires 2 SDTs, one for configuring the SRS and one for reporting the measurements.

  - Even though the 2nd SDT appears unavoidable, since the UE has to report the measurements for UE-assted RTT, the 1st SDT procedure may be unnecessary. For example, instead of having to do a MSG3-based request of Location services, RAN1 could work on a MSG1-based request. Furthermore, instead of configuring/triggering the SRS in the RRC Release message, one could envision a paging-type of operation, where a UE only monitors a PDCCH+PDSCH.

- Furthermore, power consumption could be reduced if the UE avoids retuning between PRS and SRS every time it needs to receive and transmit. Requesting PRS and SRS to have the same center frequency may reduce the power consumption of the DL+UL RRC Inactive operation. Another similar constraint could be that PRS and SRS are close by in time so that the UE really wakes up once for both.

- Lastly, even though intra-band RTT has been specified in NR Rel-16/17, having PRS and SRS on different bands is another direction, that depending on the scenario, may provide power saving gains. To be more precise, if we are in a scenario that DL-PRS & UL-SRS in FR2 is being used, but the SRS is power/coverage limited, if a device is calibrated across bands, it could enable transmitting the SRS on an FR1 band so that it can fight against the increased pathloss of FR2.

| Scenario | I-DRX and PRS/SRS Instances | RA-SDT with legacy network delay | RA-SDT with minimal network delay (SRS pre-configuration) | Percent Reduction in average power consumption |
| --- | --- | --- | --- | --- |
| UL-only | I-DRX=1.28s, N=1 | 6.99 | 3.60 | 48% |
| DL+UL w/ CG-SDT for reporting | I-DRX=1.28s, N=1 | 7.76 | 4.37 | 44% |
| UL-only | I-DRX=1.28s, N=8 | 2.18 | 1.76 | 19% |
| DL+UL w/ CG-SDT for reporting | I-DRX=1.28s, N=8 | 2.26 | 1.84 | 19% |
| UL-only | I-DRX=10.24s, N=1 | 1.75 | 1.33 | 24% |
| DL+UL w/ CG-SDT for reporting | I-DRX=10.24s, N=1 | 1.85 | 1.42 | 23% |

Furthermore, based on the power consumption analysis in Section 3, we make the following proposals:

Proposal 2: For the purpose of reduced power consumption in RRC Inactive, the following can be beneficial:

- Study ways of optimizing the SRS configuration/activation/request procedure(s) included in the UL/DL+UL RRC inactive positioning (e.g. SRS pre-configuration, RACH-based SRS request from the UE, paging-based SRS activation).

- Study ways for SRS transmission continuation after cell change in RRC Inactive (e.g., continuity of the configured SRS across cell change).

- Study PRS/SRS configuration restrictions & corresponding new UE capabilities for enabling reduced power consumption for RTT positioning.

# Power Consumption Analysis

## DL-only Positioning without reporting (UE-based)

For the case DL-only positioning where the location is needed at the UE (e.g. UE-based), there is no need to include in the power consumption analysis a power component related to the reporting of the location report.

Based on the above, and the previous agreements on the channels to include (shown below), we provide the tables shown in Sections 2.2.1-2.2.3.

DL positioning

- For the SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - DL PRS measurement with 0.5 ms duration; - 1 Number of PFL; - 8 DL PRS resources per slot are measured; - DL PRS instance of smaller than or equal to 1 slot duration;

##### 2.3.1 PRS every N=1 I-DRX cycles with 1.28 sec

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 1 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Deep sleep | 1 | 2535 | 2535 |
|  | Deep State transit | 450 | 2 | 900 |
|  | Light State transit | 100 | 1 | 100 |
|  | Total Power | 4403 |  |  |
|  | Average Power per slot | 1.72 |  |  |
|  | Battery life (in month) | K=1: 0.43 (Type A), 2.42 (Type B) K=4: 1.72 (Type A), 9.69 (Type B) |  |  |

##### 2.3.2 PRS every 8 I-DRX cycles with 1.28 sec

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 2 | Paging Occasion (2 ms) | 57 | 32 | 1824 |
|  | SSB serv. cell (2 ms) | 50 | 32 | 1600 |
|  | Light sleep (8 ms) | 20 | 128 | 2560 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Deep sleep | 1 | 20287 | 20287 |
|  | Deep State transit | 450 | 9 | 4050 |
|  | Light State transit | 100 | 8 | 800 |
|  | Total Power | 31241 |  |  |
|  | Average Power per slot | 1.53 |  |  |
|  | Battery life (in month) | K=1: 0.48 (Type A), 2.72 (Type B) K=4: 1.93 (Type A), 10.89 (Type B) |  |  |

##### 2.3.3 PRS every I-DRX cycles with 10.24 sec

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 3 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Deep sleep | 1 | 20455 | 20455 |
|  | Deep State transit | 450 | 2 | 900 |
|  | Light State transit | 100 | 1 | 100 |
|  | Total Power | 22323 |  |  |
|  | Average Power per slot | 1.09 |  |  |
|  | Battery life (in month) | K=1: 0.68 (Type A), 3.82 (Type B) K=4: 2.72 (Type A), 15.29 (Type B) |  |  |

## DL-only Positioning with reporting (UE-based or UE-assisted)

For DL-Only Positioning with location reporting to the network, either UE-based or UE-assisted, we need to include the power consumption due to reporting. We assume that a CG UL can be used for that as was agreed in the previous meeting ((UE-A) CG-SDT with 1ms duration and the periodicity of positioning interval).

##### PRS every N=1 I-DRX cycles with 1.28 sec

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 4 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Deep sleep | 1 | 2533 | 2533 |
|  | Deep State transit | 450 | 3 | 1350 |
|  | Light State transit | 100 | 1 | 100 |
|  | CG UL | 700 | 2 | 1400 |
|  | Total Power | 6251 |  |  |
|  | Average Power per slot | 2.44 |  |  |
|  | Battery life (in month) | K=1: 0.3 (Type A), 1.71 (Type B) K=4: 1.21 (Type A), 6.83 (Type B) |  |  |

##### PRS every 8 I-DRX cycles with 1.28 sec

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 5 | Paging Occasion (2 ms) | 57 | 32 | 1824 |
|  | SSB serv. cell (2 ms) | 50 | 32 | 1600 |
|  | Light sleep (8 ms) | 20 | 128 | 2560 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Deep sleep | 1 | 20285 | 20285 |
|  | Deep State transit | 450 | 10 | 4500 |
|  | Light State transit | 100 | 8 | 800 |
|  | CG UL | 700 | 2 | 1400 |
|  | Total Power | 33089 |  |  |
|  | Average Power per slot | 1.62 |  |  |
|  | Battery life (in month) | K=1: 0.45 (Type A), 2.57 (Type B) K=4: 1.82 (Type A), 10.29 (Type B) |  |  |

##### PRS every I-DRX cycles with 10.24 sec

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 6 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Deep sleep | 1 | 20453 | 20453 |
|  | Deep State transit | 450 | 1 | 450 |
|  | Light State transit | 100 | 1 | 100 |
|  | CG UL | 700 | 2 | 1400 |
|  | Total Power | 23271 |  |  |
|  | Average Power per slot | 1.14 |  |  |
|  | Battery life (in month) | K=1: 0.65 (Type A), 3.66 (Type B) K=4: 2.6 (Type A), 14.62 (Type B) |  |  |

## UL-only Positioning

With regards to the UL-only Positioning, we consider it useful to consider to scenarios of power cycle computation.

- 1st case: No SDT in the power cycle. This corresponds to the case that SRS has been configured before, and it is still valid in the power cycle that is being analyzed.

- 2nd case: Legacy SDT in the power cycle. This corresponds to the case that SRS has not been configured before, and a new SDT is needed to be performed.

##### SRS every N=1 I-DRX cycles with 1.28 sec

###### 2.5.1.1 No SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 7 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep sleep | 1 | 2535 | 2535 |
|  | Deep State transit | 450 | 2 | 900 |
|  | Light State transit | 100 | 1 | 100 |
|  | Total Power | 4493 |  |  |
|  | Average Power per slot | 1.76 |  |  |
|  | Battery life (in month) | K=1: 0.42 (Type A), 2.37 (Type B) K=4: 1.68 (Type A), 9.47 (Type B) |  |  |

###### 2.5.1.2 Legacy SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 8 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep sleep | 1 | 2302 | 2302 |
|  | Deep State transit | 450 | 3 | 1350 |
|  | SDT w/o network delay | See SDT breakdown | 233 | 13190 |
|  | Light State transit | 100 | 1 | 100 |
|  | Total Power | 17900 |  |  |
|  | Average Power per slot | 6.99 |  |  |
|  | Battery life (in month) | K=1: 0.1 (Type A), 0.6 (Type B) K=4: 0.42 (Type A), 2.39 (Type B) |  |  |

##### SRS every 8 I-DRX cycles with 1.28 sec

###### 2.5.2.1 No SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 9 | Paging Occasion (2 ms) | 57 | 32 | 1824 |
|  | SSB serv. cell (2 ms) | 50 | 32 | 1600 |
|  | Light sleep (8 ms) | 20 | 128 | 2560 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep sleep | 1 | 20287 | 20287 |
|  | Deep State transit | 450 | 9 | 4050 |
|  | Light State transit | 100 | 8 | 800 |
|  | Total Power | 31331 |  |  |
|  | Average Power per slot | 1.53 |  |  |
|  | Battery life (in month) | K=1: 0.48 (Type A), 2.72 (Type B) K=4: 1.94 (Type A), 10.89 (Type B) |  |  |

###### 2.5.2.2 Legacy SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 10 | Paging Occasion (2 ms) | 57 | 32 | 1824 |
|  | SSB serv. cell (2 ms) | 50 | 32 | 1600 |
|  | Light sleep (8 ms) | 20 | 128 | 2560 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep sleep | 1 | 20054 | 20054 |
|  | Deep State transit | 450 | 10 | 4500 |
|  | SDT | See SDT breakdown | 233 | 13190 |
|  | Light State transit | 100 | 8 | 800 |
|  | Total Power | 44738 |  |  |
|  | Average Power per slot | 2.18 |  |  |
|  | Battery life (in month) | K=1: 0.34 (Type A), 1.36 (Type B) K=4: 1.36 (Type A), 7.64 (Type B) |  |  |

##### SRS every I-DRX cycles with 10.24 sec

###### 2.5.3.1 No SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 11 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep sleep | 1 | 20455 | 20455 |
|  | Deep State transit | 450 | 2 | 900 |
|  | Light State transit | 100 | 1 | 100 |
|  | Total Power | 22413 |  |  |
|  | Average Power per slot | 1.09 |  |  |
|  | Battery life (in month) | K=1: 0.68 (Type A), 3.82 (Type B) K=4: 2.72 (Type A), 15.29 (Type B) |  |  |

###### 2.5.3.2 Legacy SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 12 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep sleep | 1 | 20222 | 20222 |
|  | Deep State transit | 450 | 3 | 1350 |
|  | SDT with legacy network delay | See SDT breakdown | 233 | 13190 |
|  | Light State transit | 100 | 1 | 100 |
|  | Total Power | 35820 |  |  |
|  | Average Power per slot | 1.75 |  |  |
|  | Battery life (in month) | K=1: 0.42 (Type A), 2.38 (Type B) K=4: 2.38 (Type A), 9.52 (Type B) |  |  |

## DL+UL Positioning

Similar to the UL-only Positioning, we consider 2 power cycle cases with regards to the SDT, one that doesn’t have SDT, and one that has a legacy SDT procedure considered in the power consumption computation.

##### 2.5.1 PRS/SRS every N=1 I-DRX cycles with 1.28 sec

###### 2.5.1.1 No SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 13 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep State transit | 450 | 4 | 1800 |
|  | Light State transit | 100 | 1 | 100 |
|  | CG UL | 700 | 2 | 1400 |
|  | Deep sleep | 1 | 2532 | 2532 |
|  | Total Power | 6910 |  |  |
|  | Average Power per slot | 2.70 |  |  |
|  | Battery life (in month) | K=1: 0.27 (Type A), 1.54 (Type B) K=4: 1.09 (Type A), 6.17 (Type B) |  |  |

###### 2.5.1.2 Legacy SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 14 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep State transit | 450 | 4 | 1800 |
|  | Light State transit | 100 | 1 | 100 |
|  | SDT w/o network delay | See SDT breakdown | 233 | 13190 |
|  | CG UL | 700 | 2 | 1400 |
|  | Deep sleep | 1 | 2299 | 2299 |
|  | Total Power | 19867 |  |  |
|  | Average Power per slot | 7.76 |  |  |
|  | Battery life (in month) | K=1: 0.09 (Type A), 0.54 (Type B) K=4: 0.38 (Type A), 2.14 (Type B) |  |  |

##### 2.5.2 PRS/SRS every 8 I-DRX cycles with 1.28 sec

###### 2.5.2.1 No SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 15 | Paging Occasion (2 ms) | 57 | 32 | 1824 |
|  | SSB serv. cell (2 ms) | 50 | 32 | 1600 |
|  | Light sleep (8 ms) | 20 | 128 | 2560 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep State transit | 450 | 10 | 4500 |
|  | Light State transit | 100 | 8 | 800 |
|  | CG UL | 700 | 2 | 1400 |
|  | Deep sleep | 1 | 20284 | 20284 |
|  | Total Power | 33298 |  |  |
|  | Average Power per slot | 1.63 |  |  |
|  | Battery life (in month) | K=1: 0.45 (Type A), 2.55 (Type B) K=4: 1.88 (Type A), 10.22 (Type B) |  |  |

###### 2.5.2.2 Legacy SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 16 | Paging Occasion (2 ms) | 57 | 32 | 1824 |
|  | SSB serv. cell (2 ms) | 50 | 32 | 1600 |
|  | Light sleep (8 ms) | 20 | 128 | 2560 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep State transit | 450 | 10 | 4500 |
|  | Light State transit | 100 | 8 | 800 |
|  | SDT | See SDT breakdown | 233 | 13190 |
|  | CG UL | 700 | 2 | 1400 |
|  | Deep sleep | 1 | 20051 | 20051 |
|  | Total Power | 46255 |  |  |
|  | Average Power per slot | 2.26 |  |  |
|  | Battery life (in month) | K=1: 0.33 (Type A), 1.86 (Type B) K=4: 1.32 (Type A), 7.47 (Type B) |  |  |

##### 2.5.3 PRS/SRS every I-DRX cycles with 10.24 sec

###### 2.5.3.1 No SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 17 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep State transit | 450 | 3 | 1350 |
|  | Light State transit | 100 | 1 | 100 |
|  | CG UL | 700 | 2 | 1400 |
|  | Deep sleep | 1 | 20452 | 20452 |
|  | Total Power | 24380 |  |  |
|  | Average Power per slot | 1.19 |  |  |
|  | Battery life (in month) | K=1: 0.62 (Type A), 3.5 (Type B) K=4: 2.49 (Type A), 14 (Type B) |  |  |

###### 2.6.3.2 Legacy SDT in the power cycle

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 18 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep State transit | 450 | 4 | 1800 |
|  | Light State transit | 100 | 1 | 100 |
|  | SDT with legacy network delay | See SDT breakdown | 233 | 13190 |
|  | CG UL | 700 | 2 | 1400 |
|  | Deep sleep | 1 | 20219 | 20219 |
|  | Total Power | 37787 |  |  |
|  | Average Power per slot | 1.85 |  |  |
|  | Battery life (in month) | K=1: 0.4 (Type A), 2.25 (Type B) K=4: 1.60 (Type A), 9 (Type B) |  |  |

## Ultra-Deep sleep state

The following agreement was reached with regards to including an ultra-deep sleep state in the power consumption analysis for LPHAP:

Agreement For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed: - Option 1: - The relative power unit: 0.015 - Additional transition energy: 2000 - Total transition time: 400ms - Option 2: - The relative power unit: 0.01 - Additional transition energy: 450; - Total transition time: 25ms - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state - Notes: the values above can be further discussed Agreement For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value.

With regards to the options above and the previous discussions during the RAN1 meeting, we make the following observations:

- Option 1 has a starting poing the assumptions of NB-IoT’s power saving mode (R1-1714993) in which a UE consumes less power than that in RRC_IDLE state, due to the reason that, e.g., UE shuts down most of its power supplies to maintain a very low current. In this case, accurate time/frequency synchronization with the network is not guaranteed.

- Option 2 has a starting point that a dedicated positioning device, with little communication service, with dedicated hardware has mainly positioning functionalities. Such a device might have optimized values with regards to the transition time and total transition time.

With regards to Option 2, our understanding is that it is not in scope to consider a set of very specifialized device(s) with only positioning capabilities, beyond the already defined devices (e.g. regular device or Redcap device). Such an additional fragmentation of the devices needs good justification and overall support from the ecosystem, which we do not see at this point. From our side, an LPHAP device is a regular UE that has legacy communication capabilities, likely of IoT nature (e.g. Redcap or eRedcap), with some additional positioning-specific capabilities, some of which may be new, depending on the progress in this subagenda.

Furthermore, it should also be noted that ultra-deep sleep state will also be considered in the Low-Power WUS, and therefore, we expect that characterization of ultra-deep sleep

Proposal 1: For the purpose of evaluating LPHAP Positioning, with regards to the ultra-deep sleep state, consider Option 1 with a transition energy of 9000:

  - Option 1:

    - The relative power unit: 0.015

    - Additional transition energy: 20000

    - Total transition time: 400ms

In this analysis we assume that the PRS and PO are close to each other

### DL-only Positioning without reporting (UE-based)

##### eDRX Periodicity – 10.24s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 19 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (16 ms) | 20 | 32 | 640 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Ultra Deep sleep state | 0.015 | 20439 | 306.585 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | Total Power | 21694.585 |  |  |
|  | Average Power per slot | 1.06 |  |  |
|  | Battery life (in month) | K=1: 0.7 (Type A), 3.93 (Type B) K=4: 2.79 (Type A), 15.72 (Type B) |  |  |

##### eDRX Periodicity – 20.48s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 20 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (16 ms) | 20 | 32 | 640 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Ultra Deep sleep state | 0.015 | 40919 | 613.785 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | Total Power | 22001.785 |  |  |
|  | Average Power per slot | 0.54 |  |  |
|  | Battery life (in month) | K=1: 1.37 (Type A), 7.72 (Type B) K=4: 5.49 (Type A), 30.86 (Type B) |  |  |

##### eDRX Periodicity – 30.72s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 21 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 32 | 640 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Ultra Deep sleep state | 0.015 | 61399 | 920.985 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | Total Power | 22308.985 |  |  |
|  | Average Power per slot | 0.36 |  |  |
|  | Battery life (in month) | K=1: 2.05 (Type A), 11.57 (Type B) K=4: 8.23 (Type A), 46.29 (Type B) |  |  |

### DL-only Positioning with reporting (UE-based or UE-assisted)

##### 2.6.1.1 eDRX Periodicity – 10.24s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 22 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (16 ms) | 20 | 32 | 640 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Ultra Deep sleep state | 0.015 | 20437 | 306.555 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | CG UL | 700 | 2 | 1400 |
|  | Total Power | 23094.555 |  |  |
|  | Average Power per slot | 1.13 |  |  |
|  | Battery life (in month) | K=1: 0.66 (Type A), 3.69 (Type B) K=4: 2.62 (Type A), 14.75 (Type B) |  |  |

##### 2.6.1.2  eDRX Periodicity – 20.48s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 23 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (16 ms) | 20 | 32 | 640 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Ultra Deep sleep state | 0.015 | 40917 | 613.755 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | CG UL | 700 | 2 | 1400 |
|  | Total Power | 23401.755 |  |  |
|  | Average Power per slot | 0.57 |  |  |
|  | Battery life (in month) | K=1: 1.29 (Type A), 7.31 (Type B) K=4: 5.19 (Type A), 29.24 (Type B) |  |  |

##### 2.6.1.3  eDRX Periodicity – 30.72s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 24 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (16 ms) | 20 | 32 | 640 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | Ultra Deep sleep state | 0.015 | 61397 | 920.955 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | CG UL | 700 | 2 | 1400 |
|  | Total Power | 23708.955 |  |  |
|  | Average Power per slot | 0.39 |  |  |
|  | Battery life (in month) | K=1: 1.90 (Type A), 10.68 (Type B) K=4: 7.60 (Type A), 42.73 (Type B) |  |  |

### UL-only Positioning (No SDT in power cycle)

##### eDRX Periodicity – 10.24s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 25 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (16 ms) | 20 | 32 | 640 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Ultra Deep sleep state | 0.015 | 20439 | 306.585 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | Total Power | 21784.585 |  |  |
|  | Average Power per slot | 1.06 |  |  |
|  | Battery life (in month) | K=1: 0.7 (Type A), 3.93 (Type B) K=4: 2.79 (Type A), 15.72 (Type B) |  |  |

##### eDRX Periodicity – 20.48s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 26 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 32 | 640 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Ultra Deep sleep state | 0.015 | 40919 | 613.785 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | Total Power | 22091.785 |  |  |
|  | Average Power per slot | 0.54 |  |  |
|  | Battery life (in month) | K=1: 1.38 (Type A), 7.71 (Type B) K=4: 5.48 (Type A), 30.86 (Type B) |  |  |

##### eDRX Periodicity – 30.72s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 27 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 32 | 640 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Ultra Deep sleep state | 0.015 | 61399 | 920.985 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 2 | 200 |
|  | Total Power | 22398.985 |  |  |
|  | Average Power per slot | 0.37 |  |  |
|  | Battery life (in month) | K=1: 2 (Type A), 11.26 (Type B) K=4: 8.01 (Type A), 45.01 (Type B) |  |  |

### DL+UL Positioning

##### eDRX Periodicity – 10.24s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 28 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (24 ms) | 20 | 48 | 960 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 3 | 300 |
|  | CG UL | 700 | 2 | 1400 |
|  | Ultra Deep sleep | 0.015 | 20420 | 306.3 |
|  | Total Power | 23724.3 |  |  |
|  | Average Power per slot | 1.16 |  |  |
|  | Battery life (in month) | K=1: 0.64 (Type A), 3.59 (Type B) K=4: 2.56 (Type A), 14.37 (Type B) |  |  |

##### eDRX Periodicity – 20.48s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 29 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (24 ms) | 20 | 48 | 960 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 3 | 300 |
|  | CG UL | 700 | 2 | 1400 |
|  | Ultra Deep sleep | 0.015 | 40900 | 613.5 |
|  | Total Power | 24031.5 |  |  |
|  | Average Power per slot | 0.59 |  |  |
|  | Battery life (in month) | K=1: 1.25 (Type A), 7.06 (Type B) K=4: 5.02 (Type A), 28.25 (Type B) |  |  |

##### eDRX Periodicity – 30.72s

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 30 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (24 ms) | 20 | 48 | 960 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Ultra Deep State transit | 20000 | 1 | 20000 |
|  | Light State transit | 100 | 3 | 300 |
|  | CG UL | 700 | 2 | 1400 |
|  | Ultra Deep sleep | 0.015 | 61380 | 920.7 |
|  | Total Power | 24338.7 |  |  |
|  | Average Power per slot | 0.40 |  |  |
|  | Battery life (in month) | K=1: 1.85 (Type A), 10.41 (Type B) K=4: 7.4 (Type A), 41.67 (Type B) |  |  |

## 3.6 UL or DL+UL & SDT with SRS preconfiguration

As it was described in Section 2.1 and Observation 1, the SDT procedure used to configure SRS may be significantly longer than what it should be due to the high layer signaling between the network and the LMF that is currently required. In the sections below, we make the assumption of a SDT procedure with reduced latency between the messages (as an example, 2 msec gaps are being used, as shown in the figure below), and evaluate the potential power consumption gains.

### 3.6.1 PRS/SRS every I-DRX cycles with 10.24 sec

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 31 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep sleep | 1 | 20418 | 20418 |
|  | Deep State transit | 450 | 3 | 1350 |
|  | SDT with minimal network delay | See SDT breakdown | 37 | 4320 |
|  | Light State transit | 100 | 1 | 100 |
|  | Total Power | 27146 |  |  |
|  | Average Power per slot | 1.33 |  |  |
|  | Battery life (in month) | K=1: 0.56 (Type A), 3.13 (Type B) K=4: 2.23 (Type A), 12.53 (Type B) |  |  |

| Evaluation case | Power states | Relative Power unit | Duration (in slots) | Total Power |
| --- | --- | --- | --- | --- |
| Case 32 | Paging Occasion (2 ms) | 57 | 4 | 228 |
|  | SSB serv. cell (2 ms) | 50 | 4 | 200 |
|  | Light sleep (8 ms) | 20 | 16 | 320 |
|  | PRS Instance (0.5 ms) | 120 | 1 | 120 |
|  | SRS (0.5 msec) | 210 | 1 | 210 |
|  | Deep State transit | 450 | 4 | 1800 |
|  | Light State transit | 100 | 1 | 100 |
|  | SDT with minimal network delay | See SDT breakdown | 32 | 4320 |
|  | CG UL | 700 | 2 | 1400 |
|  | Deep sleep | 1 | 20420 | 20415 |
|  | Total Power | 29113 |  |  |
|  | Average Power per slot | 1.42 |  |  |
|  | Battery life (in month) | K=1: 0.52 (Type A), 2.93 (Type B) K=4: 2.08 (Type A), 11.73 (Type B) |  |  |

# Summary of Power Consumption Evaluation

We  now provide the summary Tables:

Table 3.3-1: Evaluation cases and assumptions

| Evaluation assumption | Case 1, FR1, DL-only, 1.28 s I-DRX, N=1, UE-B | Case 2, FR1, DL-only, 1.28 s I-DRX, N=8, UE-B | Case 3, FR1, DL-only, 10.24 s I-DRX, N=1, UE-B | Case 4, FR1, DL-only, 1.28 s I-DRX, N=1 | Case 5, FR1, DL-only, 1.28 s I-DRX, N=8 | Case 6, FR1, DL-only, 10.24 s I-DRX, N=1 | Case 7, FR1, UL-only, 1.28 s I-DRX, N=1, No SDT | Case 8, FR1, UL-only, 1.28 s I-DRX, N=1. w SDT | Case 9, FR1, UL-only, 1.28 s I-DRX, N=8, No SDT | Case 10, FR1, UL-only, 1.28 s I-DRX, N=8, w SDT | Case 11, FR1, UL-only, 10.24 s I-DRX, N=1, no SDT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sleep state | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 1.28 s | 1.28 s | 10.24 s | 1.28 s | 1.28 s | 1.28 s | 1.28 s | 10.24 s |
| paging reception | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate |
| RS periodicity | 1.28 s | 1.28 s | 10.24 s | 1.28 s | 1.28 s | 10.24 s | 1.28 s | 1.28 s | 1.28 s | 1.28 s | 10.24 s |
| M-sample | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| RRM measurement | - | - | - | - | - | - | - | - | - | - | - |
| BWP switching | - | - | - | - | - | - | - | - | - | - | - |
| Measurement reporting | No(UE-B) | No(UE-B) | No(UE-B) | CG-UL | CG-UL | CG-UL | No | No | No | No | No |
| implementation factor K | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} |

| Evaluation assumption | Case 12, FR1, UL-only, 10.24 s I-DRX, N=1, w SDT | Case 13, FR1,DL+ UL, 1.28 s I-DRX, N=1, No SDT | Case 14, FR1, DL+UL, 1.28 s I-DRX, N=1, w SDT | Case 15, FR1, DL+UL, 1.28 s I-DRX, N=8, No SDT | Case 16, FR1, DL+UL, 1.28 s I-DRX, N=8, w SDT | Case 17, FR1, DL+UL, 10.24 s I-DRX, N=1, no SDT | Case 18, FR1, DL+UL, 10.24 s I-DRX, N=1, w SDT | Case 19, FR1, DL-only, 10.24 s I-DRX, N=1, UE-B, Ultra deep sleep | Case 20, FR1, DL-only, 20.48 s I-DRX, N=1, UE-B, Ultra deep sleep | Case 21, FR1, DL-only, 30.72 s I-DRX, N=1, UE-B, Ultra deep sleep |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sleep state | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep |
| DRX cycle | 10.24 s | 1.28 s | 1.28 s | 1.28 s | 1.28 s | 10.24 s | 10.24 s | 10.24 s | 20.48 s | 30.72 s |
| paging reception | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate |
| RS periodicity | 10.24 s | 1.28 s | 1.28 s | 1.28 s | 1.28 s | 10.24 s | 10.24 s | 10.24 s | 20.48 s | 30.72 s |
| M-sample | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| RRM measurement | - | - | - | - | - | - | - | - | - | - |
| BWP switching | - | - | - | - | - | - | - | - | - | - |
| Measurement reporting | No | No | No | No | No | No | No | No(UE-B) | No(UE-B) | No(UE-B) |
| implementation factor K | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} |

| Evaluation assumption | Case 22, FR1, DL-only, 10.24 s I-DRX, N=1, Ultra deep sleep | Case 23, FR1, DL-only, 20.48 s I-DRX, N=1, Ultra deep sleep | Case 24, FR1, DL-only, 30.72 s I-DRX, N=1, Ultra deep sleep | Case 25, FR1, UL-only, 10.24 s I-DRX, N=1, Ultra deep sleep | Case 26, FR1, UL-only, 20.48 s I-DRX, N=1, Ultra deep sleep | Case 27, FR1, UL-only, 30.72 s I-DRX, N=1, Ultra deep sleep | Case 28, FR1, DL+UL, 10.24 s I-DRX, N=1, Ultra deep sleep | Case 29, FR1, DL+UL, 20.48 s I-DRX, N=1, Ultra deep sleep | Case 30, FR1, DL+UL, 30.72 s I-DRX, N=1, Ultra deep sleep | Case 31, FR1, UL-only, 10.24 s I-DRX, N=1, SRS preconfig. | Case 18, FR1, DL+UL, 10.24 s I-DRX, N=1, SRS preconfig |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sleep state | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Ultra Deep sleep | Deep sleep | Deep sleep |
| DRX cycle | 10.24 s | 20.48 s | 30.72 s | 10.24 s | 20.48 s | 30.72 s | 10.24 s | 20.48 s | 30.72 s | 10.24 s | 10.24 s |
| paging reception | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate | 2ms, I-DRX periodicity, 10% paging rate |
| RS periodicity | 10.24 s | 20.48 s | 30.72 s | 10.24 s | 20.48 s | 30.72 s | 10.24 s | 20.48 s | 30.72 s | 10.24 s | 10.24 s |
| M-sample | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| RRM measurement | - | - | - | - | - | - | - | - | - | - | - |
| BWP switching | - | - | - | - | - | - | - | - | - | - | - |
| Measurement reporting | CG-UL | CG-UL | CG-UL | No | No | No | No | No | No | No | No |
| implementation factor K | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} | K={1,4} |

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| Case 1, Rel-17 | 1.72 | K=1: 0.43 (Type A), 2.42 (Type B) K=4: 1.72 (Type A), 9.69 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 2.21) |
| Case 2, Rel-17 | 1.53 | K=1: 0.48 (Type A), 2.72 (Type B) K=4: 1.93 (Type A), 10.89 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 1.11) |
| Case 3, Rel-17 | 1.09 | K=1: 0.68 (Type A), 3.82 (Type B) K=4: 2.72 (Type A), 15.29 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 4, Rel-17 | 2.44 | K=1: 0.3 (Type A), 1.71 (Type B) K=4: 1.21 (Type A), 6.83 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 5.17) |
| Case 5, Rel-17 | 1.62 | K=1: 0.45 (Type A), 2.57 (Type B) K=4: 1.82 (Type A), 10.29 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 1.71) |
| Case 6, Rel-17 | 1.14 | K=1: 0.65 (Type A), 3.66 (Type B) K=4: 2.6 (Type A), 14.62 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4) |
| Case 7, Rel-17 | 1.76 | K=1: 0.42 (Type A), 2.37 (Type B) K=4: 1.68 (Type A), 9.47 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 2.53) |
| Case 8, Rel-17 | 6.99 | K=1: 0.1 (Type A), 0.6 (Type B) K=4: 0.42 (Type A), 2.39 (Type B) | No (min gap: 3.61) | No (min gap: 9.61) |
| Case 9, Rel-17 | 1.53 | K=1: 0.48 (Type A), 2.72 (Type B) K=4: 1.94 (Type A), 10.89 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 1.11) |
| Case 10, Rel-17 | 2.18 | K=1: 0.34 (Type A), 1.36 (Type B) K=4: 1.36 (Type A), 7.64 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 4.36) |
| Case 11, Rel-17 | 1.09 | K=1: 0.68 (Type A), 3.82 (Type B) K=4: 2.72 (Type A), 15.29 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4) |
| Case 12, Rel-17 | 1.75 | K=1: 0.42 (Type A), 2.38 (Type B) K=4: 2.38 (Type A), 9.52 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 4.36) |
| Case 13, Rel-17 | 2.70 | K=1: 0.27 (Type A), 1.54 (Type B) K=4: 1.09 (Type A), 6.17 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 5.83) |
| Case 14, Rel-17 | 7.76 | K=1: 0.09 (Type A), 0.54 (Type B) K=4: 0.38 (Type A), 2.14 (Type B) | No (min gap: 3.86) | No (min gap: 9.86) |
| Case 15, Rel-17 | 1.63 | K=1: 0.45 (Type A), 2.55 (Type B) K=4: 1.88 (Type A), 10.22 (Type B) | Yes (Type B with K=4), No for the rest | No (min gap: 1.78) |
| Case 16, Rel-17 | 2.26 | K=1: 0.33 (Type A), 1.86 (Type B) K=4: 1.32 (Type A), 7.47 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 17, Rel-17 | 1.19 | K=1: 0.62 (Type A), 3.5 (Type B) K=4: 2.49 (Type A), 14 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 18, Rel-17 | 1.85 | K=1: 0.4 (Type A), 2.25 (Type B) K=4: 1.60 (Type A), 9 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 19, Rel-17 | 1.06 | K=1: 0.7 (Type A), 3.93 (Type B) K=4: 2.79 (Type A), 15.72 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 20, eDRX enhancement | 0.54 | K=1: 1.37 (Type A), 7.72 (Type B) K=4: 5.49 (Type A), 30.86 (Type B) | Yes (Type B with K=1 or 4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 21, eDRX enhancement | 0.36 | K=1: 2.05 (Type A), 11.57 (Type B) K=4: 8.23 (Type A), 46.29 (Type B) | Yes (Type B with K=1 or 4, or Type A with K=4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 22, Rel-17 | 1.13 | K=1: 0.66 (Type A), 3.69 (Type B) K=4: 2.62 (Type A), 14.75 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 23, eDRX enhancement | 0.57 | K=1: 1.29 (Type A), 7.31 (Type B) K=4: 5.19 (Type A), 29.24 (Type B) | Yes (Type B with K=1 or 4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 24, eDRX enhancement | 0.39 | K=1: 1.90 (Type A), 10.68 (Type B) K=4: 7.60 (Type A), 42.73 (Type B) | Yes (Type B with K=1 or 4, or Type A with K=4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 25, Rel-17 | 1.06 | K=1: 0.7 (Type A), 3.93 (Type B) K=4: 2.79 (Type A), 15.72 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 26, eDRX enhancement | 0.54 | K=1: 1.38 (Type A), 7.71 (Type B) K=4: 5.48 (Type A), 30.86 (Type B) | Yes (Type B with K=1 or 4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 27, eDRX enhancement | 0.37 | K=1: 2 (Type A), 11.26 (Type B) K=4: 8.01 (Type A), 45.01 (Type B) | Yes (Type B with K=1 or 4, or Type A with K=4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 28, Rel-17 | 1.16 | K=1: 0.64 (Type A), 3.59 (Type B) K=4: 2.56 (Type A), 14.37 (Type B) | Yes (Type B with K=4) | Yes (Type B with K=4), No for the rest |
| Case 29, eDRX enhancement | 0.59 | K=1: 1.25 (Type A), 7.06 (Type B) K=4: 5.02 (Type A), 28.25 (Type B) | Yes (Type B with K=1 or 4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 30, eDRX enhancement | 0.40 | K=1: 1.85 (Type A), 10.41 (Type B) K=4: 7.4 (Type A), 41.67 (Type B) | Yes (Type B with K=1 or 4, or Type A with K=4)), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 31, SRS preconfiguration | 1.33 | K=1: 0.56 (Type A), 3.13 (Type B) K=4: 2.23 (Type A), 12.53 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |
| Case 32, SRS preconfiguration | 1.42 | K=1: 0.52 (Type A), 2.93 (Type B) K=4: 2.08 (Type A), 11.73 (Type B) | Yes (Type B with K=4), No for the rest | Yes (Type B with K=4), No for the rest |

Table 3.3-3: Summary for UE power consumption results

Observation 1: Reducing the latencies involved in the legacy SDT procedure may significantly reduce the power consumption.

Observation 2: Time-domain proximity of the PRS/SRS/Paging/Reporting-Opportunity reduces the power consumption by ensuring the UE stays in sleep mode longer times and reducing the need of sleep mode switches.

Observation 3: Increasing I-DRX and/or SRS periodicities would reduce the power consumption while keeping the latency-related QoS within the required targets (e.g. 20.48, 30.72 SRS periodicities and/or I-DRX).

Observation 4: If the location is needed at the UE, the smallest Power consumption is achieved for UE-based DL Positioning

Observation 5: If the location is needed at the network, the smallest Power consumption is achieved for UL-only Positioning

Observation 6: Positioning-related (re-)configuration(s) (e.g. SDT) increase significantly the power consumption, mainly due to increased latency and longer awake time for the devices.

# Conclusions

With regards to the topic of LPHAP positioning we make the following observations and proposals:

Observation 1: Reducing the latencies involved in the legacy SDT procedure may significantly reduce the power consumption.

Observation 2: Time-domain proximity of the PRS/SRS/Paging/Reporting-Opportunity reduces the power consumption by ensuring the UE stays in sleep mode longer times and reducing the need of sleep mode switches.

Observation 3: Increasing I-DRX and/or SRS periodicities would reduce the power consumption while keeping the latency-related QoS within the required targets (e.g. 20.48, 30.72 SRS periodicities and/or I-DRX).

Observation 4: If the location is needed at the UE, the smallest Power consumption is achieved for UE-based DL Positioning

Observation 5: If the location is needed at the network, the smallest Power consumption is achieved for UL-only Positioning

Observation 6: Positioning-related (re-)configuration(s) (e.g. SDT) increase significantly the power.

Proposal 1: Support Positioning measurements in RRC Idle state.

Proposal 2: For the purpose of reduced power consumption in RRC Inactive, the following can be beneficial:

- Study ways of optimizing the SRS configuration/activation/request procedure(s) included in the UL/DL+UL RRC inactive positioning (e.g. SRS pre-configuration, RACH-based SRS request from the UE, paging-based SRS activation).

- Study ways for SRS transmission continuation after cell change in RRC Inactive (e.g., continuity of the configured SRS across cell change).

- Study PRS/SRS configuration restrictions & corresponding new UE capabilities for enabling reduced power consumption for RTT positioning.

# References

[1] RP-213588, “Revised SID on Study on expanded and improved NR positioning ”, RAN#94e, December 6-17, 2021.