# 3GPP TSG RAN WG1 110-bis-e							R1-2210007

# e-meeting, October 10th – October 19th, 2022

Agenda item:	9.11.4

Source: 	Qualcomm Incorporated

Title: 	Improved GNSS Operations for IoT-NTN

Document for:	Discussion and Decision

# Background

In RAN#94-e, the following objectives with RAN1 impact were included in the WID for eMTC/NB-IoT over NTN in Release 18:

IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

- Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In RAN1#110, the following agreements were made, with regards to this agenda item:

Agreement

GNSS assistance information that UE reports to eNB at least consists of:

- GNSS position fix time duration for measurement

- GNSS validity duration

Agreement

When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix

- FFS details of signalling

- FFS how UE reports GNSS assistance information after eNB trigger and the detailed content

- Note: further discuss whether a UE is expected to handle all eNB triggers

In this contribution, we provide our views on aspects related to improved GNSS operations for IoT-NTN.

# Baseline power consumption due to GNSS fixes

  - Parameters for evaluation of power consumption

To evaluate the power consumption impact of reading GNSS to maintain uplink synchronization (by appropriately pre-compensating the time and frequency offsets), we provide some reference parameters associated with the different transmit and receive operations that a UE may perform during establishing and maintaining a connection.

The key elements in evaluating power consumption associated with a transmit/receive operation at the UE are the current drawn by the UE (e.g., in milliamperes) and time-duration of the operation (e.g., in milliseconds). In Table 1 below, we report these numbers (with current values relative to a reference downlink reception current) for a typical NB-IoT over NTN scenario (e.g., a good coverage LEO satellite setting for Set 2 in [1]) corresponding to a downlink SNR (for 15 kHz 1 PRB reception) of 0 dB and an uplink SNR (for 15 kHz 1 PRB transmission) of -5 dB (with a PC5 UE transmitting at the max. power of 20 dBm).

Table 1: Parameters for evaluating power consumption in IoT over NTN.

| Operation | Current (Referenced to downlink current CDL) | Duration |
| --- | --- | --- |
| GNSS reception | CGNSS=1.1×CDL | 2000 ms |
| Downlink Reception | CDL | - PDCCH: 40 ms - PDSCH (RAR): 8 ms - PDSCH (Msg4): 8 ms - PDSCH (Conn. Release): 8 ms |
| Uplink Transmission | CUL=4.3×CDL | - PRACH: 1.6 ms - Msg3: 8 ms - PUSCH (data): 8 ms per ~1000 bits (simulated with 8000 bits per ON-duration) - HARQ-ACK: 8 ms |
| Sleep | Csleep=5.5×10-5×CDL | - PSM: 8 hrs - CDRX: ~10s |

  - Power consumption—short, sporadic connections

We model a short, sporadic connection according to Fig. 1 below. Essentially, the IoT UE is assumed to transmit its payload once every 8 hrs, once every 24 hrs, etc; after transmitting the payload, the UE’s connection is released, and it goes back into deep sleep mode, until before the next transmission occasion.

Figure 1: Short, sporadic transmissions for IoT over NTN.

In this setting, we observe that, acquiring a GNSS fix before every connection results in ~34% of the UE’s total power consumption resulting from GNSS alone. While there may be subtle differences depending on exact implementation, the general observation that GNSS consumes a significant chunk of the UE’s power—and thereby, directly reduces the expected battery life of the UE—is valid.

Observation 1: Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy.

For UEs that are mobile, e.g., say tracking devices, etc., that are operating in this short, sporadic connection paradigm, this power penalty due to GNSS cannot be mitigated significantly, under the purview of Release 17 assumptions of GNSS-based uplink pre-compensation.

However, for UEs that are fixed, e.g., smart meters, etc, these UEs may be able to save power by having a much more relaxed (e.g., once a week, or once a month, depending on the setting) GNSS fixing schedule.

  - Power consumption—long connections (e.g., based on CDRX)

We model an example long connection according to Fig. 2 below. In this instance, the IoT UE may remain in connected mode for a significantly longer duration of time than the short, sporadic connections described above. These may be facilitated e.g., via connected mode DRX (CDRX), wherein much larger payloads are transmitted or received by the UE during the longer connection.

Figure 2: Long connection with connected mode DRX for IoT over NTN.

In this setting, we observe that a GNSS fix before every uplink transmission occasion (which, in the absence of other mechanisms, may end up being required to maintain uplink synchronization accuracy within specified limits) results in ~45% of the UE’s total power consumption resulting from GNSS fixes. While (as described above) we can mitigate this somewhat for fixed UEs, for mobile UEs (especially UEs moving at high speeds), without other enhancements to connected mode synchronization, we may not be able to avoid this hit to the UE’s power consumption on account of GNSS fixes.

Observation 2: Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization.

- This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix

# Closed loop time and frequency correction

  - Power savings

We show, in this section, that the above penalty to UE power consumption (during long connections) from GNSS fixes can be mitigated significantly by facilitating closed-loop time and frequency corrections issued by the base-station.

This may be achieved by a (potentially periodic, or prior to each uplink transmission) NPRACH transmission from the UE, followed by a timing and/or frequency correction command issued by the network in a response message. Especially in the setting where these NPRACH resources are robust to time and frequency synchronization errors (see, e.g., the design proposed in Section 4 of this contribution), such solutions can dramatically reduce the power consumption penalty from GNSS fixing during a long connection, by essentially replacing a GNSS fix with an NPRACH followed by a closed loop correction.

Such a strategy is (approximately) schematically depicted in Fig. 3 below.

Figure 3: Relaxed GNSS fixing using NPRACH-based closed loop corrections.

In Table 2, we demonstrate how the power penalty due to GNSS fixes diminishes with an increase in the GNSS relaxation factor, which essentially denotes—relative to the baseline scheme in Fig. 2—the factor by which we reduce the total number of GNSS fixes required and replace it with adequate close-loop corrections.

Table 2: Reduction in power penalty due to GNSS, by relaxing required GNSS fixes and enabling NPRACH-driven closed-loop time and frequency corrections.

| GNSS Relaxation factor | Power penalty due to GNSS in connected mode for long connections |
| --- | --- |
| No Relaxation (Baseline, as shown in Fig. 4) | 45% |
| 2 | 29% |
| 4 | 17% |

Based on the results above, we make the following observations and proposals with regards to supporting long connections in eMTC and NB-IoT over NTN.

Observation 3: For long connections in eMTC and NB-IoT over NTN, NPRACH-driven closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections.

- An NPRACH design that is robust to time and frequency errors (e.g., the one based on restricted preambles in Section 4 of this contribution) is especially suitable for this.

Proposal 1: RAN1 to specify NPRACH-driven closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes.

  - Modifications to base TA for PRACH transmission

Currently, a UE uses the following formula to determine the timing advance—i.e., the offset applied to an uplink frame w.r.t a corresponding downlink frame

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs

where NTA denotes the TA component based on accumulating “TA commands” received from the base-station.

In response to NPRACH (received in a RAR), the TA command specifies the “entire NTA” (as opposed to “incremental” updates).

The implication of this is that any time a UE transmits a NPRACH, it uses a value of NTA=0 (i.e., it flushes any accumulated TA commands that it may have received, e.g., via MAC-CEs). Thus, as an example, an NPRACH triggered by a “PDCCH order” in connected mode also uses NTA=0.

Observation 4:  According to current specifications, any time a UE transmits a NPRACH, it uses a value of NTA=0 in calculating the TA for the NPRACH transmission.

In NTN, NTA, adjUE denotes the TA component specific to satellite communications and is determined by the UE (to “pre-compensate” large UE-satellite round-trip delays) based on the UE’s own location (e.g., by acquiring a “GNSS position fix”), the NTN serving satellite’s location (e.g., by reading the satellite ephemeris info broadcast on SIBs), and the common TA.

The accuracy (over time) of NTA, adjUE depends on the accuracy of location info. For example, if a considerable amount of time has passed since the last GNSS position fix, say for a mobile UE, the accuracy of this term can be thrown off.

Observation 5: If a considerable amount of time has passed since the last GNSS position fix, e.g., for a mobile UE, the accuracy of NTA, adjUE becomes progressively worse over time, i.e., the error in NTA, adjUE increases over time.

One solution to this problem is for the eNB to issue TA commands so that NTA compensates the error introduced in NTA, adjUE. This approach, however, has the drawback of not being usable for NPRACH, since NPRACH uses an NTA=0. Therefore, if the error introduced by NTA, adjUE is larger than the maximum correction capability of NPRACH, the UE would fail random access in connected mode (in case of, e.g., PDCCH order or SR).

Observation 6: Although the eNB can progressively correct (by issuing TA commands) the timing error due to a stale UE location, this correction is not applied when transmitting NPRACH (which currently uses NTA=0). This may cause the timing error to go beyond the NPRACH correction capability.

To mitigate this, several solutions can be considered, such as updating the NTA, adjUE term after each NPRACH transmission, so that it acts as a “proxy” for an updated GNSS fix, without needing to spend precious power in acquiring a GNSS fix every time. Other solutions may include—for NPRACH in connected mode for NTN—allowing “accumulation” in the NTA term, after every response (from the network) corresponding to a NPRACH transmission.

With these solutions, it is feasible to realize a “power-optimal” scenario, wherein, a UE only reads GNSS (ideally) once before a connection, and subsequently, with the above adjustments to the TA for NPRACH, receives “closed loop fixes” to keep its TA synchronized for uplink transmissions.

We also note that, the discussion above, in the context of TA, would also need to be extended to “(doppler) frequency correction”, for maintenance of uplink synchronization—which comprises both time and frequency pre-compensation.

In the light of the above discussion, we make the following proposals:

Proposal 2: RAN1 to discuss solutions such that the residual TA that is to be corrected by a “closed loop” command (upon transmission of a NPRACH in connected mode), does not exceed the correction capability afforded by such a NPRACH transmission. Candidate solutions include:

- Updating the NTA, adjUE term with every TA command received in response to transmitting a NPRACH in connected mode

- Accumulating prior TA commands into the NTA term, as opposed to universally setting it to 0 for a NPRACH transmission

Proposal 3: Proposal 2—related to TA—shall also translate to an analogous solution for closed-loop residual (doppler) frequency correction.

  - Doppler frequency errors due to outdated GNSS

We consider a LEO satellite at an orbit of 1200 km, and depict below, the progression of the residual (uncorrected) Doppler frequency shift that may occur, as a function of time that has elapsed since the last GNSS fix. For this depiction, we assume that the starting position (the position at which the UE has a fresh GNSS fix) is at the nadir beam, where the satellite is directly above the UE (elevation angle 90 degree); thereafter, the satellite and the UE move in opposite directions with their respective velocities. The error (uncorrected Doppler shift) is then calculated by the difference between the “exact” Doppler shift (had the UE known the GNSS in real time) and the “estimated” Doppler shift—assuming the UE doesn’t update its GNSS location after the initial time point. Note here, we assume that we know the satellite’s ephemeris (e.g., PVT) “exactly” at all points for this depiction, which is optimistic.

We see above that, for high UE speeds, even within a timespan of 2 minutes, there can be an approximately 220 Hz frequency shift on account of an outdated GNSS, which would result in significant performance degradation for an NB-IoT UE with a subcarrier spacing of 3.75 kHz and may make it difficult for the UE to meet the requirements set by RAN4.

Observation 7: For mobile UEs, with a GNSS fix that is outdated by 2 minutes, there may be a residual doppler frequency error of 220 Hz, with LEO satellites at a 1200 km orbit operating in the S band.

- This error may be outside the synchronization requirements set by RAN4.

- This may cause significant interference among UEs in the uplink when a 3.75 kHz subcarrier spacing is used for NB-IoT.

- This may be mitigated by introducing a closed-loop frequency correction mechanism, as proposed in this contribution.

- Requiring fresh GNSS fixes within such short intervals (e.g., 2 minutes) severely degrades battery life of the UE.

# Robust NPRACH design to enable reducing GNSS fixes

For the following aspects:

- Accessing a cell from IDLE mode (without essentially being required to always get a GNSS fix immediately before attempting to transmit PRACH)

- NPRACH-driven closed loop corrections in connected mode (e.g., during a long connection, after some inactive time)

a robust NPRACH design that can correct relatively large time and frequency offsets without significant specification impact is important.

Such a design may allow a relaxation of the time/frequency synchronization requirements (for NPRACH transmission) for a UE, which would translate to a reduction in the number and frequency of GNSS fixes required before and during each connection.

Proposal 4: RAN1 to consider specifying (at least a subset of) NPRACH resources with increased robustness to time and frequency errors, to facilitate:

- Accessing a cell from IDLE mode, while relaxing the requirement of an “immediately preceding” GNSS fix in all instances.

- Closed-loop corrections (e.g., after periods of UE inactivity), thereby reducing the number of GNSS fixes required during a connection.

One such design is to restrict NPRACH starting subcarriers (e.g., over a subset of all available starting subcarriers) that a UE can use for contention-based random access (CBRA). The simplest example is shown in Fig. 4 below, where “alternate starting subcarriers” can be selected by UEs for CBRA.

Figure 4: Example of "restrictions" on starting NPRACH subcarriers for CBRA. Alternate starting subcarriers may be selected for NPRACH transmission by a UE.

We show in Fig. 5 that with this restriction, the robustness of NPRACH to time and frequency errors improves significantly. This is due to the increased resiliency to ICI among preambles that is afforded by this design. Even with relatively large initial uplink frequency errors (e.g., up to 1kHz), the base station can determine and correct for these errors effectively (as demonstrated by the CDFs of residual timing and frequency errors after base-station processing of the NPRACH preamble). In the CDFs in Fig. 5, we see that with the alternate subcarrier restriction scheme (the red curve), the probability of the residual timing errors (after base-station processing) being outside +/- CP/2 (of NPUSCH) is very close to that of “unloaded” transmission—i.e., when only one preamble is transmitted—even with an up to 1 kHz initial frequency synchronization error. In contrast, in the “fully loaded” setting—i.e., when all preambles are used for NPRACH transmission by (e.g., other) UEs (represented by the cyan curve), the probability of residual timing errors after base-station processing being greater than +/- CP/2 increases significantly.

Figure 5: Cumulative distribution functions (CDFs) of residual timing error after base-station processing of NPRACH preambles transmitted by UE(s). In this setting, initial frequency errors can be up to +/- 1 kHz, and the power levels of different UEs are within +/- 10 dB of each other.

Observation 8: Restricting alternate starting subcarriers for NPRACH transmissions allows to correct for potentially large initial uplink frequency synchronization errors (e.g., up to 1 kHz)

- Such a scheme may facilitate UE power savings by relaxing the frequency and accuracy of GNSS fixes required.

- Such a scheme may also facilitate NPRACH-driven closed-loop corrections of time and frequency errors in connected mode, thereby reducing the power penalty from frequent GNSS fixes.

# Connected Mode GNSS (re)acquisition

  - Connected mode GNSS measurement as a fallback option

In RAN1#109-e, the following agreement was made with regards to GNSS acquisition in connected mode.

Agreement

At least the following options can be considered on GNSS measurement in connected for potential enhancements for improved GNSS operations:

- Option 1: UE re-acquires GNSS position fix during RLF procedure

- Option 2: UE re-acquires GNSS position fix with a new gap

Note: this does not imply that a Rel-18 IoT NTN UE is mandated to support one or both of the options.

In our view, we believe that connected mode GNSS measurement should ideally be a “fallback” feature—i.e., if (after an initial GNSS fix before initiating connection), closed-loop time and frequency corrections fail to keep the UE synchronized for the purposes of uplink synchronization, the UE may be provided with an opportunity to attempt to re-acquire GNSS in connected mode, without having to tear down the entire connection and re-start from IDLE.

A connected mode GNSS measurement should not be a “routine” occurrence, that is somehow mandated multiple times during a connection, since the obvious disadvantages from a power consumption standpoint of such an approach are well understood (see for example, the analyses in Sections 2 and 3.1 of this contribution).

Observation 9: GNSS measurement in connected mode should be considered a fallback procedure (as opposed to a routine, expected occurrence), e.g., when closed-loop time and frequency commands fail to maintain uplink synchronization in time and frequency within stipulated requirements.

To this end, we believe that connected-mode GNSS measurement should be specified in the context of a “recovery” procedure, prior to declaration of RLF—in much a similar fashion to what is done in Release 17 for recovering from an uplink synchronization expiry due to satellite ephemeris and/or common TA.

Proposal 5: A UE may re-acquire GNSS as part of a “recovery” procedure before the declaration of an eventual RLF (if the recovery fails within a certain time), in a similar fashion to Release 17 recovery procedures for ephemeris and common TA parameters upon validity expiry.

  - Backwards compatibility with Release 17 networks

Note that Release 17 UE behavior upon GNSS validity expiry was for the UE to go back to IDLE mode. Hence, that is the behavior that a Release 17 base-station will expect from any NB-IoT or eMTC UE (including Release 18 UEs) that connect to it.

To make sure that there are no incompatibility issues between a Release 18 UE that is “capable” of performing connected mode GNSS measurements as part of recovery procedure, such a capability must—in some sense—be “ratified” by an indication of support of such UE behavior by the base station. In other words, a Release 18 base station may indicate that it supports UEs that can perform a connected mode GNSS acquisition. This, interpreted in conjunction with a UE’s capability to perform the same, will result in an unambiguous understanding between the network and the UE, as to how the UE is expected to behave, should uplink synchronization be lost on account of a GNSS validity expiry event.

Proposal 6: The network shall indicate whether it supports UE behavior associated with connected mode GNSS measurements, as part of a recovery procedure upon expiry of GNSS validity.

- This avoids backwards compatibility issues w.r.t a Rel 17 base-station, that always expects UEs to go to IDLE mode upon GNSS validity expiry.

# Conclusion

In this contribution we presented our views on enhancements to NB-IoT/eMTC over NTN as it relates to improved GNSS operations. We summarize our proposals below.

On power consumption due to GNSS fixes, we make the following observations:

Observation 1: Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy.

Observation 2: Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization.

- This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix

Observation 3: For long connections in eMTC and NB-IoT over NTN, NPRACH-driven closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections.

On specifying, and facilitating closed-loop time and frequency corrections to minimize the number of GNSS fixes required, we make the following proposals and observations:

Proposal 1: RAN1 to specify NPRACH-driven closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes.

Observation 4:  According to current specifications, any time a UE transmits a NPRACH, it uses a value of NTA=0 in calculating the TA for the NPRACH transmission.

Observation 5: If a considerable amount of time has passed since the last GNSS position fix, e.g., for a mobile UE, the accuracy of NTA, adjUE becomes progressively worse over time, i.e., the error in NTA, adjUE increases over time.

Observation 6: Although the eNB can progressively correct (by issuing TA commands) the timing error due to a stale UE location, this correction is not applied when transmitting NPRACH (which currently uses NTA=0). This may cause the timing error to go beyond the NPRACH correction capability.

Proposal 2: RAN1 to discuss solutions such that the residual TA that is to be corrected by a “closed loop” command (upon transmission of a NPRACH in connected mode), does not exceed the correction capability afforded by such a NPRACH transmission. Candidate solutions include:

- Updating the NTA, adjUE term with every TA command received in response to transmitting a NPRACH in connected mode

- Accumulating prior TA commands into the NTA term, as opposed to universally setting it to 0 for a NPRACH transmission

Proposal 3: Proposal 2—related to TA—shall also translate to an analogous solution for closed-loop residual (doppler) frequency correction.

Observation 7: For mobile UEs, with a GNSS fix that is outdated by 2 minutes, there may be a residual doppler frequency error of 220 Hz, with LEO satellites at a 1200 km orbit operating in the S band.

- This error may be outside the synchronization requirements set by RAN4.

- This may cause significant interference among UEs in the uplink when a 3.75 kHz subcarrier spacing is used for NB-IoT.

- This may be mitigated by introducing a closed-loop frequency correction mechanism, as proposed in this contribution.

- Requiring fresh GNSS fixes within such short intervals (e.g., 2 minutes) severely degrades battery life of the UE.

With regards to increasing the robustness of NPRACH and facilitating relaxations to GNSS measurements, we make the following proposal and observation:

Proposal 4: RAN1 to consider specifying (at least a subset of) NPRACH resources with increased robustness to time and frequency errors, to facilitate:

- Accessing a cell from IDLE mode, while relaxing the requirement of an “immediately preceding” GNSS fix in all instances.

- Closed-loop corrections (e.g., after periods of UE inactivity), thereby reducing the number of GNSS fixes required during a connection.

Observation 8: Restricting alternate starting subcarriers for NPRACH transmissions allows to correct for potentially large initial uplink frequency synchronization errors (e.g., up to 1 kHz)

- Such a scheme may facilitate UE power savings by relaxing the frequency and accuracy of GNSS fixes required.

- Such a scheme may also facilitate NPRACH-driven closed-loop corrections of time and frequency errors in connected mode, thereby reducing the power penalty from frequent GNSS fixes.

With regards to GNSS measurements in connected mode, we make the following observation and proposals:

Observation 9: GNSS measurement in connected mode should be considered a fallback procedure (as opposed to a routine, expected occurrence), e.g., when closed-loop time and frequency commands fail to maintain uplink synchronization in time and frequency within stipulated requirements.

Proposal 5: A UE may re-acquire GNSS as part of a “recovery” procedure before the declaration of an eventual RLF (if the recovery fails within a certain time), in a similar fashion to Release 17 recovery procedures for ephemeris and common TA parameters upon validity expiry.

Proposal 6: The network shall indicate whether it supports UE behavior associated with connected mode GNSS measurements, as part of a recovery procedure upon expiry of GNSS validity.

- This avoids backwards compatibility issues w.r.t a Rel 17 base-station, that always expects UEs to go to IDLE mode upon GNSS validity expiry.

# References

[1] R1-2103070, “Scenarios applicable to NB-IoT/eMTC” by Qualcomm Incorporated, RAN1 104-bis-e.