| 3GPP TSG RAN WG1 Meeting #110-bis-e | R1-2210051 |
| --- | --- |
| Online, 10th – 19th October 2022 |  |

| Agenda Item: | 9.13.1 |
| --- | --- |
| Source: | EURECOM |
| Title: | Discussion on Evaluation on Low power WUS |
| Document for: | Discussion and Decision |

# Introduction

The SI [1] on low-power (LP) wake-up signal (WUS) and receiver for NR has been approved in RAN#94e with the main goal to study WUS and receiver architectures that allow for an independent low-power receiver implementation.

Objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms and their coverage availability, as well as latency impact. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

The following objectives are included in the SI [1]:

The agenda item focuses on the first two points in the SI objectives and our contribution primarily concentrates on link-level evaluations.

# Use cases and KPIs

As stated in the objectives, the primary use cases target devices with significant power constraints. LP-WUS strives to enable longer battery life and hence less maintenance costs for these type of devices. Moreover, we distinguish two high level use cases, namely low-latency and latency-tolerant use cases.

Low-latency use-cases require higher LP-WUS duty cycles or monitoring occasions and hence the overall power-consumption of the wake-up radio (WUR) will be higher compared to latency-tolerant applications. Consequently, there is a fundamental trade-off between latency and power consumption.

Examples of low-latency use cases are: Actuator control (e.g. electronic valves), On-demand (near real-time) measurements from sensors (temperature, pressure, location, etc).

Latency-tolerant uses cases include: Wearables, smart metering or control of health devices like connected insulin pumps in a pediatric setting, where the patient cannot make treatment decisions and/or is unable to operate/carry a cell-phone with Bluetooth connection to the pump.

Important KPIs include:

- Power consumption

- Latency

- Unnecessary wakeup: UEs that are woken up because they are part of a group of UEs where other UEs are concerned by the wakeup. This is due to UE grouping which is necessary if the WUS capacity is insufficient to independently indicate individual UEs.

- False wakeup: UE wakes up because of erroneous WUS decoding

- False alarm: UE wakes up because it detected a WUS when there was one transmitted

The unnecessary and false wakeup events all impact the overall power consumption and latency. However, we consider false alarms to be a rare event (compared to a false wakeup) if the UE grouping and duty cycles are dimensioned correctly.

# WUS Link-Level evaluation assumptions

Our basis assumptions for the link-level evaluation are the following:

WUS capacity: We consider the WUS capacity to be in the range of 3-12 bits. More precisely, we assume as in previous WUS designs that some kind of UE grouping is performed. A low capacity has the advantage that the WUS signaling is more efficient, i.e. the WUS is shorter and more reliable.

Modulation: We primarily focus on multi-carrier OOK as it enables seamless integration into the existing NR transmitting chain as well low-power receiver architectures.

The simulations are carried out in base-band, i.e. a low-pass filter is used to extract the WUS from the overall system bandwidth followed by a decimation to reduce the sampling rate.

We focus on BLER performance, i.e. zero error is declared if all the bits have been received correctly and the block is in error if at least one bit is wrong.

More detailed assumptions are given in Table 1.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2 GHz |
| System BW | 10MHz (52 PRBs) |
| SCS | 15kHz |
| Antenna configuration | 1Tx, 1Rx |
| WUS BW | K = 64 SCs |
| Low Pass Filter Type | 3rd order Butterworth |
| LPF Cut-off Frequency | 2*(K/2+0.5)*SCS Hz |
| WUS Waveform | MC-OOK, DFT-precoded MC-OOK |
| Channel | AWGN, TDL-C 300ns |
| Doppler | 0 |
| Receiver | Energy Detector |
| Payload | 3-12 bits |

Table 1: Link-Level simulation assumptions.

# Link-Level Simulation Results

In this section, we provide link level simulation results. These results are not intended for absolute reference but to show certain trade-offs in the choice of design parameters as well as to compare different WUS schemes. The reader is referred to our companion contributions [2] and [3] for more details on receive algorithms and WUS design, respectively.

## MC-OOK

In Figure 1, we simulate the performance of multi-carrier OOK with various number of allocated sub-carriers K and different receive algorithms. ED refers to energy/envelope detection, CD to coherent detection and NCD to non-coherent detection, cf. [2].

Figure 1: MC-OOK, AWGN, 10 symbols

From Figure 1, we observe that K has a significant impact on the performance of the ED. Larger values of K require wider filter BWs and hence more noise is captured. If all power is concentrated into a single sub-carrier, K=1, the performance is best since less noise is captured after filtering. This suggests that smaller values of K are to be preferred, however as it is shown in the next results for fading channels (cf. Figure 3), small values of K are unable to captured the multi-path diversity.

On the other hand, CD and NCD are independent of K.

In Figure 2, we fix K=64 and apply a rate R=1/2 Manchester code to explore the impact on the receiver performance. Instead of threshold detection, the ED compares the power of the 2 encoded bits.

Figure 2: MC-OOK, AWGN, 10 symbols, Manchester code R=1/2

From Figure 2, we observe that the performance of ED (ED-MC) significantly improves compared to threshold detection (ED), about 4.8dB resulting from a SNR gain of 3dB, since double the power is transmitted, as well as a gain due to improved detection. Both CD and NCD only achieve the 3dB power gain.

We conclude that Manchester coding significantly increases the performance of ED and avoids complex threshold detection which depends also on the channel.

Observation 1: Manchester coding significantly improves performance of ED.

In Figure 3, we compare the performance of the different receivers and K=12, 64 for the TDL-C channel with 300ns delay spread.

Figure 3: MC-OOK, TDL-C 300ns, 10 symbols, Manchester code R=1/2.

From Figure 3, K=64 has better performance than K=12, because more multi-path diversity can be exploited. For low SNR, the ED with K=12 outpeforms K=64, since in that region the noise is dominant and K=12 rejects more noise due to the narrower filter. However, for higher SNR the diversity gain of K=64 outweighs the SNR gain of K=12.

Observation 2: For MC-OOK, the number of allocated sub-carriers is a trade-off between SNR gain and multi-path diversity gain.

## DFT-precoded MC-OOK

In this section we evaluate the proposed DFT-precoded MC-OOK, [3], where multiple bits are transmitted per OFDM symbol.

Figure 4 shows the performance of ED for DFT-precoded MC-OOK for various code rates. Rate R=1/2 refers to conventional Manchester coding where one bit is encoded into 2 bits. R=2/4 has the same code rate but denotes that 2 bits are encoded into 4 bits.

As expected, there is a 3dB gain of R=2/4 compared to R=1/2. Further decreasing the code rate to R=3/8 achieves a gain of 3.3dB (power gain plus coding gain). For comparison, if we decrease the number of SCs per coded bit from KB=8 to KB=4, the performance loss is significant since the diversity of the channel cannot be exploited.

Figure 4: DFT-precoded MC-OOK, TDL-C 300ns, 1 symbol, different coding schemes, K=64, KB=8.

Observation 3: Jointly encoding multiple bits results in significant performance gain.

## DFT-precoded MC-OOK with overlay code in time-domain

In Figure 5 we compare the performance of DFT-precoded MC-OOK with the vertical-horizontal coding strategy presented in [3]. Here, we fix the payload to 12 bits over 12 symbols. For DFT-precoded MC-OOK we encode 3 bits per symbol and repeat that symbol three times. The VH-code uses the same encoding as the DFT-precoded MC-OOK but additionally encode 3 bits through different sequences. We use cyclically shifted Zadoff-Chu sequences in this example. Moreover, 6 bits are encoded in time-domain via a non-coherent overlay code which encodes 6 bits into 12 complex symbols. The generator matrix G in GF4 is given by

G= 100122333333010323123333001212220123

The complex modulated codeword entries wmn are generate as

wmn=ei2πcn/4

where cn is the nth entry of c=dG, with d being the sequence of input bits in base 4.

We simulate two receivers, the full complexity receiver (‘VH, Full’) which correlates the received signal with all possible messages and a low-complexity receiver (‘VH, LowComp’). The latter does a staged detection, i.e. ED, sequence detection and detection of overlay code, with 2 hypothesis for the last 2 stages. Thus, significantly reducing the computational complexity, cf. [2].

From Figure 5, we observe a gain of almost 6dB between DFT-precoded MC-OOK with repetition and the low-complexity VH detection. Compared to the full detection, the low-complexity receiver loses about 2.3dB.

The simulation results suggest that a significant performance gain can be achieved with more elaborate coding strategies at the expense of increases receiver complexity.

Figure 5: VH-code, AWGN, 12 symbols

Observation 4: More elaborate coding strategies significantly increase spectral efficiency while maintaining moderate decoding complexity.

# Conclusion

In this contribution the following proposals and observations have been made:

Observation 1: Manchester coding significantly improves performance of ED.

Observation 2: For MC-OOK, the number of allocated sub-carriers is a trade-off between SNR gain and multi-path diversity gain.

Observation 3: Jointly encoding multiple bits results in significant performance gain.

Observation 4: More elaborate coding strategies significantly increase spectral efficiency while maintaining moderate decoding complexity.

# References

[1] RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94e

[2] R1-2210052, “Discussion on Low power WUS receiver architectures”, EURECOM, RAN1#110b-e

[3] R1-2210054, “Discussion on L1 signal design and procedure for low power WUS“, EURECOM, RAN1#110b-e