| 3GPP TSG RAN WG1 Meeting #110-bis-e | R1-2210052 |
| --- | --- |
| Online, 10th – 19th October 2022 |  |

| Agenda Item: | 9.13.2 |
| --- | --- |
| Source: | EURECOM |
| Title: | Discussion on Low power WUS receiver architectures |
| Document for: | Discussion and Decision |

# Introduction

The SI [1] on low-power (LP) wake-up signal (WUS) and receiver for NR has been approved in RAN#94e with the main goal to study WUS and receiver architectures that allow for an independent low-power receiver implementation.

LP-WUS has been studied and specified in other RATs, most notably in IEEE 802.11ba.

In 3GPP wake-up signals are specified in both LTE-M/NB-IoT as well as NR-Rel-17 under the name of paging early indication (PEI). The key difference between those WUS and a LP-WUS is that the LP-WUS is received with a wake-up receiver (WUR) independent of the main radio, i.e. the main radio can be turned off.

Objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms and their coverage availability, as well as latency impact. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

The following objectives are included in the SI [1]:

This agenda item discusses possible low power receiver architectures. In this contribution we focus on OOK modulation since it is assumed by most low-power receivers [5]. Moreover, we concentrate on various receive algorithms rather than detailed receiver hardware description.

# Receiver algorithms for OOK

Consider a narrow-band multi-carrier transmission with K sub-carriers, where the received vector of OFDM symbol l on receive antenna p is given by

rl,p=Hl,pFsl,m+nl,p

With sl,m the transmit signal for message, Hl,p=diag(hl,p,1,hl,p,2,…,hl,p,K), hl.p,k is the complex channel coefficient on sub-carrier k, k=1,2,…,K and F is the DFT matrix of size K incase DFT-precoded MC-OOK is used. The noise vector nl,p=[nl,p,1,nl,p,2,…,nl,p,K] is i.i.d. complex Gaussian with zero mean and variance σ2, i.e. nk~NC0,σ2.

## Envelope Detection

A non-coherent receiver only has access to the magnitude (or envelope) r(t) of the received signal r(t) in time-domain, that is, the phase is unknown.  This has the significant advantage of not requiring a local oscillator (PLL) which is one of the most power consuming components of a receiver. Consider the case of AWGN with base-band system model

r=sm+n

In case of OOK, s0=0 and s1=A, the envelope r has the following distribution p(r), [2] (Equation 7-4-6)

pr=2rσ2e-r2+A2σ2I02rAσ2

where I0(z) is the modified Bessel function of the first kind and zeroth order. The optimal threshold b to decide if  s0 or s1 was transmitted is given as the solution to [1] (Equation 7-4-13)

e-γI0b02γ=1

with SNR γ=A2σ2 and b0=b2/σ2.  The above equation can be well approximated by [2] (Equation 7-4-14)

b=σ1+γ4

( 1)

In the limiting cases we have limγ→0b=σ and limγ→∞b=A2, i.e. for high SNR, the envelope detector is the same as the coherent detector. Hence, an Envelope detector decides s1if

r>b

and s0 otherwise. An Energy detector or quadratic envelope detector decides s1 if

r2>b2=σ21+γ4

Equation 1: Threshold detection rule for energy detector in AWGN.

In case of fading channels, the threshold detection is non-trivial but can be based on channel statistics to determine optimal threshold levels through simulations, which are subsequently stored in the receiver.

If the transmitted bits are not encoded, a threshold detection is utilized as depicted in Figure 1.

Figure 1: Block Diagram of an Energy detector with threshold detection.

The base-band signal r(t) is obtained in the usual way by down-conversion, analog-digital converter and filtering of the WUS. Subsequently, the energy is computed by squaring the magnitude of the samples. Then, the energy is accumulated over the samples T that represent one coded bit at offset ti.

uin=p=1Pt=titi+Trpt2

Denote un={u0n,u1n,…,uN-1n} the accumulated energy values of each coded bit in codeword n. Recall that P is the number of receive antennas. The accumulated energy ui of bit i is compared to a threshold g as

bi=0,  if ui≤g1,  if &ui>g

For instance, in AWGN, the threshold in equation ( 1) can be used with g=b2 normalized to number of receiver antennas.

If the bits are encoded, e.g. via a Manchester code, see [3], decoding has to be carried out instead of threshold detection as Figure 2.

Figure 2: Block-diagram of envelope detector with coded WUS.

Then, the corresponding estimated input message mn is given by

mn= argmaxi=0,1,..,N-1uin

The bits b are obtained by transforming mn into its binary representation.

## Coherent Detection

The optimal MAP detector for the narrow-band SISO model is given by [4] (Chapter 4.2-1)

m=argmaxmσ22lnPm-12r-Hsm2

where Pm is the probability of message m being transmitted and H=diag(h1,h2,…,hK) is the channel estimate. The channel estimate can obtained via the known preamble sequence assumed to be transmitted before the WUS data part, [3]. Note that we drop the symbol index l. The above equation can be rewritten as

m=argmaxmηm+R(smHr)

with matched filter output r= HHr and ηm given by

ηm=σ22lnPm-12Hsm2

For equally likely symbols sm, i.e. PM=1/M, we obtain

m=argminmr-Hsm

The decision region Dm is given by [4] (Chapter 4.2-1)

Dm=r∈CK: RsmHr+ηm> Rsm'Hr+ηm',∀1≤m'≤M,m'≠m

In case of MC-OOK the two transmit signals are s0=0 and s1=a. Note that the modulation symbols for the ON-signal a must be known for a coherent receiver. The decision rule that s1 has been transmitted reads

Rs1Hr>η1

For the special case where ak=1K, ∀k and PM=1/M we obtain

k=1KRrk>h22K

where  H=diag(h).

If a rate R=1/2 Manchester code is used, the transmit messages are given by s0=0,aTT and  s1=aT,0T. Consider r=r1,r2T, where r1 and r2 are the matched filer outputs associated with the first and second set of sub-carriers, respectively. The decision region D1 reads

D1=r∈CK: RaHr1-r2>η0-η1

Assuming ak=1K, ∀k and PM=1/M we obtain

D1=r∈CK: k=1KRr1k-Rr2k>-h22-h122K

In case of DFT-MC-OOK, there are B coded bits transmitted per OFDM symbol, i.e. we have M=2B possible messages sm. Those bits are decoded jointly using the MAP metric above.

## Coherent reception with unknown channel

We consider a coherent reception but without access to a channel estimate. In this case the optimal receiver is given by

m=argmaxmsmHr2

The receiver correlates the received signal with all possible transmit messages.

# Receiver algorithms for VH-coding structure

The optimal ML receiver is given by

m=argmaxmp=1Pl=1Lrl,pHsl,m2

where sl,m is the hypothesis of message m in symbol l.

Figure 3: Hierarchical reduced complexity detection.

However, the VH-code structure allows for a hierarchical detection, reduced complexity, where messages mv0, mv1 and mh are detected separately. The block-diagram is shown in Figure 3.

mv0=argmaxmv0p=1Pl=1Lk=mv0KB+1mv0KB+1+KBrk,l,p2

First, mv0 is decoded with an energy detector, i.e. after aligning the offsets per OFDM symbol, we have

In the next step, mv1 is detected under different hypothesis mv0, by correlating with all possible sequences smv1.

mv1=argmaxmv1p=1Prp,mv0Hsmv12

where rp,mv0 are the extracted ON-signals under hypothesis mv0 of the received vector on antenna p.

In the last step, the horizontal code mh is detected as

mh=argmaxmhp=1Pl=1Lrl,pHsl,mh,mv0,mv12

where sl,mh,mv0,mv1are the hypothesis on mh given the hypotheses on the vertical dimension mv0,mv1.

From the hypotheses, and depending on the bit mapping, the overall message m is obtained as

m=mv0+mv12Bv0+mh2Bv0+Bv1

This receiver can significantly reduce the complexity. For instance, consider Bv0=3, Bv1=3 and Bh=4, that is B=10 bits of payload. The full complexity receiver requires 210=1024 correlations, where as the low complexity receiver does ED followed by 23=8 correlations for Bv1 and 24=16 correlations for Bh assuming only one hypothesis per step.

# Discussion

Envelope detection allows for the simplest and least power consuming receive architecture implementation since it does not require a PLL for coherent detection.

A correlation receiver requires the ON-signal to be known as well as the carrier phase for coherent demodulation. This will results in an implementation with significantly higher power consumption. On top, a coherent detection requires channel estimation adding to the hardware complexity.

However, as described in [3] the WUS may be designed to allow for a hierarchical detection. That is, the most crucial information, e.g. the group ID, can be detected with a simple low-power ED. Depending on the result, the device either shuts down (it is not in the detected group) or turns on additional HW for the decoding of additional information.

Proposal 1: Consider receiver architectures for hierarchical decoding of the WUS.

# Conclusion

In this contribution we presented and discussed several receive algorithms:

Proposal 1: Consider receiver architectures for hierarchical decoding of the WUS.

# References

[1] RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94e

[2] M. Schwartz, W. R. Bennett, and S. Stein, “Communication systems and techniques”, John Wiley & Sons, 1995.

[3] R1-2210054, “Discussion on L1 signal design and procedure for low power WUS“, EURECOM, RAN1#110b-e

[4] J. G. Proakis and S. M., Digital Communications, vol. 4, New York: McGraw-hill, 2001.

[5] D.D. Wentzloff, A. Alghaihab, and J. Im, “Ultra-Low Power Receivers for IoT Applications: A Review”, IEEE, 2020