3GPP TSG RAN WG1 #110bis-e Meeting	R1-2210062

e-Meeting, October 10th – 19th, 2022

Agenda item:		9.1.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Two TAs for UL multi-DCI multi-TRP operation

Document for:		Discussion and Decision

# Introduction

Rel-18 MIMO scope was finalized in RAN#94 where the corresponding objectives are captured in RP-213598. One of these objectives is to study and, if justified, specify the use of two TAs (timing advance) considering UL multi-DCI for multi-TRP operation. This objective is copied below:

7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

RAN1#109-e was the first meeting where the above Rel-18 objective was discussed. During this meeting, the general support of the feature of two TAs for UL multi-DCI multi-TRP was agreed considering both intra-cell and inter-cell multi-TRP scenarios, as can be seen in the agreements copied below:

Agreement Enhancement on two TAs for UL multi-DCI for multi-TRP operation is supported in Rel-18. Note 1: whether (1) the network signals two TACs or (2) the network signals one TAC and the UE deriving the second TA can be further studied. Note 2: evaluations can be considered on as-needed basis.

Agreement Support two TA enhancement for both intra-cell and inter-cell multi-DCI multi-TRP scenarios in Rel-18.

Agreement Enhancements on two TAs for UL multi-DCI for multi-TRP operation are applicable to both FR1 and FR2.

In this contribution, we continue the discussion on two TAs for UL multi-DCI for multi-TRP operation considering the discussions and agreements made in RAN1#109-e and RAN1#110, for which the related FL summaries can be found in R1-2205209 and R1-2207800 & R1-2208016, respectively.

# Discussion

## Considerations on DL reference timing(s)

### One or two reference timings

In RAN1#109-e, whether to adopt one or two (DL) reference timings was discussed, where the agreement copied below was made. This aspect was further discussed in RAN1#110 but without reaching a consensus.

Agreement For multi-DCI multi-TRP operation with two TAs, study the following alternatives: - Alt 1: two reference timings are considered - Alt 2: one reference timing is considered Note: reference timing above is the timing of the DL reception

For the support of two TAs for multi-DCI multi-TRP, Alt.1 seems a more natural extension compared to Alt.2. Specifically, with Alt.1 each TA is determined/applied relative to the DL Rx timing of the corresponding TRP, as illustrated in Figure 1. And this alternative would not be prone to the synchronization error or the propagation delay difference between the TRPs. In addition, it’s to be noted that the UE is also expected to perform autonomous uplink timing adjustment [3GPP TS 38.133, section 7.1.2]. So, UE’s autonomous uplink timing adjustment would require UE to track reference timing RS from both TRPs in the case of UL transmissions to different TRPs. It’s worth noting that Alt.1 may require some specs effort, for instance, to define the association between TA loop or TAG and DL RSs (such as TRSs), nevertheless we don’t think this would be a showstopper. Also, note that at least two TRSs, each from a TRP, would be potentially needed for channel estimation, so there is no additional TRS(s) required with Alt.1 in this case.

Figure 1: Example illustrating applying two TAs considering two respective reference timings (i.e., Alt.1).

On the other side, performance degradation is expected with Alt.2 and would depend on TRPs synchronization error and the propagation delay difference between the TRPs. The impact of these factors, and more generally a comparison between Alt.2 and Alt.1, was analyzed in R1-2203062 where it’s concluded that the use of two reference timings is better than a single reference timing. Specifically, it was observed that with two TA offsets and two reference timings, the UL receive timing error is within a tolerable range, thus resulting in no performance degradation. In addition, with Alt.2, it should be discussed and defined which reference timing to consider, i.e., the reference timing corresponding to which TRP, given that there are two TRPs. And how to update/apply TA for the loop for which the (single) reference timing is considered/maintained.

Furthermore, Alt.1 may be easier to implement considering that there is no need to exchange information between the TRPs with this alternative. Whereas with Alt.2 the second TA may be a function of the first TA (for which the (single) reference timing is considered/maintained), so some level of coordination may be needed between the TRPs.

Finally, it’s worth recalling that the support of two TAGs within a serving cell was agreed in the last meeting (i.e., RAN1#110). This could be seen as an additional reason to adopt two reference timings, each of which for a TAG.

Based on the above observations and discussions, in our view Alt.1 should be supported.

Proposal 1: For multi-DCI multi-TRP operation with two TAs, two reference timings are considered.

### Enabling reference timing per TAG or TRP

If Alt.1 is adopted, i.e., two reference timings are considered, in order to enable/define a reference timing per TRP, an association of at least one DL RS, such as TRSs or SSBs, to a respective TAG (given that the support of two TAGs within a serving cell is agreed) and thus TRP would be needed. Then, for a given TA loop / TAG, the TA would be adjusted with respect to the corresponding DL reference timing.

Proposal 2: Discuss the association of DL RSs to TAGs/TRPs if Alt.1 is adopted, i.e., if two reference timings are considered.

## Association of UL channels/signals to TAGs

In RAN1#110, the association of UL channels/signals to TAGs has been discussed where the following agreement listing various options was made:

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, downselect one of the options in RAN1#110bis-e: - Option 1: Associate TAG to TCI-state/spatial relation - Option 2: Associate TAG to CORESETPoolIndex - Option 3: Associate TAG to DL RS group. For a UL transmission, UE adopts the TAG associated with the DL RS group to which the PL RS of the UL transmission belongs. - Option 4: Associate TAG to target UL channels/RSs directly for semi-static UL channels/RSs (e.g. P CSI PUCCH, P SRS, CG PUSCH), and further discuss how to associate TAG to dynamic UL channels/RSs(e.g. via associating TAG to CORESETPoolIndex additionally, etc.)

In the following, we discuss the various options listed in the above agreement.

- Option 1 would require defining and specifying association to a TAG considering the Rel-18/Rel-17 unified TCI framework as well as the existing spatial relation framework. This could be achieved by (i) either defining a direct association of a TAG to a TCI state or spatial relation, or (ii) by associating a TAG to a set of DL RSs, i.e., SSBs/CSI-RSs, where this association is used to determine the association of TAG to TCI state or spatial relation. From another perspective, some companies pointed out that this option is more future-proof compared to other options, e.g., considering the Rel-18 mobility objectives.

- Option 2 would already work for the PUSCH/PUCCH/SRS with a corresponding PDCCH, as in this case the association to a CORESETPoolIndex can be obtained through the PDCCH scheduling the PUSCH/PUCCH/SRS. For configured UL transmissions, an association of periodic and maybe some semi-persistent UL resources to CORESETPoolIndex would be needed. On the other hand, Option 2 may result is some scheduling flexibility limitation since, in addition to the fact that CORESETPoolIndex has been so far defined for DL only, using CORESETPoolIndex for UL would not really allow scheduling from one TRP an UL transmission towards another TRP. However, this should not be a problem especially given that the focus is on the multi-DCI mode (as stated in the Rel-18 objective copied in Section 1), since this mode is typically more suitable for non-ideal backhaul between the TRPs which suggests a more independent operation of the TRPs.

- Option 3 relies on association of DL RSs to a TAG and relies on the PL RS of the UL transmission to determine the corresponding TAG. This is, in a way, similar to one interpretation of Option 1, under which a TAG is associated to a set of DL RSs and this association is used to determine the association of TAG to TCI state.

- Option 4 seems similar to Option 2, where the key difference being that, under Option 4, configured UL resources/transmissions can be directly associated to a TAG instead of a CORESETPoolIndex.

Based on the above observations, it can be noticed that all four options could work and that there is some overlap/similarity between some of these options. In our view, it would be enough to select between Option 1 and Option 2 (with or without considering some aspects from Option 4).

Proposal 3: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, select one of the following options:

- Option 1: Associate TAG to TCI-state/spatial relation.

  - This could be achieved by (i) either defining direct association of a TAG to a TCI state or spatial relation, or (ii) by associating a TAG to a set of DL RSs where this association is used to determine the association of TAG to TCI state or spatial relation.

- Option 2 (with or without aspects from Option 4): Associate TAG to CORESETPoolIndex.

  - configured UL resources/transmissions can be associated to a CORESETPoolIndex (based on Option 2) or to a TAG (based on Option 4).

## Overlapping UL transmissions

On the aspects related to handling overlapping UL transmissions due to the application of two TAs, the following agreement listing some potential handling ways was agreed in RAN1#110:

Agreement For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes: - whether to introduce scheduling restriction in overlapping part - whether to introduce dropping rules - whether specification impact is needed, or if the issue can be handled via implementation - whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18)

First, it’s worth recalling that in legacy operation (with one TA in a cell), when two slots overlap due to TA adjustment, the later slot is reduced in duration relative to the earlier slot.

Considering the multi-DCI M-TRP operation with two TAs, handling overlapping part between UL transmissions associated with two TAs could consist in simply dropping one of the UL transmissions, where the decision on which UL transmission to drop could be based on some simple rules (e.g., always drop the later or earlier UL transmission) or could be even left up to UE implementation (which is less preferable). An alternative handling approach could be to introduce some scheduling constraints in the time domain to avoid the overlap. One might argue that the scheduling constraints may not be feasible given that the focus is on the multi-DCI mode and thus the two TRPs may not be closely coordinating, however still the overlap could be avoided assuming that there would be some level of coordination. We don’t have a strong preference regarding which handling approach to adopt between the two above approaches, we thus propose:

Proposal 4: For handling overlapping part between two UL transmissions associated with two TAs, if the UE doesn’t support simultaneous UL transmissions, support any of the following approaches:

- Drop one UL transmission, where the decision of which UL transmission to drop is based on some simple rule(s).

- Introduce some scheduling constraints in the time domain to avoid the overlaps, where this is guaranteed by the network.

We now consider the case where the UE supports simultaneous UL transmissions, assuming these UL transmissions could be partially overlapping in time. In principle, for this case, the overlap discussed above should not be problematic as anyhow the UE can transmit the overlapping UL transmissions – even if based on some UE capability. Hence, no special considerations seem required in this case.

Observation 1: In case of two TAs and if the UE supports simultaneous UL transmissions, which could be partially overlapping in time, no special handling seems required for the scenarios where two UL transmissions overlap due to the application of two TAs.

## Enhancements on random-access procedures

In RAN1#109-e and RAN1#110, there had been some discussions on whether enhancements are needed for the contention-free random access (CFRA) and the contention-based random access (CBRA) procedures, but without reaching any consensus. The related RAN1#110 agreement is copied below.

Agreement For multi-DCI based multi-TRP operation with two TAs, study the impact of two TAs for the following: - RACH triggered by PDCCH order in intra-cell MTRP case - RACH triggered by PDCCH order in inter-cell MTRP case - Which might require RACH enhancement as well - UE triggered RACH by CBRA or CFRA in RRC connected mode Further details of enhancements needed (if any)

Recall that based on legacy operation, when the gNB detects a problem in the UL synchronization of a UE, the gNB can indicate the UE to initiate a random-access procedure through a PDCCH order. The PDCCH order is triggered by DCI format 1_0 (with CRC scrambled by C-RNTI). This PDCCH can trigger either CFRA procedure or CBRA procedure. Further, in legacy, RRC configures the UE with a timer (timeAlignmentTimer) per TAG. The UE restarts the timer when it receives a new TA value. If the timer expires, the UE will need to perform a random-access procedure before it’s possible for the UE to perform UL transmissions in the cells, where at least CBRA could be used.

Next, we discuss what potential enhancements related to the above random-access procedures would be needed, considering the multi-TRP scenarios and under the assumption that PRACH towards each TRP would be needed in order for each TRP to determine its corresponding TA (based on the triggers discussed above).

PDCCH order:

For CFRA, the PDCCH order can already indicate the preamble index, SSB index, and RACH occasion (RO) using which the UE transmits PRACH. Then, for both intra-cell and inter-cell scenarios, the PDCCH order can be used to trigger PRACH towards a TRP. However, some aspects need to be further discussed. Specifically, for inter-cell scenarios, for triggering PRACH for the TRP in a non-serving cell with a given PCI, the UE would potentially need to be provided some CFRA configuration corresponding to this PCI and (implicit/explicit) indication indicating the PCI, and thus TRP, for which the PRACH is being triggered.

For CBRA, PDCCH order should allow triggering CBRA per TRP, where the contention-based aspect is kept but per TRP. For inter-cell cases, this would suggest the need to provide CBRA configuration per each (additional) PCI and a way to indicate the PCI through the PDCCH order. For intra-cell cases, this would require splitting RACH resources into two groups each corresponding to a TRP/CORESETPoolIndex. It’s worth noting that some companies mentioned that CFRA would be enough and there is no need to also support CBRA. However, recall that, based on legacy procedures, a PDCCH order could trigger any of CBRA and CFRA. Also, note that contention-based designs are in general suitable for scenarios with a large number of UEs in the cell. Anyhow, we are open to further discuss this aspect and, if needed, to also send an LS to RAN2 asking their view on the matter (e.g., if RAN2 would foresee significant RAN2-related changes in order to support CBRA here).

In addition, it should be discussed whether PDCCH order operation needs to be enhanced so that a PDCCH order from a first TRP can trigger PRACHs towards both TRPs at a time or towards any TRP. In our view, such enhancements are useful and needed as they would decrease the DL overhead in some cases or at least provide good flexibility (but might also require some level of coordination between the TRPs).

Proposal 5: To enable the determination of two TAs for inter-cell multi-TRP scenarios, for CFRA procedures: support providing the UE with CFRA configuration corresponding to a non-serving cell PCI.

Proposal 6: To enable the determination of two TAs for intra-cell and inter-cell multi-TRP scenarios, for CBRA procedures: support at least triggering PRACH per TRP.

- FFS provide the UE with CBRA configuration per each PCI and define a way to indicate the PCI through the PDCCH order for inter-cell cases.

- FFS splitting RACH resources into two groups each corresponding to a TRP/CORESETPoolIndex and define a way to indicate corresponding TRP/CORESETPoolIndex for intra-cell cases.

Proposal 7: To enable the determination of two TAs for multi-TRP scenarios, for CBRA and CFRA procedures, RAN1 should discuss for both intra-cell and inter-cell cases:

- if the PDCCH order operation should be enhanced so that a PDCCH order from a first TRP can trigger PRACHs towards the two TRPs at a time or towards any TRP, or

- if the PDCCH order operation should be made TRP specific, i.e., each TRP can send a corresponding PDCCH order triggering PRACH transmission towards that TRP.

UE-triggered RACH (due to time-alignment timer expiry):

For CBRA, similar enhancement as for the PDCCH order triggering CBRA would be required. Specifically, for inter-cell cases, this would suggest the need to provide CBRA configuration per each (additional) PCI and a way to indicate the PCI through the PDCCH order. For intra-cell cases, this would require splitting RACH resources into two groups each corresponding to a TRP/CORESETPoolIndex. Hence, when the time alignment timer for a TRP expires, the UE would transmit PRACH corresponding to this TRP.

Proposal 8: To enable the determination of two TAs for multi-TRP scenarios, for CBRA procedures: support UE-based PRACH triggering per TRP when the corresponding time alignment timer expires.

- FFS provide CBRA configuration per PCI for inter-cell cases.

- FFS splitting RACH resources into two groups each corresponding to a TRP/CORESETPoolIndex for intra-cell cases.

case of PCI activation and TCI state/ spatial relation switch update:

## Considerations on new PCI activation and TCI state/ spatial relation update

In this section, we will discuss potential considerations when a new PCI is activated (for inter-cell cases) and when at least one TCI state or spatial relation is updated.

### When a new additional PCI is activated via TCI state or spatial relation activation/indication

Recall that Rel-17 agreed that for inter-cell multi-TRP, one additional PCI, other than the serving cell’s PCI, is activated when one or more activated TCI states for PDSCH/PDCCH (which is associated with a CORESETPoolIndex) are associated with this additional PCI. Similar consideration, which may also be generalized to cover UL/joint TCI states under the unified TCI framework, on when an additional PCI is considered as activated could be used in Rel-18.

Assuming that two TAs are maintained, when a new additional PCI is activated, e.g., through the activation of at least one TCI state associated with that PCI, it should be discussed how the second TA will be updated; here we assume that the first TA corresponds to the TRP in the serving cell. One might argue that PDCCH order could be always used to trigger PRACH towards the TRP corresponding to the new PCI. However, this would increase the DL overhead and there is always a risk that the UE doesn’t correctly receive the PDCCH order. Thus, enabling the UE to autonomously decide to trigger PRACH could be considered in this case. Besides, if the propagation delay difference between the TRP corresponding to previously activated additional PCI (if any) and the newly activated additional PCI is relatively small, then PRACH may not be even needed in this case since the UE could autonomously adjust the TA. The UE could simply start or be allowed to transmitting other UL transmissions considering the latest update of the second TA.

Observation 2: When an additional PCI is activated, PRACH transmission towards the TRP corresponding to this PCI may not be always needed. Specifically, sending PRACH may not be always needed e.g., depending on whether the UE determines that autonomous adjustment of the TA is enough to correct the misalignment that has resulted due to the switch to the new PCI. Besides, when PRACH is needed, always using a PDCCH order would increase the DL overhead and there is anyway a risk that the UE doesn’t correctly receive the PDCCH order.

Proposal 9: Discuss considerations for when an additional PCI is activated, including the UE deciding whether a PRACH transmission is needed.

### Considerations in case of indicated TCI state update

With Option 1 (listed in the agreement in Section 2.2), a TCI state is associated to a TAG. When a new TCI state is indicated per a given TRP/CORESETPoolIndex, and even if associated with the same TAG, the propagation delay difference between the previously indicated TCI state and the newly indicated TCI state may range from relatively small to relatively large. And this could be determined/estimated by the UE. When, the propagation delay difference is relatively large, the UE would need to transmit PRACH. However, transmitting PRACH would clearly incur delay. Another possibility would be to exploit the occasions with longer CP duration, which occur (somewhat more frequently) each 0.5 ms. Such occasions could be used to transmit some e.g., configured UL channel/signal such an SRS, which is then used by the network to determine/calculate the corresponding TA or TA adjustment. Clearly, this doesn’t fully replace PRACH which would still be needed at least as a fallback option.

Observation 3: When a new TCI state is indicated for a given TAG, the UE could determine whether or not a relatively large TA adjustment is required for the TA loop of that TAG.

Observation 4: Transmission occasions with longer CP duration, which occur each 0.5 ms, are more frequent than RACH occasions. Transmissions with longer CP allows the network to calculate/determine TA (or TA adjustment) after a TCI state update e.g., even when the TA misalignment is relatively large.

Proposal 10: In case of update/switch of (indicated) TCI state, discuss efficient ways, based at least partially on UE decision/evaluation, to allow the network determining the TA (or TA adjustment) corresponding to this TCI state.

- FFS whether to use PRACH and/or UL transmission(s) in longer CP occasion(s), etc.

Further, in case of update/switch of TCI state, such as a new indicated TCI state or new activated TCI state which is associated with a new additional PCI (for inter-cell cases), the UE behavior regarding corresponding UL transmissions should be clearly defined during the transition period before this TCI state becomes applicable as well as immediately after it becomes applicable. For instance, after the TCI state becomes applicable, if the UE hasn’t yet received (relative or absolute) TA update or determined a TA adjustment, it should be clarified which TA loop, or TA corresponding to which TAG, the UE should use (if any), for corresponding UL transmissions – or even if the UE should not transmit these UL transmissions at least in some cases.

Proposal 11: Discuss whether there is a need to clarify the impact of TCI state update/switch on some UL transmissions at least before a corresponding TA update is obtained.

## TA loop activation, assistance information, and indication of one or two TA commands

### TA loop activation/deactivation and assistance information

First, for multi-TRP scenarios, our assumption is that it would be up to the network to determine whether, in addition to a first TA loop, a second TA loop needs to be activated. This may, for instance, depend on the UE location with respect to the two TRPs, i.e., whether the propagation delay difference (corresponding to the links between the UE and TRPs) is big or not. However, the network may not be aware of how big the propagation delay difference is at a given time, whereas the UE can be aware of such a difference based on DL RS (such as TRSs) measurements. Thus, it should be discussed whether some (simple) assistance information from the UE would be beneficial so that the network is made aware of whether the propagation delay difference is sufficiently big/small or not. The network could then use such information in order to decide whether to activate/deactivate a TA loop of the two TA loops. One might argue that the network could always operate with two TA loops for a cell as long as it’s configured with multi-TRP, however this may not be efficient.

Observation 5: Assistance information from the UE on whether the propagation delay difference between DL RSs from the two TRPs is sufficiently big/small could help the network in deciding whether one or two TA loops should be activated for the UE.

Proposal 12: RAN1 to discuss the importance of assistance information, provided by the UE, to assist the network in the decision of activating/deactivating a second TA loop, if such TA loop adaption is adopted.

Regardless of whether the assistance information (discussed above) is adopted or not, the activation by the network of a second TA loop, in addition to a first active TA loop, should be discussed. Specifically, it should be discussed how the UE is activated with a second TA loop, or whether both TA loops would be active from the beginning, e.g., after RRC (re)configuration indicating two (activated) TA loops, and stay active until some RRC reconfiguration, e.g., indicating a single (activated) TA loop.

Proposal 13: RAN1 to discuss the need and enablers for indicating the activation/deactivation of a second TA loop, when a first TA loop is active.

### Indication of one or two TA commands

Considering the Rel-18 support of two TAs for multi-TRP, the network may not necessarily always indicate the UE with two TACs (timing advance commands). In legacy, the UE receives the initial (absolute) timing advance value in a RAR (random access response) message. Timing advance value can then be updated with a MAC CE giving a TAC with a relative TA value.

When the network decides to indicate two TACs for the UE, similar ways of carrying one TAC in legacy can be essentially used, i.e., through RAR and MAC CE. For MAC-CE updates of the relative TA, the legacy MAC-CE could be for instance associated with CORESETPoolIndex#0 and a new MAC-CE, following a similar design as the legacy, could be introduced and be associated with CORESETpoolIndex#1. Another approach would be to provide one or two TACs using the same MAC CE, where an indication could be used to inform the UE whether the MAC CE contains one or two TACs.

Proposal 14: Discuss ways for indicating one or two TA commands (at a time) for multi-TRP scenarios.

# Conclusion

In this contribution, we discuss the use of two TAs considering UL multi-DCI for multi-TRP operation. The following observations and proposals are made:

Proposal 1: For multi-DCI multi-TRP operation with two TAs, two reference timings are considered.

Proposal 2: Discuss the association of DL RSs to TAGs/TRPs if Alt.1 is adopted, i.e., if two reference timings are considered.

Proposal 3: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, select one of the following options:

- Option 1: Associate TAG to TCI-state/spatial relation.

  - This could be achieved by (i) either defining direct association of a TAG to a TCI state or spatial relation, or (ii) by associating a TAG to a set of DL RSs where this association is used to determine the association of TAG to TCI state or spatial relation.

- Option 2 (with or without aspects from Option 4): Associate TAG to CORESETPoolIndex.

  - configured UL resources/transmissions can be associated to a CORESETPoolIndex (based on Option 2) or to a TAG (based on Option 4).

Proposal 4: For handling overlapping part between two UL transmissions associated with two TAs, if the UE doesn’t support simultaneous UL transmissions, support any of the following approaches:

- Drop one UL transmission, where the decision of which UL transmission to drop is based on some simple rule(s).

- Introduce some scheduling constraints in the time domain to avoid the overlaps, where this is guaranteed by the network.

Observation 1: In case of two TAs and if the UE supports simultaneous UL transmissions, which could be partially overlapping in time, no special handling seems required for the scenarios where two UL transmissions overlap due to the application of two TAs.

Proposal 5: To enable the determination of two TAs for inter-cell multi-TRP scenarios, for CFRA procedures: support providing the UE with CFRA configuration corresponding to a non-serving cell PCI.

Proposal 6: To enable the determination of two TAs for intra-cell and inter-cell multi-TRP scenarios, for CBRA procedures: support at least triggering PRACH per TRP.

- FFS provide the UE with CBRA configuration per each PCI and define a way to indicate the PCI through the PDCCH order for inter-cell cases.

- FFS splitting RACH resources into two groups each corresponding to a TRP/CORESETPoolIndex and define a way to indicate corresponding TRP/CORESETPoolIndex for intra-cell cases.

Proposal 7: To enable the determination of two TAs for multi-TRP scenarios, for CBRA and CFRA procedures, RAN1 should discuss for both intra-cell and inter-cell cases:

- if the PDCCH order operation should be enhanced so that a PDCCH order from a first TRP can trigger PRACHs towards the two TRPs at a time or towards any TRP, or

- if the PDCCH order operation should be made TRP specific, i.e., each TRP can send a corresponding PDCCH order triggering PRACH transmission towards that TRP.

Proposal 8: To enable the determination of two TAs for multi-TRP scenarios, for CBRA procedures: support UE-based PRACH triggering per TRP when the corresponding time alignment timer expires.

- FFS provide CBRA configuration per PCI for inter-cell cases.

- FFS splitting RACH resources into two groups each corresponding to a TRP/CORESETPoolIndex for intra-cell cases.

Observation 2: When an additional PCI is activated, PRACH transmission towards the TRP corresponding to this PCI may not be always needed. Specifically, sending PRACH may not be always needed e.g., depending on whether the UE determines that autonomous adjustment of the TA is enough to correct the misalignment that has resulted due to the switch to the new PCI. Besides, when PRACH is needed, always using a PDCCH order would increase the DL overhead and there is anyway a risk that the UE doesn’t correctly receive the PDCCH order.

Proposal 9: Discuss considerations for when an additional PCI is activated, including the UE deciding whether a PRACH transmission is needed.

Observation 3: When a new TCI state is indicated for a given TAG, the UE could determine whether or not a relatively large TA adjustment is required for the TA loop of that TAG.

Observation 4: Transmission occasions with longer CP duration, which occur each 0.5 ms, are more frequent than RACH occasions. Transmissions with longer CP allows the network to calculate/determine TA (or TA adjustment) after a TCI state update e.g., even when the TA misalignment is relatively large.

Proposal 10: In case of update/switch of (indicated) TCI state, discuss efficient ways, based at least partially on UE decision/evaluation, to allow the network determining the TA (or TA adjustment) corresponding to this TCI state.

- FFS whether to use PRACH and/or UL transmission(s) in longer CP occasion(s), etc.

Proposal 11: Discuss whether there is a need to clarify the impact of TCI state update/switch on some UL transmissions at least before a corresponding TA update is obtained.

Observation 5: Assistance information from the UE on whether the propagation delay difference between DL RSs from the two TRPs is sufficiently big/small could help the network in deciding whether one or two TA loops should be activated for the UE.

Proposal 12: RAN1 to discuss the importance of assistance information, provided by the UE, to assist the network in the decision of activating/deactivating a second TA loop, if such TA loop adaption is adopted.

Proposal 13: RAN1 to discuss the need and enablers for indicating the activation/deactivation of a second TA loop, when a first TA loop is active.

Proposal 14: Discuss ways for indicating one or two TA commands (at a time) for multi-TRP scenarios.