# 3GPP TSG RAN WG1 #110bis-e                                                                       R1-2210094

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CSI-RS validation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, CSI-RS validation under FR2-2 only concerns the case that the UE is under LBT mode and CSI-RS verification is not performed for UE under No-LBT mode. However, CSI-RS could be part of discivery bust which could be exempted from LBT as short control signalling according to TS 37.213. When CSI-RS is exempted from LBT, gNB could transmit the CSI-RS without channel occupancy and perform CSI-RS validation for this situation would lead to errorneuos dropping of CSI-RS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Exclude the case of CSI-RS where short control signaling exemption is applicable from CSI-RS validation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errorneuous dropping of CSI-RS when CSI-RS is eligible for short control signaling exemption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis - If it is not implemented in UE while implemented in Network, there is interoperatability issue since UE and Network may not have a same understanding whther the CSI-RS is dropped when short control signaling exemption is applicable. - If it is not implemented in network while implemented in UE, there is interoperatability issue since UE and Network may not have a same understanding whther the CSI-RS is dropped when short control signaling exemption is applicable. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

11.1.1	UE procedure for determining slot format

<< unchanged part omitted>>

For a UE operation with shared spectrum channel access in FR1, or in FR2-2 when the UE is provided ChannelAccessMode2-r17 = 'enabled', if a UE is configured by higher layers to receive a CSI-RS and the UE is provided CO-DurationsPerCell, for a set of symbols of a slot that are indicated as downlink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided, the UE cancels the CSI-RS reception in the set of symbols of the slot that are not within the remaining channel occupancy duration unless the CSI-RS is exempted from sensing, as described in [15, TS 37.213].

<< unchanged part omitted>>