# 3GPP TSG RAN WG1 #110bis-e                                                                       R1-2210096

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on configuration of PDSCH aggregation factor for MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to RRC spec, pdsch-AggregationFactor (if present) for DCI format 4_1 or 4_2 should be included in MBS-RNTI-SpecificConfig of g-RNTI-ConfigToAddModList rather than in PDSCH-ConfigMulticast as shown below from the latest TS 38.331: “– CFR-ConfigMulticast The IE CFR-ConfigMulticast indicates UE specific common frequency resource configuration for multicast for one dedicated BWP. CFR-ConfigMulticast information element -- ASN1START -- TAG-CFR-CONFIGMULTICAST-START CFR-ConfigMulticast-r17::= SEQUENCE { […] pdsch-ConfigMulticast-r17 PDSCH-Config OPTIONAL, -- Need M ” “– PDSCH-Config The PDSCH-Config IE is used to configure the UE specific PDSCH parameters. If this IE is used for MBS CFR, the following fields shall be absent: […], pdsch-AggregationFactor” “– MAC-CellGroupConfig The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX. MAC-CellGroupConfig information element -- ASN1START -- TAG-MAC-CELLGROUPCONFIG-START MAC-CellGroupConfig ::= SEQUENCE { […] g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 OPTIONAL, -- Need N […] MBS-RNTI-SpecificConfig-r17 ::= SEQUENCE { […] pdsch-AggregationFactor-r17 ENUMERATED {n2, n4, n8} OPTIONAL -- Cond G-RNTI } ” - The second hyphen in “PDSCH-Config-Multicast”, “PDSCH-Config-Broadcast”, “PDSCH-Config-MCCH”, and “PDSCH-Config-MTCH” in many places should be removed. (Also in many other sections of TS 38.214 as well as in 38.212,213) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change configuration providing PDSCH aggregation factor for multicast from “PDSCH-ConfigMulticast” to “g-RNTI-ConfigToAddModList” and remove “Multicast” from” pdsch-AggregationFactorMulticast ”. 2. Align the parameter name for “pdsch-AggregationFactor-r17”, “PDSCH-ConfigMulticast”, “PDSCH-ConfigBroadcast”, ““PDSCH-ConfigMCCH”, ““PDSCH-ConfigMTCH,“PDSCH-ConfigPTM”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incosistent and unclear specification regarding PDSCH aggregation for multicast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 5,1.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.2.1	Resource allocation in time domain

<<unchanged part omitted>>

When receiving PDSCH scheduled by DCI format 4_1, or 4_2 in PDCCH with CRC scrambled by G-RNTI, if the UE is configured with pdsch-AggregationFactor-r17 in the g-RNTI-ConfigToAddModList associated with the corresponding G-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor-r17 consecutive slots. When receiving PDSCH scheduled by DCI format 4_1 or 4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI, or PDSCH without corresponding PDCCH transmission using associated SPS-Config and activated by the DCI format 4_1 or 4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor, in associated SPS-Config if configured, or 1 otherwise, consecutive slots. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor-r17 -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using SPS-Config and activated by DCI format 4_1 or 4_2. When receiving PDSCH scheduled by DCI format 4_0 in PDCCH with CRC scrambled by G-RNTI for MTCH, if the UE is configured with pdsch-AggregationFactor in the PDSCH-ConfigPTM, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots, and the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2.

When receiving PDSCH scheduled by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception or G-RNTI, if the DCI field 'Time domain resource assignment' indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the PDSCH-ConfigMulticast or PDSCH-ConfigBroadcast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. When receiving PDSCH scheduled without corresponding PDCCH transmission using associated SPS-Config and activated by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception, if the DCI field 'Time domain resource assignment' of the activating DCI indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the PDSCH-ConfigMulticast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots.

<<unchanged part omitted>>

##### 5.1.2.1.1	Determination of the resource allocation table to be used for PDSCH

Table 5.1.2.1.1-1 and Table 5.1.2.1.1-1A define which PDSCH time domain resource allocation configuration to apply. Either a default PDSCH time domain allocation A, B or C according to tables 5.1.2.1.1-2, 5.1.2.1.1-3, 5.1.2.1.1-4 and 5.1.2.1.1-5 is applied, or the higher layer configured pdsch-TimeDomainAllocationList or [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] or pdsch-TimeDomainAllocationListDCI-1-2 is applied. For operation with shared spectrum channel access in frequency range 1, as described in [16, TS 37.213], UE reinterprets S and L in row 9 of Table 5.1.2.1.1-2 as S=6 and L=7.

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 4_0, 4_1 and 4_2

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationList | PDSCH-ConfigMCCH/PDSCH-ConfigMTCH includes pdsch-TimeDomainAllocationList Or pdsch-ConfigMulticast includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationListForMultiPDSCH-r17 | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | - | - | Default A for normal CP |
|  |  | 2 | - | - | - | - | Default B |
|  |  | 3 | - | - | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| RA-RNTI, MSGB-RNTI, TC-RNTI | Type1 common | 1,2,3 | No | - | - | - | Default A |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| MCCH-RNTI | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMCCH |
| G-RNTI for broadcast | Type 0/0B common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigMTCH, if configured, otherwise TimeDomainAllocationList provided in PDSCH-ConfigMCCH |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | - | - | Default A |
|  |  | 1,2,3 | Yes | No | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
|  |  | 1,2,3 | No/Yes | - | - | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config (Note 2) |
| G-RNTI, G-CS-RNTI (for multicast) | Type-X common search space for multiast | 1,2,3 | No | - | No | - | Default A |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1) |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMulticast (Note 1) |
| Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs and G-CS-RNTIs (configured for multicast) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 only. |  |  |  |  |  |  |  |