3GPP TSG RAN WG1 #110bis-e	R1-2210100

e-meeting, 10 – 19 October, 2022

Agenda item:		8.16.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues for Rel-17 UE features

Document for:		Discussion and Decision

# Introduction

In RAN1#110 the RAN1 UE features have been further developed [1]. In this contribution we present our views on the remaining issues not covered by other agenda items.

# Discussion

In RAN1#110 the following agreements have been made [2]:

Agreement

Confirm the following working assumption:

Working Assumption

A new UE capability is defined as below,

- For each “source-target” pair (as indicated by srs-SwitchingTimesListNR), the UE can indicate which other bands in the band combination are affected by the SRS switch. If this new indication is missing, the UE defaults to Rel-15 behavior.

- If the UE indicates the new list of bands, the dropping rules / timelines apply to the bands indicated by the list (requires update in RAN1 specs).

Note: the new UE capability has no impact on the legacy capability txSwitchImpactToRx and txSwitchWithAnotherBand

Agreement

The CR R1-2207960 is endorsed in principle by deleting “and the SRS is not dropped according to the priority rules defined in this subclause.”

Hence, FGs 39-3-1 and 39-3-2 can be confirmed and the indication they are working assumptions should be removed.

Proposal: Remove the text “this is a working assumption” from FGs 39-3-1 and 39-3-2.

# Conclusion

In this contribution we have presented our views on UE feature issues not covered by other agenda items, with the following proposal:

Proposal: Remove the text “this is a working assumption” from FGs 39-3-1 and 39-3-2.

# References

- R1-2207923, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2022

- “RAN1 Chair’s Notes”, RAN1#110, Aug. 2022