3GPP TSG RAN WG1 Meeting #110bis-e	R1-2210104

e-meeting, October 10th – 19th, 2022

Source:             CEWiT

Title:	             Discussion on Unified TCI framework extension for multi-TRP

Agenda Item:	 9.1.1.1

Document for: Discussion

# Introduction

In RAN1#110 [1], potential issues have been identified to support extension of unified TCI framework for supporting multi-TRPs. Several agreements were made in last RAN1# 110 [1] meeting related to required enhancements for extending the specifications for unified TCI framework supporting M-TRP. In this contribution, we provide our views on the Rel-18 extension for indication of multiple DL/UL TCI states, potential issues related to indicating TCI states during dynamic switching between STRP and MTRP operation modes. In this document, two TRPs are considered for M-TRP operation.

In Rel-18 unified TCI framework extension to multi-TRP transmission schemes, the following agreements were made in RAN1#110 meeting [1].

Agreement:

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings:

- Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s)

  - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group

  - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions

- Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

Switching between multi-TRP and single TRP operation is not precluded

Agreement

On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation

- FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP

- Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1

- Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17

Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based

- Discussion

  - Unified TCI framework for multi-TRP

In Rel-15, beam management was introduced to support independent analog beam selection especially in high frequency range. In Rel-16/17, TCI framework used for beam indication has been further enhanced to achieve low overhead and low latency for single TRP use cases. In Rel-17 NR, unified TCI framework was introduced for further reduction in overhead and latency for high intra-/inter-cell mobility and large number of configured TCI states. In unified TCI framework, BS supports L1-based beam indication (TCI update) to enable common beam for DL data and control and common beam for UL data and control beam operation. There are two types of unified TCI framework defined based on the condition of existence of beam correspondence between between DL and UL. They are separate TCI state indication, used for DL reception and UL transmission if beam correspondence between DL and UL is not assumed, and joint (common) TCI state indication used for DL reception and UL transmission when beam correspondence between DL and UL is assumed.

But in multi TRP scenario, latency and overhead is still an issue in case of beam indication for UL and DL when large number of TCI states are configured. Hence, in Rel.18 it has been agreed to extend the unified TCI framework for M-TRP scenario as well. All the M-TRP scenarios including SDM, FDM, TDM based PDSCH, TDM based PUSCH/PUCCH, SFN based PDCCH/PDSCH, and PDCCH repetition can be considered for Rel. 18 unified TCI framework extension for M-TRP.

    - Potential enhancement to L2- based signaling

In Rel-16 NR, if a UE is configured for M-TRP operation, BS configures the UE with up to 128 TCI states using RRC signaling. Then UE is signaled with a MAC-CE that consists of up to eight code points where each code point maps to one or two TCI states out of the configured TCI states. BS signals one out of the eight code points (from MAC CE) using a DCI to the UE for activating the corresponding TCI states. In Rel-17 NR, with the introduction of unified TCI, UE can be configured with up to 128 unified TCI states for joint/DL TCI state indication and 64 TCI states for UL TCI state indication. Further, the BS signals a MAC CE to the UE that consists of up to eight code points where each code point maps to either one joint TCI state in case of a UE configured with joint unified TCI operation or one DL TCI state and one UL TCI state in case of a UE is configured with separate unified TCI operation.

When a M-TRP architecture is combined with unified TCI framework, BS needs to signal a TCI state common for data/channel/RS for each TRP in case of both separate and joint TCI indication to the UE. In order to signal the Common TCI state to a UE using DCI for each TRP, for both separate TCI and joint TCI operation, codepoints should map to multiple DL/UL TCI states in MAC CE structure. Therefore, the MAC CE structure needs to be further studied and modified for indicating multiple DL/UL TCI states to support unified TCI framework for M-TRP transmission.

For example, as shown in Fig.1 if joint TCI indication is configured through RRC, one codepoint in MAC CE maps to up to two TCI states corresponding to two TRPs respectively and if separate TCI indication is configured through RRC, one codepoint in MAC CE maps to up to four TCI states corresponding to two TRPs respectively. Therefore, for separate and joint TCI indication one codepoint in MAC CE can be associated with up to four TCI states corresponding to two TRPs respectively.

Fig1. Beam indication for joint and separate TCI operation

As shown in Fig.2, multiple TCI states are mapped to one TCI codepoint and needs an indicator to indicate whether the octet is containing joint/DL TCI states and/or UL TCI states in the configured MAC CE structure. One joint/DL unified TCI state configured for a UE out of the 128 Joint/DL TCI states  and one UL unified TCI state configured for a UE out of the 64 UL TCI states configured for the UE. To support unified TCI framework extension for M-TRP, existing number of codepoints in MAC CE are sufficient to indicate multiple DL/UL TCI states therefore the bit field in DCI to indicates TCI code point can be reused as 3 bits.

Fig.2 Example of enhanced MAC CE for TCI states activation/deactivation for cases

Of M>1 and /or N>1

Proposal 1: Support enhanced MAC CE structure, where one codepoint in MAC CE structure can be associated to up to four states and further study is required to select type of TCI states from different TCI state pool.

    - TCI configuration enhancement of PDCCH/PDSCH

In Rel-15/16 NR, each CORESET is configured with up to 64 possible TCI states and one out of these 64 TCI states is activated using a MAC-CE. When a UE monitors a search space set associated with a CORESET, the UE uses the activated TCI state for that corresponding CORESET. In Rel-17 NR, a UE is configured with a set of 128 unified TCI states and one of these TCI states is activated using a DCI for a UE. UE uses the activated unified TCI state to decode following CORESETs.

If a UE is configured to be operating in unified TCI and is connected to M-TRP, BS activates multiple unified TCI states to, support M-TRP operation. Then UE needs to determine which one of the two TCI states to be used to receive PDCCH. So, there is a need to investigate this impact on PDCCH monitoring when COEREST is associated with multiple TCI states.

In Rel 16, CORESETPoolIndex variable was introduced in M-TRP transmission to support the various M-TRP schemes. A CORESET pool index is a variable that will be present inside the ControlResourceSet IE. This variable can take a value of either 0 or 1. If this variable is not configured for a CORESET, UE assumes default value that is 0 for that CORESET.

For unified TCI framework extension for M-DCI based M-TRP transmission, some fixed rules should be defined for TCI state update. The rules should be defined for an association between the TCI state indicated in the DCI and the CORESETPoolIndex. As described in the Fig. 3, a UE is activated with a set of TCI states in case of M-DCI based M-TRP transmission scenario.

If a UE is configured with two CORESET pool indices i.e., index ‘0 (or 1)’ and ‘1 (or 0)’ in RRC configuration, then each of the activated unified TCI states maps to one CORESET pool index. UE determines the TCI state of a CORESET based on its associated pool index. To determine the TCI state of CORESET, need to define fixed/mapping rules between the indicted joint/DL/UL TCI states and a CORESETPoolIndex.

CORESETPoolIndex value associated with TCI states can be used to determine which TRP the TCI state should be applied.

Fig3. TCI state indication for M-DCI based MTRP transmission

Proposal 2: For M-DCI based M-TRP transmission, support the use of CORESETPoolIndex to determine the TCI states of a CORESET.

- There should be fixed/mapping rules between the indicted joint/DL/UL TCI states and a CORESETPoolIndex.

For unified TCI framework extension for S-DCI based M-TRP transmission, a UE is configured with only one CORESETPoolIndex and it is activated with two unified TCI states as shown in Fig. 4. To determine the TCI state of CORESET, methods can be defined like as mapping pattern and fixed association rule based on CORESETIndex.

Fig3. TCI state indication for S-DCI based M-TRP transmission

Proposal 3: For S-DCI based M-TRP transmission, further study is required to determine the active TCI state of CORESET.

- For example, any mapping pattern and fixed association rule based on CORESETIndex.

In Rel-17, M-TRP framework was extended to support PDCCH transmission for more reliability. In this PDCCH repetition was introduced where PDCCH is repeated multiple times in TDM manner and each repetition from different TRP maps to one out of two TCI states. This framework can be a baseline for further extension in supporting unified TCI framework for M-TRP.

In the previous meeting, three alternatives were proposed to determine the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN. Out of these three alternatives we feel that Alt 2 is best suited alternative. According to our view, Alt 2 requires lesser spec change compared to other alternatives.

Proposal 4:  Alt 2 should be supported for unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s).

In Rel-16 NR, there are various M-TRP schemes that are introduced by 3GPP. In the case of a M-TRP scenario, the transmission and reception to and from different TRPs can be multiplexed in any one of time, frequency, and space, leading to FDM Scheme A, FDM scheme B, TDM scheme A, SDM scheme etc. When a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. Similarly, when CORESETPoolIndex is configured as for a UE by higher layer parameter, then the UE may expect to be scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by one scheduling PDCCH.

If a DCI schedules a M-TRP PDSCH with any of FDM scheme A, FDM scheme B, TDM scheme A, then this DCI is referred as S-DCI and if a DCI schedules a Single-TRP PDSCH, then it referred as S-TRP DCI.

Based on configuration of CORESETPoolIndex, UE can receive any type of DCI i.e., M-DCI, S-DCI and S-TRP DCI. BS activates a UE with a pair of TCI states (TCI state 1, TCI state 2) using a DCI. UE activates these two TCI states after BeamApp time from the last symbol of the PUCCH corresponding to the DCI activating the TCI states.

In Rel-16, a UE differentiates whether it is scheduled to receive M-TRP PDSCH or S-TRP PDSCH based on the number of TCI states that are being signaled by the corresponding DCI. But, if a UE configured with unified TCI states, then UE can be scheduled with PDSCH without explicit indication of TCI states in the DCI. If a UE receives a DCI without any explicit indication of TCI states, then the UE may not be able to understand whether the DCI schedules M-TRP PDSCH or S-TRP PDSCH. To solve this issue, we need to study some signalling schemes to indicate a UE whether the scheduled PDSCH is configured to receive 1 TCI state (S-TRP PDSCH) or 2 TCI states (M-TRP PDSCH).

Proposal 5: Support further studies to update the TCI state for PDSCH during dynamic switching in different scenarios such as:

- UE is scheduled to receive from single TRP alone,

- UE is scheduled to receive form M-TRP but with S-DCI

- UE is scheduled to receive from M-TRP using M-DCI

    - Collision between channel and signals

A UE that is connected with multiple TRPs, can be scheduled to receive signals/channels simultaneously from both the TRPs using full or partially overlapping resources sometimes. If the UE is not capable of receiving with different spatial Rx filters simultaneously, then collision occurs. Therefore, mechanism to handle collisions between channels and signals should be investigated to support unified TCI framework for M-TRP.

Proposal 6: Need to investigate the methods to avoid collisions between different channels and signals.

# 3   Conclusion

This contribution we discussed potential enhancements to support unified TCI framework for M- TRP.

Further, we summarize the proposals as follows:

Proposal 1: Support enhanced MAC CE structure, where one codepoint in MAC CE structure can be associated to up to four states and further study is required to select type of TCI states from different TCI state pool.

Proposal 2: For M-DCI based M-TRP transmission, support the use of CORESETPoolIndex to determine the TCI states of a CORESET.

- There should be define fixed/mapping rules between the indicted joint/DL/UL TCI states and a CORESETPoolIndex.

Proposal 3: For S-DCI based M-TRP transmission, further study is required to determine the active TCI state of CORESET.

- For example, any mapping pattern and fixed association rule based on CORESETIndex.

Proposal 4:  Alt 2 should be supported for unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s).

Proposal 5: Support further studies to update the TCI state for PDSCH during dynamic switching in different scenarios such as:

- UE is scheduled to receive from single TRP alone,

- UE is scheduled to receive form M-TRP but with S-DCI

- UE is scheduled to receive from M-TRP using M-DCI

Proposal 6: Need to investigate the methods to avoid collisions between different channels and signals.

Reference

[1] 3GPP, “RAN1#110 Meeting chairman’s notes”, Toulouse, France, August 22nd- 26th, 2022,