3GPP TSG RAN WG1 #110bis-e			R1-2210140

e-Meeting, October 10th – 19th, 2022

Source:	WILUS Inc.

Title:	Discussion on PHY channel design framework for SL on unlicensed spectrum

Agenda item:	9.4.1.2

Document for:	Discussion/Decision

- Introduction

From the RAN1#109-e meeting to RAN1#110 meeting, the followings were agreed on physical channel design framework for SL-U [1]-[2].

Agreement at RAN1#109-e SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc. Agreement at RAN1#109-e For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission Agreement at RAN1#109-e For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement at RAN1#109-e For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure Agreement at RAN1#110 For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported Agreement at RAN1#110 For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool Agreement at RAN1#110 To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement at RAN1#110 If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior Agreement at RAN1#110 Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives Agreement at RAN1#110 If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded - FFS details of above alternatives Agreement at RAN1#110 For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS Agreement at RAN1#110 For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

In this contribution, we discuss a PHY channel design for NR sidelink on unlicensed spectrum and provide our views.

Discussion on channel access for NR sidelink on unlicensed spectrum

From the RAN guidance, RAN1 study phase should focus on FR1 unlicensed bands such as n46 and n96/n012 and is to be completed by RAN#98. According to the regulation on FR1 unlicensed spectrum, devices shall meet Occupied Channel Bandwidth (OCB) requirement that transmissions in a channel shall occupy 80% or more of the channel bandwidth. From the coverage perspective, it might be beneficial for the device to maintain the high output power and fulfill the regulations on maximum EIRP, and power spectral density. For these aspects, the interlaced allocation as one of resource allocation mechanisms was adopted in NR-U Rel-16 and LTE-LAA Rel-14. Considering available subchannel size(s) for each transmission of PSCCH/PSSCH which can be allocated on one or more contiguous sub-channels, PSCCH/PSSCH in sidelink unlicensed cannot satisfy the OCB requirement. Therefore, it was agreed at the previous RAN1#110 meeting that interlaced RB-based transmission for PSCCH/PSSCH on unlicensed spectrum is supported. The details how to support interlaced transmission such as RB based interlaced transmission or sub-channel based interlaced transmission was discussed at the previous RAN1#110 meeting. Similar to the method in which RB-based interlaced defined in NR-U is allocated across RB-sets regardless of RBs for intra-cell guard band, when one subchannel can occupy at least one or K interlaced in SL-U, it might be desirable to be able to span 1 or multiple RB set(s) allocated in a resource pool, rather than being confined within 1 RB set.

- Proposal 1: For interlace RB-based transmission for PSCCH and PSSCH in SL-U,

  - 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

Considering that LBT is performed per 20 MHz channel bandwidth for a fair-coexistence on shared spectrum with other existing technologies such as Wi-Fi, LAA, and NR-U, interlaced allocation should be reused for PSSCH resource allocation and RB set indication might be also reused if intra-cell guard configuration is configured for PSSCH resource allocation for wideband operation on SL-U. And also PRBs within intra-cell guard band of two adjacent RB sets are used in the same way as Rel-16 NR-U. The receiver UE cannot know whether or not channel access procedures by the transmitter UE to transmit PSCCH/PSSCH is successful. From the perspective of the UE receiving the PSCCH, it would be difficult to know in which RB set the transmitter UE transmits the PSCCH or whether the transmission is performed in one or more RB sets after the transmitter UE performs channel access successfully. In order to reduce blind detection for determining the number of RB sets in which the PSCCH is transmitted from the standpoint of the UE receiving the PSCCH, a method of limiting the transmission of the PSCCH to at least one RB set except intra-cell guard band PRBs will need to be further considered.

- Proposal 2: For interlace RB-based transmission for PSCCH and PSSCH in SL-U,

  - The RB set indication might be used for wideband operation on SL-U if intra-cell guard configuration is configured for PSSCH resource allocation.

  - PRBs within intra-cell guard band of two adjacent RB sets are used in the same way as Rel-16 NR-U.

  - PSCCH allocation should be restricted within a RB set except intra-cell guard band PRBs similar to NR-U PDCCH operation.

Different from PSCCH/PSSCH which can be allocated on one or more contiguous sub-channels, each PSFCH associated with PSSCH is occupied with 1 PRB using PUCCH format 0. To meet OCB requirement, it may be desirable to allocate the PSFCH in the form of an RB-based interlaced transmission. In Rel-16 NR-U, enhanced PUCCH format 0 can be considered as a starting point for RB-based interlaced PSFCH transmission. Unlike the method in NR-U in which PUCCH resources are dynamically indicated by scheduling DCI, in NR sidelink, a set of continuous PRBs for PSFCH occasions is used for PSFCH transmission. This is to simplify the sidelink HARQ-ACK feedback procedure by allowing all sidelink UEs to have a common understanding of HARQ-ACK feedback, and to improve the reliability of SL HARQ-ACK feedback by avoiding interference from non-HARQ feedback SL transmission to HARQ feedback SL transmission. Therefore, PUCCH format 0 is considered as a starting point for RB-based interlace PSFCH transmission, but it should be further studied and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for NR sidelink unlicensed operation.

- Proposal 3: PUCCH format 0 using RB-based interlace for PSFCH transmission should be supported, but it should be further discussed and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for SL-U.

- Proposal 4: To meet OCB and PSD requirement for PSFCH transmission, each PSFCH transmission occupies an interlace similar to PUCCH in NR-U. Different PRBs within an interlace should be applied to allocate rotating CS shift for the PAPR/CM reduction.

- Proposal 5: Considering LBT failure for PSFCH transmission, it should be further investigated that PSFCH resource is dynamically indicated.

Conclusion

In this contribution, we have discussed a PHY channel design for NR sidelink on unlicensed spectrum. As a conclusion, we summarize our view as follows:

- Proposal 1: For interlace RB-based transmission for PSCCH and PSSCH in SL-U,

  - 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

- Proposal 2: For interlace RB-based transmission for PSCCH and PSSCH in SL-U,

  - The RB set indication might be used for wideband operation on SL-U if intra-cell guard configuration is configured for PSSCH resource allocation.

  - PRBs within intra-cell guard band of two adjacent RB sets are used in the same way as Rel-16 NR-U.

  - PSCCH allocation should be restricted within a RB set except intra-cell guard band PRBs similar to NR-U PDCCH operation.

- Proposal 3: PUCCH format 0 using RB-based interlace for PSFCH transmission should be supported, but it should be further discussed and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for SL-U.

- Proposal 4: To meet OCB and PSD requirement for PSFCH transmission, each PSFCH transmission occupies an interlace similar to PUCCH in NR-U. Different PRBs within an interlace should be applied to allocate rotating CS shift for the PAPR/CM reduction.

- Proposal 5: Considering LBT failure for PSFCH transmission, it should be further investigated that PSFCH resource is dynamically indicated.

References

[1] Final Report of 3GPP TSG RAN WG1 #109-e meeting v1.0.0

[2] Draft Report of 3GPP TSG RAN WG1 #110 meeting v0.2.0